Department of Veterans Affairs

Medication Order Check Healthcare Application
(MOCHA)
Enhancements (Enh) 2

PSJ*5*281

Software Design Document (SDD)

February 2014

Version 1.0

.Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 08/14/2013 | 1.0 | Format/edit |
| 05/28/13 | 1.0 | Added CCR6584,6587 information |
| 05/24/13 | 1.0 | Added CCR6454,6493,6586,6585 information |
| 05/15/13 | 1.0 | Added CCR6124,6398,6396,6466 information |
| 05/07/13 | 1.0 | Added CCR5980 information |
| 05/07/13 | 1.0 | Added CCR2987 information |
| 04/15/13 | 1.0 | Added CCR6198 information |
| 04/01/13 | 1.0 | Added CCR6312 information |
| 03/25/13 | 1.0 | Added CCR6205 information |

Table of Contents

1	Introduction	1

1.1	Purpose	1

1.2	Scope	1

1.3	Definitions and Acronyms	1

1.4	References	2

2	Conceptual Design	2

2.1	Product Perspective	2

2.1.1	User Interfaces	2

2.1.2	Hardware Interfaces	2

2.1.3	Software Interfaces	2

2.1.4	Communications Interfaces	2

2.1.5	Memory Constraints	2

2.1.6	Special Operations	2

2.2	Product Features	2

2.3	User Characteristics	2

2.4	Dependencies and Constraints	2

3	Specific Requirements	3

3.1	Database Repository	3

3.2	System Features	3

3.3	Design Element Tables	3

3.3.1	Routines (Entry Points)	3

3.3.2	Templates	25

3.3.3	Bulletins	25

3.3.4	Data Dictionaries (DDs)	25

3.3.5	Field Definition Information	25

3.3.6	Data Entries Affected by the Design	25

3.3.7	Unique Record(s) [Commonly the .01 field]	25

3.3.8	File/Global Size Changes	25

3.3.9	Mail Groups	25

3.3.10	Security Keys	26

3.3.11	Options	26

3.3.12	Protocols	26

3.3.13	Remote Procedure Call (RPC)	26

3.3.14	Constants Defined in Interface	26

3.3.15	Variables Defined in Interface	26

3.3.16	Types Defined in Interface	26

3.3.17	Graphic User Interface (GUI)	26

3.3.18	GUI Classes Defined	26

3.3.19	Current Form	26

3.3.20	Modified Form	26

3.3.21	Components on Form	26

3.3.22	Events	26

3.3.23	Methods	26

3.3.24	Special References	26

3.3.25	Class Events	27

3.3.26	Class Methods	27

3.3.27	Class Properties	27

3.3.28	Uses Clause	27

3.3.29	Form	27

3.3.30	Function	27

3.3.31	Dialog	27

3.3.32	Help Frame	27

3.3.33	Health Level Seven (HL7) Application Parameter	27

3.3.34	Health Level Seven (HL7) Logical Link	27

3.3.35	COTS Interface	27

4	Attachment A - Approval Signatures	A-1

# Introduction

This Software Design Document (SDD) will outline the changes to the software introduced by Patch PSJ*5*281 delivered with Medication Order Check Healthcare Application (MOCHA) Enhancements (Enh) 2.

## Purpose

The purpose of this SDD is to plan and document the Inpatient Medications Package updates introduced with MOCHA Enh 2. The intended audience of this document is the MOCHA Workgroup.

## Scope

The scope of this SDD is PSJ*5*281 delivered with MOCHA Enh 2. This SDD documents changes made to VistA M routines to support the requested functionality as documented in the MOCHA Enh 2 Requirments Specification Document (RSD).

## Definitions and Acronyms

| Term | Definition |
| --- | --- |
| ADPAC | Automated Data Processing Application Coordinator |
| ADR | Adverse Drug Reaction |
| BSA | Body Surface Area |
| CodeCR | Code Change Request |
| IP | Inpatient |
| MOCHA | Medication Order Check Healthcare Application |
| PBM | Pharmacy Benefits Management |
| PDM | Pharmacy Data Management |
| PMAS | Program Management Accountability System |
| POC | Proof of Concept |
| PRE | Pharmacy Reengineering |
| RSD | Requirements Specification Document |
| SAC | Standards and Conventions |
| SDD | Software Design Document |
| TSPR | Technical Services Project Repository |
| UFT | User Functional Test |
| VA | Department of Veterans Affairs |
| VistA | Veterans Health Information Systems and Technology Architecture |

## References

MOCHA Enh 2 Requirements Specification Document

# Conceptual Design

## Product Perspective

Patch PSJ*5.0*281, addresses the following Code Change Requests (CCRs) that are considered to be the second enhancements to MOCHA v 1.0: CCR2987, CCR5980, CCR6312, CCR6205, CCR6198, CCR6124, CCR6398, CCR6396, CCR6466, CCR6454, CCR6493, CCR6586, CCR6584, CCR6587.

### User Interfaces

MOCHA Enh 2 does not introduce any changes to the standard VistA Interface.

### Hardware Interfaces

MOCHA Enh 2 does not introduce any changes to the standard VistA architecture.

### Software Interfaces

MOCHA Enh 2 does not introduce any changes to the existing interfaces between VistA Pharmacy packages.

### Communications Interfaces

N/A

### Memory Constraints

N/A

### Special Operations

N/A

## Product Features

N/A

## User Characteristics

N/A

## Dependencies and Constraints

This patch will require PSJ*5*260 and PSJ*5*275.

# Specific Requirements

## Database Repository

N/A

## System Features

The following functionality will be delivered with MOCHA Enh 2. Detailed requirements and subsequent acceptance criteria can be found on the PRE SharePoint site:

## Design Element Tables

### Routines (Entry Points)

#### CCR2987 (This patch will add enhancements to Inpatient prescription edit sequence to perform order checks when the dosage field is modified.)
CCR5980 (The Order Status will be displayed with all orders in the Order Check messages.)

| Routine Name | PSJOCDI |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| SRS Traceability | CR2831, 5377 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary References | N/A |  |  |  |  |  |  |  |  |
| Related Protocols | N/A |  |  |  |  |  |  |  |  |
| Related Integration Agreements |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition |  |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic |  |  |  |  |  |  |  |  |  |
| HDR(PSJDNM) ;Display the intro text on drug interaction W !,"This patient is receiving the following order(s) that have a " W $S($G(PSJSEV)="C":"CRITICAL",$G(PSJSEV)="S":"SIGNIFICANT",1:"")_" Drug" W !,"Interaction with "_$G(PSJDNM)_":",! Q DSPDRG(PSJPON,PSJDNM,PSJCLINF) ;Display order info or drug name from prospective. Q:$G(PSJPON)="" I $P(PSJPON,";",3)="PROSPECTIVE" W !?8,$G(PSJDNM),! Q I $D(PSJDSPON(PSJPON)) Q S PSJDSPON(PSJPON)="" I ($Y+8)>IOSL D PAUSE^PSJMISC(1,0) W @IOF D DSPORD^PSJOC($P(PSJPON,";",2),"",PSJCLINF) Q |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| HDR(PSJDNM) ;Display the intro text on drug interaction NEW PSJPON,PSJCNT,PSJIND,PSJON,PSJSTAT SET PSJPON="",PSJCNT=0,PSJIND="",PSJON="",PSJSTAT="" ; ;**Display order status - CCR 5980 ; IF $G(PSJDGCK) D Q .F SET PSJPON=$O(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJPON)) Q:$G(PSJPON)="" D ..IF $G(PSJDNM)=$P(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJPON),U,4),$P(PSJPON,";",1)="P",$P(PSJPON,";",3)="PROSPECTIVE" S PSJSTAT=" (OP Pending)" Q ..IF $G(PSJDNM)=$P(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJPON),U,4),$P(PSJPON,";",1)'="O",$P(PSJPON,";",1)'="N",$P(PSJPON,";",3)="PROSPECTIVE" S PSJSTAT=" (Prospective)" Q ..IF $G(PSJDNM)=$P(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJPON),U,4),$P(PSJPON,";",1)="O" D Q ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=0 SET PSJON="Active" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=1 SET PSJON="Non-Verified" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=2 SET PSJON="Refill" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=3 SET PSJON="Hold" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=4 SET PSJON="Drug Interactions" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=5 SET PSJON="Suspended" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=10 SET PSJON="Done" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=11 SET PSJON="Expired" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=12 SET PSJON="Discontinued" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=13 SET PSJON="Deleted" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=14 SET PSJON="Discontinued by provider" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=15 SET PSJON="Discontinued (Edit)" ...IF $P(^PSRX($P(PSJPON,";",2),"STA"),U,1)=16 SET PSJON="Provider Hold" ...SET PSJSTAT=" (Local Rx #"_$P(^PSRX($P(PSJPON,";",2),0),U,1)_" ("_PSJON_"))" Q ..IF $G(PSJDNM)=$P(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJPON),U,4),$P(PSJPON,";",1)="N" D Q ...SET PSJSTAT=" (Non-VA)" Q ..IF $G(PSJDNM)=$P(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJPON),U,4) S PSJCNT=$L($P(PSJPON,";",2)),PSJIND=$E($P(PSJPON,";",2),PSJCNT),PSJON=$E($P(PSJPON,";",2),-1,(PSJCNT-1)) ..IF $G(PSJIND)="P" D Q ...IF $P(^PS(53.1,PSJON,0),U,9)="A" SET PSJSTAT=" (IP Active)" Q ...IF $P(^PS(53.1,PSJON,0),U,9)="D" SET PSJSTAT=" (IP Discontinued)" Q ...IF $P(^PS(53.1,PSJON,0),U,9)="I" SET PSJSTAT=" (IP Incomplete)" Q ...IF $P(^PS(53.1,PSJON,0),U,9)="N" SET PSJSTAT=" (IP Non-Verified)" Q ...IF $P(^PS(53.1,PSJON,0),U,9)="U" SET PSJSTAT=" (IP Unreleased)" Q ...IF $P(^PS(53.1,PSJON,0),U,9)="P" SET PSJSTAT=" (IP Pending)" Q ..IF $G(PSJIND)="U" D Q ...IF $P(^PS(55,$G(DFN),5,PSJON,0),U,9)="A" SET PSJSTAT=" (IP Active)" Q ...IF $P(^PS(55,$G(DFN),5,PSJON,0),U,9)="D" SET PSJSTAT=" (IP Discontinued)" Q ...IF $P(^PS(55,$G(DFN),5,PSJON,0),U,9)="E" SET PSJSTAT=" (IP Expired)" Q ...IF $P(^PS(55,$G(DFN),5,PSJON,0),U,9)="H" SET PSJSTAT=" (IP Hold)" Q ...IF $P(^PS(55,$G(DFN),5,PSJON,0),U,9)="R" SET PSJSTAT=" (IP Renewed)" Q ...IF $P(^PS(55,$G(DFN),5,PSJON,0),U,9)="RE" SET PSJSTAT=" (IP Reinstated)" Q ...IF $P(^PS(55,$G(DFN),5,PSJON,0),U,9)="DE" SET PSJSTAT=" (IP Discontinued (Edit))" Q ...IF $P(^PS(55,$G(DFN),5,PSJON,0),U,9)="DR" SET PSJSTAT=" (IP Discontinued (Renewal))" Q ..IF $G(PSJIND)="V" D Q ...IF $P(^PS(55,$G(DFN),"IV",PSJON,0),U,17)="A" SET PSJSTAT=" (IP Active)" Q ...IF $P(^PS(55,$G(DFN),"IV",PSJON,0),U,17)="H" SET PSJSTAT=" (IP Hold)" Q ...IF $P(^PS(55,$G(DFN),"IV",PSJON,0),U,17)="R" SET PSJSTAT=" (IP Renewed)" Q ...IF $P(^PS(55,$G(DFN),"IV",PSJON,0),U,17)="D" SET PSJSTAT=" (IP Discontinued)" Q ...IF $P(^PS(55,$G(DFN),"IV",PSJON,0),U,17)="E" SET PSJSTAT=" (IP Expired)" Q ...IF $P(^PS(55,$G(DFN),"IV",PSJON,0),U,17)="P" SET PSJSTAT=" (IP Purge)" Q ...IF $P(^PS(55,$G(DFN),"IV",PSJON,0),U,17)="O" SET PSJSTAT=" (IP On call)" Q ...IF $P(^PS(55,$G(DFN),"IV",PSJON,0),U,17)="N" SET PSJSTAT=" (IP Non-Verified)" Q .W !,"This patient is receiving the following order(s) that have a " .W $S($G(PSJSEV)="C":"CRITICAL",$G(PSJSEV)="S":"SIGNIFICANT",1:"")_" Drug" .W !,"Interaction with "_$G(PSJDNM)_$G(PSJSTAT)_":",! ; ELSE D .W !,"This patient is receiving the following order(s) that have a " .W $S($G(PSJSEV)="C":"CRITICAL",$G(PSJSEV)="S":"SIGNIFICANT",1:"")_" Drug" .W !,"Interaction with "_$G(PSJDNM)_":",! Q DSPDRG(PSJPON,PSJDNM,PSJCLINF) ;Display order info or drug name from prospective. CCR 6454 Q:$G(PSJPON)="" I $P(PSJPON,";",3)="PROSPECTIVE" W !?8,$G(PSJDNM)_" (Prospective)",! Q I $D(PSJDSPON(PSJPON)) Q S PSJDSPON(PSJPON)="" I ($Y+8)>IOSL D PAUSE^PSJMISC(1,0) W @IOF D DSPORD^PSJOC($P(PSJPON,";",2),"",PSJCLINF) Q |  |  |  |  |  |  |  |  |  |

#### CCR5980 (Add order status to order check messages – Inpatient.)
CCR6628 (2nd line of the routine needs to be updated with the correct list of patch numbers.)

| Routine Name | PSJOCDT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| SRS Traceability | CR2831, 5377 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary References | N/A |  |  |  |  |  |  |  |  |
| Related Protocols | N/A |  |  |  |  |  |  |  |  |
| Related Integration Agreements |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition |  |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic: N/A |  |  |  |  |  |  |  |  |  |
| New Routine |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| New Routine |  |  |  |  |  |  |  |  |  |

#### CCR5980, 6628

| Routine Name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| Requirement Traceability Matrix | CR2831, 5377 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary (DD) References |  |  |  |  |  |  |  |  |  |
| Related Protocols |  |  |  |  |  |  |  |  |  |
| Related Integration Control Registrations (ICRs) |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic |  |  |  |  |  |  |  |  |  |
| HDR ; NEW PSJHDR,PSJDNM I $D(^TMP($J,"PSJPRE","OUT","DRUGDRUG")) W @IOF D LINE^PSJMISC("=",81) S PSJHDR="This patient is already receiving the following INPATIENT and/or OUTPATIENT order(s) for a drug in the same therapeutic class(es)" S:$D(PSJDGCK) PSJHDR="This patient is already receiving the following INPATIENT and/or OUTPATIENT order(s) for drugs in the same therapeutic class(es)" S PSJDNM=$O(PSJPROSP("UD","")) I PSJDNM]"" S PSJHDR=PSJHDR_" as "_PSJDNM_":" D WRITE^PSJMISC(PSJHDR,1,77) Q D WRITE^PSJMISC(PSJHDR_":",1,77) |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| HDR ; NEW PSJHDR,PSJDNM,PSJSTAT SET PSJSTAT="" IF $G(PSJDGCK) SET PSJSTAT=" (Prospective)" ; I $D(^TMP($J,"PSJPRE","OUT","DRUGDRUG")) W @IOF D LINE^PSJMISC("=",81) S PSJHDR="This patient is already receiving the following INPATIENT and/or OUTPATIENT order(s) for a drug in the same therapeutic class(es)" S PSJDNM=$O(PSJPROSP("UD","")) I PSJDNM]"" S PSJHDR=PSJHDR_" as "_PSJDNM_PSJSTAT_":" D WRITE^PSJMISC(PSJHDR,1,77) Q D WRITE^PSJMISC(PSJHDR_":",1,77) |  |  |  |  |  |  |  |  |  |

#### CCR6205 (IP CK Option when a supply item is first on med profile, the CK option displays supply item message.)
CCR6312 (Add functionality to use Clinical Reminder based order checks in pharmacy order entry.)
CCR6396 (Continues with Order after selecting to not continue.)
CCR6398 (Clinical Reminder Order Check has two Press Return messages.)
CCR6586 (Clinical Reminder (IP) - Invalid Drug Stored in Intervention File.)
CCR6585 (CROC Message does not display when processing IP IV solution.)
CCR6645 (System does not accept NO Entry at prompts Intervene and Continue.)
CCR6646 (System does not accept NO Entry at prompts Intervene and Continue.)
CCR6696 (Invalid CROC messages display during processing of IP IV order.)

| Routine Name | PSJCROC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| Requirement Traceability Matrix | CR3286 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary (DD) References |  |  |  |  |  |  |  |  |  |
| Related Protocols |  |  |  |  |  |  |  |  |  |
| Related Integration Control Registrations (ICRs) | DBIA5531, DBIA10050 |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic |  |  |  |  |  |  |  |  |  |
| PSJCROC ;HP/MJE - CLINICAL REMINDER ORDER CHECKS FOR IP OC ;09/22/11 5:00pm ;;5.0;INPATIENT MEDICATIONS ;**281**;16 DEC 97;Build 4 ; ;Reference to ORDERCHK^PXRMORCH supported by DBIA 5531 ;Reference to SIG^XUSESIG supported by DBIA 10050 ; CK(CDRG1) ;CHECK FOR CROCS AGAINST PROSPECTIVE DRUG S $P(CROCLN,"=",60)="=",$P(CROCLN2,"-",60)="-" W @IOF,!,"Now processing Clinical Reminder Order Checks. Please wait ...",!!,CROCLN D ORDERCHK^PXRMORCH(DFN,-1,0,CDRG1,0) S (CROCSTA,CROCDNM,CROCNUM,CROCSTR,CROCHFLG)="" I $D(^TMP($J,CDRG1)) D .S CROCHFLG=2,CROCNR=0 .F S CROCSTA=$O(^TMP($J,CDRG1,CROCSTA)) Q:CROCSTA=""!$G(PSGORQF) D ..I CROCSTA=1 S CROCHFLG=1 ..F S CROCDNM=$O(^TMP($J,CDRG1,CROCSTA,CROCDNM)) Q:CROCDNM=""!$G(PSGORQF) D ...S CROCSTA2=$S(CROCSTA=1:"HIGH",CROCSTA=2:"MEDIUM",CROCSTA=3:"LOW",1:"UNKNOWN") ...W !,"*** Clinical Reminder Order Check ¦ Severity: "_CROCSTA2_" ***",! ...W !,CROCDNM,! ...F S CROCNUM=$O(^TMP($J,CDRG1,CROCSTA,CROCDNM,CROCNUM)) Q:CROCNUM=""!$G(PSGORQF) D ....I ($Y+7)>IOSL,$E(IOST)="C" D .....W ! K DIR S DIR(0)="E",DIR("A")="Press Return to Continue..." D ^DIR K DIR W @IOF ....W !,^TMP($J,CDRG1,CROCSTA,CROCDNM,CROCNUM) ...W !!,CROCLN2 W ! K DIR S DIR(0)="E",DIR("A")="Press Return to Continue..." D ^DIR K DIR W @IOF K ^TMP($J,CDRG1) I $G(PSJDGCK) K CROCSTA,CROCDNM,CROCNUM,CROCSTR,CROCHFLG,CROCLN,CROCLN2,CDRG1 Q I CROCHFLG=1 S DIR(0)="SA^1:YES;0:NO",DIR("A")="Do you want to Continue? ",DIR("B")="Y" D ^DIR I CROCHFLG=1,Y["^"!(Y=0)!($D(DIRUT)) S (PSGORQF)=1 Q I CROCHFLG=1 D SIG^XUSESIG I X1="" W !!,"Signature Code not valid" S (PSGORQF)=1 H 2 Q I CROCHFLG=1 I X1'="" D EN3^PSJRXI("CLINICAL REMINDER",CDRG1) I CROCHFLG>1 W ! S DIR(0)="SA^1:YES;0:NO",DIR("A")="Do you want to Intervene? ",DIR("B")="N" D ^DIR I CROCHFLG>1,Y=1 D EN3^PSJRXI("CLINICAL REMINDER",CDRG1) K CROCSTA,CROCDNM,CROCNUM,CROCSTR,CROCHFLG,CROCLN,CROCLN2,CDRG1 Q ; CKIVI ;Log IV interventions I $G(PSJDGCK) K CROCSTA,CROCDNM,CROCNUM,CROCSTR,CROCHFLG,CROCLN,CROCLN2,CDRG1 Q I CROCHFLG=1 D .S CROCIXN1="" F S CROCIXN1=$O(^TMP("CROCDRG",1,CROCIXN1)) Q:CROCIXN1=""!($G(PSGORQF)=1) D ..F CROCIXN2=0:0 S CROCIXN2=$O(^TMP("CROCDRG",1,CROCIXN1,CROCIXN2)) Q:'CROCIXN2!($G(PSGORQF)=1) D ...S DIR(0)="SA^1:YES;0:NO",DIR("A")="Do you want to Continue with "_^TMP("CROCDRG",1,CROCIXN1,CROCIXN2)_"? ",DIR("B")="Y" D ^DIR ...I Y["^"!(Y=0)!($D(DIRUT)) S PSGORQF=1 Q ...D SIG^XUSESIG I X1="" W !!,"Signature Code not valid" S PSGORQF=1 H 2 Q ...I X1'="" D EN3^PSJRXI("CLINICAL REMINDER",^TMP("CROCDRG",1,CROCIXN1,CROCIXN2)) I CROCHFLG=2!($D(^TMP("CROCDRG",2)))!($D(^TMP("CROCDRG",3))) D .I $G(PSGORQF)=1 Q .F CROCIXN1=1:0 S CROCIXN1=$O(^TMP("CROCDRG",CROCIXN1)) Q:'CROCIXN1 D ..S CROCIXN2="" F S CROCIXN2=$O(^TMP("CROCDRG",CROCIXN1,CROCIXN2)) Q:CROCIXN2="" D ...I CROCIXN1=2,$D(^TMP("CROCDRG",1,CROCIXN2)) Q ...I CROCIXN1=3,$D(^TMP("CROCDRG",2,CROCIXN2))!($D(^TMP("CROCDRG",1,CROCIXN2))) Q ...F CROCIXN3=0:0 S CROCIXN3=$O(^TMP("CROCDRG",CROCIXN1,CROCIXN2,CROCIXN3)) Q:'CROCIXN3 D ....S DIR(0)="SA^1:YES;0:NO",DIR("A")="Do you want to Intervene with "_^TMP("CROCDRG",CROCIXN1,CROCIXN2,CROCIXN3)_"? ",DIR("B")="N" D ^DIR ....I Y=1 D EN3^PSJRXI("CLINICAL REMINDER",^TMP("CROCDRG",CROCIXN1,CROCIXN2,CROCIXN3)) K CROCSTA,CROCDNM,CROCNUM,CROCSTR,CROCHFLG,CROCLN,CROCLN2,CDRG1,CROCIXN1,CROCIXN2,CROCIXN3,^TMP("CROCDRG") Q ; CKIV(CDRG2,IVFLG) ;CHECK FOR CROCS AGAINST PROSPECTIVE IV ORDER S (CROCSTA2,CROCDNM2,CROCNUM2)="" S CROCCTR=1 I IVFLG="A" S PHROI=$$GET1^DIQ(52.6,CDRG2,15,"I") I IVFLG="S" S PHROI=$$GET1^DIQ(52.7,CDRG2,9,"I") S ODRI=$O(^ORD(101.43,"ID",PHROI_";99PSP",0)) D ORDERCHK^PXRMORCH(DFN,ODRI,0,-1,0) I $D(^TMP($J,$G(ODRI))) D .S CROCNR=0 .F S CROCSTA2=$O(^TMP($J,ODRI,CROCSTA2)) Q:CROCSTA2="" D ..;I IVFLG="A" S ^TMP("CROCDRG",CROCSTA2,CROCCTR,CRIV)=$P(TMPDRG1("AD",CRIV),"^",2)_" "_$P(TMPDRG1("AD",CRIV),"^",3) ..;I IVFLG="S" S ^TMP("CROCDRG",CROCSTA2,CROCCTR,CRIV)=$P(TMPDRG1("SOL",CRIV),"^",2)_" "_$P(TMPDRG1("SOL",CRIV),"^",3) ..I IVFLG="A" S ^TMP("CROCDRG",CROCSTA2,$P(TMPDRG1("AD",CRIV),"^",2)_" "_$P(TMPDRG1("AD",CRIV),"^",3),CRIV)=$P(TMPDRG1("AD",CRIV),"^",2)_" "_$P(TMPDRG1("AD",CRIV),"^",3) ..I IVFLG="S" S ^TMP("CROCDRG",CROCSTA2,$P(TMPDRG1("SOL",CRIV),"^",2)_" "_$P(TMPDRG1("SOL",CRIV),"^",3),CRIV)=$P(TMPDRG1("SOL",CRIV),"^",2)_" "_$P(TMPDRG1("SOL",CRIV),"^",3) ..S CROCCTR=CROCCTR+1 ..F S CROCDNM2=$O(^TMP($J,ODRI,CROCSTA2,CROCDNM2)) Q:CROCDNM2="" D ...F S CROCNUM2=$O(^TMP($J,ODRI,CROCSTA2,CROCDNM2,CROCNUM2)) Q:CROCNUM2="" D ....S ^TMP("CROCIV",CROCSTA2,CROCDNM2,CROCNUM2)=^TMP($J,ODRI,CROCSTA2,CROCDNM2,CROCNUM2) K ^TMP($J,$G(ODRI)),CROCSTA2,CROCDNM2,CROCNUM2,CROCSTR2,CROCHFLG2,ODRI,IVFLG,CDRG2 Q ; CKIVD ;DISPLAY IV CROCS S (CROCSTA,CROCDNM,CROCNUM,CROCSTR,CROCHFLG)="" S CROCNMF=0 I $D(^TMP("CROCIV")) D .S CROCHFLG=2 .F S CROCSTA=$O(^TMP("CROCIV",CROCSTA)) Q:CROCSTA=""!$G(PSGORQF) D ..I CROCSTA=1 S CROCHFLG=1 ..F S CROCDNM=$O(^TMP("CROCIV",CROCSTA,CROCDNM)) Q:CROCDNM=""!$G(PSGORQF) D ...S CROCSTA2=$S(CROCSTA=1:"HIGH",CROCSTA=2:"MEDIUM",CROCSTA=3:"LOW",1:"UNKNOWN") ...W !,"*** Clinical Reminder Order Check ¦ Severity: "_CROCSTA2_" ***",! ...W !,CROCDNM,! S:'CROCNMF CDRG1=CROCDNM S CROCNMF=1 ...F S CROCNUM=$O(^TMP("CROCIV",CROCSTA,CROCDNM,CROCNUM)) Q:CROCNUM=""!$G(PSGORQF) D ....I ($Y+7)>IOSL,$E(IOST)="C" D .....W ! K DIR S DIR(0)="E",DIR("A")="Press Return to Continue..." D ^DIR K DIR W @IOF ....W !,^TMP("CROCIV",CROCSTA,CROCDNM,CROCNUM) ...W !!,CROCLN2 W ! K DIR S DIR(0)="E",DIR("A")="Press Return to Continue..." D ^DIR K DIR W @IOF K ^TMP("CROCIV"),CROCNMF G CKIVI Q |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |

#### CCR6205 (IP Option when a supply item is first on med profile, the CK option displays supply item message and will not complete order checks.)
CCR6628 (2nd line of the routine needs to be updated with the correct list of patch numbers.)

| #### Routine Name | PSGOE7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| SRS Traceability | CR1007 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary References | N/A |  |  |  |  |  |  |  |  |
| Related Protocols | N/A |  |  |  |  |  |  |  |  |
| Related Integration Agreements |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition |  |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic: N/A |  |  |  |  |  |  |  |  |  |
| New Routine |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| New Routine |  |  |  |  |  |  |  |  |  |

#### CCR6205, 6628

| Routine Name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| Requirement Traceability Matrix |  |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary (DD) References |  |  |  |  |  |  |  |  |  |
| Related Protocols |  |  |  |  |  |  |  |  |  |
| Related Integration Control Registrations (ICRs) |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic |  |  |  |  |  |  |  |  |  |
| PSGOE7 ;BIR/CML3-SELECT DRUG ;15 MAY 00 / 1:43 PM ;;5.0;INPATIENT MEDICATIONS;**9,26,34,52,55,50,87,111,181,254,267,260,288**;16 DEC 97;Build 93 ; ; Reference to ^PS(50.7 is supported by DBIA 2180 ; Reference to ^PS(59.7 is supported by DBIA 2181 ; Reference to ^PSDRUG( is supported by DBIA 2192 ; NFI-UD chgs for FR#: 1 ; ;S PSGDICS="U"_$S($D(PSJOERR):",I",1:"") S PSGDICS="U" ; AD ; Ask Drug K PSJDOSE,PSJDOX ;var array use in ^PSJDOSE K PSGODO,^TMP("PSJINTER",$J) D KILL^PSJBCMA5(+$G(PSJSYSP)) K DIC S DIC="^PS(50.7,",DIC(0)="EMQZVT",D="B^C" I '$P(PSJSYSU,";",4) S DIC("S")="I $$ENOISC^PSJUTL(Y,""U"")" E D .I '$D(PSJDGCK) S DIC("T")="",DIC="^PSDRUG(",DIC("S")="I +$G(^PSDRUG(+Y,2)),$P($G(^PSDRUG(+Y,2)),""^"",3)[""U"" S X(1)=+$G(^(""I"")) I $S('X(1):1,1:X(1)>DT)",D="B^C^VAPN^VAC^NDC^XATC" .I $D(PSJDGCK) S DIC("T")="",DIC="^PSDRUG(",DIC("S")="I +$G(^PSDRUG(+Y,2)),$$GCN^PSGOE7(+Y),$$PKGFLG^PSGOE7($P($G(^PSDRUG(+Y,2)),""^"",3)) S X(1)=+$G(^(""I"")) I $S('X(1):1,1:X(1)>DT)",D="B^C^VAPN^VAC^NDC^XATC" ; AD1 ; K PSGORD,PSJORD,PSJALLGY,^TMP("PSJINTER",$J),^PS(53.45,+$G(PSJSYSP),5),^PS(53.45,+$G(PSJSYSP),6) S PSGORQF=0 R !!,"Select DRUG: ",X:DTIME I '$T W $C(7) S X="^" I $D(PSJDGCK) I ($G(PSJOCNT)=1&(X="")) D Q .W !!,"Not enough active profile drugs to perform drug check",! .K DIR S DIR(0)="E",DIR("A")="Press Return to Continue..." D ^DIR K DIR W @IOF I $D(PSJDGCK),X="" N PSGDGCKF S PSGDGCKF=1 G DGCKX I ("^"[X)!(X="") S PSGORQF=1 G DONE G:X?1"S."1.E DONE I X?1."?" W !!?2,"Select the medication you wish the patient to receive." W:PSJSYSU<3 " You should consult",!,"with your pharmacy before ordering any non-formulary medication." W ! D MIX^DIC1 G:X?1."?" AD1 G:"^"[X!(Y'>0) AD1 G:$$PSJSUPCK^PSJDGCK(+Y) AD1 S (PSGDO,PSGDRG,PSGDRGN,PSGNEDFD,PSGPDRG,PSGPDRGN)="" DGCKX I $P(PSJSYSU,";",4) D G DO .S:'$D(PSJDGCK) PSGDRG=+Y,PSGDRGN=Y(0,0) .S:$D(PSJDGCK)&'$D(PSGDGCKF) PSGDRG=+Y,PSGDRGN=Y(0,0) .S:$D(PSJDGCK)&$D(PSGDGCKF) PSGDRG=$P($$DGCKIEN^PSJDGCK(),";",2),PSGDRGN=$P($G(^PSDRUG(PSGDRG,0)),U) .D DIN^PSJDIN(+$G(^PSDRUG(PSGDRG,2)),PSGDRG) .I '$D(PSJDGCK) I $P(Y(0),"^",9) D NF S:Y>0 PSGDRG=+Y(0),PSGDRGN=Y(0,0) D SNFM .I $D(PSJDGCK)&'$D(PSGDGCKF) I $P(Y(0),"^",9) D NF S:Y>0 PSGDRG=+Y(0),PSGDRGN=Y(0,0) D SNFM .S PSGPDRG=+$G(^PSDRUG(PSGDRG,2)),PSGPDRGN=$$OINAME^PSJLMUTL(PSGPDRG) I '$D(PSJDGCK) S PSGPDRG=+Y,PSGPDRGN=$$OINAME^PSJLMUTL(PSGPDRG) I $D(PSJDGCK)&'$D(PSGDGCKF) S PSGPDRG=+Y,PSGPDRGN=$$OINAME^PSJLMUTL(PSGPDRG) ;F Q=1:1:$L(PSGDICS) S X=$P(PSGDICS,",",Q) Q:X="" S PSJLUAPP=$O(^PS(50.3,PSGPDRG,1,"B",X,0)) I PSJLUAPP S X=$G(^PS(50.3,PSGPDRG,1,PSJLUAPP,0)) Q S X=$O(^PSDRUG("ASP",PSGPDRG,0)) I X,'$O(^(X)) S PSGDRG=X,PSGDRGN=$$ENDDN^PSGMI(X) ; DO ; dosage ordered NEW PSJALLGY S PSGNEDFD=$$GTNEDFD("U",PSGPDRG) I $G(PSGDRG),$P(PSJSYSU,";",4) S PSJALLGY(PSGDRG)="" D ENDDC^PSGSICHK(PSGP,PSGDRG) G:$G(PSGORQF) AD I '$P(PSJSYSU,";",4) S PSGX=PSGPDRG D END^PSGSICHK G:Y<0 AD S PSGDO="" ; DONE ; K DIC,%,%Y,PSGDICS,PSJLUAPP,Q1,Q2,Q3,Z,PSJALLGY Q ; ; NF ; S Y=0 W $C(7),!!,"PLEASE NOTE: The selected item is not currently on your medical center's",!?13,"formulary." Q:'$P(PSJSYSU,";",2) N CNT S CNT=0 F Q1=0:0 S Q1=$O(^PSDRUG(PSGDRG,65,Q1)) Q:'Q1 I $$CHKDRG(+$G(^(Q1,0))) S CNT=CNT+1 I CNT=0 W !!,"There are no formulary alternatives entered for this item." W:PSJSYSU>2 " You should consult",!,"with your pharmacy before ordering this item." S Y=0 Q I CNT=1 S Q1=$O(^PSDRUG(PSGDRG,65,0)),Q1=+$G(^(Q1,0)),Q3=$P(^PSDRUG(Q1,0),"^") W !!,Q3," has been entered as a formulary " W:$X>67 ! W "alternative." I F Q=1:1 S %=2 W !!,"Is ",$S(Q=1:"this",1:Q3)," acceptable" D YN^DICN Q:% D NFOH I CNT=1 S:%=1 (Y(0),Y)=Q1,Y(0,0)=Q3 S:%<0 Y=-1 Q K DA,DIC S DA(1)=PSGDRG,DIC="^PSDRUG("_PSGDRG_",65,",DIC(0)="AEMQZ",DIC("A")="Select FORMULARY ALTERNATIVE: " W ! D ^DIC K DIC Q ; NFOH ; S X="Answer 'YES' to order this formulary alternative ("_Q3_") for the patient instead of the non-formulary item originally selected. Answer 'NO' to use the drug originally selected." W !!?2 F Y=1:1:$L(X," ") S Z=$P(X," ",Y) W:$L(Z)+$X+2>IOM ! W Z," " Q CHKDRG(DRG) ; Determine if dispense drug is valid for Unit Dose. I $D(^PSDRUG(DRG,0)),$P($G(^(2)),U,3)["U" S X=+$G(^("I")) I 'X!(X>DT) Q DRG Q 0 ; SNFM ; show non-formulary message S Y=1 Q:PSJSYSU=3!'$O(^PS(59.7,1,21,0)) W $C(7),! S Q=0 F S Q=$O(^PS(59.7,1,21,Q)) Q:'Q W !,$G(^(Q,0)) W ! D READ^PSJUTL S Y=1 Q ; GTNEDFD(APP,PDRG) ; Find defaults from Orderable Item. Q $P($G(^PS(50.7,+PDRG,0)),"^",5,8) N Q,X S X="" F Q=1:1:$L(APP) S X=$E(APP,Q) Q:X="" S X=$O(^PS(50.3,+PDRG,1,"B",X,0)) I X S X=$P($G(^PS(50.3,+PDRG,1,X,0)),"^",5,8) Q Q X ; PKGFLG(PKF) ;Return 0 for not in range of acceptable package flags, 1 for within range I $S(PKF["U":1,1:0) Q 1 I $S(PKF["I":1,1:0) Q 1 Q 0 ; GCN(PSGIENID) ;Return 0 for not matched, 1 for matched with no GCNSEQNO, 1^1 for matched with a GCNSEQNO N PSGNDFID,PSGGCNPT,PSGGCNID S PSGNDFID=$P($G(^PSDRUG(PSGIENID,"ND")),"^"),PSGGCNPT=$P($G(^PSDRUG(PSGIENID,"ND")),"^",3) I 'PSGNDFID!('PSGGCNPT) Q 0 S PSGGCNID=$$PROD0^PSNAPIS(PSGNDFID,PSGGCNPT) I $P(PSGGCNID,"^",7) Q PSGIENID_";"_PSGNDFID_";"_$P(PSGGCNID,"^",7) Q PSGIENID_";"_PSGNDFID |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| ******************************************************************************* ¦"MDEVC"¦PSGOE7.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS;**9,26,34,52,55,50,87,111,181,254,267,2 60,281**;16 DEC 97;Build 4 ................... ¦"MDEV"¦PSGOE7.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS;**9,26,34,52,55,50,87,111,181,254,267,2 60,288**;16 DEC 97;Build 93 --------------------------------------- ¦"MDEVC"¦PSGOE7.INT.1 +30 D MIX^DIC1 G:X?1."?" AD1 G:"^"[X!(Y'>0) AD1 S (PSGDO,PSGDRG,PSGDRGN ,PSGNEDFD,PSGPDRG,PSGPDRGN)="" +31 I $D(PSJDGCK) I $$PSJSUPCK^PSJDGCK(+Y) G AD1 ................... ¦"MDEV"¦PSGOE7.INT.1 +30 D MIX^DIC1 G:X?1."?" AD1 G:"^"[X!(Y'>0) AD1 G:$$PSJSUPCK^PSJDGCK(+Y ) AD1 S (PSGDO,PSGDRG,PSGDRGN,PSGNEDFD,PSGPDRG,PSGPDRGN)="" ******************************************************************************* |  |  |  |  |  |  |  |  |  |

#### CCR2987 (Received the aminoglycoside order check on free text dosage in the unit dose package when entering it new but not on edit.)
CCR6124 (IP BUN scenario and the “Dangerous meds” alert did not fire.)

| #### Routine Name | PSGOEE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| SRS Traceability | CR2831 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary References | N/A |  |  |  |  |  |  |  |  |
| Related Protocols | N/A |  |  |  |  |  |  |  |  |
| Related Integration Agreements |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition |  |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic: |  |  |  |  |  |  |  |  |  |
| PSGOEE ;BIR/CML3-EDIT ACTIVE OR NON-VERIFIED ORDERS ; 8/12/10 5:59pm ;;5.0;INPATIENT MEDICATIONS;**4,7,29,47,64,58,82,91,110,111,112,142,179,181,254,267,268**;16 DEC 97;Build 9 ; ; Reference to ^PS(55 is supported by DBIA# 2191. ; Reference to ^PSSLOCK is supported by DBIA# 2789. ; D NOW^%DTC S PSGDT=% K PSGEFN,PSGOEEF S PSGOEEF=0 I PSGORD["A"!(PSGORD["O") G ACT 531 ; edit orders in 53.1 ENF ; Entry point D EN2^PSGOEEW K PSJACEPT D EDLOOP G:'$G(PSJACEPT) OUT I $G(PSGOEENO) D . N PSGOEENO S PSGOEENO=1 D NEW E D . N PSGOEENO S PSGOEENO=0 D UPD I $G(PSGOEAV) D ACT1 Q D DONE1 S PSGOEEF=0,PSJORD=PSGORD D GETUD^PSJLMGUD(PSGP,PSGORD),ENSFE^PSGOEE0(PSGP,PSGORD) Q ACT ; Perform Edit NEW PSJALGY1 K PSGOEER D EN2^PSGOEEW,EDLOOP G:'$G(PSJACEPT) OUT I $G(PSGOEENO) D . N PSGOEENO S PSGOEENO=1 D NEW E D . N PSGOEENO S PSGOEENO=0 D UPD S:$D(PSGOEF)!$G(PSGOEENO) PSGCANFL=-1 ACT1 ; Continue editing D DONE1 S PSGOEEF=0 D GETUD^PSJLMGUD(PSGP,PSGORD),ENSFE^PSGOEE0(PSGP,PSGORD) D:PSGOEAV UNL^PSSLOCK(PSGP,PSGORD) Q EDIT ; Edit I $G(Y) D ASKOVR(Y,$G(PSGORD),.PSJSTARI) D FULL^VALM1 W ! S PSGOEER="" F Q=1:1 S Q1=$P(Y,",",Q) Q:'Q1 S X=$P($T(@(PSGOEEG_Q1)),";",3),PSGOEER=PSGOEER_X_";",PSGOEEF(+X)=Q S:Q1=1 PSJALGY1=1 S LIMIT=$L(PSGOEER,";")-1,(PSGDEF,PSGOEE)=0 F S PSGOEE=PSGOEE+1 Q:PSGOEE>LIMIT I +$P(PSGOEER,";",PSGOEE)=101 S PSGDEF=1 S PSGOEER=$E(PSGOEER,1,$L(PSGOEER)-1),(MSG,PSGOEE)=0 F S PSGOEE=PSGOEE+1 Q:PSGOEE>$L(PSGOEER,";") S F1=$S(PSGOEEG=3:53.1,1:55.06) I 'PSGDEF!((PSGDEF)&(+$P(PSGOEER,";",PSGOEE)'=2)) D @$P(PSGOEER,";",PSGOEE) Q:'PSGOEE Q EDLOOP ; Continue prompting for fields to edit. K PSJNOO D:$G(Y) EDIT D ENNOU^PSGOEE0 I '$G(PSGOEENO),DR="" S VALMBCK="R" Q K VALMSG I '$G(PSGOEENO),$G(PSGPDNX) D CKDT I $G(PSGOEENO) D .S VALMSG="This change will cause a new order to be created." D GTSTATUS,CHKDD,CKDT .S PSGEBN=$$ENNPN^PSGMI(DUZ),PSGLIN=$$ENDD^PSGMI(PSGDT)_U_$$ENDTC^PSGMI(PSGDT) D CHK^PSGOEV("^^"_PSGMR_"^^^^"_PSGST,PSGPDRG_U_PSGDO,PSGSCH_U_PSGSD_"^^"_PSGFD) K VALMBCK,PSJACEPT,PSGPDNX D EN^VALM("PSJU LM ACCEPT") Q:'$G(PSJACEPT) I $G(PSGS0XT)="D",'$G(PSGS0Y) I ((",P,R,")'[(","_$G(PSGST)_",")) D Q .S PSJACEPT=0 W !!,"This is a 'DAY OF THE WEEK' schedule and MUST have admin times." D PAUSE^VALM1 I ($G(PSGOEER)["26^PSGOE9")!($G(PSGOEER)["26^PSGOE8")!($G(PSGOEER)["109^PSGOE9")!($G(PSGOEER)["109^PSGOE8")!($G(PSGOEER)["3^PSGOE9")!($G(PSGOEER)["3^PSGOE8")!($G(PSGOEER)["101^PSGOE9")!($G(PSGOEER)["101^PSGOE8") S PSGOEENO=1 I $G(PSGOEENO)!($G(PSGOEER)["2^PSGOE92")!($G(PSGOEER)["2^PSGOE82") D OC I $G(PSGORQF) S PSJNOO=-1 I '$G(PSJNOO),$G(PSGOEENO) S PSJNOO=$$ENNOO^PSJUTL5("E") D K1 S PSJACEPT=$S($G(PSJNOO)<0:0,1:1) S VALMBCK=$S('PSJACEPT:"R",'PSGOEAV:"R",1:"Q") Q OC ;Perform OC (only when OI was edited) & dosing check NEW PSJDD,PSJALLGY K PSGORQF D FULL^VALM1 S PSJDD=+$$DD53P45^PSJMISC() I 'PSJDD S PSGORQF=1 Q I $G(PSJALGY1)!$G(PSGOEENO) D . D ENDDC^PSGSICHK(PSGP,PSJDD) D:'$G(PSGORQF) IN^PSJOCDS($G(PSGORD),"UD",PSJDD) Q CHKDD ;*** Check inactive Dispense drug within the order. D CHKDRG^PSGOE2 Q CKDT ; Check if new start/stop dates should be calculated. S PSGS0Y=$S($D(PSGS0Y):PSGS0Y,1:$G(PSGAT)) ;PSJ*5*179 Recalc start date if Before last given I ($P($G(PSBSTR),"^")>PSGSD)!('$G(PSGNEWDT)&(PSGSD=$G(PSGOSD))&(PSGFD=$G(PSGOFD)))!($G(PSGOST)'=PSGST)!(PSGSCH'=$G(PSGOSCH))!($G(PSGPDNX)) D .N PSGOES S PSGOES=1,PSGOFD=PSGFD D ^PSGNE3 I $G(PSGOFD) S PSGNEFD=PSGFD .S PSGSD=PSGNESD,PSGSDN=$$ENDD^PSGMI(PSGNESD)_U_$$ENDTC^PSGMI(PSGNESD),PSGFD=PSGNEFD,PSGFDN=$$ENDD^PSGMI(PSGNEFD)_U_$$ENDTC^PSGMI(PSGNEFD),PSGNEWDT=1 .I $D(PSGOFD),PSGOFD]"",PSGFD'=PSGOFD S PSGOEEF(25)=1 .I $D(PSGOSD),PSGOSD]"",PSGSD'=PSGOSD S PSGOEEF(10)=1 ;BHW;PSJ*5*179;Add EFD call here, removed from PSGOE91 D EFDACT^PSJUTL Q NEW3 ; ;S:PSGOEAV PSGOEAV="0^1" NEW ; I $D(^PS(53.45,+$G(PSJSYSP),5)) N PSJFSI S PSJFSI=1 D FILESI^PSJBCMA5(DFN,PSGORD) N SIARRAY S SIARRAY="" D .I PSGORD["P" M SIARRAY=^PS(53.1,+PSGORD,15) D NEWNVAL^PSGAL5(PSGORD,6000,"SPECIAL INSTRUCTIONS",,.SIARRAY) .I PSGORD["U" M SIARRAY=^PS(55,DFN,5,+PSGORD,15) D NEWUDAL^PSGAL5(PSGORD,6000,"SPECIAL INSTRUCTIONS",,.SIARRAY) W !,"...discontinuing original order..." I PSGORD["P" S PSJCOM=+$P($G(^PS(53.1,+PSGORD,.2)),"^",8) I PSJCOM D NEW^PSJCOM1 Q ;DC and Unlock order. S PSGEDIT="DE" D ENOR^PSGOECS,UNL^PSSLOCK(PSGP,PSGORD) K PSGEDIT W !!," ...creating new order..." W:'PSGOEAV "(you will now work on this new order)" S PSGS0Y=PSGAT,PSGNESD=PSGSD,PSGNEFD=PSGFD,PSGOEPR=PSGPR,PSGPDRG=PSGPD,PSGPDRGN=PSGPDN,PSGOEE="E" S PSGOORD=PSGORD D ^PSGOETO K PSGOEOS I PSGOORD["U" S $P(^PS(55,PSGP,5,+PSGOORD,0),"^",26,27)=PSGORD_"^E" E S $P(^PS(53.1,+PSGOORD,0),"^",26,27)=PSGORD_"^E" I $G(PSJFSI) I $$GETSI^PSJBCMA5(DFN,PSGOORD) D FILESI^PSJBCMA5(DFN,PSGORD) I 'PSGOEAV,($G(PSGORD)["P"),'$G(^PS(53.1,+PSGORD,2.5)),$G(^PS(53.1,+PSGORD,0)) D . N DUR S DUR=$$GETDUR^PSJLIVMD(PSGP,PSGORD,$S(PSGORD["P":"P",1:5),1) I DUR]"" K DA,DR,DIE S DIE="^PS(53.1,",DA=+PSGORD,DR="116////"_DUR D ^DIE I PSGOEAV,+PSJSYSU=3,'$D(PSGOES) D EN^PSGPEN(PSGORD),UNL^PSSLOCK(PSGP,PSGORD) Q S PSJORD=PSGORD,PSGACT=$$ENACTION^PSGOE1(PSGP,PSGORD) Q UPD ; K DA W !!,"...updating order..." I PSGORD["P" S PSJCOM=+$P($G(^PS(53.1,+PSGORD,.2)),"^",8) I PSJCOM D UPD^PSJCOM Q I $$DIFFSI^PSJBCMA5(DFN,PSGORD) D .N SIARRAY M:PSGORD["P" SIARRAY=^PS(53.1,+PSGORD,15) M:PSGORD["U" SIARRAY=^PS(55,DFN,5,+PSGORD,15) .Q:'$D(SIARRAY) .I PSGORD["P" D NEWNVAL^PSGAL5(PSGORD,6000,"SPECIAL INSTRUCTIONS",,.SIARRAY) .I PSGORD["U" D NEWUDAL^PSGAL5(DFN,PSGORD,6000,"SPECIAL INSTRUCTIONS",,.SIARRAY) ; Set trigger for FIELD (12) Dispense Drug to print a updated pick list. I PSGORD["U",$D(^PS(53.45,PSJSYSP,2,1,0)),$D(^PS(55,PSGP,5,+PSGORD,1,1,0)) D .N PSJX12,PSJF12 S PSJF12=0 .F PSJX12=0:1 S PSJX12=$O(^PS(53.45,PSJSYSP,2,PSJX12)) Q:+PSJX12=0 S:$G(^PS(53.45,PSJSYSP,2,PSJX12,0))'=$G(^PS(55,PSGP,5,+PSGORD,1,PSJX12,0)) PSJF12=1 .S:PSJF12 ^PS(55,"AUE",PSGP,+PSGORD)="" N TMP,PSGSIF S TMP=PSGOEENO N PSGOEENO S PSGOEENO=TMP N II F II=1:1:$L($G(DR),";") I $E($P($G(DR),";",II),1,7)="122////" S PSGSIF=$P(PSGSI,"^",2),PSGSI=$P(PSGSI,"^") Q I $G(PSJCOM),$G(PSJCOMSI) K PSJCOMSI,^TMP("PSGSI",$J) M ^TMP("PSGSI",$J,5)=^PS(53.45,PSJSYSP,5) D .D FILESI^PSJBCMA5(DFN,PSGORD) .N PSJCHILD,PSJOEORD S PSJOEORD=0 F S PSJOEORD=$O(^PS(55,"ACX",PSJCOM,PSJOEORD)) Q:'PSJOEORD D .. S PSJCHILD=0 F S PSJCHILD=$O(^PS(55,"ACX",PSJCOM,PSJOEORD,PSJCHILD)) Q:'PSJCHILD D ... Q:PSJCHILD=PSGORD N DR,DA,DIE,ORD S DR=$S(PSJCHILD["V":"31////"_$G(P("OPI")),1:"8////"_$G(PSGSI)) S DR=DR_";"_$S(PSJCHILD["V":146,1:122)_"////"_+$G(PSGSIF) ... I '$D(^PS(53.45,+$G(PSJSYSP),5)) M ^PS(53.45,+$G(PSJSYSP),5)=^TMP("PSGSI",$J,5) ... D FILESI^PSJBCMA5(DFN,PSJCHILD) ... ;PSJ*5*179 Comment edits ... S DR=$TR(DR,"*") I DR'="" S DA=+PSJCHILD,DIE=$S(PSJCHILD["U":"^PS(55,"_PSGP_",5,",1:"^PS(53.1,") S:DIE["^PS(55," DA(1)=PSGP D ^DIE W "." D EN1^PSJHL2(PSGP,"XX",+PSJCHILD_"U") . K ^TMP("PSGSI",$J) S DR=$TR(DR,"*") I DR'="" S DA=+PSGORD,DIE=$S(PSGORD["U":"^PS(55,"_PSGP_",5,",1:"^PS(53.1,") S:DIE["^PS(55," DA(1)=PSGP D ^DIE W "." F Q=1,3 K @(PSGOEEWF_Q_")") S %X="^PS(53.45,"_PSJSYSP_","_$S(Q=1:2,1:1)_",",%Y=PSGOEEWF_Q_"," K @(PSGOEEWF_Q_")") D %XY^%RCR W "." S $P(@(PSGOEEWF_"1,0)"),"^",2)=$S(PSGORD["U":55.07,1:53.11)_"P" I $D(^PS(53.45,+$G(PSJSYSP),5)) D FILESI^PSJBCMA5(DFN,PSJORD) ; Naked reference on the line below refers to full reference using indirection to either ^PS(55 or ^PS(53.1, S ND=$G(@($S(PSGORD["U":"^PS(55,"_PSGP_",5,",1:"^PS(53.1,")_+PSGORD_",0)")) I $P(ND,"^",21) S ORIFN=$P(ND,"^",21),ND1=$G(^(.2)),ND2=$G(^(2)) W !,"...updating OE/RR..." D EN1^PSJHL2(PSGP,"XX",PSGORD) I $$ENACTION^PSGOE1(PSGP,PSGORD)["V" S VALMBCK="R" I PSJSYSL,PSJSYSL<3 S $P(@($S(PSGORD["U":"^PS(55,"_PSGP_",5,",1:"^PS(53.1,")_+PSGORD_",7)"),"^",1,2)=PSGDT_"^"_$E("D",PSGOEENO)_"E",PSGTOL=2,PSGUOW=DUZ,PSGTOO=PSGORD'["U"+1,DA=+PSGORD D ENL^PSGVDS ; **This is where the Automated Dispensing Machine hook is called. Do NOT DELETE or change this location ** D EDIT^PSJADM ; **END of Interface Hook ** Q OUT ; D ABORT K PSGNEWDT S PSGCANFL=1 D GETUD^PSJLMGUD(PSGP,PSGORD),ENSFE^PSGOEE0(PSGP,PSGORD),INIT^PSJLMUDE(PSGP,PSGORD) Q DONE ; I PSGORD["P",'$D(PSGOEF),PSGSCH]"",$O(^PS(53.1,+PSGORD,1,0)) D ENF^PSGOEE0 DONE1 ; I PSGORD["U" S X=+PSGORD L -^PS(55,PSGP,5,X) E L -^PS(53.1,+PSGORD) K ^PS(53.45,+PSJSYSP,1),^(2),^(5),^(6) I '$D(PSGOEF) K PSGSD,PSGSCH,PSGST,PSGFD K DA,DIE,DIR,DP,DR,DRG,ND,ND0,ND1,ND2,ORIFN,PSGAL,PSGALEF,PSGAT,PSGOEE,PSGOEEF,PSGOEEG,PSGOEEWF,PSGEFN,PSGTOL,PSGTOO,PSGUOW,XREF,PSGEFN,PSGMR,PSGMRN,PSGOROE1,PSGPD,PSGPDN,PSGSI,PSGPR,PSGSM,PSGHSM,PSGSTN,PSGSDN,PSGFDN,PSGPRN K PSGDO,PSGOEENO Q K1 ; K BACK,F1,F2,PSGF2,MSG,PSGEFN,PSGNEWDT,PSGOEEND,PSGOPD,PSGOPDN,PSGOMR,PSGOMRN,PSGOSCH,PSGOSI,PSGOPR,PSGOSM,PSGOHSM,PSGOSD,PSGOFD,PSGOST,PSGOPRN,PSGOSTN,PSGOSDN,PSGOFDN,PSGODO,PSGPDRG,PSGPDRGN,PSGOEER Q ; ABORT ; Display no change message and pause. S (PSGDI,PSGDFLG)='$$DDOK^PSGOE2(PSGOEEWF_"1,",+$G(@(PSGOEEWF_".2)"))) S PSGPFLG='$$OIOK^PSGOE2(+$G(@(PSGOEEWF_".2)"))) W !!,$C(7),"No changes made to this order." D PAUSE^VALM1 K PSGOEEF S PSGOEEF=0 Q ; GTSTATUS ; Determine status of new order and set LM title. S PSGSTAT=$S($P($G(PSJSYSP0),U,9):"ACTIVE",1:"NON-VERIFIED") S VALM("TITLE")=PSGSTAT_" UNIT DOSE "_$S(PSGSTAT="PENDING":"("_PSGPRIO_")",1:"") Q ; ASKOVR(Y,PSJOVRON,PSJSTARI) ; Check to see if any starred fields are being edited. If so, ask if they wish to view overrides/interventions if they exist Q:'$D(Y)!$D(PSJSTARI) N II,I3,YY S YY=$S(Y:Y,1:$TR($P(Y,"^",4),"=")) Q:'YY S PSJOVRON=$S($G(PSJOVRON):PSJOVRON,1:$G(PSJORD)) Q:'$G(PSJOVRON) N PSJORD S PSJORD=PSJOVRON I '$G(PSJSTARI) F II=1:1:$L(YY,",") Q:$G(PSJSTARI) S I3=$P(YY,",",II) I I3 S:$G(PSGEFN(I3))!($G(PSJSTAR)[("("_I3_")")) PSJSTARI=1 I $G(PSJSTARI) I ($G(PSJORD)&$G(PSGP)) I $$ASKDISP^PSGSICH1 D FULL^VALM1 D OVRDISP^PSGSICH2(PSGP,PSJORD,3) Q ; FIELDS ; 31 ;;101^PSGOE8;PSGOPD;PSGPD;101;1 32 ;;109^PSGOE8;PSGODO;PSGDO;109;PSGODO]"" 33 ;;10^PSGOE81;PSGOSD;PSGSD;10;0 34 ;;3^PSGOE8;PSGOMR;PSGMR;3;1 35 ;;25^PSGOE81;PSGOFD;PSGFD;25;0 36 ;;7^PSGOE8;PSGOST;PSGST;7;0 37 ;;5^PSGOE82;PSGOSM;PSGSM;5;0 38 ;;26^PSGOE8;PSGOSCH;PSGSCH;26;1 39 ;;39^PSGOE81;PSGOAT;PSGAT;39;0 310 ;;1^PSGOE82;PSGOPR;PSGPR;1;1 311 ;;8^PSGOE81;PSGOSI;PSGSI;8;0 312 ;;2^PSGOE82;;;2;0 313 ;;40^PSGOE82;;;40;0 51 ;;101^PSGOE9;PSGOPD;PSGPD;101;1 52 ;;109^PSGOE9;PSGODO;PSGDO;109;PSGODO]"" 53 ;;10^PSGOE91;PSGOSD;PSGSD;10;1 54 ;;3^PSGOE9;PSGOMR;PSGMR;3;1 55 ;;34^PSGOE91;PSGOFD;PSGFD;34;1 56 ;;7^PSGOE9;PSGOST;PSGST;7;0 57 ;;5^PSGOE92;PSGOSM;PSGSM;5;0 58 ;;26^PSGOE9;PSGOSCH;PSGSCH;26;1 59 ;;41^PSGOE91;PSGOAT;PSGAT;41;0 510 ;;1^PSGOE92;PSGOPR;PSGPR;1;1 511 ;;8^PSGOE91;PSGOSI;PSGSI;8;0 512 ;;2^PSGOE92;;;2;0 513 ;;15^PSGOE92;;;15;0 |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| ******************************************************************************* ¦"MDEVC"¦PSGOEE.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS;**4,7,29,47,64,58,82,91,110,111,112,142 ,179,181,254,267,268,281**;16 DEC 97;Build 4 ................... ¦"MDEV"¦PSGOEE.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS;**4,7,29,47,64,58,82,91,110,111,112,142 ,179,181,254,267,268**;16 DEC 97;Build 9 --------------------------------------- ¦"MDEVC"¦PSGOEE.INT.1 +54 I $G(PSGOEENO)!($G(PSGOEER)["2^PSGOE92")!($G(PSGOEER)["2^PSGOE82")! ($G(PSGOEER)["109^PSGOE8") D OC ................... ¦"MDEV"¦PSGOEE.INT.1 +54 I $G(PSGOEENO)!($G(PSGOEER)["2^PSGOE92")!($G(PSGOEER)["2^PSGOE82") D OC --------------------------------------- ¦"MDEVC"¦PSGOEE.INT.1 +60 OC ;Perform OC (only when OI or Dosage was edited) & dosing check ................... ¦"MDEV"¦PSGOEE.INT.1 +60 OC ;Perform OC (only when OI was edited) & dosing check ******************************************************************************* |  |  |  |  |  |  |  |  |  |

#### CCR6628 (2nd line of the routine needs to be updated with the correct list of patch numbers.)

| Routine Name | PSJDGCK |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| Requirement Traceability Matrix | CR1007, 5184 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary (DD) References |  |  |  |  |  |  |  |  |  |
| Related Protocols |  |  |  |  |  |  |  |  |  |
| Related Integration Control Registrations (ICRs) |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic |  |  |  |  |  |  |  |  |  |
| PSJDGCK ;HP/MJE - Drug Interaction Utility ;09/22/11 5:00pm ;;5.0;INPATIENT MEDICATIONS;**260,288**;16 DEC 97;Build 93 ;External references to ^PSSDSAPM supported by DBIA 5570 ; DGCK ; S PSJDGCK=1 D FULL^VALM1 I $D(PSJDGCK)&($G(VALM("TITLE"))="IV Order Entry") D EN^PSJO1(2) I $$DGCKIEN^PSJDGCK()="" D .W !!,"Not enough active profile drugs to perform drug check",! .K DIR S DIR(0)="E",DIR("A")="Press Return to Continue..." D ^DIR K DIR W @IOF E D ^PSGOE7 DGCKEND S VALMBCK="R" K PSJDGCK Q ; DGCKIEN() ; N PSJDGCK7 S PSJDGCK7="" I '$D(^TMP("PSJ",$J)) Q PSJDGCK7 I $D(PSJDGCK) D .N PSJDGCK1,PSJDGCK2,PSJDGCK3,PSJDGCK4,PSJDGCK5,PSJDGCK6 S (PSJDGCK1,PSJDGCK2,PSJDGCK3,PSJDGCK4,PSJDGCK5,PSJDGCK6)="" .S PSJDGCK1=$O(^TMP("PSJ",$J,PSJDGCK1)),PSJDGCK2=$O(^TMP("PSJ",$J,PSJDGCK1,PSJDGCK2)),PSJDGCK3=$O(^TMP("PSJ",$J,PSJDGCK1,PSJDGCK2,PSJDGCK3)),PSJDGCK4=$O(^TMP("PSJ",$J,PSJDGCK1,PSJDGCK2,PSJDGCK3,PSJDGCK4)) .I PSJDGCK4["U" S PSJDGCK5=$$GET1^DIQ(55.06,+PSJDGCK4_","_DFN_",",108,"I"),PSJDGCK6=$$DRG^PSSDSAPM(+PSJDGCK5,"I"),PSJDGCK7=$$GET1^DIQ(50,$P(PSJDGCK6,";"),.01,"E") .I PSJDGCK4["V" S PSJDGCK5=$$GET1^DIQ(55.01,+PSJDGCK4_","_DFN_",",130,"I"),PSJDGCK6=$$DRG^PSSDSAPM(+PSJDGCK5,"I"),PSJDGCK7=$$GET1^DIQ(50,$P(PSJDGCK6,";"),.01,"E") .I PSJDGCK4["P"!(PSJDGCK4["N") S PSJDGCK5=$$GET1^DIQ(53.1,+PSJDGCK4_","_DFN_",",108,"I"),PSJDGCK6=$$DRG^PSSDSAPM(+PSJDGCK5,"I"),PSJDGCK7=$$GET1^DIQ(50,$P(PSJDGCK6,";"),.01,"E") .S PSJDGCK7=PSJDGCK7_";"_$P(PSJDGCK6,";") Q PSJDGCK7 ; PSJSUPCK(CHK) ; I '$D(PSJDGCK) K CHK Q 0 I '($P($G(^PSDRUG(CHK,0)),"^",3)["S"!($E($P($G(^PSDRUG(CHK,0)),"^",2),1,2)="XA")) K CHK Q 0 W !,"You have selected a supply item, please select another drug" W !,"or leave blank and hit enter for Profile Order Checks." K CHK Q 1 |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| ******************************************************************************* ¦"MDEVC"¦PSJDGCK.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS;**260,288,281**;16 DEC 97;Build 4 ................... ¦"MDEV"¦PSJDGCK.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS;**260,288**;16 DEC 97;Build 93 --------------------------------------- ¦"MDEVC"¦PSJDGCK.INT.1 +29 I $D(PSGDGCKF) Q 0 ................... ¦"MDEV"¦PSJDGCK.INT.1 +29 I '$D(PSJDGCK) K CHK Q 0 ******************************************************************************* |  |  |  |  |  |  |  |  |  |

#### CCR6585 (CROC Message does not display when processing IP IV solution.)
CCR6398 (Clinical Reminder Order Check has two Press Return messages.)
CCR6646 (System does not accept NO Entry at prompts Intervene and Continue.)

| Routine Name | PSJOC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| SRS Traceability | CR3286 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary References | N/A |  |  |  |  |  |  |  |  |
| Related Protocols | N/A |  |  |  |  |  |  |  |  |
| Related Integration Agreements |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition |  |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic: N/A |  |  |  |  |  |  |  |  |  |
| PSJOC ;BIR/MV - NEW ORDER CHECKS DRIVER ;6 Jun 07 / 3:37 PM ;;5.0;INPATIENT MEDICATIONS ;**181,260**;16 DEC 97;Build 94 ; ; Reference to ^PSODDPR4 is supported by DBIA# 5366. ; Reference to ^PSSHRQ2 is supported by DBIA# 5369. ; OC(PSPDRG,PSJPTYP) ; ;PSPDRG - Drug array in format of PDRG(n)=IEN (#50) ^ Drug Name ;Where n is a sequential number. The Drug Name can be OI, Generic name form #50, or Add/Sol name ;PSJPTYP - P1 ; P2 ; Where P1 is "I" for Inpatient, "O" for Outpatient ; P2 is the Inpatient Order Number (for PSJ use only) ;PSJOCERR(DRUG NAME)="Reason text". Where Drug Name can be either OI name or AD/SOL name. NEW PSJOCERR ;Quit OC if FDB link is down. PSGORQF is defined if user wish to stop the order process I $$SYS^PSJOCERR() Q ; I $D(PSJDGCK) W !!,"Building MEDS profile please wait...",! D BLD^PSODDPR4(DFN,"PSJPRE",.PSPDRG,PSJPTYP) D DISPLAY K ^TMP($J,"PSJPRE") Q DISPLAY ; NEW PSJPAUSE,PSJOLDSV,PSJDNM,PSJMON,PSJOC,PSJOCDT,PSJOCDTL,PSJOCLST,PSJP,PSJS,PSJPON,PSJDN,PSJSEV,PSJECNT D GMRAOC Q:'$$DSPSERR() D FULL^VALM1 W @IOF,"Now Processing Enhanced Order Checks! Please wait...",! ; If there are no OC or errors to display, this var will trigger a pause before continue /w the order S PSJPAUSE=1 D DRUGERR I $D(PSJDGCK) W:'$D(^TMP($J,"PSJPRE","OUT","DRUGDRUG"))&'$D(^TMP($J,"PSJPRE","OUT","THERAPY",1)) !,"No Order Check Warnings Found",! ;Process drug interaction & drug interception D DI^PSJOCDI Q:$G(PSGORQF) D DSPERR^PSJOCERR("DRUGDRUG") ;Process duplicate therapy order checks D:'$D(PSJDGCK) DT^PSJOCDT I $D(PSJDGCK) D:$D(^TMP($J,"PSJPRE","OUT","THERAPY")) DTDGCK^PSJOCDT D:'$G(PSGORQF) DSPERR^PSJOCERR("THERAPY") I $D(PSJDGCK),'$D(^TMP($J,"PSJPRE","OUT","THERAPY")) D PAUSE^PSJLMUT1 Q ;DX action Q:$G(PSGORQF) D:$G(PSJPAUSE) PAUSE^PSJLMUT1 Q ; GMRAOC ;Display allergy & CPRS OC regardless if FDB is connected D ALLERGY D CPRS^PSJOCOR(.PSPDRG) Q ALLERGY ;Do allergy order check ;The allergy check will be processed for each of the dispense drug stores in the PSJALLGY array ;PSJALLGY(X)="" Where X is the disp drug IEN. PSJALLGY array store all dispense drugs use in an order ; I '$D(PSJDGCK) D .NEW PSJDD .F PSJDD=0:0 S PSJDD=$O(PSJALLGY(PSJDD)) Q:'PSJDD D EN^PSJGMRA(DFN,PSJDD) I $D(PSJDGCK) D .S PSJXX="" S PSJYY=1 .F S PSJXX=$O(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJXX)) Q:PSJXX="" D ..S PSJALLGY(PSJYY,$P(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJXX),U,3))="" ..S PSJYY=PSJYY+1 .F PSJDD=0:0 S PSJDD=$O(PSJALLGY(PSJDD)) Q:'PSJDD F PSJCC=0:0 S PSJCC=$O(PSJALLGY(PSJDD,PSJCC)) Q:'PSJCC D EN^PSJGMRA(DFN,PSJCC) K PSJXX,PSJYY,PSJDD,PSJCC,PSJALLGY Q DSPORD(ON,PSJNLST,PSJCLINF) ;Display the order data ;ON - ON_U/V/P ex: 21V ;PSJNLST - It's number list and also use to trigger pg break, line break NEW PSJCOL,PSJX,PSJOC,PSJLINE,X Q:ON="" S:'$D(PSJCLINF) PSJCLINF=";0" S PSJLINE=1,PSJCOL=1 I $P(PSJCLINF,";",2) D CLNDISP^PSJCLNOC(.PSJCLINF) D Q .I $G(PSJNLST)="",(($Y+6)>IOSL) D PAUSE^PSJLMUT1 W @IOF I ON'["V" D DSPLORDU^PSJLMUT1(DFN,ON) I ON["V" D DSPLORDV^PSJLMUT1(DFN,ON) F PSJX=0:0 S PSJX=$O(PSJOC(ON,PSJX)) Q:'PSJX D . I $G(PSJNLST)="",(($Y+6)>IOSL) D PAUSE^PSJLMUT1 W @IOF . W ! . I $G(PSJNLST) W:(PSJX=1) PSJNLST W:(PSJX>1) ?$L(PSJNLST) . S X=PSJOC(ON,PSJX) . W $E(X,9,$L(X)) W ! Q ; DRUGERR ;Display drug level errors NEW PSJPON,PSJN,PSJNV,PSJDSPFG,PSJPERR,PSJX ;Only display the exceptions once per patient. ;PSJEXCPT(PSJDNM_REASON) - Array for invalid drugs that already display to once within a pt selection S PSJDSPFG=0 S PSJPERR=$$PROSPERR() S PSJPON="" F S PSJPON=$O(^TMP($J,"PSJPRE","OUT","EXCEPTIONS",PSJPON)) Q:PSJPON="" D . F PSJN=0:0 S PSJN=$O(^TMP($J,"PSJPRE","OUT","EXCEPTIONS",PSJPON,PSJN)) Q:'PSJN D .. S PSJNV=$G(^TMP($J,"PSJPRE","OUT","EXCEPTIONS",PSJPON,PSJN)) .. I ($P(PSJPON,";",3)="PROSPECTIVE") S PSJX='$$ERRCHK("PROSPECTIVE",$P(PSJNV,U,3)_$P(PSJNV,U,10)) .. I PSJPERR,($P(PSJPON,";",3)="PROFILE") Q .. I ($P(PSJPON,";",3)="PROFILE"),'$$ERRCHK("PROFILE",$P(PSJNV,U,3)_$P(PSJNV,U,10)) Q .. D DSPDRGER() I PSJDSPFG D PAUSE^PSJLMUT1 W @IOF Q DSPDRGER(PSJDSFLG) ; NEW PSJTXT S PSJTXT=$P(PSJNV,U,7) S X="Enhanced Order Checks cannot " I $G(PSJDSFLG),(PSJTXT[X) S PSJTXT="Dosing Checks could not "_$P(PSJTXT,X,2) S PSJDSPFG=1 K PSJPAUSE I ($Y+6)>IOSL D PAUSE^PSJLMUT1 W @IOF W ! D WRITE^PSJMISC(PSJTXT,,79) D:$P(PSJNV,U,10)]"" WRITE^PSJMISC("Reason: "_$P(PSJNV,U,10),3,79) W ! Q ERRCHK(PSJTYPE,PSJX) ; ;PSJTYPE - Either "PROFILE" or "PROSPECTIVE" ;PSJX - Drug name_Error reason ;Return 1 if this error drug has not displayed to the user. I $G(PSJX)="" Q 0 I $G(PSJTYPE)="" Q 0 I PSJTYPE="PROFILE",'$D(PSJEXCPT(PSJTYPE,PSJX)) S PSJEXCPT(PSJTYPE,PSJX)="" Q 1 I PSJTYPE="PROSPECTIVE",'$D(PSJEXCPT(PSJTYPE,PSJX)) S PSJEXCPT(PSJTYPE,PSJX)="" Q 1 Q 0 PING(PSJMSG) ;Check if FDB is down. Return 0 if it is ;pass in a message to customize the display S ^TMP($J,"PSJPRE","IN","PING")="" D IN^PSSHRQ2("PSJPRE") Q $$DSPSERR($G(PSJMSG)) DSPSERR(PSJMSG) ;Display system errors NEW X S X=$G(^TMP($J,"PSJPRE","OUT",0)) I $P(X,U)=-1 D NOFDB($P(X,U,2),$G(PSJMSG)) Q $S($P(X,U)=-1:0,1:1) NOFDB(PSJX,PSJMSG) ;Display connection down message Q:$G(PSJX)="" I $G(PSJMSG)]"" W !!,"No dosing checks can be performed" I $G(PSJMSG)="" W !!,"No Enhanced Order Checks can be performed." W !," Reason: ",PSJX,!! K PSJX Q PROSPERR() ;Return 1 if only prospective exception should be displayed. NEW PSJPERR,PSJPON S PSJPERR=1 ;If all prospectives are caught in the exception then display them only and omit the profile drugs S PSJPON="" F S PSJPON=$O(^TMP($J,"PSJPRE","IN","PROSPECTIVE",PSJPON)) Q:PSJPON="" Q:'PSJPERR D . I $D(^TMP($J,"PSJPRE","OUT","EXCEPTIONS",PSJPON)) Q . S PSJPERR=0 Q PSJPERR |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| ******************************************************************************* ¦"MDEVC"¦PSJOC.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS ;**181,260,281**;16 DEC 97;Build 93 ................... ¦"MDEV"¦PSJOC.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS ;**181,260**;16 DEC 97;Build 94 --------------------------------------- ¦"MDEVC"¦PSJOC.INT.1 +26 S $P(CROCLN,"=",60)="=",$P(CROCLN2,"-",60)="-",CROCNR=1 +27 I '$D(PSJDGCKX) D +28 .I '$D(TMPDRG1("AD",0))&('$D(TMPDRG1("SOL",0)))&($G(PSPDRG(1))) D C K^PSJCROC($P(PSPDRG(1),"^")) I $G(PSGORQF) Q +29 .I $G(TMPDRG1("AD",0))>0!$G(TMPDRG1("SOL",0))>0 W @IOF,!,"Now proce ssing Clinical Reminder Order Checks. Please wait ...",!!,CROCLN +30 .I $G(TMPDRG1("AD",0))>0 F CRIV=0:0 S CRIV=$O(TMPDRG1("AD",CRIV)) Q :'CRIV D CKIV^PSJCROC($P(TMPDRG1("AD",CRIV),"^",1),"A") +31 .I $G(TMPDRG1("SOL",0))>0 F CRIV=0:0 S CRIV=$O(TMPDRG1("SOL",CRIV)) Q:'CRIV D CKIV^PSJCROC($P(TMPDRG1("SOL",CRIV),"^",1),"S") +32 .I $G(TMPDRG1("AD",0))>0!$G(TMPDRG1("SOL",0))>0 D CKIVD^PSJCROC +33 .I CROCNR W !! K DIR S DIR(0)="E",DIR("A")="Press Return to Continu e..." D ^DIR K DIR +34 K CRIV,CROCPFLG,CROCNR +35 I $G(PSGORQF) W $C(7)," ...order not entered..." Q ................... ¦"MDEV"¦PSJOC.INT.1 --------------------------------------- ¦"MDEVC"¦PSJOC.INT.1 +53 I '$D(PSJDGCK) D:$G(PSJPAUSE) PAUSE^PSJLMUT1 ................... ¦"MDEV"¦PSJOC.INT.1 +43 D:$G(PSJPAUSE) PAUSE^PSJLMUT1 ******************************************************************************* |  |  |  |  |  |  |  |  |  |

#### CCR6312 (Add functionality to use Clinical Reminder based order checks in pharmacy order entry.)

| Routine Name | PSJRXI |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhancement Category | New |  | Modify | Delete |  |  | No Change |  |  |
| Requirement Traceability Matrix | CR3286 |  |  |  |  |  |  |  |  |
| Related Options |  |  |  |  |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  |  |  | Routines “Called” |  |  |  |
| Data Dictionary (DD) References |  |  |  |  |  |  |  |  |  |
| Related Protocols |  |  |  |  |  |  |  |  |  |
| Related Integration Control Registrations (ICRs) |  |  |  |  |  |  |  |  |  |
| Data Passing | Input | Output Reference |  |  | Both |  |  | Global Reference | Local |
| Input Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Output Attribute Name and Definition | Name: Definition: |  |  |  |  |  |  |  |  |
| Current Logic |  |  |  |  |  |  |  |  |  |
| Modified Logic (Changes are in bold) |  |  |  |  |  |  |  |  |  |
| ******************************************************************************* ¦"MDEVC"¦PSJRXI.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS;**3,181,254,267,281**;16 DEC 97;Build 4 ................... ¦"MDEV"¦PSJRXI.INT.1 +2 ;;5.0;INPATIENT MEDICATIONS;**3,181,254,267**;16 DEC 97;Build 158 --------------------------------------- ¦"MDEVC"¦PSJRXI.INT.1 +88 ; +89 EN3(PSJIVDST,CDRG2) ; Entry Point for Clinical Reminders +90 ;PSJIVDST = intervention text +91 N PSJIVDSN S PSJIVDSN=0 +92 ;D LOOK I 'PSJIVDSN Q 1 +93 S PSJRXREQ=PSJIVDST +94 S PSJDD=CDRG2 +95 D START K PSJIVDSN,CDRG2 +96 EN3X Q 0 +97 ; +98 LOOK ;Find Internal Number of 32.3 file +99 S PSJRXIEN=$$FIND1^DIC(9009032.3,"","X",PSJIVDST,"B") +100 Q ................... ¦"MDEV"¦PSJRXI.INT.1 ******************************************************************************* |  |  |  |  |  |  |  |  |  |

### Templates

| Template Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enhancement Category | New | Modify | Delete |  | No Change |
| Requirements Traceability Matrix |  |  |  |  |  |
| Template Type | Sort | Input | Print |  | Other |
| Related Options |  |  |  |  |  |
| Related Routines | Routines “Called By” |  |  | Routines “Called” |  |
| Data Dictionary (DD) References |  |  |  |  |  |
| Global References |  |  |  |  |  |

### Bulletins

N/A

### Data Dictionaries (DDs)

N/A

### Field Definition Information

N/A

### Data Entries Affected by the Design

N/A

### Unique Record(s) [Commonly the .01 field]

N/A

### File/Global Size Changes

N/A

### Mail Groups

N/A

### Security Keys

N/A

### Options

N/A

### Protocols

N/A

### Remote Procedure Call (RPC)

N/A

### Constants Defined in Interface

N/A

### Variables Defined in Interface

N/A

### Types Defined in Interface

N/A

### Graphic User Interface (GUI)

N/A

### GUI Classes Defined

N/A

### Current Form

N/A

### Modified Form

N/A

### Components on Form

N/A

### Events

N/A

### Methods

N/A

### Special References

N/A

### Class Events

N/A

### Class Methods

N/A

### Class Properties

N/A

### Uses Clause

N/A

### Form

N/A

### Function

N/A

### Dialog

N/A

### Help Frame

N/A

### Health Level Seven (HL7) Application Parameter

N/A

### Health Level Seven (HL7) Logical Link

N/A

### COTS Interface

N/A

# Attachment A - Approval Signatures

This section is used to document the approval of the Software Design Document during the Formal Review.  The review should be ideally conducted face to face where signatures can be obtained ‘live’ during the review however the following forms of approval are acceptable:

1.  Physical signatures obtained face to face or via fax

2.  Digital signatures tied cryptographically to the signer

3.  /es/ in the signature block provided that a separate digitally signed e-mail indicating the signer’s approval is provided and kept with the document

/es/

Signed:	Date:

Program Manager, Pharmacy Reengineering

< IT Program Manager & Integrated Project Team (IPT) Chair >

/es/

Signed:	Date:

PBM, Director, Clinical Informatics/Reengineering

< Business Sponsor >

/es/

Signed:	Date:

VA Project Manager

< Project Manager >

/es/

Signed:	Date:

ASD/EA Representative

< Technical and Enterprise Architectural Review Team >