EDI New Standards and Operating Rules VHA Provider-side Technical Compliance Requirements (VA118-1001-1018)

eBilling Build 2

Interface Control Document

Patch IB*2*517

X12N 5010 Health Care Services Review – Request for Review and Response (278)

June 2014

Version 2.0

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 5/5/14 | 0.1 | Initial document |  |
| 5/19/14 | 0.2 | Abbreviation updates, Peer review initial findings |  |
| 5/20/14 | 0.3 | Spelling corrections, table format clean-up, Peer Review findings |  |
| 5/23/14 | 1.0 | Initial submission to the VA |  |
| 6/3/14 | 1.1 | Changes requested by VA reviewers |  |
| 6/18/14 | 2.0 | Change request updates, removed NS rows from tables. |  |

Table of Contents

1	Introduction	1

1.1	Project Description	1

1.2	Document Scope	1

1.3	Document Overview	1

1.4	Assumptions	1

1.5	Abbreviations/Acronyms	2

2	Referenced Documents	3

2.1	Governing Standards	3

3	Illustrations	3

3.1	Data Flow Diagram	3

4	Data Exchange Technologies	4

4.1	Messaging Standards	4

4.1.1	HL7 v2.6	4

5	Communications	4

5.1	Mode of Interface	4

5.1.1	Interface Initiation	4

5.2	High-Level Protocols	4

5.3	Low-Level Protocols	5

5.4	Error/Failure Management and Automated Recovery Procedures	5

6	Concept and Sequence of Operations	5

6.1	Identity of Systems	5

6.1.1	Sending Application	5

6.1.2	Receiving Application	5

6.1.3	Third-Party Applications	5

6.2	Data Descriptions	5

7	Message Specification	6

7.1	Conventions	6

7.1.1	HL7 Messages	6

7.1.2	Date/Time of Message Format	6

7.1.3	Message and Segment Boundaries	6

7.1.4	Segment Tables	6

7.1.5	Message Notations	7

7.2	Network Connections	7

7.3	X12 278 Request: Message Type RQA_I08	7

7.3.1	RQA_I08 Request MSH segment (Required)	9

7.3.2	RQA_I08 Request EVN segment (Required)	11

7.3.3	RQA_I08 Request IN1 segment (Required)	12

7.3.4	RQA_I08 Request PRD segment (Required)	14

7.3.5	RQA_I08 Request CTD segment (Optional)	17

7.3.6	RQA_I08 Request PRD segment (Optional)	19

7.3.7	RQA_I08 Request GT1 segment (Required)	20

7.3.8	RQA_I08 Request PID segment (Required)	27

7.3.9	RQA_I08 Request G2R Segment Group – PRB segment (Required)	31

7.3.10	RQA_I08 Request G2R Segment Group – PV1 segment (Optional)	36

7.3.11	RQA_I08 Request AUT segment (Optional)	38

7.3.12	RQA_I08 Request ZTP segment (Optional) (Repeating)	39

7.3.13	RQA_I08 Request DG1 segment (Optional) (Repeating)	41

7.3.14	RQA_I08 Request ZHS segment (Optional)	43

7.3.15	RQA_I08 Request PV1 segment (Optional) (Repeating)	48

7.3.16	RQA_I08 Request OBR segment (Optional)	52

7.3.17	RQA_I08 Request G20 Segment Group – RXA segment (Optional)	54

7.3.18	RQA_I08 Request G20 Segment Group – NTE segment (Optional)(Repeating)	57

7.3.19	RQA_I08 Request RXE segment (Optional)	58

7.3.20	RQA_I08 Request PRB segment (Optional)	63

7.3.21	RQA_I08 Request PSL segment (Optional)(Repeating)	68

7.3.22	RQA_I08 Request NTE segment (Optional)	71

7.3.23	RQA_I08 Request G30 Segment Group (Optional)(Repeating) – PRD segment (Optional)				72

7.3.24	RQA_I08 Request G30 Segment Group (Optional)(Repeating) – AUT segment (Optional)				80

7.3.25	RQA_I08 Request G30 Segment Group (Optional) (Repeating) – ZTP segment (Optional)				82

7.3.26	RQA_I08 Request G50 Segment Group (Optional) (Repeating) – PRB segment (Optional)				83

7.3.27	RQA_I08 Request G50 Segment Group (Optional) (Repeating) – AUT segment (Optional)				86

7.3.28	RQA_I08 Request G50 Segment Group (Optional) (Repeating) – ZTP segment (Optional)				87

7.3.29	RQA_I08 Request G50 Segment Group (Optional) (Repeating) – PSL segment (Optional) (Repeating)	88

7.3.30	RQA_I08 Request G50 Segment Group (Optional) (Repeating) – ZHS segment (Optional)				101

7.3.31	RQA_I08 Request G50 Segment Group (Optional) (Repeating) – PSL segment (Optional) (Repeating)	106

7.3.32	RQA_I08 Request G50 Segment Group (Optional) (Repeating) – NTE segment (Optional)				109

7.3.33	RQA_I08 Request G50 Segment Group (Optional) (Repeating) – PRD segment (Optional) (Repeating)	110

7.4	X12 278 Response: Message Type RPA_I08	117

7.4.1	RPA_I08 Response MSH segment (Required)	119

7.4.2	RPA_I08 Response EVN segment (Required)	122

7.4.3	RPA_I08 Response MSA segment (Optional) (Repeating)	123

7.4.4	RPA_I08 Response IN1 segment (Required)	125

7.4.5	RPA_I08 Response MSA segment (Optional) (Repeating)	130

7.4.6	RPA_I08 Response PRD segment (Required)	133

7.4.7	RPA_I08 Response MSA segment (Optional) (Repeating)	134

7.4.8	RPA_I08 Response PRD segment (Optional)	136

7.4.9	RPA_I08 Response GT1 segment (Required)	137

7.4.10	RPA_I08 Response MSA segment (Optional) (Repeating)	146

7.4.11	RPA_I08 Response PID segment (Optional)	148

7.4.12	RPA_I08 Response MSA segment (Optional) (Repeating)	154

7.4.13	RPA_I08 Response G2R Segment Group – PRB segment (Required)	158

7.4.14	RPA_I08 Response G2R Segment Group – PV1 segment (Optional)	161

7.4.15	RPA_I08 Response AUT segment (Optional) (Repeating)	162

7.4.16	RPA_I08 Response ZTP segment (Optional) (Repeating)	165

7.4.17	RPA_I08 Response DG1 segment (Optional) (Repeating)	167

7.4.18	RPA_I08 Response ZHS segment (Optional)	169

7.4.19	RPA_I08 Response PV1 segment (Optional)	174

7.4.20	RPA_I08 Response OBR segment (Optional)	176

7.4.21	RPA_I08 Response RXA segment (Optional)	178

7.4.22	RPA_I08 Response RXE segment (Optional)	180

7.4.23	RPA_I08 Response PRB segment (Optional)	184

7.4.24	RPA_I08 Response PSL segment (Optional) (Repeating)	187

7.4.25	RPA_I08 Response NTE segment (Optional)	190

7.4.26	RPA_I08 Response G3O Segment Group (optional) (repeating) – PRD segment (Optional)	191

7.4.27	RPA_I08 Response G3O Segment Group (optional) (repeating) – MSA segment (Optional) (Repeating)	197

7.4.28	RPA_I08 Response G3O Segment Group (optional) (repeating) – NK1 segment (Optional)				199

7.4.29	RPA_I08 Response G3O Segment Group (optional) (repeating) – CTD segment (Optional)				206

7.4.30	RPA_I08 Response G3O Segment Group (optional) (repeating) – PRD segment (Optional) (Repeating)	209

7.4.31	RPA_I08 Response G3O Segment Group (optional) (repeating) – MSA segment (Optional) (Repeating)	213

7.4.32	RPA_I08 Response G5O Segment Group (optional) (repeating) – MSA segment (Optional) (Repeating)	215

7.4.33	RPA_I08 Response G5O Segment Group (optional) (repeating) – PRB segment (Optional)				218

7.4.34	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – AUT segment (Optional) (Repeating)	221

7.4.35	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – ZTP segment (Optional) (Repeating)	224

7.4.36	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – DG1 segment (Optional) (Repeating)	225

7.4.37	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – PSL segment (Optional) (Repeating)	226

7.4.38	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – ZHS segment (Optional)	239

7.4.39	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – PSL segment (Optional) (Repeating)	243

7.4.40	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – NTE segment (Optional)	246

7.4.41	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – PRD segment (Optional) (Repeating)	247

7.4.42	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – MSA segment (Optional) (Repeating)	254

7.4.43	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – PRD segment (Optional) (Repeating)	256

7.4.44	RPA_I08 Response G5O Segment Group (Optional) (Repeating) – CTD segment (Optional)	262

# Introduction

This document describes the interface between a VistA system at a VA Medical Center and a Trusted Link Enterprise (TLE) system at the VA Financial Services Center (FSC) in Austin, TX.

## Project Description

The purpose of this Interface Control Document (ICD) is to define the message structure and protocols which govern the interchange of data between the Integrated Billing (IB) module within a VistA system and the VA FSC related to the electronic processing of ASC X12N/005010 Health Care Services Review – Request for Review and Response (278).

## Document Scope

This ICD specifies the interface between VistA IB and FSC. This document provides details on the functional, performance, operational and design requirements for the interface between VistA IB and FSC. This document defines the record layouts for the data that the FSC receives from VistA IB.  This document is intended for all parties requiring such information, including business stakeholders, end-users, software developers, system designers, testers and anyone else responsible for implementing this interface.

## Document Overview

This ICD describes the interface between VistA IB and a Trusted Link Enterprise (TLE) system at FSC.

VistA is health care system installed at VA Medical Centers.  The IB module is software designed to support the creation of health care services claims.  This interface supports the electronic transmission of 278 Health Care Services Review / Request for Review and Response message(s) to Emdeon, the VA’s clearinghouse.  The messages are initialed mapped to an HL7 formated message and transmitted to FSC, where these HL7 messages are translated into ASC X12N/005010 278 Health Care Services Review / Request for Review and Response messages and transmitted to the respective Payer.

## Assumptions

- Communication currently can be established via Transmission Control Protocol / Internet Protocol (TCP/IP) between the VistA application at each VAMC and FSC.

- Both the VistA application and an application at FSC are capable of exchanging TCP/IP messages.

- Both the VistA application and an application at FSC are capable of creating and parsing messages that are based on the HL7 v. 2.6 standard.

- FSC will be able to then parse the HL7 message(s) received from the VistA application at each VAMC and create the appropriate X12N 5010 Health Care Services Review – Request for Review (278) message and submit them to the HCCH.

- FSC will be able to then receive a corresponding X12N 5010 Health Care Services Review – Request for Review (278) Response message and parse that message and build an HL7 RPA_I08 response message and send it to the VAMC who initiated the request.

## Abbreviations/Acronyms

| Acronyms | Description |
| --- | --- |
| ASCII | American Standard Code for Information Interchange |
| AUT | Authorization Information |
| CMS | Center for Medicare and Medicaid Services |
| COTS | Commercial Off-the-Shelf |
| CTD | Contact Data (HL7 Segment) |
| DG1 | Diagnosis (HL7 Segment) |
| EDI | Electronic Data Interchange |
| EDT | Eastern Daylight Time |
| EVN | Event Type (HL7 Segment) |
| FSC | Financial Services Center, Austin, TX |
| GT1 | Guarantor Data (HL7 Segment) |
| HL7 | Health Level Seven |
| HPID | Health Plan Identifier |
| IB | Integrated Billing |
| ICD | Interface Control Document |
| IN1 | Insurance (HL7 segment) |
| MLLP | Minimal Lower-Layer Protocol |
| MSA | Message Acknowledgement (HL7 Segment) |
| MSH | Message Header (HL7 Segment) |
| NTE | Notes and Comments (HL7 Segment) |
| OBR | Observation Request (HL7 Segment) |
| PID | Patient Identification (HL7 Segment) |
| PRB | Problem Details (HL7 Segment) |
| PRD | Provider Data (HL7 Segment) |
| PSL | Product/Service Line Item (HL7 Segment) |
| PV1 | Patient Visit (HL7 Segment) |
| RSD | Requirements Specification Document |
| RPA | Response (HL7 Message Type) |
| RQA | Request (HL7 Message Type) |
| RXA | Pharmacy/Treatment Administration (HL7 Segment) |
| RXE | Pharmacy/Treatment Encoded Order (HL7 Segment) |
| ZHS | Health Care Services Delivery (HSD) (HL7 Segment) |
| ZTP | Subscriber Date (HL7 Segment) |

# Referenced Documents

| Document | Owner | Location |
| --- | --- | --- |
| ASC X12N/005010X217, Based on Version 5, Release 1 ASC X12 Standards for Electronic Data Interchange Technical Report Type 3 Health Care Services Review – Request for Review and Response (278) May 2006 | Washington Publishing Company |  |

## Governing Standards

- ASC X12N/005010X217, Based on Version 5, Release 1.  ASC X12 Standards for Electronic Data Interchange Technical Report Type 3.  Health Care Services Review – Request for Review and Response (278).  May 2006

# Illustrations

## Data Flow Diagram

This diagram illustrates the brand new interface this project will create.

# Data Exchange Technologies

When a VistA user generates a Heath Care Services Request for Review (278) it is sent to FSC using the existing VistA Health Level Seven (HL7) module.  FSC then translates the HL7 RQA_I08 message to an X12 278 Health Care Services Review / Request for Review transaction and sends the message to the HCCH.  The HCCH sends the 278 Request message to the Payer for review and receives the 278 Response message back from the Payer.  The HCCH forwards the X12 278 Response to the FSC, where FSC translates the X12 message into a RPA_I08 HL7 message and sends the HL7 response message to the VAMC who submitted the initial request.

This is a generic implementation of the HL7 protocol for use by all VistA applications. The HL7 protocol provides a single, standard interface between computer systems for the exchange of information.

## Messaging Standards

### HL7 v2.6

HL7 is an all-volunteer organization involved in the development of international healthcare standards. “HL7” is also used to refer to some of the specific standards created by the organization (i.e. HL7 v2.x, v3.0, HL7, etc.).

HL7 and its members provide a framework (and related standards) for the exchange, integration, sharing and retrieval of electronic health information.

# Communications

Included in this section is an introduction of the high-level communication requirements of the applications involved. This introduction discusses the applicable TCP/IP requirements for participating systems.

## Mode of Interface

### Interface Initiation

Connection is established via HL7 logical link to the Financial Services Center:

| VL Trader Domain | IP | Port |
| --- | --- | --- |
| (Test) xxx.fsc.va.gov | TBD | TBD |
| (Production) xxx.fsc.va.gov | TBD | TBD |

## High-Level Protocols

HL7 v 2.6 is the high-level protocol that will be used for this interface.

## Low-Level Protocols

TCP/IP is the low-level protocol that will be used for this interface.

## Error/Failure Management and Automated Recovery Procedures

The VistA HL7 module automatically handles communication errors between VistA and FSC by resending the message if and when it is needed.

# Concept and Sequence of Operations

## Identity of Systems

The implemented interface affects the Integrated Billing module within the VistA system.

### Sending Application

The Integrated Billing module within VistA initiates the X12N 5010 Health Care Services Review / Request for Review (278) query messages sent to FSC.

The TLE at the FSC receives the HL7 278 Request message and translates the message into the proper X12 format and forwards the message on to the HCCH.

The X12N 5010 Health Care Services Review / Request for Review (278) response message is sent back to FSC, FSC translates the X12 formatted message back to HL7 and returns the HL7 formatted Response message to the originating VAMC.

### Receiving Application

The Integrated Billing module within VistA receives the X12N 5010 Health Care Service Review / Request for Review (278) response messages returned by the FSC.

### Third-Party Applications

The X12N 5010 Health Care Services Review / Request for Review (278) Request and Response messages utilize the existing HL7 interface to the FSC by using the Vitria interface engine.  This is a Commercial Off-the-shelf (COTS) product suite that handles communication between the VAMCs and FSC (Vitria Corporate Headquarters – 945 Stewart Drive, Sunnyvale, CA 94085).

## Data Descriptions

Vista sends 278 Health Care Services Review / Request for  Review data to the FSC where the data is translated to HIPAA compliant format.  VistA receives 278 Health Care Services Review / Response data from the FSC.

The messages include the following type of data:

- UMO Data

- Requester Data

- Subscriber Data

- Dependent Data

- Patient Event and Patient Event Providers Data

- Services and Services Providers Data

# Message Specification

## Conventions

### HL7 Messages

HL7 messages shall follow the standard HL7 version 2.6 format as noted in the document ‘An Application Protocol for Electronic Data Exchange in Healthcare Environments’.  The Z segments included in the message are based on the standard HL7 version 2.4 library of Data Types.

### Date/Time of Message Format

- The format for MSH-7, Date/Time of Message, is YYYYMMDDHHMMSSZZZZZ where:

  - YYYY is the 4-digit year, e.g. “2002”.

  - MM is the month number, ranging from “01” to “12”.

  - DD is the day number within the month, ranging from “01” to “31”.

  - HH is the hour from “00” to “23”.

  - MM is the minute from “00” to “59”.

  - SS is the second from “00” to “59”.

  - ZZZZZ represents the time zone as a leading signed value time offset from Greenwich Mean Time of the form HHMM using the same convention for HH and MM as given above. For example, Eastern Daylight Time (EDT) is given as “–0400”.

### Message and Segment Boundaries

Message starts with American Standard Code for Information Interchange (ASCII) character 1110, vertical tab (VT).

Segment terminates with ASCII character 1310, carriage return (CR).

Message terminates with ASCII character 2810, file separator (FS), followed by ASCII character 1310, carriage return (CR).

The prototypical form is:

- <VT>segment<CR>segment<CR><FS><CR>

<VT> as the prefix and <FS><CR> as the suffix are part of the HL7 Minimal Lower-Layer Protocol, (MLLP).

### Segment Tables

For the Segment tables that follow in section 7.3, the following statements provide some additional information for the meaning of the data found in each column.

The Seq column contains the sequence number of the element.

The Name column gives the HL7 name for the element or sub-element.

Following the element name, data type information drawn from the HL7 2.6 standard may appear in the DT column.

The Len column contains the length of the element.

In the Opt column, value “R” indicates that the element requires a value, so the sender must place a non-empty value into this HL7 element. The value “O” marks those elements in which a value is optional. The value of optional elements may be empty. “RE” indicates conditional use and “NS” means not supported.

The Rep column contains “TRUE” if the element is repeatable and “FALSE” (or blank) if it isn’t.

The Example Value column contains an example value for this element.

The VistA Input/Storage column (TDMD) indicates where in the VistA database the value is stored and/or retrieved.

### Message Notations

A message notation presents the segment IDs in the correct order. Curly brackets ( { } ) surround one or more segments that are to be repeated one or more times. Square brackets ( [ ] ) surround optional segments. Using both kinds of brackets declares that the enclosed segments may appear zero or more times.

## Network Connections

VistA connects to the designated TCP port and IP domain, listed in section 5.1.1, using TCP/IP to send HL7 messages to the FSC.

The FSC sends HL7 messages to the TCP port xxxx on the destination VistA system using TCP/IP.

## X12 278 Request: Message Type RQA_I08

The RQA-I08 Message Type query is constructed of the following HL7 segments in the order listed:

- MSH – Message Header: Information for X12 BHT Header segment

- EVN – Event Type: Information for X12 BHT Header segment

- IN1 – Insurance: Information for X12 NM1 UMO Name segment

- PRD – Provider Data: Information for X12 NM1 Requester Name segment, N3 Requester Address Segment, N4 Requester City, State, Zip

Code segment

- [CTD] – Contact Data: Information for X12 PER Requester Contact Information segment

- [PRD] – Provider Data: Information for X12 PRV Requester Provider Information segment

- GT1 – Guarantor: Information for X12 NM1 Subscriber segment, REF Subscriber segment, N3 Subscriber segment N4 Subscriber City,

State, Zip Code segment, DMG Subscriber Demographic segment, INS Subscriber segment

- PID – Patient Identification: Information for X12 NM1 Dependent Name segment, REF Dependent Supplemental Identification segment, N3

Dependent Address segment, N4 Dependent City, State, Zip Code segment, DMG Dependent Demographic Information segment,

INS Dependent Relationship segment

- [(G2R segment group)] –

  - PRB – Problem Details: Information for X12 UM Health Care Services Review Information segment

  - [PV1] – Patient Visit: Information for X12 UM Health Care Services Review Information segment

- [AUT] – Authorization Information: Information for X12 REF Previous Review Authorization Number segment

- {[ZTP]} – Subscriber Date: Information for X12 DTP Accident Date segment, DTP Last Menstrual Period Date segment, DTP Estimated Date

of Birth segment, DTP Onset of Current Symptoms or Illness Date segment, DTP Event Date segment, DTP Admission Date

segment, DTP Discharge Date segment

- {[DG1]} – Diagnosis: Information for X12 HI Patient Diagnosis segment

- [ZHS] - Health Care Services Delivery (HSD): Information for X12 HSD Health Care Services Delivery segment

- {[PV1]} – Patient Visit: Information for X12 CRC Certification Information segment, CL1 Institutional Claim Code segment,

- [OBR] – Observation Request: Information for X12 CR1 Ambulance Transport segment

- [(G20 segment group)] –

  - [RXA] - Pharmacy/Treatment Administration: Information for X12 CR2 Spinal Manipulation Service Information segment

  - {[NTE]} – Notes and Comments: Information for X12 CR2 Spinal Manipulation Service Information segment

- [RXE] – Pharmacy/Treatment Encoded Order: Information for X12 CR5 Home Oxygen Therapy segment

- [PRB] – Problem Details: Information for X12 CR6 Home Health Care Information segment

- {[PSL]} – Product/Service Line Item: Information for X12 PWK Additional Patient Information segment

- [NTE] – Notes and Comments: Information for X12 MSG Message Text segment

- {[G30 segment group]} –

  - [PRD] – Provider Data: Information for X12 NM1 Patient Event Provider Name segment, N3 Patient Event Provider Address

segment, N4 Patient Event Provider City, State, Zip Code segment. PRV Patient Event Provider Information segment, NM1

Patient Event Transport Information segment, N3 Patient Event Transport Address segment, N4 Patient Event Transport

City/State/Zip Code segment, NM1 Patient Event Other UMO Name segment

  - [AUT] – Authorization Information: Information for X12 REF Other UMO Denial Reason segment

  - [ZTP] – Subscriber Date: Information for X12 DTP Other UMO Denial Date segment

- {[G50 segment group]} –

  - [PRB] – Problem Details: Information for X12 UM Health Care Services Review Information segment

  - [AUT] – Authorization Information: Information for X12 REF Previous Review Authorization Number segment, REF Administrative

Reference Number segment

  - [ZTP] – Subscriber Date: Information for X12 DTP Service Date segment

  - {[PSL]} – Product/Service Line Item: Information for X12 SV1 Professional Service segment, SV2 Institutional Service Line segment,

SV3 Dental Service segment, TOO Tooth Information segment

  - [ZHS] – Health Care Services Delivery (HSD): Information for X12 HSD Health Care Service Delivery segment

  - {[PSL]} – Product/Service Line Item: Information for X12 PWK Additional Service Information segment

  - [NTE] – Notes and Comments: Information for X12 MSG Message Text segment

  - {[PRD]} – Provider Data: Information for X12 NM1 Service Provider Name segment, N3 Service Provider Address segment, N4

Service Provider City, State, Zip Code segment, PRV Service Provider Information segment

The notation is:

MSH,EVN,IN1,PRD,[CTD],[PRD],GT1,PID,[(G2R)PRB,[PV1]][AUT],{[ZTP]},{[DG1]},[ZHS],{[PV1]},[OBR],[(G2O)[RXA],{[NTE]}][RXE],[PRB],{[PSL]},[NTE],{[(G3O)[PRD],[AUT],[ZTP]]}{[(G5O)[PRB],[AUT],[ZTP],{[PSL]},[ZHS],{[PSL]},[NTE],{[PRD]}]}

### RQA_I08 Request MSH segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | Vista Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Field Separator | 1 | ST | 1 |  | FALSE | ¦ |  |  |
| Encoding Characters | 2 | ST | 4 | R | FALSE | ^~\& |  |  |
| Sending Application | 3 | HD | 180 | R | FALSE |  |  |  |
| Namespace ID | 3.1 | IS | 10 | R |  | VISTA 278 |  |  |
| Sending Facility | 4 | HD | 180 | R | FALSE |  |  |  |
| Namespace ID | 4.1 | IS | 3 | R |  | 509 | File: 869.3 (HL COMMUNICATION SERVER PARAMETERS) Field: .04 (INSTITUTION), File: 4 (INSTITUTION) Field: .01 (NAME) |  |
| Universal ID | 4.2 | ST | 30 | R |  | AUGUSTA.MED.VA.GOV | File 869.3 (HL COMMUNICATION SERVER PARAMETERS) Field: .02 (DOMAIN) |  |
| Universal ID Type | 4.3 | ID | 3 | R |  | DNS |  |  |
| Receiving Application | 5 | HD | 180 | R | FALSE |  |  |  |
| Namespace ID | 5.1 | IS | 3 | R |  | FSC |  |  |
| Receiving Facility | 6 | HD | 180 | R | FALSE |  |  |  |
| Universal ID | 6.2 | ST | 60 | R |  | INV.VITRIA-EDI.AAC.VA.GOV | File: 870 (HL LOGICAL LINK) Field: .02 (INSTITUTION) |  |
| Universal ID Type | 6.3 | ID | 3 | R |  | DNS |  |  |
| Date/Time of Message | 7 | DTM | 24 | R | FALSE | 20040328134602.1234+0600 | File: 356.22 (HCS REVIEW TRANSMISSION) Field: .1 (DATE MARKED ‘IN PROGRESS’) | BHT04 and BHT05 Transaction Set Creation Date and Time |
| Implementation Note: |  |  |  |  |  |  |  |  |
| YYYYMMDD[HHHMM[SS[.SSSS]]][+-ZZZZ] |  |  |  |  |  |  |  |  |
| Message Type | 9 | MSG | 15 | R | FALSE |  |  |  |
| Message Code | 9.1 | ID | 3 | R |  | RQA |  |  |
| Trigger Event | 9.2 | ID | 3 | R |  | I08 |  |  |
| Message Control ID | 10 | ST | 50 | R | FALSE | 509772 | File: 356.22 (HCS REVIEW TRANSMISSION) Field: .12 (MESSAGE CONTROL ID) | BHT03 Submitter Transaction ID |
| Implementation Note: |  |  |  |  |  |  |  |  |
| VistA application will set this value per each 278 request transaction |  |  |  |  |  |  |  |  |
| Processing ID | 11 | PT | 3 | R | FALSE |  |  |  |
| Processing ID | 11.1 | ID | 3 | R |  | T for Test P for Production |  |  |
| Version ID | 12 | VID | 60 | R | FALSE |  |  |  |
| Version ID | 12.1 | ID | 5 | R |  | 2.6 |  |  |
| Accept Acknowledgment Type | 15 | ID | 2 | R | FALSE | AL |  |  |
| Application Acknowledgment Type | 16 | ID | 2 | R | FALSE | NE |  |  |
| Country Code | 17 | ID | 3 | R | FALSE | USA |  |  |

### RQA_I08 Request EVN segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | Vista Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Recorded Date/Time | 2 | DTM | 24 | R | FALSE | 20140424 | Date of message creation |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not required for this X12 278 Request Message, However, it is required by HL7, so VistA will populate this field with the Date/Time of message creation |  |  |  |  |  |  |  |  |
| Event Reason Code | 4 | IS | 3 | R | FALSE | 13 | When user generates the Request message, this value will be selected by the user. Not stored in the database. | BHT02 Transaction Set Purpose Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| '01' for Cancellation, |  |  |  |  |  |  |  |  |
| '13' for Request, '36' for Authority to Deduct |  |  |  |  |  |  |  |  |

### RQA_I08 Request IN1 segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Val | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID - IN1 | 1 | SI | 4 | R | FALSE | 1 | Fixed value |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Not required for X12 message, however it is required by HL7, therefore, VistA application will set this value equal to ‘1’ |  |  |  |  |  |  |  |  |
| Insurance Plan ID | 2 | CWE | 250 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 7 | R |  | PLAN ID | Fixed value |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message; however, it is a required field for this HL7 segment. Therefore, set as fixed value 'PLAN ID' |  |  |  |  |  |  |  |  |
| Insurance Company ID | 3 | CX | 250 | R | TRUE |  |  |  |
| ID Number | 3.1 | ST | 3 | R |  | VA1 | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Field: 2.312 (INSURANCE TYPE), File: 36 (PAYER) Field: 3.1 (PAYER), File: 365.12 (PAYER) Field: Either .02 (VA NATIONAL ID) or .03 (CMS NATIONAL ID) | Loop 2010A: NM109 UMO (Utilization Management Org) ID |
| Identifier Type Code | 3.5 | ID | 3 | R |  | PI | VistA application will load this value. 'PI' for Payer Identification or 'XV' for CMS Plan ID | Loop 2010A: NM108 Identification Code Qualifier |
| Insurance Company Name | 4 | XON | 250 | RE | TRUE |  |  |  |
| Organization Name | 4.1 | ST | 60 | RE |  | INSURANCE NAME | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY) File: 2 (PATIENT) Field: 2.312 (INSURANCE TYPE), File: 36 (INSURANCE COMPANY) Field: .01 (NAME) | Loop 2010A: NM103 UMO Last or Organization Name |
| Release Information Code | 27 | IS | 1 | RE | FALSE | Y | File: 356.22 (278) Field: 2.16 (RELEASE OF INFORMATION CODE) | UM09 Release of Information Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| VistA application will populate this field value with one of the following: |  |  |  |  |  |  |  |  |
| 'M' (limited or restricted), |  |  |  |  |  |  |  |  |
| 'Y' (provider has signed statement to permit release of medical billing data) |  |  |  |  |  |  |  |  |

### RQA_I08 Request PRD segment (Required)

| Name |  | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role |  | 1 | CWE | 250 | R | TRUE |  |  |  |
| Alternate Identifier |  | 1.4 | ST | 10 | R |  | NM1 2010B | Fixed: 'NM1 2010B' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA application will set this field equal to 'NM1 2010B' to indicate this is for X12 segment NM1 in loop 2010B |  |  |  |  |  |  |  |  |  |
| Provider Name | 2 |  | XPN | 250 | RE | TRUE |  |  |  |
| Family Name | 2.1 |  | FN | 60 | R |  |  |  |  |
| � Surname | 2.1.1 |  | ST | 60 | R |  | VAMC NAME | File: 4 (INSTITUTION) Field: 100 (OFFICIAL VA NAME) | Loop 2010B: NM103 Requester Last or Organization Name |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA application will set this field value equal to the VAMC facility name |  |  |  |  |  |  |  |  |  |
| Provider Address | 3 |  | XAD | 292 | RE | TRUE |  |  |  |
| Street Address | 3.1 |  | SAD | 184 | RE |  |  |  |  |
| Street or Mailing Address | 3.1.1 |  | ST | 55 | RE |  | STREET ADDRESS LINE 1 | File: 4 (INSTITUTION) Field: 4.01 (STREET ADDR. 1 (MAILING)) | Loop 2010B: N301 Requester Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Requester Address Line 1 information |  |  |  |  |  |  |  |  |  |
| Other Designation | 3.2 |  | ST | 55 | RE |  | SUITE 200 | File: 4 (INSTITUTION) Field: 4.02 (STREET ADDR. 2 (MAILING)) | Loop 2010B: N302 Requester Address Line (2) |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA application will populate this field with Street Address 2 from Institution file for VAMC |  |  |  |  |  |  |  |  |  |
| City | 3.3 |  | ST | 30 | RE |  | CITY | File: 4 (INSTITUTION) Field: 4.03 (CITY (MAILING)) | Loop 2010B: N401 Requester City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA application will populate this field with the Requester City from the Institution (4) file, field City (4.03) |  |  |  |  |  |  |  |  |  |
| State or Province | 3.4 |  | ST | 2 | RE |  | VA | File: 4 (INSTITUTION) Field: 4.04 (STATE (MAILING)) | Loop 2010B: N402 Requester State Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA application will populate this field with the Requester State or Province code from the Institution File (4), field State (4.04) |  |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 3.5 |  | ST | 15 | RE |  | 12345 | File: 4 (INSTITUTION) Field: 4.05 (ZIP (MAILING)) | Loop 2010B: N403 Requester Postal Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA application will populate this field with the Requester Zip Code from the Institution File (4) field Zip (4.05) |  |  |  |  |  |  |  |  |  |
| Country | 3.6 |  | ID | 3 | RE |  | USA | Not available in the Vista database for file 4 (INSTITUTION) | Loop 2010B: N404 Requester Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA application will populate this field with the Requester Country Code |  |  |  |  |  |  |  |  |  |
| Other Geographic Designation |  | 3.8 | ST | 3 | RE |  | SUB | Not available in the Vista database for file 4 (INSTITUTION) | Loop 2010B: N407 Requester Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA will populate this field with the Request Country Subdivision Code if the requester is not in the US |  |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 |  | PLN | 100 | R | TRUE |  |  |  |
| ID Number | 7.1 |  | ST | 100 | R |  | 4380000016 | File: 4 (INSTITUTION) Field: 41.99 (NPI) | Loop 2010B: NM109 Requester ID |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| The VistA application will populate this field value with the VAMC's NPI value |  |  |  |  |  |  |  |  |  |

### RQA_I08 Request CTD segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Role | 1 | CWE | 10 | R | FALSE |  |  |  |
| Identifier | 1.1 | ST | 10 | R |  | PER 2010B | Fixed value: 'PER 2010B' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| VistA application will set this field equal to 'PER 2010B' to indicate this is for X12 segment PER in loop 2010B |  |  |  |  |  |  |  |  |
| Contact Name | 2 | XPN | 240 | R | TRUE |  |  |  |
| Family Name | 2.1 | FN | 194 | R |  |  |  |  |
| � Surname | 2.1.1 | ST | 60 | R |  | Doe, Jane RN | File: 356.22 (278) Field: .11 (REQUESTED BY), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010B: PER02 Requester Contact Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Requester Contact name, which will usually be the nurse initiating the 278 request |  |  |  |  |  |  |  |  |
| Contact Communication Information | 5 | XTN | 260 | RE | TRUE |  |  |  |
| Telecommunication Equipment Type | 5.2 | ID | 2 | RE |  | PH | File: 356.22 (278) Field: .11 (REQUESTED BY), File: 200 (NEW PERSON) Field: .132 (OFFICE PHONE), Field: .136 (FAX NUMBER), Field: .151 (EMAIL ADDRESS) | Loop 2010B: PER03 Requester Number Qualifier, PER05 Communication Number Qualifier, PER07 Communication Number Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field value with the Requester Contact Qualifier. One of the following: |  |  |  |  |  |  |  |  |
| 'EM' (Electronic Mail), |  |  |  |  |  |  |  |  |
| 'EX' (Telephone Extension), |  |  |  |  |  |  |  |  |
| 'FX' (Facsimile), |  |  |  |  |  |  |  |  |
| 'TE' (Telephone), |  |  |  |  |  |  |  |  |
| 'UR' (URL) |  |  |  |  |  |  |  |  |
| A mapping will be required because this field is linked to an HL7 table. |  |  |  |  |  |  |  |  |
| 'EM' (Electronic Mail) ==> X.400 |  |  |  |  |  |  |  |  |
| 'EX' (Telephone Extension), ==> MD |  |  |  |  |  |  |  |  |
| 'FX' (Facsimile), ==> FX |  |  |  |  |  |  |  |  |
| 'TE' (Telephone), ==> PH |  |  |  |  |  |  |  |  |
| 'UR' (URL) ==> Internet |  |  |  |  |  |  |  |  |
| Any Text | 5.8 | ST | 256 | RE |  | (999) 123-1234 | File: 356.22 (278) Field: .11 (REQUESTED BY), File: 200 (NEW PERSON) Field: .132 (OFFICE PHONE), Field: .136 (FAX NUMBER), Field: .151 (EMAIL ADDRESS) | Loop 2010B: PER04 Requester Contact Communication Number, PER06 Requester Contact Communication Number, PER08 Requester Contact Communication Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field may be populated with a telephone number, Email address, Facsimile number, phone extension number, URL |  |  |  |  |  |  |  |  |

### RQA_I08 Request PRD segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role | 1 | CWE | 250 | R | FALSE |  |  |  |
| Alternate Identifier | 1.4 | ST | 10 | R |  | PRV 2010B | Fixed value 'PRV 2010B' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| VistA application will set this field equal to ‘PRV 2010B’ to indicate this data is for X12 segment PRV in loop 2010B |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 | PLN | 100 | RE | FALSE |  |  |  |
| ID Number | 7.1 | ST | 50 | R |  | 31231323 | File: 4 (INSTITUTION) Field: 4.043 (TAXONOMY CODE) | Loop 2010B: PRV03 Provider Taxonomy Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Provider Taxonomy Code |  |  |  |  |  |  |  |  |

### RQA_I08 Request GT1 segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID - GT1 | 1 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 837 message. Since it is required, the VistA application will set this field value equal to ‘1’ |  |  |  |  |  |  |  |  |
| Guarantor Number | 2 | CX | 250 | RE | TRUE |  |  |  |
| ID Number | 2.1 | ST | 80 | R |  | CU7698F | File: 356.22 (278) Field: 02 (PATIENT), File 2 (PATIENT) Field: SUBSCRIBER ID; (2nd field value occurrence) FIle: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Sub File .3121 (INSURANCE TYPE) Field: 2 (GROUP NUMBER) or File 2 (PATIENT) Field .09 (SOCIAL SECURITY NUMBER) | Loop 2010C: NM109 Subscriber Primary ID and REF02 Subscriber Supplemental ID |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will set this field value equal to the Subscriber Primary ID from the PATIENT file (2), sub file 2.3121. The 2nd occurrence of this field will be the Subscriber Supplement ID |  |  |  |  |  |  |  |  |
| Identifier Type Code | 2.5 | ID | 3 | RE |  | XX | File: 356.22 (278) Field: .02 (PATIENT), File 2 (PATIENT) Sub File: 2.312 (INSURANCE TYPE) Field: 2 (GROUP NUMBER) or Field .09 (SOCIAL SECURITY NUMBER) | Loop 2010C: REF01 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| VistA application will populate this field (the 2nd repeating occurrence) with the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
| '6P' (Group Number) |  |  |  |  |  |  |  |  |
| 'SY' (SSN) |  |  |  |  |  |  |  |  |
| This field has an associated HL7 table, so a mapping is required. |  |  |  |  |  |  |  |  |
| '6P' ==> 'XX' |  |  |  |  |  |  |  |  |
| 'SY' ==> 'SS' |  |  |  |  |  |  |  |  |
| Guarantor Name | 3 | XPN | 274 | R | FALSE |  |  |  |
| Family Name | 3.1 | FN | 194 | R |  |  |  |  |
| Surname | 3.1.1 | ST | 60 | R |  | SMITH | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: 17 (NAME OF INSURED) | Loop 2010C: NM103 Subscriber Last Name |
| Given Name | 3.2 | ST | 35 | RE |  | JOHN | File: 356.22 (278) Field: .02 (PATIENT), File 2 (PATIENT) Field 17 (NAME OF INSURED) | Loop 2010C: NM104 Subscriber First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber First Name from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 3.3 | ST | 25 | RE |  | W | File: 356.22 (278) Field: .02 (PATIENT), FIle: 2 (PATIENT) Field 17 (NAME OF INSURED) | Loop 2010C: NM105 Subscriber Middle Name or Initial |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber Middle Name or Initial from the PATIENT (2) file. |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) | 3.4 | ST | 10 | RE |  | JR | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: 17 (NAME OF INSURED) | Loop 2010C: NM107 Subscriber Name Suffix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber Name Suffix from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| Prefix (e.g., DR) | 3.5 | ST | 10 | RE |  | DR | File: 356.22 (278) Field: .02 (PATIENT), File 2 (PATIENT) Field: 17 (NAME OF INSURED) | Loop 2010C: NM106 Subscriber Name Prefix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber Name Prefix from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| Guarantor Address | 5 | XAD | 292 | RE | TRUE |  |  |  |
| Street Address | 5.1 | SAD | 184 | RE |  |  |  |  |
| Street or Mailing Address | 5.1.1 | ST | 55 | RE |  | 123 MAIN STREET | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), FIle: 2 (PATIENT) Subfile 2.312 (INSURANCE POLICY) Field: 3.06 (INSURED’S STREET 1) | Loop 2010C: N301 Subscriber Address Line |
| Other Designation | 5.2 | ST | 55 | RE |  | SUITE 200 | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Subfile 2.312 (INSURANCE POLICY) Field: 3.07 (INSURED’S STREET 2) | Loop 2010C: N302 Subscriber Address Line |
| City | 5.3 | ST | 30 | RE |  | CITY | File 356.22 (278) Field .02 (PATIENT) and .03 (INSURANCE POLICY), File 2 (PATIENT) Subfile 2.312 (INSURANCE TYPE) Field: 3.08 (INSURED’S CITY) | Loop 2010C: N401 Subscriber City Name |
| State or Province | 5.4 | ST | 2 | RE |  | VA | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Subfile 2.312 (INSURANCE TYPE) Field: 3.09 (INSURED’S STATE) | Loop 2010C: N402 Subscriber State Code |
| Zip or Postal Code | 5.5 | ST | 15 | RE |  | 23433 | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Subfile 2.312 (INSURANCE TYPE) Field: 3.1 (INSURED’S ZIP) | Loop 2010C: N403 Subscriber Zip Code |
| Country | 5.6 | ID | 3 | RE |  | ALB | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Subfile 2.312 (INSURANCE TYPE) File: 3.13 (INSURED’S COUNTRY) | Loop 2010C: N404 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber Country Code, if not USA |  |  |  |  |  |  |  |  |
| Other Geographic Designation | 5.8 | ST | 3 | RE |  | 312 | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Subfile 2.312 (INSURANCE TYPE) Field: 3.14 (INSURED’S COUNTRY SUBDIVISION) | Loop 2010C: N407 Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber Country Subdivision Code, if the Subscriber's address is outside the US and there are Subdivision Codes |  |  |  |  |  |  |  |  |
| Guarantor Date/Time Of Birth | 8 | DTM | 8 | RE | FALSE | 19451212 | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Subfile 2.312 (INSURANCE TYPE) Field: 3.01 (Insured's DOB) | Loop 2010C: DMG02 Subscriber Birth Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| YYYYMMDD - this field will be populated with the Subscriber's Birth Date from the PATIENT (2) file. |  |  |  |  |  |  |  |  |
| Guarantor Administrative Sex | 9 | IS | 1 | RE | FALSE | M | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Subfile: 2.312 (INSURANCE TYPE) Field: 3.12 (INSURED’S SEX) | Loop 2010C: DMG03 Subscriber Gender Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber Gender Code |  |  |  |  |  |  |  |  |
| 'F' (Female) |  |  |  |  |  |  |  |  |
| 'M' (Male) |  |  |  |  |  |  |  |  |
| 'U' (Unknown) |  |  |  |  |  |  |  |  |
| Guarantor Employment Status | 20 | IS | 2 | RE | FALSE | AU |  | Loop 2010C: INS08 Employment Status Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber's Employment Status Code to indicate military status |  |  |  |  |  |  |  |  |
| 'AO' (Active Military Overseas) |  |  |  |  |  |  |  |  |
| 'AU' (Active Military USA) |  |  |  |  |  |  |  |  |
| 'DI' (Deceased) |  |  |  |  |  |  |  |  |
| 'PV' (Previous) |  |  |  |  |  |  |  |  |
| 'RU' (Retired Military USA) |  |  |  |  |  |  |  |  |
| Contact Relationship | 48 | IS | 2 | RE | FALSE | 1 | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Subfile: 2.312 (INSURANCE TYPE) Field: 16 (PT.RELATIONSHIP TO INSURED) | Loop 2010D: INS02 Relationship to Insured Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Dependent Relationship to Insured Code |  |  |  |  |  |  |  |  |
| '01' (Spouse) |  |  |  |  |  |  |  |  |
| '19' (Child) |  |  |  |  |  |  |  |  |
| 'G8' (Other Relationship) |  |  |  |  |  |  |  |  |

### RQA_I08 Request PID segment (Required)

| Name |  | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Patient Identifier List |  | 3 | CX | 250 | R | TRUE |  |  |  |
| ID Number |  | 3.1 | ST | 50 | R |  | ACCT NUMBER | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Subfile 2..312 (INSURANCE TYPE) Field: 2 (GROUP NUMBER) or File: 2 (PATIENT) Field: .09 (Social Security Number) | Loop 2010D: REF02 Dependent Supplemental Identifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Dependent Supplemental Identifier, either the Patient Account Number or the Patient SSN |  |  |  |  |  |  |  |  |  |
| Assigning Authority |  | 3.4 | HD | 3 | RE |  |  |  |  |
| Namespace ID |  | 3.4.1 | IS | 3 | RE |  | EJ | FIle: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Subfile: 2.312 (INSURANCE TYPE) Field: 2 (GROUP NUMBER) or File: 2 (PATIENT) Field: .09 (SOCIAL SECURITY NUMBER) | Loop 2010D: REF01 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Dependent Supplemental Identification Qualifier, which will identify the value in PID.3.1 |  |  |  |  |  |  |  |  |  |
| 'EJ' (Patient Account Number) |  |  |  |  |  |  |  |  |  |
| 'SY' (SSN) |  |  |  |  |  |  |  |  |  |
| Patient Name |  | 5 | XPN | 264 | R | FALSE |  |  |  |
| Family Name |  | 5.1 | FN | 194 | R |  |  |  |  |
| Surname |  | 5.1.1 | ST | 60 | R |  | SMITH | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .01 (NAME) | Loop 2010D: NM103 Dependent Last Name |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| If the Patient is Not the Subscriber, the Patient's Last Name will be placed into this field from the PATIENT (2) file. |  |  |  |  |  |  |  |  |  |
| Given Name |  | 5.2 | ST | 35 | RE |  | JOHN | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .01 (NAME) | Loop 2010D: NM104 Dependent First Name |
| Second and Further Given Names or Initials Thereof |  | 5.3 | ST | 25 | RE |  | W | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .01 (NAME) | Loop 2010D: NM105 Dependent Middle Name |
| Suffix (e.g., JR or III) |  | 5.4 | ST | 10 | RE |  | JR | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .01 (NAME) | Loop 2010D: NM107 Dependent Name Suffix |
| Date/Time of Birth |  | 7 | DTM | 8 | RE | FALSE | 20040508 | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .03 (DATE OF BIRTH) | Loop 2010D: DMG02 Dependent Birth Date |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| YYYYMMDD - If the Patient is not the Subscriber, the Patient's Birth Date will be placed in this field, from the PATIENT (2) File |  |  |  |  |  |  |  |  |  |
| Administrative Sex | 8 |  | IS | 1 | RE | FALSE | F | File: 356.22 (278) Field: .02 (PATIENT), File 2 (PATIENT) Field: .02 (SEX) | Loop 2010D: DMG03 Dependent Gender Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Dependent Gender Code |  |  |  |  |  |  |  |  |  |
| 'F' (Female) |  |  |  |  |  |  |  |  |  |
| 'M' (Male) |  |  |  |  |  |  |  |  |  |
| 'U' (Unknown) |  |  |  |  |  |  |  |  |  |
| Patient Address | 11 |  | XAD | 292 | RE | FALSE |  |  |  |
| Street Address | 11.1 |  | SAD | 184 | RE |  |  |  |  |
| Street or Mailing Address | 11.1.1 |  | ST | 55 | RE |  | 123 MAIN STREET | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .111 (STREET ADDRESS [LINE 1]) | Loop 2010D: N301 Dependent Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, this field will be populated with the Dependent Address Line 1 from the PATIENT (2) file |  |  |  |  |  |  |  |  |  |
| Other Designation |  | 11.2 | ST | 55 | RE |  | Apt 5 | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .112 (STREET ADDRESS [LINE 2]) | Loop 2010D: N302 Dependent Address Line (2) |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, this field will be populated with the Dependent Address Line 2 from the PATIENT (2) file, if one exists |  |  |  |  |  |  |  |  |  |
| City |  | 11.3 | ST | 30 | RE |  | RESTON | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .114 (CITY) | Loop 2010D: N401 Dependent City Name |
| State or Province |  | 11.4 | ST | 2 | RE |  | VA | File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .115 (STATE) | Loop 2010D: N402 Dependent State |
| Zip or Postal Code |  | 11.5 | ST | 15 | RE |  | 21121 | File 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .116 (ZIP CODE) | Loop 2010D: N403 Dependent Zip Code |
| Country |  | 11.6 | ID | 3 | RE |  | ALB | FIle: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Field: .1173 (COUNTRY) | Loop 2010D: N404 Dependent Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber and the Patient address is outside the US, this field will contain the country code for the Dependent Country Code |  |  |  |  |  |  |  |  |  |
| Other Geographic Designation |  | 11.8 | ST | 3 | RE |  | DIF | Not available in the Vista database for file 2 (PATIENT) | Loop 2010D: N407 Dependent Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber and the Patient address is outside the US and they have Subdivision Codes, this field will contain the Dependent Country Subdivision Code |  |  |  |  |  |  |  |  |  |
| Birth Order |  | 25 | NM | 9 | RE | FALSE | 1 |  | Loop 2010D: INS17 Birth Sequence Number |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber and there is a need to report Birth Order, this field will contain the Birth Sequence Number of the Patient |  |  |  |  |  |  |  |  |  |
| Field length has been expanded to 9 characters to allow for X12 value |  |  |  |  |  |  |  |  |  |

### RQA_I08 Request G2R Segment Group – PRB segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action Code | 1 | ID | 2 | R | FALSE | CO |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application is not using this field for this X12 message, however, it is required so it will be set equal to 'CO' |  |  |  |  |  |  |  |  |
| Action Date/Time | 2 | DTM | 8 | R | FALSE | 20040328 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| YYYYMMDD - The VistA application is not using this field for this X12 message, however, this field is required, so it will be set equal to the date of the request |  |  |  |  |  |  |  |  |
| Problem ID | 3 | CWE | 250 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 1 | R |  | 1 | File: 356.22 (278) Field: 2.02 (CERTIFICATION TYPE CODE) | Loop 2000E: UM02 Certification Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will set this value equal to the Certification Type Code value |  |  |  |  |  |  |  |  |
| '1' (appeal - immediate), |  |  |  |  |  |  |  |  |
| '2' (appeal - standard), |  |  |  |  |  |  |  |  |
| '3' (cancel), |  |  |  |  |  |  |  |  |
| '4' (extension), |  |  |  |  |  |  |  |  |
| 'I' (initial), |  |  |  |  |  |  |  |  |
| 'N' (reconsideration), |  |  |  |  |  |  |  |  |
| 'R' (renewal), |  |  |  |  |  |  |  |  |
| 'S' (revised) |  |  |  |  |  |  |  |  |
| Problem Instance ID | 4 | EI | 60 | R | FALSE |  |  |  |
| Entity Identifier | 4.1 | ST | 3 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application is not using this field for this X12 message, however, it is required, so it will be set equal to 1 |  |  |  |  |  |  |  |  |
| Episode of Care ID | 5 | EI | 60 | R | FALSE |  |  |  |
| Entity Identifier | 5.1 | ST | 10 | R |  | UM 2000E |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will set this field value equal to 'UM 2000E' to indicate to FSC that this data is for X12 segment UM, loop 2000E |  |  |  |  |  |  |  |  |
| Problem Classification | 10 | CWE | 250 | RE | FALSE |  |  |  |
| Identifier | 10.1 | ST | 3 | RE |  | AA | File: 356.22 (278) Field: 2.08 (RELATED CAUSES #1) | Loop 2000E: UM05-1 Related Causes Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to the Related Causes Code, if applicable |  |  |  |  |  |  |  |  |
| 'AA' (auto accident), |  |  |  |  |  |  |  |  |
| 'AP' (another party responsible), |  |  |  |  |  |  |  |  |
| 'EM' (employment) |  |  |  |  |  |  |  |  |
| Text | 10.2 | ST | 3 | RE |  | EM | File: 356.22 (278) Field: 2.09 (RELATED CAUSES #2) | Loop 2000E: UM05-2 Related Causes Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to the Related Causes Code, if applicable |  |  |  |  |  |  |  |  |
| 'AP' (another party responsible), |  |  |  |  |  |  |  |  |
| 'EM' (employment) |  |  |  |  |  |  |  |  |
| Alternate Identifier | 10.4 | ST | 3 | RE |  | AP | File: 356.22 (278) Field: 2.1 (RELATED CAUSES #3) | Loop 2000E: UM05-3 Related Causes Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to the Related Causes Code, if applicable |  |  |  |  |  |  |  |  |
| 'AP' (another party responsible) |  |  |  |  |  |  |  |  |
| Problem Management Discipline | 11 | CWE | 250 | RE | TRUE |  |  |  |
| Identifier | 11.1 | ST | 3 | RE |  | HS | File: 356.22 (278) Field: 2.01 (REQUEST CATEGORY) or 2.03 (SERVICE TYPE CODE) | Loop 2000E: UM01 Request Category Code and UM03 Service Type Code (2nd occurrence) |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The 1st occurrence of this field will be populated with the Request Category Code |  |  |  |  |  |  |  |  |
| 'IN' (Individual when BHT06 = RU), |  |  |  |  |  |  |  |  |
| 'AR' (Admission review), |  |  |  |  |  |  |  |  |
| 'HS' (health services review), |  |  |  |  |  |  |  |  |
| 'SC' (specialty review) |  |  |  |  |  |  |  |  |
| The 2nd occurrence of this field will be used for the Service Type Code. |  |  |  |  |  |  |  |  |
| Examples: |  |  |  |  |  |  |  |  |
| '1' (Medical Care), |  |  |  |  |  |  |  |  |
| '2' (Surgical), |  |  |  |  |  |  |  |  |
| '3' (Consultation), |  |  |  |  |  |  |  |  |
| '4' (Diagnostic X-Ray), |  |  |  |  |  |  |  |  |
| Etc,,, |  |  |  |  |  |  |  |  |
| Problem Life Cycle Status | 14 | CWE | 250 | RE | FALSE |  |  |  |
| Identifier | 14.1 | ST | 1 | RE |  | 3 | File: 356.22 Field: 2.14 (CURRENT HEALTH CONDITION) | Loop 2000E: UM07 Current Health Condition Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Current Health Condition Code |  |  |  |  |  |  |  |  |
| examples: |  |  |  |  |  |  |  |  |
| '1' (acute), |  |  |  |  |  |  |  |  |
| '2' (stable), |  |  |  |  |  |  |  |  |
| '3' (chronic), |  |  |  |  |  |  |  |  |
| Etc… |  |  |  |  |  |  |  |  |
| Problem Ranking | 18 | CWE | 250 | RE | FALSE |  |  |  |
| Identifier | 18.1 | ST | 3 | RE |  | U | File: 356.22 Field: 2.13 (LEVEL OF SERVICE) | Loop 2000E: UM06 Level of Service Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Level of Service Code |  |  |  |  |  |  |  |  |
| '03' (emergency), |  |  |  |  |  |  |  |  |
| 'E' (elective), |  |  |  |  |  |  |  |  |
| 'U' (urgent) |  |  |  |  |  |  |  |  |
| Problem Prognosis | 22 | CWE | 250 | RE | FALSE |  |  |  |
| Identifier | 22.1 | ST | 1 | RE |  | 4 | File: 356.22 Field: 2.15 (PROGNOSIS CODE) | Loop 2000E: UM08 Prognosis Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Prognosis Code |  |  |  |  |  |  |  |  |
| examples: |  |  |  |  |  |  |  |  |
| '1' (poor), |  |  |  |  |  |  |  |  |
| '2' (guarded), |  |  |  |  |  |  |  |  |
| '3' (fair), |  |  |  |  |  |  |  |  |
| '4' (good), |  |  |  |  |  |  |  |  |
| Etc… |  |  |  |  |  |  |  |  |
| Alternate Text | 22.5 | ST | 2 | RE |  | 3 | File: 356.22 (278) Field: 2.17 (RELAY REASON) | Loop 2000E: UM010 Delay Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Delay Reason Code |  |  |  |  |  |  |  |  |
| examples: |  |  |  |  |  |  |  |  |
| '1' (proof of eligibility unknown), |  |  |  |  |  |  |  |  |
| '2' (litigation), |  |  |  |  |  |  |  |  |
| '3' (authorization delays), |  |  |  |  |  |  |  |  |
| Etc… |  |  |  |  |  |  |  |  |

### RQA_I08 Request G2R Segment Group – PV1 segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Patient Class | 2 | IS | 1 | R | FALSE | U |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will not be using this field for this X12 message, however, it is required by HL7, so it will be set equal to 'U' |  |  |  |  |  |  |  |  |
| Assigned Patient Location | 3 | PL | 2816 | RE | FALSE |  |  |  |
| Point of Care | 3.1 | IS | 2 | RE |  | 34 | File: 356.22 (278) Field: 2.05 (FACILITY TYPE) | Loop 2000E: UM04-1 Facility Type Code |
| Person Location Type | 3.6 | IS | 20 | RE |  | A | File: 356.22 (278) Field: 2.04 (FACILITY TYPE QUALIFIER) | Loop 2000E: UM04-2 Facility Code Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Facility Code Qualifier for field PV1.3.1 |  |  |  |  |  |  |  |  |
| 'A' (uniform billing claim form bill type), |  |  |  |  |  |  |  |  |
| 'B' (place of service codes for professional or dental services) |  |  |  |  |  |  |  |  |
| Hospital Service | 10 | IS | 2 | RE | FALSE | VA | File: 356.22 (278) Field: 2.11 (STATE) | Loop 2000E: UM05-4 State or Province Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the State of Province Code for the location of the Related Causes Information |  |  |  |  |  |  |  |  |
| Temporary Location | 11 | PL | 2816 | RE | FALSE |  |  |  |
| Location Description | 11.9 | ST | 3 | RE |  | ALB | File: 356.22 (278) Field: 2.12 (COUNTRY) | Loop 2000E: UM05-5 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Country Code for the Related Causes Information, if an accident occurred outside the USA |  |  |  |  |  |  |  |  |

### RQA_I08 Request AUT segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authorizing Payer, Company ID | 2 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 10 | R |  | REF 2000E | Fixed value 'REF 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If this segment exists, this field will be populated with 'REF 2000E', which indicates to FSC that this data is for X12 segment REF, loop 2000E |  |  |  |  |  |  |  |  |
| Text | 2.2 | ST | 50 | RE |  | REF number | File: 356.22 (278) Field: 17.01 (PREV. AUTHORIZATION #) or Field: 17.02 (PREV. ADMIN. REFERENCE #) | Loop 2000E: REF02 Previous Review Authorization Number or REF02 Previous Administration Reference Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Previous Administration Reference Number that this request is providing additional information for or follow-up on. |  |  |  |  |  |  |  |  |
| Alternate Text | 2.5 | ST | 3 | RE |  | BB | File: 356.22 (278) Field: 17.01 (PREV. AUTHORIZATION #) or Field: 17.02 (PREV. ADMIN. REFERENCE #) | Loop 2000E: REF01 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
| 'BB' (authorization number for additional information request), |  |  |  |  |  |  |  |  |
| 'NT' (authorization number for follow-up request) |  |  |  |  |  |  |  |  |

### RQA_I08 Request ZTP segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID | 1 | SI | 4 | R | FALSE | 1 | Fixed value of '1' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, but it is required, so a fixed value of ' 1 ' will be placed here |  |  |  |  |  |  |  |  |
| Date/Time Qualifier | 2 | CWE | 230 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 3 | R |  | 431 | File: 356.22 (278) Field: 2.18 (ACCIDENT DATE) or 2.19 (LAST MENSTRUAL PERIOD DATE) or 2.2 (ESTIMATED DOB) or 2.21 (ILLNESS DATE) or .07 (EVENT DATE) or 2.22 (DISCHARGE DATE) | Loop 2000E: DTP01 Date/Time Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| 439 - Accident Related |  |  |  |  |  |  |  |  |
| 484 - Last Menstrual Period |  |  |  |  |  |  |  |  |
| ABC - Estimated Date of Birth |  |  |  |  |  |  |  |  |
| 431 - Onset of Current Symptoms |  |  |  |  |  |  |  |  |
| AAH - when 278 is for Admission Review and Proposed or Actual Admission date is known |  |  |  |  |  |  |  |  |
| 435 - when 278 is for Admission Review and Proposed or Actual Admission date is unknown |  |  |  |  |  |  |  |  |
| 096 - Discharge Date when 278 is for Admission Review and Actual Discharge Date is known |  |  |  |  |  |  |  |  |
| Date Time Period | 3 | DR | 51 | R | FALSE |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Date or range of dates being transmitted. |  |  |  |  |  |  |  |  |
| Range start date/time | 3.1 | DTM | 8 | R |  | 20100228 | File: 356.22 (278) Field: 2.18 (ACCIDENT DATE) or 2.19 (LAST MENSTRUAL PERIOD DATE) or 2.2 (ESTIMATED DOB) or 2.21 (ILLNESS DATE) or .07 (EVENT DATE) or 2.22 (DISCHARGE DATE) | Loop 2000E: DTP03 Accident Date, DTP03 Last Menstrual Date, DTP03 Estimated Date of Birth, DTP03 Onset of Current Symptoms or Illness Date, DTP03 Proposed or Actual Event Date, DTP03 Proposed or Actual Admission Date, DTP03 Proposed or Actual Discharge Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Date will be in the following format: |  |  |  |  |  |  |  |  |
| CCYYMMDD |  |  |  |  |  |  |  |  |
| Loop ID | 4 | ST | 10 | R | FALSE | DTP 2000E |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populated this field with 'DTP 2000E', indicating the data in this segment is for X12 segment DTP, loop 2000E |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |

### RQA_I08 Request DG1 segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID - DG1 | 1 | SI | 4 | R | FALSE | 1 | Fixed value 1 through 12 |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to a sequential numerical value, from 1 through 12 |  |  |  |  |  |  |  |  |
| Diagnosis Code - DG1 | 3 | CWE | 250 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 30 | R |  | 473.1 | File: 356.22 (278) Subfile: 356.223 (PATIENT DIAGNOSIS) Field: .02 (DIAGNOSIS CODE) | Loop 2000E: HI01-2 through HI12-2 Diagnosis Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Diagnosis Code related to this Patient Diagnosis |  |  |  |  |  |  |  |  |
| Diagnosis Date/Time | 5 | DTM | 8 | RE | FALSE | 20140410 | File: 356.22 (278) Subfile: 356.223 (PATIENT DIAGNOSIS) Field: .03 (DIAGNOSIS DATE) | Loop 2000E: HI01-4 through HI12-4 Diagnosis Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| YYYYMMDD - This date is the Diagnosis Date |  |  |  |  |  |  |  |  |
| Diagnosis Type | 6 | IS | 2 | R | FALSE | W |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, so it will be set to 'W' |  |  |  |  |  |  |  |  |
| Diagnosis Classification | 17 | IS | 3 | RE | FALSE | ABF | File: 356.22 (278) Subfile: 356.223 (PATIENT DIAGNOSIS) Field: .01 (DIAGNOSIS TYPE) | Loop 2000E: HI01-1 through HI12-1 Diagnosis Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Diagnosis Type Code, identifying the code list from which field DG1.3.1 is from. |  |  |  |  |  |  |  |  |
| example: |  |  |  |  |  |  |  |  |
| 'ABF' (International Classification of Diseases Clinical Mod. Diagnosis (ICD-10 -CM)), |  |  |  |  |  |  |  |  |
| 'ABJ' (International Classification of Diseases Clinical Mod. Admitting Diagnosis (ICD-10-CM)), |  |  |  |  |  |  |  |  |
| etc... |  |  |  |  |  |  |  |  |

### RQA_I08 Request ZHS segment (Optional)

| Name | Seq | DT | Len |  | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID | 1 | SI | 10 |  | RE | FALSE | HSD 2000E | Fixed value 'HSD 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the value 'HSD 2000E', which indicates to FSC that this data is for X12 segment HSD, loop 2000E |  |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |  |
| Quantity Qualifier | 2 | CWE | 99 |  | RE | FALSE |  |  |  |
| Identifier | 2.1 | ST | 2 |  | R |  | DY | File: 356.22 (278) Field: 4.01 (HCSD QUANTITY QUALIFIER) | Loop 2000E: HSD01 Quantity Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is populated with the Health Care Services Delivery Quantity Qualifier |  |  |  |  |  |  |  |  |  |
| 'DY' (Days) |  |  |  |  |  |  |  |  |  |
| 'FL' (Units) |  |  |  |  |  |  |  |  |  |
| 'HS' (Hours) |  |  |  |  |  |  |  |  |  |
| 'MN' (Month) |  |  |  |  |  |  |  |  |  |
| 'VS' (Visits) |  |  |  |  |  |  |  |  |  |
| Benefit Quantity | 3 | NM | 15 |  | RE | FALSE | 123 | File: 356.22 (278) Field: 4.02 (HCSD SERVICE UNIT COUNT) | Loop 2000E: HSD02 Service Unit Count |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Health Care Services Delivery Service Unit Count |  |  |  |  |  |  |  |  |  |
| Unit or Basis for Measurement Code | 4 | CWE | 20 |  | RE | FALSE |  |  |  |
| Identifier | 4.1 | ST | 2 |  | R |  | DA | File: 356.22 (278) Field: 4.03 (HCSD UNITS OF MEASUREMENT) | Loop 2000E: HSD03 Unit or Basis for Measurement Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Health Care Services Delivery Unit or Basis for Measurement Code |  |  |  |  |  |  |  |  |  |
| 'DA' (Days) |  |  |  |  |  |  |  |  |  |
| 'MO' (Months) |  |  |  |  |  |  |  |  |  |
| 'VS' (Visit) |  |  |  |  |  |  |  |  |  |
| 'WK' (Week) |  |  |  |  |  |  |  |  |  |
| 'YR' (Years) |  |  |  |  |  |  |  |  |  |
| Sample Selection Modulus | 5 | ST | 6 |  | RE | FALSE | 1.5 | File 356.22 (278) Field: 4.04 (HCSD SAMPLE SELECTION MODULUS) | Loop 2000E: HSD04 Sample Selection Modulus |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| To specify the sampling frequency in terms of a modulus of the Unit of Measure, e.g., every fifth bag, every 1.5 Minutes |  |  |  |  |  |  |  |  |  |
| Time Period Qualifier | 6 | CWE | 20 |  | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 |  | R |  | 25 | File: 356.22 (278) Field: 4.05 (HCSD TIME PERIOD QUALIFIER) | Loop 2000E: HSD05 Time Period Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is populated with the Health Care Services Delivery Time Period Qualifier. It is used if needed to provide further information about the number and |  |  |  |  |  |  |  |  |  |
| frequency of benefits. |  |  |  |  |  |  |  |  |  |
| '6' (Hour) |  |  |  |  |  |  |  |  |  |
| '7' (Day) |  |  |  |  |  |  |  |  |  |
| '21' (Years) |  |  |  |  |  |  |  |  |  |
| '22' (Service Year) |  |  |  |  |  |  |  |  |  |
| '23' (Calendar Year) |  |  |  |  |  |  |  |  |  |
| '24' (Year to Date) |  |  |  |  |  |  |  |  |  |
| '25' (Contract) |  |  |  |  |  |  |  |  |  |
| '26' (Episode) |  |  |  |  |  |  |  |  |  |
| '27' (Visit) |  |  |  |  |  |  |  |  |  |
| '28' (Outlier) |  |  |  |  |  |  |  |  |  |
| '29' (Remaining) |  |  |  |  |  |  |  |  |  |
| '30' (Exceeded) |  |  |  |  |  |  |  |  |  |
| '31' (Not Exceeded) |  |  |  |  |  |  |  |  |  |
| '32' (Lifetime) |  |  |  |  |  |  |  |  |  |
| '33' (Lifetime Remaining) |  |  |  |  |  |  |  |  |  |
| '34' (Month) |  |  |  |  |  |  |  |  |  |
| '35' (Week) |  |  |  |  |  |  |  |  |  |
| Number of Periods | 7 | NM | 20 |  | RE | FALSE | 5 | File: 356.22 (278) Field: 4.06 (HCSD PERIOD COUNT) | Loop 2000E: HSD06 Period Count |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Health Care Services Delivery Period Count. This is used if needed to provide further information about the |  |  |  |  |  |  |  |  |  |
| number and frequency of benefits. |  |  |  |  |  |  |  |  |  |
| Ship/Delivery or Calendar Pattern Code | 8 | CWE | 20 |  | RE | FALSE |  |  |  |
| Identifier | 8.1 | ST | 2 |  | R |  | A | File: 356.22 (278) Field: 4.07 (HCSD DELIVERY FREQUENCY) | Loop 2000E: HSD07 Delivery Frequency Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is populated with the Heath Care Services Delivery Frequency Code. This is used if needed to provide further information about the number and |  |  |  |  |  |  |  |  |  |
| frequency of benefits. |  |  |  |  |  |  |  |  |  |
| '1' - 1st Week of the Month |  |  |  |  |  |  |  |  |  |
| '2' - 2nd Week of the Month |  |  |  |  |  |  |  |  |  |
| '3' - 3rd Week of the Month |  |  |  |  |  |  |  |  |  |
| '4' - 4th Week of the Month |  |  |  |  |  |  |  |  |  |
| '5' - 5th Week of the Month |  |  |  |  |  |  |  |  |  |
| '6' - 1st & 3rd Weeks of the Month |  |  |  |  |  |  |  |  |  |
| '7' - 2nd & 4th Weeks of the Month |  |  |  |  |  |  |  |  |  |
| '8' - 1st Working Day of Period |  |  |  |  |  |  |  |  |  |
| '9' - Last Working Day of Period |  |  |  |  |  |  |  |  |  |
| 'A' - Monday through Friday |  |  |  |  |  |  |  |  |  |
| 'B' - Monday through Saturday |  |  |  |  |  |  |  |  |  |
| 'C' - Monday through Sunday |  |  |  |  |  |  |  |  |  |
| 'D' – Monday |  |  |  |  |  |  |  |  |  |
| 'E' – Tuesday |  |  |  |  |  |  |  |  |  |
| 'F' – Wednesday |  |  |  |  |  |  |  |  |  |
| 'G' – Thursday |  |  |  |  |  |  |  |  |  |
| 'H' – Friday |  |  |  |  |  |  |  |  |  |
| 'J' – Saturday |  |  |  |  |  |  |  |  |  |
| 'K' – Sunday |  |  |  |  |  |  |  |  |  |
| 'L' - Monday through Thursday |  |  |  |  |  |  |  |  |  |
| 'M' – Immediately |  |  |  |  |  |  |  |  |  |
| 'N' - As Directed |  |  |  |  |  |  |  |  |  |
| 'O' - Daily Mon. through Fri. |  |  |  |  |  |  |  |  |  |
| 'P' - 1/2 Mon. & 1/2 Thurs. |  |  |  |  |  |  |  |  |  |
| 'Q' - 1/2 Tues. & 1/2 Thurs. |  |  |  |  |  |  |  |  |  |
| 'R' - 1/2 Wed. & 1/2 Fri. |  |  |  |  |  |  |  |  |  |
| 'S' - Once Anytime Mon. through Fri. |  |  |  |  |  |  |  |  |  |
| 'SG' - Tuesday through Friday |  |  |  |  |  |  |  |  |  |
| 'SL' - Monday, Tuesday and Thursday |  |  |  |  |  |  |  |  |  |
| 'SP' - Monday, Tuesday and Friday |  |  |  |  |  |  |  |  |  |
| 'SX' - Wednesday and Thursday |  |  |  |  |  |  |  |  |  |
| 'SY' - Monday, Wednesday and Thursday |  |  |  |  |  |  |  |  |  |
| 'SZ' - Tuesday, Thursday and Friday |  |  |  |  |  |  |  |  |  |
| 'T' - 1/2 Tue. & 1/2 Fri. |  |  |  |  |  |  |  |  |  |
| 'U' - 1/2 Mon. & 1/2 Wed. |  |  |  |  |  |  |  |  |  |
| 'V' - 1/3 Mon., 1/3 Wed., 1/3 Fri. |  |  |  |  |  |  |  |  |  |
| 'W' - Whenever Necessary |  |  |  |  |  |  |  |  |  |
| 'X' - 1/2 By Wed., Bal. By Fri. |  |  |  |  |  |  |  |  |  |
| Ship/Delivery Pattern Time Code | 9 | CWE | 20 | RE |  | FALSE |  |  |  |
| Identifier | 9.1 | ST | 2 | R |  |  | A | File: 356.22 (278) Field: 4.08 (HCSD DELIVERY TIME PATTERN) | Loop 2000E: HSD08 Delivery Pattern Time Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Health Care Services Delivery Pattern Time Code. This is used if needed to provide further information about the |  |  |  |  |  |  |  |  |  |
| number and frequency of benefits. |  |  |  |  |  |  |  |  |  |
| 'A' - 1st Shift (Normal Working Hours) |  |  |  |  |  |  |  |  |  |
| 'B' - 2nd Shift |  |  |  |  |  |  |  |  |  |
| 'C' - 3rd Shift |  |  |  |  |  |  |  |  |  |
| 'D' - A.M. |  |  |  |  |  |  |  |  |  |
| 'E' - P.M. |  |  |  |  |  |  |  |  |  |
| 'F' - As Directed |  |  |  |  |  |  |  |  |  |
| 'G' - Any Shift |  |  |  |  |  |  |  |  |  |
| 'Y' - None (Also Used to Cancel or Override a Previous Pattern) |  |  |  |  |  |  |  |  |  |

### RQA_I08 Request PV1 segment (Optional) (Repeating)

| Name | Seq |  | DT |  | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Patient Class | 2 |  | IS |  | 2 | R | FALSE | 7 | File: 356.22 (278) Field: 4.09 (AMBULANCE CERT. CONDITION) or 5.01 (CHIROPRACTIC CERT. CONDITION) or 5.07 (DME CERT. CONDITION) or 5.13 (OXYGEN CERT. CONDITION) or 6.01 (FUNCT. LIMIT CERT. CONDITION) or 6.07 (ACTIVITIES CERT. CONDITION) or 6.13 (MENTAL STATUS CERT. CONDITION) | Loop 2000E: CRC01 Condition Code Category |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field will be populated with a Condition Code Category |  |  |  |  |  |  |  |  |  |  |
| '07' (ambulance certification), |  |  |  |  |  |  |  |  |  |  |
| '08' (chiropractic cert), |  |  |  |  |  |  |  |  |  |  |
| '09' (DME cert), |  |  |  |  |  |  |  |  |  |  |
| '11' (Oxygen Therapy cert), |  |  |  |  |  |  |  |  |  |  |
| '75' (functional limit), |  |  |  |  |  |  |  |  |  |  |
| '76' (activities permitted), |  |  |  |  |  |  |  |  |  |  |
| '77' (mental status) |  |  |  |  |  |  |  |  |  |  |
| Field length has been expanded to 2 characters to allow for X12 value |  |  |  |  |  |  |  |  |  |  |
| Assigned Patient Location | 3 |  | PL |  | 2816 | RE | FALSE |  |  |  |
| Point of Care | 3.1 |  | IS |  | 10 | RE |  | CRC 2000E | Fixed value 'CRC 2000E' or 'CL1 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field will be set to 'CRC 2000E' or'CL1 2000E'. This indicates to FSC that this data is for X12 segment CRC in loop2000E or segment CL1 in loop 2000E. |  |  |  |  |  |  |  |  |  |  |
| Admission Type | 4 | IS |  | 1 |  | RE | FALSE | 1 | File: 356.22 (278) Field: 7.01 (ADMISSION TYPE) | Loop 2000E: CL101 Admission Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Admission Type Code |  |  |  |  |  |  |  |  |  |  |
| '1' (EMERGENCY), |  |  |  |  |  |  |  |  |  |  |
| '2' (URGENT), |  |  |  |  |  |  |  |  |  |  |
| '3' (ELECTIVE), |  |  |  |  |  |  |  |  |  |  |
| '4' (NEWBORN), |  |  |  |  |  |  |  |  |  |  |
| '5' (TRAUMA), |  |  |  |  |  |  |  |  |  |  |
| '9' (INFORMATION NOT AVAILABLE) |  |  |  |  |  |  |  |  |  |  |
| Admit Source | 14 |  | IS |  | 1 | RE | FALSE | 1 | File: 356.22 (278) Field: 7.02 (ADMISSION SOURCE) | Loop 2000E: CL102 Admission Source Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Admission Source Code for X12 segment CL1 |  |  |  |  |  |  |  |  |  |  |
| '1' (physician referral), |  |  |  |  |  |  |  |  |  |  |
| '2' (clinic referral), |  |  |  |  |  |  |  |  |  |  |
| '3' (HMO referral), |  |  |  |  |  |  |  |  |  |  |
| '4' (transfer from hospital), |  |  |  |  |  |  |  |  |  |  |
| '5' (transfer from skilled nursing facility), |  |  |  |  |  |  |  |  |  |  |
| '6' (transfer from other health care facility), |  |  |  |  |  |  |  |  |  |  |
| '7' (emergency room), |  |  |  |  |  |  |  |  |  |  |
| '8' (court / law enforcement), |  |  |  |  |  |  |  |  |  |  |
| '9' (info not available) |  |  |  |  |  |  |  |  |  |  |
| Ambulatory Status | 15 |  | IS |  | 3 | RE | FALSE | Y | File: 356.22 (278) Field: 4.09 (AMBULANCE CERT. CONDITION) or 5.01 (CHIRIORACTIC CERT. CONDITION) or 5.07 (DME CERT. CONDITION) or 5.13 (OXYGEN CERT. CONDITION) or 6.01 (FUNCT. LIMIT CERT. CONDITION) or 6.07 (ACTIVITIES CERT. CONDITION) or 6.13 (MENTAL STATUS CERT. CONDITION), 2nd occurrence of field value will be various CONDITION fields 1 through 5 | Loop 2000E: CRC02 Certification Condition Indicator, CRC03 Condition Indicator, CRC04 Condition Indicator, CRC05 Condition Indicator, CR06 Condition Indicator, CRC07 Condition Indicator |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| The 1st occurrence of this field will be the Certification Condition Indicator. |  |  |  |  |  |  |  |  |  |  |
| 'N' (no if the condition codes in CRC03 thru CRC07 do not apply), |  |  |  |  |  |  |  |  |  |  |
| 'Y' ( they do apply) |  |  |  |  |  |  |  |  |  |  |
| The 2nd through 6th occurrences will be the Certification Condition Indicators. |  |  |  |  |  |  |  |  |  |  |
| example: |  |  |  |  |  |  |  |  |  |  |
| '01' (patient was admitted), |  |  |  |  |  |  |  |  |  |  |
| '02' (patient was bed confined before the ambulance service), |  |  |  |  |  |  |  |  |  |  |
| '03' (patient was bed confined after the ambulance service), |  |  |  |  |  |  |  |  |  |  |
| '04' (patient was moved by stretcher), |  |  |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |  |  |
| Field length has been expanded to 3 characters to allow for X12 value |  |  |  |  |  |  |  |  |  |  |
| Patient Type | 18 |  | IS |  | 1 | RE | FALSE | 1 | File: 356.22 (278) Field: 7.04 (NURSING HOME STATUS) | Loop 2000E: CL104 Nursing Home Residential Status Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field will contain the Nursing Home Residential Status Code. |  |  |  |  |  |  |  |  |  |  |
| '1' (Transferred to Intermediate Care Facility Mentally Retarded), |  |  |  |  |  |  |  |  |  |  |
| '2' (Newly Admitted), |  |  |  |  |  |  |  |  |  |  |
| '3' (newly eligible), |  |  |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |  |  |
| Discharge Disposition | 36 |  | IS |  | 2 | RE | FALSE | A | File: 356.22 (278) Field: 7.03 (PATIENT STATUS) | Loop 2000E: CL103 Patient Status Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Patient Status Code |  |  |  |  |  |  |  |  |  |  |

### RQA_I08 Request OBR segment (Optional)

| Name | Seq |  | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Universal Service Identifier | 4 |  | CWE | 705 | R | FALSE |  |  |  |
| Identifier | 4.1 |  | ST | 10 | R |  | CR1 2000E | Fixed value 'CR1 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be set to 'CR1 2000E' to indicate to FSC that this data is for X12 segment CR1 in loop 2000E |  |  |  |  |  |  |  |  |  |
| Relevant Clinical Information | 13 |  | ST | 1 | RE | FALSE | A | File: 356.22 (278) Field: 18.04 (AMBULANCE TRANSPORT REASON) | Loop 2000E: CR104 Ambulance Transport Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Ambulance Transport Reason Code |  |  |  |  |  |  |  |  |  |
| 'A' (patient was transported to nearest facility for care of symptoms, complaints, or both), |  |  |  |  |  |  |  |  |  |
| 'B' (patient was transported for the benefit of a preferred physician), |  |  |  |  |  |  |  |  |  |
| etc... |  |  |  |  |  |  |  |  |  |
| Placer Field 1 | 18 |  | ST | 80 | RE | FALSE | Up to 80 characters of free text | File: 356.22 (278) Field: 18.09 (ROUND TRIP PURPOSE DESCRIPTION) | Loop 2000E: CR109 Round Trip Purpose Description |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Round Trip Purpose Description |  |  |  |  |  |  |  |  |  |
| Placer Field 2 | 19 |  | ST | 80 | RE | FALSE | Up to 80 characters of free text | File: 356.22 (278) Field: 18.1 (STRETCHER PURPOSE DESCRIPTION) | Loop 2000E: CR110 Stretcher Purpose Description |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Stretcher Purpose Description |  |  |  |  |  |  |  |  |  |
| Quantity/ Timing |  | 27 | TQ | 2206 | RE | TRUE |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| Quantity Subcomponent of Quantity Component field: The 1st occurrence of this field will be populated with the patient's weight measurement. The 2nd |  |  |  |  |  |  |  |  |  |
| occurrence of this field will be the Transport Distance. |  |  |  |  |  |  |  |  |  |
| Units Subcomponent of Quantity Component field: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Unit or Basis of Measurement qualifier for field OBR.27.1.1 |  |  |  |  |  |  |  |  |  |
| 'KG' (Kilogram) |  |  |  |  |  |  |  |  |  |
| 'LB' (pound) |  |  |  |  |  |  |  |  |  |
| Quantity | 27.1 |  | CQ | 722 | RE |  |  |  |  |
| � Quantity | 27.1.1 |  | NM | 16 | RE |  | 175 | File: 356.22 (278) Field: 18.02 (PATIENT WEIGHT) Field: 18.06 (TRANSPORT DISTANCE) | Loop 2000E: CR102 Weight, CR106 Transport Distance (2nd occurrence) |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| The 1st occurrence of this field will be populated with the patient's weight measurement. The 2nd occurrence of this field will be the Transport Distance |  |  |  |  |  |  |  |  |  |
| � Units | 27.1.2 |  | CWE | 705 | RE |  |  | File: 356.22 (278) Field: 18.01 (WEIGHT UNITS) Field: 18.05 (DISTANCE UNITS) | Loop 2000E: CR101 Unit or Basis of Measurement, CR105 Unit or Basis of Measurement (2nd occurrence) |
| Placer Supplemental Service Information | 46 |  | CWE | 705 | RE | FALSE |  |  |  |
| Identifier | 46.1 |  | ST | 1 | RE |  | I | File: 356.22 (278) Field: 18.03 (AMBULANCE TRANSPORT CODE) | Loop 2000E: CR103 Ambulance Transport Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Ambulance Transport Code |  |  |  |  |  |  |  |  |  |
| 'I' (initial trip), |  |  |  |  |  |  |  |  |  |
| 'R' (return trip), |  |  |  |  |  |  |  |  |  |
| 'T' (transfer trip), |  |  |  |  |  |  |  |  |  |
| 'X' (round trip) |  |  |  |  |  |  |  |  |  |

### RQA_I08 Request G20 Segment Group – RXA segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Give Sub-ID Counter | 1 | NM | 9 | R | FALSE | 5 | File: 356.22 (278) Field: 7.05 (TREATMENT SERIES NUMBER) | Loop 2000E: CR201 Treatment Series Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Treatment Series Number, which is the total number of treatments in the series of treatments. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 9 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Administration Sub-ID Counter | 2 | NM | 15 | R | FALSE | 2 | File: 356.22 (278) Field: 7.06 (TREATMENT COUNT) | Loop 2000E: CR202 Treatment Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Treatment Count |  |  |  |  |  |  |  |  |
| Field length has been expanded to 15 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Date/Time Start of Administration | 3 | DTM | 24 | R | FALSE | 20140428 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for X12 message, so will be set to date of transmission |  |  |  |  |  |  |  |  |
| Date/Time End of Administration | 4 | DTM | 24 | R | FALSE | 20140428 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, it is a required HL7 field, so it will be set to the date of message transmission |  |  |  |  |  |  |  |  |
| Administered Code | 5 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 5.1 | ST | 1 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, this field is required for HL7, so it will be set to 1 |  |  |  |  |  |  |  |  |
| Administered Amount | 6 | NM | 20 | R | FALSE | 0 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, it is a required field for HL7, so it will be set to 0 (zero) |  |  |  |  |  |  |  |  |
| Administration Notes | 9 | CWE | 697 | RE | TRUE |  |  |  |
| Identifier | 9.1 | ST | 3 | RE |  | C1 | File: 356.22 (278) Field: 7.07 (SUBLUXATION LEVEL CODE #1) or 7.08 (SUBLUXATION LEVEL CODE #2) | Loop 2000E: CR203 Subluxation Level Code, CR204 Subluxation Level Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This multiple field will contain the Subluxation Level Code |  |  |  |  |  |  |  |  |
| 'C1' (cervical 1), |  |  |  |  |  |  |  |  |
| 'C2' (cervical 2), |  |  |  |  |  |  |  |  |
| 'IL' (ilium), |  |  |  |  |  |  |  |  |
| 'L1' (lumbar 1), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Indication | 19 | CWE | 697 | RE | TRUE |  |  |  |
| Identifier | 19.1 | ST | 1 | RE |  | A | File: 356.22 (278) Field: 7.09 (PATIENT CONDITION CODE) or 7.1 (COMPLICATION INDICATOR) | Loop 2000E: CR208 Patient Condition Code, CR209 Complication Indicator |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Patient Condition Code |  |  |  |  |  |  |  |  |
| 'A' (acute condition), |  |  |  |  |  |  |  |  |
| 'C' (chronic condition), |  |  |  |  |  |  |  |  |
| 'D' (non-acute), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| And the 2nd occurrence is for the Complication Indicator. |  |  |  |  |  |  |  |  |
| 'Y' (complicated condition), |  |  |  |  |  |  |  |  |
| 'N' (non-complicated condition) |  |  |  |  |  |  |  |  |
| Completion Status | 20 | ID | 2 | RE | FALSE | CP | File: 356.22 (278) Field: 7.13 (X-RAY AVAILABLE?) | Loop 2000E: CR212 X-Ray Availability Indicator |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with an X-Ray Availability Indicator |  |  |  |  |  |  |  |  |
| 'Y' (x-rays maintained), |  |  |  |  |  |  |  |  |
| 'N' (not maintained) |  |  |  |  |  |  |  |  |
| A mapping of HL7 table values to X12 is required. |  |  |  |  |  |  |  |  |
| Y ==> CP |  |  |  |  |  |  |  |  |
| N ==> NA |  |  |  |  |  |  |  |  |

### RQA_I08 Request G20 Segment Group – NTE segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Comment | 3 | FT | 80 | RE | FALSE | 80 characters of free text | File: 356.22 (278) Field: 7.11 (PAT. COND. DESCRIPTION LINE 1) or 7.12 (PAT. COND. DESCRIPTION LINE 2) | Loop 2000E: CR210 Patient Condition Description, CR211 Patient Condition Description (additional) |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Patient Condition Description |  |  |  |  |  |  |  |  |
| Comment Type | 4 | CWE | 250 | RE | FALSE |  |  |  |
| Identifier | 4.1 | ST | 10 | RE |  | CR2 2000E |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| 'CR2 2000E' |  |  |  |  |  |  |  |  |

### RQA_I08 Request RXE segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Quantity/ Timing | 1 | TQ | 2206 | RE | FALSE |  |  |  |
| Quantity | 1.1 | CQ | 15 | RE |  |  |  |  |
| � Quantity | 1.1.1 | NM | 15 | RE |  | 1 | File: 356.22 (278) Field: 8.07 (OXYGEN USE PERIOD HOUR COUNT) | Loop 2000E: CR508 Oxygen Use Period Hour Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Oxygen Use Period Hour Count. |  |  |  |  |  |  |  |  |
| This is the number of hours per period of oxygen use |  |  |  |  |  |  |  |  |
| Text | 1.8 | TX | 80 | RE |  | Up to 80 characters of free text | File: 356.22 (278) Field: 8.08 (RESPIRATORY THERAPIST ORDER) | Loop 2000E: CR509 Respiratory Therapist Order Text |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Respiratory Therapist Order Text. These are special orders for the respiratory therapist. |  |  |  |  |  |  |  |  |
| Give Code | 2 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 1 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, it is required for HL7. Set to ' 1 ' |  |  |  |  |  |  |  |  |
| Give Amount - Minimum | 3 | NM | 15 | R | FALSE | 15 | File: 356.22 (278) Field: 9.01 (ARTERIAL BLOOD GAS QUANTITY) | Loop 2000E: CR510 Arterial Blood Gas Quantity |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Arterial Blood Gas Quantity |  |  |  |  |  |  |  |  |
| Give Units | 5 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 5.1 | ST | 1 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not required for this X12 message, but it is an HL7 required field, so it will be set to ' 1 ' |  |  |  |  |  |  |  |  |
| Provider's Administration Instructions | 7 | CWE | 697 | RE | FALSE |  |  |  |
| Text | 7.2 | ST | 80 | RE |  | Up to 80 characters of free text | File: 356.22 (278) Field: 8.04 (OXYGEN EQUIPMENT REASON) | Loop 2000E: CR505 Equipment Reason Description |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Oxygen Equipment Reason Description |  |  |  |  |  |  |  |  |
| Dispense Amount | 10 | NM | 15 | RE | FALSE | 10 | File: 356.22 (278) Field: 9.02 (OXYGEN SATURATION QUANTITY) | Loop 2000E: CR511 Oxygen Saturation Quantity |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Oxygen Saturation Quantity |  |  |  |  |  |  |  |  |
| Pharmacist/Treatment Supplier's Verifier ID | 14 | XCN | 4751 | RE | TRUE |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Field length has been expanded to 4751 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| ID Number | 14.1 | ST | 1 | RE |  | A | File: 356.22 (278) Field: 8.01 (OXYGEN EQUIPMENT TYPE #1) and 8.02 (OXYGEN EQUIPMENT TYPE #2) | Loop 2000E: CR503 Oxygen Equipment Type Code, CR504 Oxygen Equipment Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Equipment Type Code |  |  |  |  |  |  |  |  |
| 'A' (concentrator), |  |  |  |  |  |  |  |  |
| 'B' (liquid stationary), |  |  |  |  |  |  |  |  |
| 'C' (gaseous stationary), |  |  |  |  |  |  |  |  |
| 'D' (liquid portable), |  |  |  |  |  |  |  |  |
| 'E' (gaseous portable), |  |  |  |  |  |  |  |  |
| 'O' (other) |  |  |  |  |  |  |  |  |
| Number of Refills Remaining | 16 | NM | 15 | RE | FALSE | 1.5 | File: 356.22 (278) Field: 9.07 (PORTABLE OXYGEN SYSTEM FLOW) | Loop 2000E: CR516 Portable Oxygen System Flow Rate |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Portable Oxygen System Flow Rate. This the oxygen flow rate for a portable oxygen system in liters per minute |  |  |  |  |  |  |  |  |
| Total Daily Dose | 19 | CQ | 15 | RE | FALSE |  |  |  |
| Quantity | 19.1 | NM | 15 | RE |  | 3 | File: 356.22 (278) Field: 8.06 (DAILY OXYGEN USE COUNT) | Loop 2000E: CR507 Daily Oxygen Use Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Daily Oxygen Use Count, which is the number of times per day the patient must use oxygen |  |  |  |  |  |  |  |  |
| Give Rate Amount | 23 | ST | 15 | RE | FALSE | 0.75 | File: 356.22 (278) Field: 8.05 (OXYGEN FLOW RATE) | Loop 2000E: CR506 Oxygen Flow Rate |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Flow Rate in liters per minute. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 15 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Give Indication | 27 | CWE | 697 | RE | TRUE |  |  |  |
| Identifier | 27.1 | ST | 1 | RE |  | E | File: 356.22 (278) Field: 9.03 (OXYGEN TEST CONDITION) | Loop 2000E: CR512 Oxygen Test Condition Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Test Condition Code. |  |  |  |  |  |  |  |  |
| 'E' (exercising), |  |  |  |  |  |  |  |  |
| 'N' (no special conditions for test), |  |  |  |  |  |  |  |  |
| 'O' (on oxygen), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Alternate Identifier | 27.4 | ST | 1 | RE |  | 1 | File: 356.22 (278) Field: 9.04 (OXYGEN TEST FINDINGS #1) or 9.05 (OXYGEN TEST FINDINGS #2) or 9.06 (OXYGEN TEST FINDINGS #3) | Loop 2000E: CR513 Oxygen Test Findings Code, CR514 Oxygen Test Findings Code, CR515 Oxygen Test Findings Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This repeating field will contain the Oxygen Test Findings Code(s) |  |  |  |  |  |  |  |  |
| '1' (dependent edema suggesting congestive heart failure), |  |  |  |  |  |  |  |  |
| '2' ("P" pulmonale on electrocardiogram (EKG)), |  |  |  |  |  |  |  |  |
| '3' (Erythrocythemia with a hematocrit greater than 56 %) |  |  |  |  |  |  |  |  |
| Dispense Package Size Unit | 29 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 29.1 | ST | 1 | RE |  | A | File: 356.22 (278) Field: 9.08 (OXYGEN DELIVERY SYSTEM) | Loop 2000E: CR517 Oxygen Delivery System Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Delivery System Code |  |  |  |  |  |  |  |  |
| 'A' (nasal cannula), |  |  |  |  |  |  |  |  |
| 'B' (oxygen conserving device), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Supplementary Code | 31 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 31.1 | ST | 1 | RE |  | A | File: 356.22 (278) Field: 8.03 (OXYGEN EQUIPMENT TYPE #3) | Loop 2000E: CR518 Oxygen Equipment Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Equipment Type Code |  |  |  |  |  |  |  |  |
| 'A' (concentrator), |  |  |  |  |  |  |  |  |
| 'B' (liquid stationary), |  |  |  |  |  |  |  |  |
| 'C' (gaseous stationary), |  |  |  |  |  |  |  |  |
| 'D' (liquid portable), |  |  |  |  |  |  |  |  |
| 'E' (gaseous portable), |  |  |  |  |  |  |  |  |
| 'O' (other) |  |  |  |  |  |  |  |  |

### RQA_I08 Request PRB segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action Code | 1 | ID | 2 | R | FALSE | UC |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 278 message, however, it is a required field for HL7, so a 'UC' will be placed into this field |  |  |  |  |  |  |  |  |
| Action Date/Time | 2 | DTM | 8 | R | FALSE | 20140422 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field for HL7, so this field will be populated with the date of the message transmission, CCYYMMDD format |  |  |  |  |  |  |  |  |
| Problem ID | 3 | CWE | 705 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 1 | R |  | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .02 (CERTIFICATION TYPE CODE) | Loop 2000E: CR608 Certification Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Certification Type Code. |  |  |  |  |  |  |  |  |
| '1' (appeal - immediate), |  |  |  |  |  |  |  |  |
| '2' (appeal - standard), |  |  |  |  |  |  |  |  |
| '3' (cancel), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Name of Coding System | 3.3 | ID | 10 | RE |  | I9C | File: 356.22 (278) Field: 10.06 (SURGICAL PROCEDURE CODE TYPE) | Loop 2000E: CR610 Product/Service ID Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Product/Service ID Qualifier |  |  |  |  |  |  |  |  |
| 'HC' (HCPCS codes), |  |  |  |  |  |  |  |  |
| 'ID' (ICD-9-CM codes) |  |  |  |  |  |  |  |  |
| This field is associated with an HL7 table, so a mapping must be used: |  |  |  |  |  |  |  |  |
| 'HC' ===> HCPCS |  |  |  |  |  |  |  |  |
| 'ID'===> I9C |  |  |  |  |  |  |  |  |
| Alternate Identifier | 3.4 | ST | 8 | RE |  | 10021 | File: 356.22 (278) Field: 10.07 (SURGICAL PROCEDURE CODE) | Loop 2000E: CR611 Surgical Procedure Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Surgical Procedure Code |  |  |  |  |  |  |  |  |
| Problem Instance ID | 4 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier | 4.1 | ST | 199 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 message, however, it is a required field for HL7, and so we will place a '1' in this field. |  |  |  |  |  |  |  |  |
| Episode of Care ID | 5 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 5.1 | ST | 10 | RE |  | CR6 2000E | Fixed value 'CR6 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the text'CR6 2000E'. This identifies to FSC that this data is for X12 segment CR6 in loop 2000E |  |  |  |  |  |  |  |  |
| Problem Established Date/Time | 7 | DTM | 8 | RE | FALSE | 20140405 | File: 356.22 (278) Field: 10.08 (PHYSICIAN ORDER DATE) | Loop 2000E: CR612 Physician Order Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Physician Order Date, in the format CCYYMMDD |  |  |  |  |  |  |  |  |
| Anticipated Problem Resolution Date/Time | 8 | DTM | 8 | RE | FALSE | 20140115 | File: 356.22 (278) Field: 10.05 (SURGERY DATE) | Loop 2000E: CR609 Surgery Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the last surgery date. Date format will be CCYYMMDD |  |  |  |  |  |  |  |  |
| Actual Problem Resolution Date/Time | 9 | DTM | 8 | RE | FALSE | 20140212 | File: 356.22 (278) Field: 10.09 (LAST VISIT DATE) | Loop 2000E: CR613 Last Visit Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the date the patient was last seen by the physician. Format of date is CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem Classification | 10 | CWE | 705 | RE | FALSE |  |  |  |
| Identifier | 10.1 | ST | 1 | RE |  | A | File: 356.22 (278) Field: 10.13 (PATIENT LOCATION) | Loop 2000E: CR617 Patient Location Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Patient Location Code, identifying the location where the patient is receiving medical treatment. |  |  |  |  |  |  |  |  |
| 'A' (acute care facility), |  |  |  |  |  |  |  |  |
| 'B' (boarding home), |  |  |  |  |  |  |  |  |
| 'C' (hospice), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Problem Life Cycle Status | 14 | CWE | 705 | RE | FALSE |  |  |  |
| Text | 14.2 | ST | 35 | RE |  | 20140401-20140405 | File: 356.22 (278) Field: 10.11 (LAST ADMISSION START DATE) and 10.12 (LAST ADMISSION END DATE) | Loop 2000E: CR616 Last Admission Period |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain a range of dates identifying the Last Admission Period for the patient. Format will be CCYYMMDD- CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem Life Cycle Status Date/Time | 15 | DTM | 8 | RE | FALSE | 20140402 | File: 356.22 (278) Field: 10.1 (PHYSICIAN CONTACT DATE) | Loop 2000E: CR614 Physician Contact Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the home health agency's most recent contact with the physician. Date format be CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem Date of Onset | 16 | DTM | 8 | RE | FALSE | 20140412 | File: 356.22 (278) Field: 10.01 (HOME HEALTH START DATE) | Loop 2000E: CR602 Home Health Start Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the date covered home health services began. The date format will be CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem Onset Text | 17 | ST | 35 | RE | FALSE | 20140401-20140408 | File: 356.22 (278) Field: 10.02 (HOME HEALTH CERT. START) and 10.03 (HOME HEALTH CERT. END) | Loop 2000E: CR604 Home Health Certification Period |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the certification period covered by this plan of treatment. Format of this date range will be CCYYMMDD-CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem Prognosis | 22 | CWE | 705 | RE | FALSE |  |  |  |
| Identifier | 22.1 | ST | 1 | RE |  | 6 | FIle: 356.22 (278) Field: 2.15 (PROGNOSIS CODE) | Loop 2000E: CR601 Prognosis Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Prognosis Code, indicating the physician's prognosis for the patient. |  |  |  |  |  |  |  |  |
| '1' (Poor) |  |  |  |  |  |  |  |  |
| '2' (Guarded) |  |  |  |  |  |  |  |  |
| '3' (Fair) |  |  |  |  |  |  |  |  |
| '4' (Good) |  |  |  |  |  |  |  |  |
| '5' (Very Good) |  |  |  |  |  |  |  |  |
| '6' (Excellent) |  |  |  |  |  |  |  |  |
| '7' (Less than 6 Months to Live) |  |  |  |  |  |  |  |  |
| '8' (Terminal) |  |  |  |  |  |  |  |  |

### RQA_I08 Request PSL segment (Optional) (Repeating)

| Name |  | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Product/Service Line Item Number |  | 1 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier |  | 1.1 | ST | 9 | R |  | PWK 2000E | Fixed value 'PWK 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with 'PWK 2000E'. This allows FSC to identify this data to be for X12 segment PWK in loop 2000E |  |  |  |  |  |  |  |  |  |
| Product/Service Line Item Sequence Number |  | 3 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 message, however, it is required for this HL7 segment, so this value will be a sequential integer, starting at 1, for each |  |  |  |  |  |  |  |  |  |
| occurrence of this segment. |  |  |  |  |  |  |  |  |  |
| Product/Service Line Item Status |  | 6 | CWE | 697 | R | FALSE |  |  |  |
| Identifier |  | 6.1 | ST | 20 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not being used for this X12 message, however, it is required for this HL7 segment, so this value will be set to '1' |  |  |  |  |  |  |  |  |  |
| Product/Service Code |  | 7 | CWE | 697 | R | FALSE |  |  |  |
| Identifier |  | 7.1 | ST | 20 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 message, however, it is required for this HL7 segment, so this field will be set to '1' |  |  |  |  |  |  |  |  |  |
| Health Document Reference Identifier |  | 19 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier |  | 19.1 | ST | 2 | RE |  | 3 | File: 356.22 (278) Field: 11.01 (REPORT TYPE CODE) | Loop 2000E: PWK01 Attachment Report Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will contain the Attachment Report Type Code, which indicates the title or contents of a document, report or supporting item. |  |  |  |  |  |  |  |  |  |
| '03' (report justifying treatment beyond utilization guidelines), |  |  |  |  |  |  |  |  |  |
| '04' (drugs administered), |  |  |  |  |  |  |  |  |  |
| '05' (treatment diagnosis), |  |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |  |
| Namespace ID |  | 19.2 | IS | 80 | RE |  | 4312 | File: 356.22 (278) Field: 11.03 (ATTACHMENT CONTROL NUMBER) | Loop 2000E: PWK06 Attachment Control Number |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will contain the Attachment Control Number |  |  |  |  |  |  |  |  |  |
| Universal ID |  | 19.3 | ST | 80 | RE |  | Up to 80 characters of free text | File: 356.22 (278) Field: 11.04 (ATTACHMENT DESCRIPTION) | Loop 2000E: PWK07 Attachment Description |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will contain the Attachment Description. This is a free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |  |
| Processing Consideration Code |  | 20 | IS | 10 | RE | FALSE | AA | File: 356.22 (278) Field: 11.02 (REPORT TRANSMISSION CODE) | Loop 2000E: PWK02 Report Transmission Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will contain the Report Transmission Code. This code defines the timing, transmission method, or format by which reports are to be sent. |  |  |  |  |  |  |  |  |  |
| 'AA' (available on request at provider site), |  |  |  |  |  |  |  |  |  |
| 'BM' (by mail), |  |  |  |  |  |  |  |  |  |
| 'EL' (electronically only), |  |  |  |  |  |  |  |  |  |
| 'EM' (E-mail), |  |  |  |  |  |  |  |  |  |
| 'FX' (by fax), |  |  |  |  |  |  |  |  |  |
| 'VO' (voice) |  |  |  |  |  |  |  |  |  |
| Restricted Disclosure Indicator | 21 |  | ID | 2 | R | FALSE | NA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message; however, it is required for this HL7 segment. Therefore, this field will be set to 'NA' |  |  |  |  |  |  |  |  |  |

### RQA_I08 Request NTE segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Comment | 3 | FT | 264 | RE | FALSE | Up to 264 characters of free form text | File: 356.22 (278) Subfile: 356.2212 (MESSAGE TEXT) Field: .01 (MESSAGE TEXT) | Loop 2000E: MSG01 Free Form Message Text |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with Free Form Message Text. This is a free form format that allows the transmission of text information to the UMO about the patient event. |  |  |  |  |  |  |  |  |
| Comment Type | 4 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 4.1 | ST | 10 | RE |  | MSG 2000E | Fixed value 'MSG 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'MSG 2000E'. This allows FSC to identify this data for the X12 segment MSG in loop 2000E |  |  |  |  |  |  |  |  |

### RQA_I08 Request G30 Segment Group (Optional) (Repeating) – PRD segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role | 1 | CWE | 705 | R | TRUE |  |  |  |
| Identifier | 1.1 | ST | 3 | RE |  | 71 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .01 (PROVIDER TYPE) or Subfile: 356.2214SA (PATIENT EVENT TRANSPORT) Field: .01 (LOCATION TYPE) or Subfile: 356.2215SA (OTHER UMO) Field: .01 (UMO TYPE) | Loop 2010EA: NM101 Entity Identifier Code, PRV01 Provider Code, Loop 2010EB: NM101 Entity Identifier Code, Loop 2010EC: NM101 Entity Identifier Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated within this repeating segment group for 4 different X12 segments. |  |  |  |  |  |  |  |  |
| For the NM1 segment in loop 2010EA, this field will contain the Entity Identifier Code. |  |  |  |  |  |  |  |  |
| '71' (attending physician), |  |  |  |  |  |  |  |  |
| '72' (operating physician), |  |  |  |  |  |  |  |  |
| 'DD' (assistant surgeon), |  |  |  |  |  |  |  |  |
| 'FA' (facility), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| For the PRV segment in loop 2010EA, this field will contain the Provider Code. |  |  |  |  |  |  |  |  |
| 'AD' (admitting), |  |  |  |  |  |  |  |  |
| 'AS' (assistant surgeon), |  |  |  |  |  |  |  |  |
| 'AT' (attending), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| For the NM1 segment in loop 2010EB, this field will contain the Entity Identifier Code. |  |  |  |  |  |  |  |  |
| '45' (drop off location), |  |  |  |  |  |  |  |  |
| 'FS' (final scheduled destination), |  |  |  |  |  |  |  |  |
| etc.. |  |  |  |  |  |  |  |  |
| For the NM1 segment in loop 2010EC, this field will contain the Entity Identifier Code. |  |  |  |  |  |  |  |  |
| '00' (Alternate Insurer), |  |  |  |  |  |  |  |  |
| 'CA' (Carrier), |  |  |  |  |  |  |  |  |
| 'GG' (Intermediary) |  |  |  |  |  |  |  |  |
| Text | 1.2 | ST | 1 | RE |  | 1 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .02 (PERSON / NON-PERSON) | Loop 2010EA: NM102 Entity Type Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For the NM1 segments in loops 2010EA, 2010EB, and 2010EC, this field will contain the Entity Type Qualifier. |  |  |  |  |  |  |  |  |
| '1' (person), |  |  |  |  |  |  |  |  |
| '2' (non-person entity) |  |  |  |  |  |  |  |  |
| Alternate Identifier | 1.4 | ST | 10 | R |  | NM1 2010EA | Fixed value 'NM1 2010EA' or 'PRV 2010EA' or 'NM1 2010EB' or 'NM1 2010EC' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain text which identifies the X12 segment and Loop the data is for. The possible values are: |  |  |  |  |  |  |  |  |
| 'NM1 2010EA' |  |  |  |  |  |  |  |  |
| 'PRV 2010EA' |  |  |  |  |  |  |  |  |
| 'NM1 2010EB' |  |  |  |  |  |  |  |  |
| 'NM1 2010EC' |  |  |  |  |  |  |  |  |
| Provider Name | 2 | XPN | 1262 | RE | TRUE |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Field length has been expanded to 1262 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Family Name | 2.1 | FN | 194 | R |  |  |  |  |
| Surname | 2.1.1 | ST | 60 | R |  | Lastname | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) or File: 4 (INSTITUTION) Field: .01 (NAME) or Subfile 356.2214SA (PATIENT EVENT TRANSPORT) Field: .02 (LOCATION NAME) or Subfile: 356.2215SA (OTHER UMO) Field: .02 (UNO NAME) | Loop 2010EA: NM103 Patient Event Provider Last or Organization Name, Loop 2010EB: NM103 Patient Event Transport Location Name, Loop 2010EC: NM103 Other UMO Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For the NM1 segments in loops 2010EA, 2010EB, and 2010EC, this field will contain the value for NM103, the Name field. |  |  |  |  |  |  |  |  |
| Given Name | 2.2 | ST | 35 | RE |  | John | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010EA: NM104 Patient Event Provider First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider First Name |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 2.3 | ST | 25 | RE |  | Middle | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010EA: NM105 Patient Event Provider Middle Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider First Name |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) | 2.4 | ST | 10 | RE |  | JR | FIle: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010EA: NM107 Patient Event Provider Name Suffix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider Name Suffix |  |  |  |  |  |  |  |  |
| Prefix (e.g., DR) | 2.5 | ST | 10 | RE |  | Dr | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010EA: NM106 Patient Event Provider Name Prefix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider Name Prefix |  |  |  |  |  |  |  |  |
| Provider Address | 3 | XAD | 513 | RE | FALSE |  |  |  |
| Street Address | 3.1 | SAD | 184 | RE |  |  |  |  |
| Street or Mailing Address | 3.1.1 | ST | 55 | RE |  | Address Line 1 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .111 (STREET ADDRESS 1) or File: 4 (INSTITUTION) Field: 4.01 (ST. ADDR.1(MAILING)) or Subfile: 356.2214SA (PATIENT EVENT TRANSPORT) File: .03 (ADDRESS LINE 1) | Loop 2010EA: N301 Patient Event Provider Address Line, Loop 2010EB: N301 Patient Event Transport Location Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA and 2010EB, this field will contain the Address Line 1 data |  |  |  |  |  |  |  |  |
| Other Designation | 3.2 | ST | 55 | RE |  | Suite 200 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .112 (STREET ADDRESS 2) or File: 4 (INSTITUTION) Field: 4.02 (ST. ADDR. 2 (MAILING)) or Subfile: 356.2214SA (PATIENT EVENT TRANSAPORT) Field: .04 (ADDRESS LINE 2) | Loop 2010EA: N302 Patient Event Provider Address Line 2, Loop 2010EB: N302 Patient Event Transport Location Address Line 2 |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA and 2010EB, this field will contain the Address Line 2 data |  |  |  |  |  |  |  |  |
| City | 3.3 | ST | 30 | RE |  | City | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .114 (CITY) or File: 4 (INSTITUTION) Field: 4.03 (CITY (MAILING)) or Subfile: 356.2214SA (PATIENT EVENT TRANSPORT) Field: .05 (CITY) | Loop 2010EA: N401 Patient Event Provider City Name, Loop 2010EB: N401 Patient Event Transport Location City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA and 2010EB, this field will contain the City name |  |  |  |  |  |  |  |  |
| State or Province | 3.4 | ST | 2 | RE |  | DC | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .115 (STATE) or File: 4 (INSTITUTION) Field: 4.04 (STATE (MAILING)) or Subfile: 356.2214SA (PATIENT EVENT TRANSPORT) Field: .06 (STATE) | Loop 2010EA: N402 Patient Event Provider State or Province Code, Loop 2010EB: N402 Patient Event Transport Location State or Province Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA and 2010EB, this field will contain the State or Province Code |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 3.5 | ST | 15 | RE |  | 21044 | File: 356.22 (278) Subfile: 256.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .116 (ZIP CODE) or File: 4 (INSTITUTION) Field: 4.05 (ZIP (MAILING)) or Subfile: 356.2214SA (PATIENT EVENT TRANSPORT) Field: .07 (ZIP / POSTAL CODE) | Loop 2010EA: N403 Patient Event Provider Zip Code, Loop 2010EB: N403 Patient Event Transport Location Postal Code or Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA and 2010EB, this field will contain the State or Province Code |  |  |  |  |  |  |  |  |
| Country | 3.6 | ID | 3 | RE |  | ALB | This value is not available in the VistA database, in either the File 200 (NEW PERSON) or File 4 (INSTITUTION) | Loop 2010EA: N404 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment N4 in loop 2010EA, this field will contain the Patient Event Provider Country Code, if outside the USA |  |  |  |  |  |  |  |  |
| Other Geographic Designation | 3.8 | ST | 3 | RE |  | HID | This value is not available in the VistA database, in either the File 200 (NEW PERSON) or File 4 (INSTITUTION) | Loop 2010EA: N407 Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment N4 in loop 2010EA, this field will contain the Country Subdivision Code if the Patent Event was outside the USA and the country has subdivision Codes |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 | PLN | 100 | RE | FALSE |  |  |  |
| ID Number | 7.1 | ST | 80 | R |  | ID12344 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: 41.99 (NPI) or File: 4 (INSTITUTION) Field: 41.99 (NPI) or Field: 4.043 (TAXONOMY CODE) | Loop 2010EA: NM109 Patient Event Provider ID, PRV03 Provider Taxonomy Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider ID. |  |  |  |  |  |  |  |  |
| For X12 segment PRV in loop 2010EA, this field will contain the Provider Taxonomy Code |  |  |  |  |  |  |  |  |

### RQA_I08 Request G30 Segment Group (Optional) (Repeating) – AUT segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authorizing Payer, Plan ID | 1 | CWE | 697 | RE | FALSE |  |  |  |
| Text | 1.2 | ST | 50 | RE |  | 60 | File: 356.22 (278) Subfile: 356.2215SA (OTHER UMO) Field: .05 (UMO DENIAL REASON #3) | Loop 2010EC: REF04-4 Other UMO Denial Reason |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain Other UMO Denial Reason |  |  |  |  |  |  |  |  |
| Alternate Text | 1.5 | ST | 50 | RE |  | 60 | File: 356.22 (278) Subfile: 356.2215SA (OTHER UMO) Field: .06 (UMO DENIAL REASON #4) | Loop 2010EC: REF04-6 Other UMO Denial Reason |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain another UMO Denial Reason |  |  |  |  |  |  |  |  |
| Authorizing Payer, Company ID | 2 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 10 | R |  | REF 2010EC | Fixed value 'REF 2010EC' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain 'REF 2010EC'. This Will allow FSC to know that this data is for X12 segment REF in loop 2010EC. |  |  |  |  |  |  |  |  |
| Text | 2.2 | ST | 50 | RE |  | 60 | File: 356.22 (278) Subfile: 356.2215SA (OTHER UMO) Field: .03 (UMO DENIAL REASON #1) | Loop 2010EC: REF02 Other UMO Denial Reason |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Other UMO Denial Reason. |  |  |  |  |  |  |  |  |
| Alternate Text | 2.5 | ST | 50 | RE |  | 60 | File: 356.22 (278) Subfile: 356.2215SA (OTHER UMO) Field: .04 (UMO DENIAL REASON #2) | Loop 2010EC: REF04-2 Other UMO Denial Reason |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Other UMO Denial Reason |  |  |  |  |  |  |  |  |

### RQA_I08 Request G30 Segment Group (Optional) (Repeating) – ZTP segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date/Time Qualifier | 2 | CWE | 230 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 3 | R |  | 598 | Fixed value '598' | Loop 2010EC: DTP01 Date/Time Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Date / Time Qualifier. |  |  |  |  |  |  |  |  |
| '598' - (Rejected) |  |  |  |  |  |  |  |  |
| Date Time Period | 3 | DR | 51 | R | FALSE |  |  |  |
| Range start date/time | 3.1 | DTM | 8 | R |  | 20100228 | File: 356.22 (278) Subfile: 356.2215SA (OTHER UMO) Field: .07 (UMO DENIAL DATE) | Loop 2010EC: DTP03 Other UMO Denial Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the date of the Other UMO Denial Date. The date will be in the format CCYYMMDD |  |  |  |  |  |  |  |  |
| Loop ID | 4 | ST | 10 | R | FALSE | DTP 2010EC | Fixed value 'DTP 2010EC' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain 'DTP 2010EC'. This allows FSC to identify this data is for X12 segment DTP in loop 2010EC |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |

### RQA_I08 Request G50 Segment Group (Optional) (Repeating) – PRB segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action Code | 1 | ID | 2 | R | FALSE | CO | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .01 (REQUEST CATEGORY) | Loop 2000F: UM01 Request Category Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Request Category Code. |  |  |  |  |  |  |  |  |
| 'HS' (Health Services Review), |  |  |  |  |  |  |  |  |
| 'SC' (Specialty Care Review) |  |  |  |  |  |  |  |  |
| A mapping is required for this field because this field is linked to an HL7 table. |  |  |  |  |  |  |  |  |
| HS ==> CO |  |  |  |  |  |  |  |  |
| SC ==> AD |  |  |  |  |  |  |  |  |
| Action Date/Time | 2 | DTM | 8 | R | FALSE | 20140422 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This X12 message is not using this field; however, it is required for this HL7 segment. Therefore, this field will be set with the message transmission date, in |  |  |  |  |  |  |  |  |
| format CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem ID | 3 | CWE | 705 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 1 | R |  | I | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .02 (CERTIFICATION TYPE CODE) | Loop 2000F: UM02 Certification Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The field will contain the Certification Type Code |  |  |  |  |  |  |  |  |
| '1' (appeal - immediate), |  |  |  |  |  |  |  |  |
| '2' (appear - standard), |  |  |  |  |  |  |  |  |
| '3' (cancel), |  |  |  |  |  |  |  |  |
| '4' (extension), |  |  |  |  |  |  |  |  |
| 'I' (initial), |  |  |  |  |  |  |  |  |
| Etc… |  |  |  |  |  |  |  |  |
| Text | 3.2 | ST | 2 | RE |  | 1 | Field: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .03 (SERVICE TYPE CODE) | Loop 2000F: UM03 Service Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Service Type Code |  |  |  |  |  |  |  |  |
| '1' (medical care), |  |  |  |  |  |  |  |  |
| '2' (surgical), |  |  |  |  |  |  |  |  |
| '3' (consultation), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Problem Instance ID | 4 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier | 4.1 | ST | 199 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This X12 message will not be using this field, however, this field is required for this HL7 segment, therefore, we will set this field equal to '1' |  |  |  |  |  |  |  |  |
| Episode of Care ID | 5 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 5.1 | ST | 10 | RE |  | UM 2000F | Fixed value 'UM 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Loop identifier 'UM 2000F' |  |  |  |  |  |  |  |  |
| Problem Classification | 10 | CWE | 705 | RE | FALSE |  |  |  |
| Identifier | 10.1 | ST | 2 | RE |  | 34 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .05 (FACILITY TYPE) | Loop 2000F: UM04-1 Facility Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Facility Type Code. This code identifies where services were or may be performed; the 1st and 2nd positions of the Uniform Bill Type |  |  |  |  |  |  |  |  |
| Code for Institutional Services, or the Place of Service Codes for Professional and Dental services |  |  |  |  |  |  |  |  |
| Text | 10.2 | ST | 2 | RE |  | A | Field 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .04 (FACILITY TYPE QUALIFIER) | Loop 2000F: UM04-2 Facility Code Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with a Facility Code Qualifier |  |  |  |  |  |  |  |  |
| 'A' (uniform billing claim form bill type), |  |  |  |  |  |  |  |  |
| 'B' (place of service codes for professional or dental services) |  |  |  |  |  |  |  |  |

### RQA_I08 Request G50 Segment Group (Optional) (Repeating) – AUT segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authorizing Payer, Company ID | 2 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 10 | R |  | REF 2000F |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'REF 2000F' This is for FSC to be able to determine that this data is for X12 segment REF in loop 2000F |  |  |  |  |  |  |  |  |
| Text | 2.2 | ST | 50 | RE |  | REV123 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 9.01 (PREV. AUTHORIZATION #) or 9.02 (PREV. ADMIN. REFERENCE #) | Loop 2000F: REF02 Previous Review Authorization Number or REF02 Previous Administration Reference Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Previous Review Authorization Number if there is one |  |  |  |  |  |  |  |  |
| Alternate Text | 2.5 | ST | 3 | RE |  | BB | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 9.01 (PREV. AUTHORIZATION #) or 9.02 (PREV. ADMIN. REFERENCE #) | Loop 2000F: REF01 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with a Reference Identification Qualifier |  |  |  |  |  |  |  |  |
| 'BB' (authorization number), |  |  |  |  |  |  |  |  |
| 'NT' (Administrator's reference number) |  |  |  |  |  |  |  |  |

### RQA_I08 Request G50 Segment Group (Optional) (Repeating) – ZTP segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date/Time Qualifier | 2 | CWE | 230 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 3 | R |  | 472 | Fixed value '472' | Loop 2000F: DTP01 Date/Time Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to the Date / Time Qualifier '472' (Service) |  |  |  |  |  |  |  |  |
| Date Time Period | 3 | DR | 51 | R | FALSE |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Date or range of dates being transmitted. |  |  |  |  |  |  |  |  |
| Range start date/time | 3.1 | DTM | 8 | R |  | 20100228 | File 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .11 (SERVICE DATE) | Loop 2000F: DTP03 Proposed or Actual Service Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Proposed or Actual Service Date in the format CCYYMMDD |  |  |  |  |  |  |  |  |
| Loop ID | 4 | ST | 10 | R | FALSE | DTP 2000F | Fixed value 'DTP 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to 'DTP 2000F'. This allows FSC to determine that this data is for X12 segment DTP in loop 2000F |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |

### RQA_I08 Request G50 Segment Group (Optional) (Repeating) – PSL segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Product/Service Line Item Number | 1 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier | 1.1 | ST | 10 | R |  | SV1 2000F | Fixed value 'SV1 2000F', 'SV2 2000F', 'SV3 2000F', or 'TOO 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with one of the following values. These values provide FSC the information to determine what X12 segment and loop this data applies to. |  |  |  |  |  |  |  |  |
| 'SV1 2000F' |  |  |  |  |  |  |  |  |
| 'SV2 2000F' |  |  |  |  |  |  |  |  |
| 'SV3 2000F' |  |  |  |  |  |  |  |  |
| 'TOO 2000F' |  |  |  |  |  |  |  |  |
| Product/Service Line Item Sequence Number | 3 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not required for this X12 message, however, it is a required field for the HL7 segment, so this field will be populated with a sequential number, starting at 1 |  |  |  |  |  |  |  |  |
| Product/Service Line Item Status | 6 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 6.1 | ST | 20 | R |  | P |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message; however, this field is required for this HL7 segment. So this field will be set to 'P' for processed |  |  |  |  |  |  |  |  |
| Product/Service Code | 7 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 7.1 | ST | 3 | R |  | HC | File 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.01 (PROCEDURE CODE TYPE) | Loop 2000F: SV101-1 Product or Service ID Qualifier or SV202-1 Product or Service ID Qualifier or SV301-1 Product or Service ID Qualifier or TOO01 Code List Qualifier Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated as follows: |  |  |  |  |  |  |  |  |
| SV101-1 Product or Service ID Qualifier |  |  |  |  |  |  |  |  |
| 'HC' (Health Care Financing Administration Common Procedural Coding System (HCPCS) codes), |  |  |  |  |  |  |  |  |
| 'N4' (national drug code in 5-4-2 format |  |  |  |  |  |  |  |  |
| SV202-1 Product or Service ID Qualifier |  |  |  |  |  |  |  |  |
| 'HC' (health care financing administration common procedural coding system (HCPCS) codes), |  |  |  |  |  |  |  |  |
| 'ID' (international classification of diseases), |  |  |  |  |  |  |  |  |
| 'N4' (national drug code in 5-4-2 format), |  |  |  |  |  |  |  |  |
| 'ZZ' (mutually defined) |  |  |  |  |  |  |  |  |
| SV301-1 Product or Service ID Qualifier |  |  |  |  |  |  |  |  |
| 'AD' (American Dental Association Codes) |  |  |  |  |  |  |  |  |
| TOO01 Code List Qualifier Code |  |  |  |  |  |  |  |  |
| 'JP' (universal national tooth designation system) |  |  |  |  |  |  |  |  |
| Text | 7.2 | ST | 48 | RE |  | D9221 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) File: 1.02 (PROCEDURE #1) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .01 (TOOTH CODE) | Loop 2000F: SV101-2 Procedure Code or SV202-2 Procedure Code or SV301-2 Procedure Code or TOO02 Tooth Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV101-2 Procedure Code |  |  |  |  |  |  |  |  |
| SV202-2 Procedure Code |  |  |  |  |  |  |  |  |
| SV301-2 Procedure Code |  |  |  |  |  |  |  |  |
| TOO02 Tooth Code |  |  |  |  |  |  |  |  |
| Alternate Text | 7.5 | ST | 48 | RE |  | D9221 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.03 (PROCEDURE #2) | Loop 2000F: SV101-8 Procedure Code or SV202-8 Procedure Code Or SV301-8 Procedure Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV101-8 Procedure Code |  |  |  |  |  |  |  |  |
| SV202-8 Procedure Code |  |  |  |  |  |  |  |  |
| SV301-8 Procedure Code |  |  |  |  |  |  |  |  |
| Product/Service Code Modifier | 8 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 8.1 | ST | 2 | RE |  | 21 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.04 (PROCEDURE MODIFIER #1) | Loop 2000F: SV101-3 Procedure Modifier or SV202-3 Procedure Modifier or SV301-3 Procedure Modifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV101-3 Procedure Modifier |  |  |  |  |  |  |  |  |
| SV202-3 Procedure Modifier |  |  |  |  |  |  |  |  |
| SV301-3 Procedure Modifier |  |  |  |  |  |  |  |  |
| Text | 8.2 | ST | 2 | RE |  | 24 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.05 (PROCEDURE MODIFIER #2) | Loop 2000F: SV101-4 Procedure Modifier or SV202-4 Procedure Modifier or SV301-4 Procedure Modifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV101-4 Procedure Modifier |  |  |  |  |  |  |  |  |
| SV202-4 Procedure Modifier |  |  |  |  |  |  |  |  |
| SV301-4 Procedure Modifier |  |  |  |  |  |  |  |  |
| Alternate Identifier | 8.4 | ST | 2 | RE |  | 25 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.06 (PROCEDURE MODIFIER #3) | Loop 2000F: SV101-5 Procedure Modifier or SV202-5 Procedure Modifier or SV301-5 Procedure Modifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV101-5 Procedure Modifier |  |  |  |  |  |  |  |  |
| SV202-5 Procedure Modifier |  |  |  |  |  |  |  |  |
| SV301-5 Procedure Modifier |  |  |  |  |  |  |  |  |
| Alternate Text | 8.5 | ST | 2 | RE |  | 27 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.07 (PROCEDURE MODIFIER #4) | Loop 2000F: SV101-6 Procedure Modifier or SV202-6 Procedure Modifier or SV301-6 Procedure Modifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV101-6 Procedure Modifier |  |  |  |  |  |  |  |  |
| SV202-6 Procedure Modifier |  |  |  |  |  |  |  |  |
| SV301-6 Procedure Modifier |  |  |  |  |  |  |  |  |
| Product/Service Code Description | 9 | ST | 80 | RE | FALSE | ORAL EVALUATION | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.08 (PROCEDURE DESCRIPTION) | Loop 2000F: SV101-7 Procedure Code Description or SV202-7 Procedure Code Description or SV301-7 Procedure Code Description |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV101-7 Procedure Code Description |  |  |  |  |  |  |  |  |
| SV202-7 Procedure Code Description |  |  |  |  |  |  |  |  |
| SV301-7 Procedure Code Description |  |  |  |  |  |  |  |  |
| Product/Service Quantity | 12 | CQ | 721 | RE | FALSE |  |  |  |
| Quantity | 12.1 | NM | 15 | RE |  | 2 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.11 (UNIT COUNT) | Loop 2000F: SV104 Service Unit Count or SV205 Service Unit Count or SV306 Service Unit Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV104 Service Unit Count |  |  |  |  |  |  |  |  |
| SV205 Service Unit Count |  |  |  |  |  |  |  |  |
| SV306 Service Unit Count |  |  |  |  |  |  |  |  |
| Units | 12.2 | CWE | 20 | RE |  |  |  |  |
| � Identifier | 12.2.1 | ST | 2 | RE |  | UN | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.1 (UNITS CODE) | Loop 2000F: SV103 Unit or Basis for Measurement Code or SV204 Unit or Basis for Measurement Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV103 Unit or Basis for Measurement Code |  |  |  |  |  |  |  |  |
| 'F2' (International Unit), |  |  |  |  |  |  |  |  |
| 'MJ' (minutes), |  |  |  |  |  |  |  |  |
| 'UN' (unit) |  |  |  |  |  |  |  |  |
| SV204 Unit or Basis for Measurement Code |  |  |  |  |  |  |  |  |
| 'DA' (days), |  |  |  |  |  |  |  |  |
| 'F2' (international unit), |  |  |  |  |  |  |  |  |
| 'UN' (unit) |  |  |  |  |  |  |  |  |
| Product/Service Unit Cost | 13 | CP | 10 | RE | FALSE |  |  |  |
| Price | 13.1 | MO | 10 | R |  |  |  |  |
| Quantity | 13.1.1 | NM | 10 | R |  | 2 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.07 (RATE) | Loop 2000F: SV206 Service Line Rate |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV206 Service Line Rate The rate per unit of associate revenue for hospital accommodation |  |  |  |  |  |  |  |  |
| Product/Service Gross Amount | 15 | CP | 765 | RE | FALSE |  |  |  |
| Price | 15.1 | MO | 20 | R |  |  |  |  |
| Quantity | 15.1.1 | NM | 18 | R |  | 100 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.09 (SERVICE LINE AMOUNT) | Loop 2000F: SV102 Service Line Amount or SV203 Service Line Amount or SV302 Service Line Amount |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV102 Service Line Amount |  |  |  |  |  |  |  |  |
| SV203 Service Line Amount |  |  |  |  |  |  |  |  |
| SV302 Service Line Amount |  |  |  |  |  |  |  |  |
| � Denomination | 15.1.2 | ID | 0 | NS |  |  |  |  |
| Product/Service Clarification Code Type | 17 | IS | 48 | RE | FALSE | 313 | File 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.06 (REVENUE CODE) or Field: 3.06 (PLACEMENT STATUS) | Loop 2000F: SV201 Service Line Revenue Code or SV305 Prosthesis, Crown, or Inlay Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV201 Service Line Revenue Code |  |  |  |  |  |  |  |  |
| SV305 Prosthesis, Crown, or Inlay Code |  |  |  |  |  |  |  |  |
| Field length has been expanded to 48 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Restricted Disclosure Indicator | 21 | ID | 2 | R | FALSE | NA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, this field is required in the HL7 segment, so this field is set to 'NA' |  |  |  |  |  |  |  |  |
| Related Product/Service Code Indicator | 22 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 22.1 | ST | 2 | RE |  | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.01 (DIAGNOSIS #1) | Loop 2000F: SV107-1 Diagnosis Code Pointer |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV107-1 Diagnosis Code Pointer |  |  |  |  |  |  |  |  |
| A pointer to the diagnosis code in the order of importance to this service |  |  |  |  |  |  |  |  |
| Text | 22.2 | ST | 2 | RE |  | 2 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.02 (DIAGNOSIS #2) | Loop 2000F: SV107-2 Diagnosis Code Pointer |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV107-2 Diagnosis Code Pointer |  |  |  |  |  |  |  |  |
| A pointer to the diagnosis code in the order of importance to this service |  |  |  |  |  |  |  |  |
| Alternate Identifier | 22.4 | ST | 20 | RE |  | 3 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.03 (DIAGNOSIS #3) | Loop 2000F: SV107-3 Diagnosis Code Pointer |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV107-3 Diagnosis Code Pointer |  |  |  |  |  |  |  |  |
| A pointer to the diagnosis code in the order of importance to this service |  |  |  |  |  |  |  |  |
| Alternate Text | 22.5 | ST | 2 | RE |  | 4 | File 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.04 (DIAGNOSIS #4) | Loop 2000F: SV107-4 Diagnosis Code Pointer |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV107-4 Diagnosis Code Pointer |  |  |  |  |  |  |  |  |
| A pointer to the diagnosis code in the order of importance to this service |  |  |  |  |  |  |  |  |
| Original Text | 22.9 | ST | 80 | RE |  | FREE FORM DESCRIPTION | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.07 (DENTAL WORK DESCRIPTION) | Loop 2000F: SV307 Description |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV307 Description |  |  |  |  |  |  |  |  |
| A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
| Side of body | 33 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 33.1 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.01 (ORAL CAVITY DESIGNATION #1) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .02 (TOOTH SURFACE #1) | Loop 2000F: SV304-1 Oral Cavity Designation Code or TOO03-1 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV304-1 Oral Cavity Designation Code |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |
| TOO03-1 Tooth Surface Code |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Text | 33.2 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.02 (ORAL CAVITY DESIGNATION #2) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .03 (TOOTH SURFACE #2) | Loop 2000F: SV304-2 Oral Cavity Designation Code or TOO03-2 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV304-2 Oral Cavity Designation |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |
| TOO03-2 Tooth Surface Code |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Alternate Identifier | 33.4 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.03 (ORAL CAVITY DESIGNATION #3) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .04 (TOOTH SURFACE #3) | Loop 2000F: SV304-3 Oral Cavity Designation Code or TOO03-3 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV304-3 Oral Cavity Designation |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |
| TOO03-4 Tooth Surface Code |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |
| etc.. |  |  |  |  |  |  |  |  |
| Alternate Text | 33.5 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.04 (ORAL CAVITY DESIGNATION #4) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .05 (TOOTH SURFACE #4) | Loop 2000F: SV304-4 Oral Cavity Designation Code or TOO03-4 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV304-5 Oral Cavity Designation Code |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |
| TOO03-4 Tooth Surface Code |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Original Text | 33.9 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.05 (ORAL CAVITY DESIGNATION #5) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .06 (TOOTH SURFACE #5) | Loop 2000F: SV304-5 Oral Cavity Designation Code or TOO03-5 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV304-5 Oral Cavity Designation |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |
| TOO03-5 Tooth Surface Code |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Main-Service | 46 | ID | 1 | RE | FALSE | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.08 (NURSING HOME STATUS) | Loop 2000F: SV209 Nursing Home Residential Status Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV209 Nursing Home Residential Status Code |  |  |  |  |  |  |  |  |
| '1' (transferred to intermediate care facility - mentally retarded), |  |  |  |  |  |  |  |  |
| '2' (newly admitted), |  |  |  |  |  |  |  |  |
| '3' (newly eligible), |  |  |  |  |  |  |  |  |
| Etc.. |  |  |  |  |  |  |  |  |
| Validation | 47 | ID | 1 | RE | FALSE | N | File: 356.22 (278) Subfile: 256.2216PA (SERVICE LINE) Field: 2.05 (EPSDT?) | Loop 2000F: SV111 EPSDT Indicator |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV111 EPSDT Indicator |  |  |  |  |  |  |  |  |
| 'N' (No), |  |  |  |  |  |  |  |  |
| 'Y' (Yes) |  |  |  |  |  |  |  |  |
| Comment | 48 | ST | 1 | RE | FALSE | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.09 (NURSING HOME LEVEL OF CARE) | Loop 2000F: SV120 Nursing Home Level of Care Or SV210 Nursing Home Level of Care |
| Implementation Note: |  |  |  |  |  |  |  |  |
| SV120 Nursing Home Level of Care '1' (skilled nursing facility), '2' (intermediate care facility), '8' (hospice), etc… SV210 Nursing Home Level of Care '1' (skilled nursing facility), '2' (intermediate care facility), '3' (intermediate care facility – mentally retarded), etc… |  |  |  |  |  |  |  |  |

### RQA_I08 Request G50 Segment Group (Optional) (Repeating) – ZHS segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID | 1 | SI | 9 | RE | FALSE | HSD 2000F | Fixed value 'HSD 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with 'HSD 2000F'. This allows FSC to identify this data is for X12 segment HSD in loop 2000F |  |  |  |  |  |  |  |  |
| Field length has been expanded to 9 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Quantity Qualifier | 2 | CWE | 99 | RE | FALSE |  |  |  |
| identifier | 2.1 | ST | 2 | R |  | DY | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.01 (HCSD QUANTITY QUALIFIER) | Loop 2000F: HSD01 Quantity Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Quantity Qualifier 'DY' (days), 'FL' (units), 'HS' (hours), 'MN' (month), 'VS' (visits) |  |  |  |  |  |  |  |  |
| Benefit Quantity | 3 | NM | 15 | RE | FALSE | 123 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.02 (HCSD SERVICE UNIT COUNT) | Loop 2000F: HSD02 Service Unit Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Unit Count |  |  |  |  |  |  |  |  |
| Unit or Basis for Measurement Code | 4 | CWE | 20 | RE | FALSE |  |  |  |
| identifier | 4.1 | ST | 2 | R |  | DA | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.03 (HCSD UNITS OF MEANSUREMENT) | Loop 2000F: HSD03 Unit or Basis for Measurement Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Unit or Basis for Measurement Code 'DA' (days), 'MO' (months), 'WK' (week) |  |  |  |  |  |  |  |  |
| Sample Selection Modulus | 5 | ST | 6 | RE | FALSE | 1.5 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.04 (HCSD SAMPLE SELECTION MODULUS) | Loop 2000F: HSD04 Sample Selection Modulus |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Sample Selection Modulus. |  |  |  |  |  |  |  |  |
| To specify the sampling frequency in terms of a modulus of the Unit of Measure, e.g., every fifth bag, every 1.5 Minutes |  |  |  |  |  |  |  |  |
| Time Period Qualifier | 6 | CWE | 20 | RE | FALSE |  |  |  |
| identifier | 6.1 | ST | 2 | R |  | 21 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.05 (HCSD TIME PERIOD QUALIFIER) | Loop 2000F: HSD05 Time Period Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Time Period Qualifier '6' (hour), '7' (day), '21' (years), '26' (episode), '27' (visit), '34' (month), '35' (week) |  |  |  |  |  |  |  |  |
| Number of Periods | 7 | NM | 3 | RE | FALSE | 5 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.06 (HCSD PERIOD COUNT) | Loop 2000F: HSD06 Period Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Period Count |  |  |  |  |  |  |  |  |
| Used if needed to provide further information about the number and frequency of benefits. |  |  |  |  |  |  |  |  |
| Ship/Delivery or Calendar Pattern Code | 8 | CWE | 20 | RE | FALSE |  |  |  |
| identifier | 8.1 | ST | 2 | R |  | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.07 (HCSD DELIVERY FREQUENCY) | Loop 2000F: HSD07 Delivery Frequency Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Delivery Frequency Code '1' (1st week of the month), '2' (2nd week of the month), 'A' (Monday through Friday), etc… |  |  |  |  |  |  |  |  |
| Ship/Delivery Pattern Time Code | 9 | CWE | 20 | RE | FALSE |  |  |  |
| identifier | 9.1 | ST | 1 | R |  | A | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.08 (HCSD DELIVERY TIME PATTERN) | Loop 2000F: HSD08 Delivery Pattern Time Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Delivery Pattern Time Code |  |  |  |  |  |  |  |  |
| '1' (1st week of the month), '2' (2nd week of the month), 'A' (Monday through Friday), etc… |  |  |  |  |  |  |  |  |

### RQA_I08 Request G50 Segment Group (Optional) (Repeating) – PSL segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Product/Service Line Item Number | 1 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier | 1.1 | ST | 10 | R |  | PWK 2000F | Fixed value 'PWK 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is set to 'PWK 2000F'. This provides FSC a way to determine that this data is for X12 segment PWK in loop2000F |  |  |  |  |  |  |  |  |
| Product/Service Line Item Sequence Number | 3 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This X12 message is not using this field, however it is a required field for this HL7 segment, so this field will be populated with a sequential numeric value, starting at 1 |  |  |  |  |  |  |  |  |
| Product/Service Line Item Status | 6 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 6.1 | ST | 20 | R |  | P |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however this field is required for this HL7 segment, so this field is populated with 'P' |  |  |  |  |  |  |  |  |
| Product/Service Code | 7 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 7.1 | ST | 1 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, it is an HL7 required field, so it will be set to ' 1 ' |  |  |  |  |  |  |  |  |
| Health Document Reference Identifier | 19 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 19.1 | ST | 2 | RE |  | 3 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 6.01 (REPORT TYPE CODE) | Loop 2000F: PWK01 Attachment Report Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Attachment Report Type Code |  |  |  |  |  |  |  |  |
| '03' (report justifying treatment beyond utilization guidelines), |  |  |  |  |  |  |  |  |
| '04' (drugs administered), |  |  |  |  |  |  |  |  |
| '05' (treatment diagnosis), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Namespace ID | 19.2 | IS | 80 | RE |  | Att123 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 6.03 (ATTACHMENT CONTROL NUMBER) | Loop 2000F: PWK06 Attachment Control Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Attachment Control Number |  |  |  |  |  |  |  |  |
| Universal ID | 19.3 | ST | 80 | RE |  | Description of Attachment | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 6.04 (ATTACHMENT DESCRIPTION) | Loop 2000F: PWK07 Attachment Description |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Attachment Description |  |  |  |  |  |  |  |  |
| Processing Consideration Code | 20 | IS | 10 | RE | FALSE | 5 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 6.02 (REPORT TRANSMISSION CODE) | Loop 2000F: PWK02 Report Transmission Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Report Transmission Code |  |  |  |  |  |  |  |  |
| '03' (report justifying treatment beyond utilization guidelines), |  |  |  |  |  |  |  |  |
| '04' (drugs administered), |  |  |  |  |  |  |  |  |
| '05' (treatment diagnosis), |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |
| Restricted Disclosure Indicator | 21 | ID | 2 | R | FALSE | NA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This X12 message is not using this field, however it is required for this HL7 segment, so this field will be populated with 'NA' |  |  |  |  |  |  |  |  |

### RQA_I08 Request G50 Segment Group (Optional) (Repeating) – NTE segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Comment | 3 | FT | 264 | RE | FALSE | Up to 264 characters of free form text | File: 356.22 (278) Subfile 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22167 (MESSAGE TEXT) Field: .01 (MESSAGE TEXT) | Loop 2000F: MSG01 Free Form Message Text |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with Free Form Message Text |  |  |  |  |  |  |  |  |
| Comment Type | 4 | CWE | 250 | RE | FALSE |  |  |  |
| Identifier | 4.1 | ST | 10 | RE |  | MSG 2000F | Fixed value 'MSG 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is set to 'MSG 2000F'. FSC can use this information to know that this data is for X12 segment MSG in loop 2000F |  |  |  |  |  |  |  |  |

### RQA_I08 Request G50 Segment Group (Optional) (Repeating) – PRD segment (Optional) (Repeating)

| Name |  | Seq |  | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role |  | 1 |  | CWE | 250 | R | TRUE |  |  |  |
| Identifier |  | 1.1 |  | ST | 3 | RE |  | 1T | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .01 (PROVIDER TYPE) | Loop 2010F: NM101 Entity Identifier Code Or PRV01 Provider Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| NM101 Entity Identifier Code |  |  |  |  |  |  |  |  |  |  |
| '1T' (physician, clinic or group practice), |  |  |  |  |  |  |  |  |  |  |
| '72' (operating physician), |  |  |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |  |  |
| PRV01 Provider Code |  |  |  |  |  |  |  |  |  |  |
| 'AS' (assistant surgeon), |  |  |  |  |  |  |  |  |  |  |
| 'OP' (operating), |  |  |  |  |  |  |  |  |  |  |
| etc… |  |  |  |  |  |  |  |  |  |  |
| Text |  | 1.2 |  | ST | 1 | RE |  | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .02 (PERSON / NON-PERSON) | Loop 2010F: NM102 Entity Type Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Entity Type Qualifier |  |  |  |  |  |  |  |  |  |  |
| '1' (person), |  |  |  |  |  |  |  |  |  |  |
| '2' (non-person entity) |  |  |  |  |  |  |  |  |  |  |
| Alternate Identifier |  | 1.4 |  | ST | 10 | R |  | NM1 2010F | Fixed value 'NM1 2010F' or 'PRV 2010F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with one of the following, which allows FSC to determine which X12 segment and loop the data belongs to. |  |  |  |  |  |  |  |  |  |  |
| 'NM1 2010F' |  |  |  |  |  |  |  |  |  |  |
| 'PRV 2010F' |  |  |  |  |  |  |  |  |  |  |
| Provider Name |  | 2 |  | XPN | 264 | RE | FALSE |  |  |  |
| Family Name |  | 2.1 |  | FN | 194 | R |  |  |  |  |
| � Surname |  | 2.1.1 |  | ST | 60 | R |  | Organization Name | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) or File: 4 (INSTITUTION) Field: .01 (NAME) | Loop 2010F: NM103 Service Provider Last Name or Organization Name |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field will be populated with the Service Provider Last Name or Organization Name |  |  |  |  |  |  |  |  |  |  |
| Given Name |  | 2.2 |  | ST | 35 | RE |  | John | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010F: NM104 Service Provider First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider First Name |  |  |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 2.3 |  |  | ST | 25 | RE |  | Middle | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010F: NM105 Service Provider Middle Name |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider Middle Name |  |  |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) |  | 2.4 |  | ST | 10 | RE |  | JR | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010F: NM107 Service Provider Name Suffix |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider Name Suffix |  |  |  |  |  |  |  |  |  |  |
| Provider Address |  | 3 |  | XAD | 292 | RE | TRUE |  |  |  |
| Street Address |  | 3.1 |  | SAD | 184 | RE |  |  |  |  |
| Street or Mailing Address |  | 3.1.1 |  | ST | 55 | RE |  | Address Line 1 | FIle: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .111 (STREET ADDRESS 1) or File: 4 (INSTITUTION) Field: 4.01 (ST. ADDR 1 (MAILING)) | Loop 2010F: N301 Service Provider Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider Address Line 1 |  |  |  |  |  |  |  |  |  |  |
| Other Designation |  | 3.2 |  | ST | 55 | RE |  | Suite 2 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .112 (STREET ADDRESS 2) or File: 4 (INSTITUTION) Field: 4.02 (ST. ADDR. 2 (MAILING)) | Loop 2010F: N302 Service Provider Address Line 2 |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Providers Address Line 2, if available |  |  |  |  |  |  |  |  |  |  |
| City |  |  | 3.3 | ST | 30 | RE |  | City | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .114 (CITY) or File: 4 (INSTITUTION) Field: 4.03 (CITY (MAILING)) | Loop 2010F: N401 Service Provider City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider City Name |  |  |  |  |  |  |  |  |  |  |
| State or Province |  |  | 3.4 | ST | 2 | RE |  | VA | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .115 (STATE), File: 4 (INSTITUTION) Field: 4.04 (STATE (MAILING)) | Loop 2010F: N402 Service Provider State or Province Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider State or Province Code |  |  |  |  |  |  |  |  |  |  |
| Zip or Postal Code |  |  | 3.5 | ST | 15 | RE |  | 20133 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .116 (ZIP CODE) or File: 4 (INSTITUTION) Field: 4.05 (ZIP (MAILING)) | Loop 2010F: N403 Service Provider Postal Zone or Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider Postal Zone or Zip Code |  |  |  |  |  |  |  |  |  |  |
| Country |  |  | 3.6 | ID | 3 | RE |  | ALB | This value is not available in the VistA database, in either the File 200 (NEW PERSON) or File 4 (INSTITUTION) | Loop 2010F: N404 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider's Country Code of location, if outside of the USA |  |  |  |  |  |  |  |  |  |  |
| Other Geographic Designation |  | 3.8 |  | ST | 3 | RE |  | JAL | This value is not available in the VistA database, in either the File 200 (NEW PERSON) or File 4 (INSTITUTION) | Loop 2010F: N407 Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider's Country Subdivision Code, if outside the USA and Subdivision Codes exist |  |  |  |  |  |  |  |  |  |  |
| Provider Identifiers |  | 7 |  | PLN | 100 | RE | FALSE |  |  |  |
| ID Number |  | 7.1 |  | ST | 80 | R |  | 123456789 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field .03 (PROVIDER), File: 200 (NEW PERSON) Field 41.99 (NPI) or File: 4 (INSTITUTION) Field: 41.99 (NPI) or Field: 4.043 (TAXONOMY CODE) | Loop 2010F: NM109 Service Provider ID or PRV03 Provider Taxonomy Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| For NM109, this field is the Service Provider ID. This will be the NPI number for the service provider |  |  |  |  |  |  |  |  |  |  |
| For PRV03, this will be the Provider Taxonomy Code |  |  |  |  |  |  |  |  |  |  |
| Type of ID Number |  | 7.2 |  | IS | 2 | R |  | XX | Fixed value 'XX' or 'PXC' depending on the segment and loop | Loop 2010F: PRV02 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |
| For NM108, this field is populated with the Provider Identification Code Qualifier. |  |  |  |  |  |  |  |  |  |  |
| 'XX' (CMS National Provider Identifier) |  |  |  |  |  |  |  |  |  |  |
| For PRV02, this field is the Reference Identification Qualifier. |  |  |  |  |  |  |  |  |  |  |
| 'PXC' (Health Care Provider Taxonomy Code) |  |  |  |  |  |  |  |  |  |  |

## X12 278 Response: Message Type RPA_I08

The RQA-I08 Message Type query is constructed of the following HL7 segments in the order listed:

- MSH – Message Header: Information for X12 BHT Header segment

- EVN – Event Type: Information for X12 BHT Header segment

- IN1 – Insurance: Information for X12 NM1 UMO Name segment

- PRD – Provider Data: Information for X12 NM1 Requester Name segment, N3 Requester Address Segment, N4 Requester City, State, Zip

Code segment

- [CTD] – Contact Data: Information for X12 PER Requester Contact Information segment

- [PRD] – Provider Data: Information for X12 PRV Requester Provider Information segment

- GT1 – Guarantor: Information for X12 NM1 Subscriber segment, REF Subscriber segment, N3 Subscriber segment N4 Subscriber City,

State, Zip Code segment, DMG Subscriber Demographic segment, INS Subscriber segment

- PID – Patient Identification: Information for X12 NM1 Dependent Name segment, REF Dependent Supplemental Identification segment, N3

Dependent Address segment, N4 Dependent City, State, Zip Code segment, DMG Dependent Demographic Information segment,

INS Dependent Relationship segment

- [(G2R segment group)] –

  - PRB – Problem Details: Information for X12 UM Health Care Services Review Information segment

  - [PV1] – Patient Visit: Information for X12 UM Health Care Services Review Information segment

- [AUT] – Authorization Information: Information for X12 REF Previous Review Authorization Number segment

- {[ZTP]} – Subscriber Date: Information for X12 DTP Accident Date segment, DTP Last Menstrual Period Date segment, DTP Estimated Date

of Birth segment, DTP Onset of Current Symptoms or Illness Date segment, DTP Event Date segment, DTP Admission Date

segment, DTP Discharge Date segment

- {[DG1]} – Diagnosis: Information for X12 HI Patient Diagnosis segment

- [ZHS] -- Health Care Services Delivery (HSD): Information for X12 HSD Health Care Services Delivery segment

- {[PV1]} – Patient Visit: Information for X12 CRC Certification Information segment, CL1 Institutional Claim Code segment,

- [OBR] – Observation Request: Information for X12 CR1 Ambulance Transport segment

- [RXA] -- Pharmacy/Treatment Administration: Information for X12 CR2 Spinal Manipulation Service Information segment

- [RXE] – Pharmacy/Treatment Encoded Order: Information for X12 CR5 Home Oxygen Therapy segment

- [PRB] – Problem Details: Information for X12 CR6 Home Health Care Information segment

- {[PSL]} – Product/Service Line Item: Information for X12 PWK Additional Patient Information segment

- [NTE] – Notes and Comments: Information for X12 MSG Message Text segment

- {[G30 segment group]} –

  - [PRD] – Provider Data: Information for X12 NM1 Patient Event Provider Name segment, N3 Patient Event Provider Address

segment, N4 Patient Event Provider City, State, Zip Code segment. PRV Patient Event Provider Information segment, NM1

Patient Event Transport Information segment, N3 Patient Event Transport Address segment, N4 Patient Event Transport

City/State/Zip Code segment, NM1 Patient Event Other UMO Name segment

  - [AUT] – Authorization Information: Information for X12 REF Other UMO Denial Reason segment

  - [ZTP] – Subscriber Date: Information for X12 DTP Other UMO Denial Date segment

- {[G50 segment group]} –

  - [PRB] – Problem Details: Information for X12 UM Health Care Services Review Information segment

  - [AUT] – Authorization Information: Information for X12 REF Previous Review Authorization Number segment, REF Administrative

Reference Number segment

  - [ZTP] – Subscriber Date: Information for X12 DTP Service Date segment

  - {[PSL]} – Product/Service Line Item: Information for X12 SV1 Professional Service segment, SV2 Institutional Service Line segment,

SV3 Dental Service segment, TOO Tooth Information segment

  - [ZHS] – Health Care Services Delivery (HSD): Information for X12 HSD Health Care Service Delivery segment

  - {[PSL]} – Product/Service Line Item: Information for X12 PWK Additional Service Information segment

  - [NTE] – Notes and Comments: Information for X12 MSG Message Text segment

  - {[PRD]} – Provider Data: Information for X12 NM1 Service Provider Name segment, N3 Service Provider Address segment, N4

Service Provider City, State, Zip Code segment, PRV Service Provider Information segment

The notation is:

MSH,EVN,{[MSA]},IN1,{[MSA]},PRD,{[MSA]},[PRD],GT1,{[MSA]},[PID],{[MSA]},[(G2R)PRB,[PV1]]{[AUT]},{[ZTP]},{[DG1]},[ZHS],[PV1],[OBR],[RXA],[RXE],[PRB],{[PSL]},[NTE],{(G3O)PRD,{[MSA]},[NK1],[CTD],{[PRD]},{[MSA]}}{(G5O){[MSA]},PRB,{[AUT]},{[ZTP]},{[DG1]},{[PSL]},[ZHS],{[PSL]},[NTE],{[PRD]},{[MSA]},{[PRD]},[CTD]}

### RPA_I08 Response MSH segment (Required)

| Name | Seq |  | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Field Separator | 1 |  | ST | 1 | R | FALSE | ¦ |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| Field separator is '¦' |  |  |  |  |  |  |  |  |  |
| Encoding Characters | 2 |  | ST | 4 | R | FALSE | ^~\& |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| Encoding Characters are "^~\&" |  |  |  |  |  |  |  |  |  |
| Sending Application | 3 |  | HD | 180 | R | FALSE |  |  |  |
| Namespace ID | 3.1 |  | IS | 10 | R |  | FSC |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| Set to "FSC" |  |  |  |  |  |  |  |  |  |
| Sending Facility | 4 |  | HD | 180 | R | FALSE |  |  |  |
| Namespace ID | 4.1 |  | IS | 3 | R |  | FSC |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| 'FSC' |  |  |  |  |  |  |  |  |  |
| Universal ID | 4.2 |  | ST | 30 | R |  | IIV.VITRIA-EDI.AAC.VA.GOV |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| Address of system at FSC |  |  |  |  |  |  |  |  |  |
| Universal ID Type | 4.3 |  | ID | 3 | R |  | DNS |  |  |
| Receiving Application | 5 |  | HD | 180 | R | FALSE |  |  |  |
| Namespace ID | 5.1 |  | IS | 20 | R |  | VISTA 278 |  |  |
| Receiving Facility | 6 |  | HD | 180 | R | FALSE |  |  |  |
| Namespace ID | 6.1 |  | IS | 4 | R |  | 122 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| The VistA site's station number |  |  |  |  |  |  |  |  |  |
| Universal ID | 6.2 |  | ST | 60 | R |  | AUGUSTA.MED.VA.GOV |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| The VistA site's Domain Name System |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |  |  |  |
| Universal ID Type | 6.3 |  | ID | 3 | R |  | DNS |  |  |
| Date/Time of Message | 7 |  | DTM | 26 | R | FALSE | 20040328134602.1234+0600 | File: 356.22 (278) Field:.1 (DATE MARKED IN PROGRESS) | BHT04 Transaction Set Creation Date and BHT05 Transaction Set Creation Time |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| YYYYMMDD[HHHMM[SS[.SSSS]]][+-ZZZZ] |  |  |  |  |  |  |  |  |  |
| Message Type | 9 |  | MSG | 15 | R | FALSE |  |  |  |
| Message Code | 9.1 |  | ID | 3 | R |  | RPA |  |  |
| Trigger Event | 9.2 |  | ID | 3 | R |  | I08 |  |  |
| Message Control ID | 10 |  | ST | 50 | R | FALSE | 509772 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) | BHT03 Submitter Transaction ID |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| VistA application will set this value per |  |  |  |  |  |  |  |  |  |
| each 278 request transaction |  |  |  |  |  |  |  |  |  |
| Processing ID | 11 |  | PT | 7 | R | FALSE |  |  |  |
| Processing ID | 11.1 |  | ID | 3 | R |  | T |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| Default Processing ID |  |  |  |  |  |  |  |  |  |
| "P" = Production |  |  |  |  |  |  |  |  |  |
| "T" = Test |  |  |  |  |  |  |  |  |  |
| Version ID |  | 12 | VID | 60 | R | FALSE |  |  |  |
| Version ID |  | 12.1 | ID | 5 | RE |  | 2.6 |  |  |
| Accept Acknowledgment Type |  | 15 | ID | 2 | R | FALSE | AL |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| AL = "Always" |  |  |  |  |  |  |  |  |  |
| Application Acknowledgment Type | 16 |  | ID | 2 | R | FALSE | NE |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| "NE" = Never |  |  |  |  |  |  |  |  |  |
| Country Code | 17 |  | ID | 3 | R | FALSE | USA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| "USA" |  |  |  |  |  |  |  |  |  |

### RPA_I08 Response EVN segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Recorded Date/Time | 2 | DTM | 24 | R | FALSE | 20140424 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not required for this X12 278 |  |  |  |  |  |  |  |  |
| Request message, so VistA will populate |  |  |  |  |  |  |  |  |
| this field with the Date/Time of message |  |  |  |  |  |  |  |  |
| creation |  |  |  |  |  |  |  |  |
| Event Reason Code | 4 | IS | 3 | R | FALSE | 19 |  | BHT06 Transaction Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| 18' - Response, no further updates to follow |  |  |  |  |  |  |  |  |
| 19' - Response, further updates to follow |  |  |  |  |  |  |  |  |
| 'AT' - administrative action |  |  |  |  |  |  |  |  |
| 'RU' - medical services reservation |  |  |  |  |  |  |  |  |

### RPA_I08 Response MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however it is an HL7 required field, so it will be populated with 'AE' for any application errors reported by FSC |  |  |  |  |  |  |  |  |
| AR' for AAA reject reasons reported by HCCH |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 4564654 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field for HL7, so it should be set to the |  |  |  |  |  |  |  |  |
| Message Control ID (MSH.10) of the original Request 278 message |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 10 | RE | FALSE | AAA 2000A | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'AAA 2000A', identifying this data is from segment AAA in the 2000A loop. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | P | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2000A: AAA04 Follow-up Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field value will be set to the value found in the AAA segment 04 Follow-up Action Code field. Value should be: |  |  |  |  |  |  |  |  |
| 'N' - resubmission not allowed |  |  |  |  |  |  |  |  |
| 'P' - please resubmit original transaction |  |  |  |  |  |  |  |  |
| Y' - do not resubmit. We will hold your request and respond again shortly |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CWE | 483 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 | RE |  | 41 | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) | Loop 2000A: AAA03 Reject Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Reject Reason Code (if applicable) |  |  |  |  |  |  |  |  |
| '04' - authorized quantity exceeded |  |  |  |  |  |  |  |  |
| '41' - authorization/access restrictions) |  |  |  |  |  |  |  |  |
| '42' - unable to respond at current time |  |  |  |  |  |  |  |  |
| '79' - invalid participant identification |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 80 | RE |  | Error text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be used for FSC to report text describing an error condition that was not reported by the HCCH via X12 |  |  |  |  |  |  |  |  |
| Alternate Identifier | 6.4 | ST | 1 | RE |  | N | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .03 (VALID REQUEST INDICATOR) | Loop 2000A: AAA01 Valid Request Indicator |
| Implementation Note: |  |  |  |  |  |  |  |  |
| 'N' - No |  |  |  |  |  |  |  |  |
| 'Y' - Yes |  |  |  |  |  |  |  |  |

### RPA_I08 Response IN1 segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID - IN1 | 1 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Not required for X12 message, Sending application will set this value and it will be |  |  |  |  |  |  |  |  |
| ignored by VistA application |  |  |  |  |  |  |  |  |
| Insurance Company ID | 3 | CX | 250 | R | TRUE |  |  |  |
| ID Number | 3.1 | ST | 3 | R |  | VA1 | File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY), File: 2 (PATIENT) Field: 2.312 (INSURANCE TYPE), File: 36 (PAYER) Field: 3.1, File: 365.12 (PAYER) Field: Either .02 (VA NATIONAL ID) or .03 (CMS NATIONAL ID) | Loop 2010A: NM109 UMO (Utilization Management Org) ID |
| Implementation Note: |  |  |  |  |  |  |  |  |
| UMO Identifier |  |  |  |  |  |  |  |  |
| Identifier Type Code | 3.5 | ID | 3 | R |  | PI | This value will quantify NM109 value. 'PI' for Payer Identification or 'XV' for CMS Plan ID | Loop 2010A: NM108 Identification Code Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| 46' - electronic transmitter identification number (ETIN) |  |  |  |  |  |  |  |  |
| 'PI' - Payer Identification before HPID |  |  |  |  |  |  |  |  |
| 'XV' - CMS Plan ID |  |  |  |  |  |  |  |  |
| Insurance Company Name | 4 | XON | 250 | RE | TRUE |  |  |  |
| Organization Name | 4.1 | ST | 60 | RE |  | INSURANCE NAME | This value will reflect the UMO Last Name as found in: File: 356.22 (278) Field: .02 (PATIENT) and .03 (INSURANCE POLICY) File: 2 (PATIENT) Field: 2.312 (INSURANCE TYPE), File: 36 (INSURANCE COMPANY) Field: .01 (NAME) | Loop 2010A: NM103 UMO Last or Organization Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM103 UMO Last or Organization Name |  |  |  |  |  |  |  |  |
| Identifier Type Code | 4.7 | ID | 3 | RE |  | XX |  | Loop 2010A: NM101 Entity Identifier Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Value from NM101 Entity Identifier Code field in X12 NM1 segment, loop 2010A |  |  |  |  |  |  |  |  |
| 'X3' - Utilization Management Organization |  |  |  |  |  |  |  |  |
| 'PR' – Payer |  |  |  |  |  |  |  |  |
| This field is associated with an HL7 table, so a mapping is required |  |  |  |  |  |  |  |  |
| 'X3' ==> 'XX' |  |  |  |  |  |  |  |  |
| 'PR' ==> 'U' |  |  |  |  |  |  |  |  |
| Insurance Co Contact Person | 6 | XPN | 250 | RE | FALSE |  |  |  |
| Family Name | 6.1 | FN | 60 | RE |  |  |  |  |
| Surname | 6.1.1 | ST | 60 | R |  | UMO Contact | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .06 (CONTACT NAME) | Loop 2010A: PER02 UMO Contact Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated from the X12 segment PER field 02 UMO Contact Name in loop 2010A |  |  |  |  |  |  |  |  |
| Insurance Co Phone Number | 7 | XTN | 250 | RE | TRUE |  |  |  |
| Telecommunication Equipment Type | 7.2 | ID | 5 | RE |  | PH | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .07 (COMMUNICATION NO. QUALIFIER-1) or .08 (COMMUNICATION NO. QUALIFIER-2) or .09 (COMMUNICATION NO. QUALIFIER-3) | Loop 2010A: 1st occurrence: If 'X.400' then PER03='EM' If 'FX' then PER03='FX' If 'PH' then PER03='TE' 2nd occurrence: If 'X.400' then PER05='EM' If 'FX' then PER05='FX' If 'PH' then PER05='TE' 3rd occurrence: If 'X.400' then PER07='EM' If 'FX' then PER07='FX' If 'PH' then PER07='TE' |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If PER03 = 'EM', then this field = 'X.400' |  |  |  |  |  |  |  |  |
| If PER03 = 'FX', then this field = 'FX' |  |  |  |  |  |  |  |  |
| If PER03 = 'TE', then this field = 'PH' |  |  |  |  |  |  |  |  |
| same for PER05 and PER07 A mapping is required because this field is linked to an HL7 table. ‘X.400’  ‘EM’ (Electronic Mail) ‘MD’  ‘EX’ (Telephone Extension) ‘FX’  ‘FX’ (Facsimile) ‘PH’  ‘TE’ (Telephone) ‘Internet’  ‘UR’ (URL) |  |  |  |  |  |  |  |  |
| Communication Address | 7.3 | ST | 256 | RE |  | name@domain.com | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 1 (CONTACT COMMUNICATION NUMBER-1) or Field: 2 (CONTACT COMMUNICATION NUMBER-2) or Field: 3 (CONTACT COMMUNICATION NUMBER-3) | Loop 2010A: If PER03='EM' or 'UR' then PER04 Communication Number If PER05='EM' or 'UR' then PER06 Communication Number If PER07='EM' or 'UR' then PER08 Communication Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated when |  |  |  |  |  |  |  |  |
| PER03='EM', this field will contain email address. |  |  |  |  |  |  |  |  |
| When PER03='UR', then this field will contain URL |  |  |  |  |  |  |  |  |
| Unformatted Telephone number | 7.11 | ST | 256 | RE |  |  | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 1 (CONTACT COMMUNICATION NUMBER-1) or Field: 2 (CONTACT COMMUNICATION NUMBER-2) or Field: 3 (CONTACT COMMUNICATION NUMBER-3) | Loop 2010A: If PER03='FX' or 'TE' then PER04 Communication Number If PER05='FX' or 'TE' then PER06 Communication Number If PER07='FX' or 'TE' then PER08 Communication Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated when |  |  |  |  |  |  |  |  |
| PER03='FX' or when ='TE', with value in |  |  |  |  |  |  |  |  |
| PER04, phone number. |  |  |  |  |  |  |  |  |
| Same for PER05 and PER07 |  |  |  |  |  |  |  |  |
| Release Information Code | 27 | IS | 1 | RE | FALSE | Y |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| VistA application will populate this field value with one of the following: |  |  |  |  |  |  |  |  |
| 'M' (limited or restricted), |  |  |  |  |  |  |  |  |
| 'Y' (provider has signed statement to permit release of medical billing data) |  |  |  |  |  |  |  |  |

### RPA_I08 Response MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however it is an HL7 required |  |  |  |  |  |  |  |  |
| field, so it will be populated with 'AE' for any application errors reported by FSC |  |  |  |  |  |  |  |  |
| 'AR' for AAA reject reasons reported by HCCH |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 6785675 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field |  |  |  |  |  |  |  |  |
| for HL7, so it should be set to the Message Control ID (MSH.10) of the |  |  |  |  |  |  |  |  |
| original Request 278 message |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 10 | RE | FALSE | AAA 2010A | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'AAA 2010A', identifying this data is from |  |  |  |  |  |  |  |  |
| segment AAA in the 2010A loop. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Expected Sequence Number | 4 | NM | 15 | NS | FALSE |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | N | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2010A: AAA04 Follow-up Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field value will be set to the value found in the AAA segment 04 Follow-up |  |  |  |  |  |  |  |  |
| Action Code field. Value should be: |  |  |  |  |  |  |  |  |
| 'N' - resubmission not allowed |  |  |  |  |  |  |  |  |
| 'P' - please resubmit original transaction |  |  |  |  |  |  |  |  |
| Y' - do not resubmit. We will hold your request and respond again shortly |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CWE | 483 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 | RE |  | 79 | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) | Loop 2010A: AAA03 Reject Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Reject Reason Code (if applicable) |  |  |  |  |  |  |  |  |
| '42' - unable to respond at current time |  |  |  |  |  |  |  |  |
| '79' - invalid participant identification |  |  |  |  |  |  |  |  |
| '80' - no response received |  |  |  |  |  |  |  |  |
| 'T4' - payer name or identifier missing |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 80 | RE |  | Error text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be used for FSC to report text describing an error condition that was not reported by the HCCH via X12 |  |  |  |  |  |  |  |  |

### RPA_I08 Response PRD segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role | 1 | CWE | 250 | R | TRUE |  |  |  |
| Alternate Identifier | 1.4 | ST | 10 | R |  | NM1 2010B |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will set this field equal to 'NM1 2010B' to indicate this is for X12 |  |  |  |  |  |  |  |  |
| segment NM1 in loop 2010B |  |  |  |  |  |  |  |  |
| Provider Name | 2 | XPN | 250 | RE | TRUE |  |  |  |
| Family Name | 2.1 | FN | 194 | RE |  |  |  |  |
| Surname | 2.1.1 | ST | 60 | R |  | VAMC NAME | This value will reflect what is found in File: 4 (INSTITUTION) Field: 100 (OFFICIAL VA NAME) | Loop 2010B: NM103 Requester Last or Organization Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will set this field value equal to the value found in NM103 |  |  |  |  |  |  |  |  |
| Requester Last or Organization Name |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 | PLN | 100 | R | TRUE |  |  |  |
| ID Number | 7.1 | ST | 100 | R |  | 123456789 | File: 4 (INSTITUTION) Field: 41.99 (NPI) | Loop 2010B: NM109 Requester ID |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field value with the value found in |  |  |  |  |  |  |  |  |
| NM1 segment, field 09 Requester ID |  |  |  |  |  |  |  |  |

### RPA_I08 Response MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however it is an HL7 required field, so it will be populated with |  |  |  |  |  |  |  |  |
| AE' for any application errors reported by FSC |  |  |  |  |  |  |  |  |
| AR' for AAA reject reasons reported by HCCH |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 56756566 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field for HL7, so it should be set to the |  |  |  |  |  |  |  |  |
| Message Control ID (MSH.10) of the original Request 278 message |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 10 | RE | FALSE | AAA 2010B | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'AAA 2010B', identifying this data is from segment AAA in the 2010B loop. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | Y | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2010B: AAA04 Follow-up Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field value will be set to the value found in the AAA segment 04 Follow-up |  |  |  |  |  |  |  |  |
| Action Code field. Value should be: |  |  |  |  |  |  |  |  |
| 'C' - please correct and resubmit |  |  |  |  |  |  |  |  |
| 'P' - resubmission not allowed |  |  |  |  |  |  |  |  |
| 'Y' - resubmission allowed |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CWE | 483 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 | RE |  | 41 | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) | Loop 2010B: AAA03 Reject Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Reject Reason Code (if applicable) |  |  |  |  |  |  |  |  |
| '35' - out of network |  |  |  |  |  |  |  |  |
| '41' - authorization/access restrictions |  |  |  |  |  |  |  |  |
| '43' - invalid/missing provider identification |  |  |  |  |  |  |  |  |
| etc... |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 80 | RE |  | Error Text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be used for FSC to report text describing an error condition that was not reported by the HCCH via X12 |  |  |  |  |  |  |  |  |

### RPA_I08 Response PRD segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role | 1 | CWE | 250 | R | FALSE |  |  |  |
| Alternate Identifier | 1.4 | ST | 10 | R |  | PRV 2010B |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| VistA application will set this field equal to 'PRV 2010B' to indicate this is for X12 |  |  |  |  |  |  |  |  |
| segment PRV in loop 2010B |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 | PLN | 100 | RE | FALSE |  |  |  |
| ID Number | 7.1 | ST | 50 | R |  | 3123123 | File: 4 (INSTITUTION) Field: 4.043 (TAXONOMY CODE) | Loop 2010B: PRV03 Provider Taxonomy Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Provider Taxonomy Code |  |  |  |  |  |  |  |  |

### RPA_I08 Response GT1 segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID - GT1 | 1 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 837 message. Since it is required, |  |  |  |  |  |  |  |  |
| FSC will set field to a value that will be ignored by VistA application |  |  |  |  |  |  |  |  |
| Guarantor Number | 2 | CX | 250 | RE | TRUE |  |  |  |
| ID Number | 2.1 | ST | 80 | R |  | CU7698F | This value will reflect data that is found in File: 356.22 (278) Field: 02 (PATIENT), File 2 (PATIENT) Field: SUBSCRIBER ID; (2nd field value occurrence) FIle: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Sub File .3121 (INSURANCE TYPE) Field: 2 (GROUP NUMBER) or File 2 (PATIENT) Field .09 (SOCIAL SECURITY NUMBER) | Loop 2010C: NM109 Subscriber Primary ID 2nd Occurrence: REF02 Subscriber Supplemental ID |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will set this field value equal to the values found in NM109 Subscriber |  |  |  |  |  |  |  |  |
| Primary ID and 2nd occurrence found in REF02 Subscriber Supplemental ID |  |  |  |  |  |  |  |  |
| Identifier Type Code | 2.5 | ID | 3 | RE |  | XX |  | Loop 2010C: Second Occurrence: REF01 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field (the 2nd repeating occurrence) with the REF01 Reference Identification Qualifier |  |  |  |  |  |  |  |  |
| '6P' (Group Number) |  |  |  |  |  |  |  |  |
| '1L' (Group or Policy Number) |  |  |  |  |  |  |  |  |
| This field is associated with an HL7 table, therefore a mapping is required. |  |  |  |  |  |  |  |  |
| '6P' ==> 'XX' |  |  |  |  |  |  |  |  |
| '1L' ==> 'U' |  |  |  |  |  |  |  |  |
| Guarantor Name | 3 | XPN | 274 | R | FALSE |  |  |  |
| Family Name | 3.1 | FN | 194 | RE |  |  |  |  |
| � Surname | 3.1.1 | ST | 60 | R |  | SMITH | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: 17 (Name of Insured) | Loop 2010C: NM103 Subscriber Last Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Last Name from |  |  |  |  |  |  |  |  |
| the NM1 segment field 03 Subscriber Last Name |  |  |  |  |  |  |  |  |
| Given Name | 3.2 | ST | 35 | RE |  | JOHN | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File 2 (Patient) Field 17 (Name of Insured) | Loop 2010C: NM104 Subscriber First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber First Name from the |  |  |  |  |  |  |  |  |
| NM1 segment, field 04 Subscriber First Name |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 3.3 | ST | 25 | RE |  | W | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), FIle: 2 (Patient) Field 17 (Name of Insured) | Loop 2010C: NM105 Subscriber Middle Name or Initial |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Middle Name or |  |  |  |  |  |  |  |  |
| Initial from the NM1 segment field 05 Subscriber Middle Name or Initial |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) | 3.4 | ST | 10 | RE |  | JR | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), FIle: 2 (Patient) Field 17 (Name of Insured) | Loop 2010C: NM107 Subscriber Name Suffix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Name Suffix from the |  |  |  |  |  |  |  |  |
| NM1 segment, field 07 Subscriber Name Suffix |  |  |  |  |  |  |  |  |
| Prefix (e.g., DR) | 3.5 | ST | 10 | RE |  | DR | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), FIle: 2 (Patient) Field 17 (Name of Insured) | Loop 2010C: NM106 Subscriber Name Prefix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Name Prefix found in segment |  |  |  |  |  |  |  |  |
| NM1 field 06 Subscriber Name Prefix |  |  |  |  |  |  |  |  |
| Guarantor Address | 5 | XAD | 292 | RE | TRUE |  |  |  |
| Street Address | 5.1 | SAD | 184 | RE |  |  |  |  |
| Street or Mailing Address | 5.1.1 | ST | 55 | RE |  | 123 MAIN STREET | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient) and .03 (Insurance Policy), FIle: 2 (Patient) Subfile 2.312 (Insurance Policy) Field: 3.06 (Insured's Street 1) | Loop 2010C: N301 Subscriber Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Address Line 1 value found in |  |  |  |  |  |  |  |  |
| the N3 Subscriber segment, field N301 Subscriber Address Line |  |  |  |  |  |  |  |  |
| Other Designation | 5.2 | ST | 55 | RE |  | SUITE 200 | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient) and .03 (Insurance Policy), File: 2 (Patient) Subfile 2.312 (Insurance Policy) Field: 3.07 (Insured's Street 2) | Loop 2010C: N302 Subscriber Address line (situational) |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Address Line 2 from the N3 |  |  |  |  |  |  |  |  |
| segment field N302 Subscriber Address line |  |  |  |  |  |  |  |  |
| City | 5.3 | ST | 30 | RE |  | CITY | This field will reflect data found in File 356.22 (278) Field .02 (Patient) and .03 (Insurance Policy), File 2 (Patient) Subfile 2.312 (Insurance Type) Field: 3.08 (Insured's City) | Loop 2010C: N401 Subscriber City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field value with the Subscriber City Name from the N4 |  |  |  |  |  |  |  |  |
| Subscriber City, State, Zip Code segment field 01 Subscriber City Name |  |  |  |  |  |  |  |  |
| State or Province | 5.4 | ST | 2 | RE |  | VA | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient) and .03 (Insurance Policy), File: 2 (Patient) Subfile 2.312 (Insurance Type) Field: 3.09 (Insured's State) | Loop 2010C: N402 Subscriber State Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber State code found in the N4 |  |  |  |  |  |  |  |  |
| Subscriber, City, State, Zip Code segment field 02 Subscriber State Code |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 5.5 | ST | 15 | RE |  | 23433 | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient) and .03 (Insurance Policy), File: 2 (Patient) Subfile 2.312 (Insurance Type) Field: 3.1 (Insured's Zip) | Loop 2010C: N403 Subscriber Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Zip code from the N4 |  |  |  |  |  |  |  |  |
| Subscriber City, State, Zip Code segment field 03 Subscriber Zip Code |  |  |  |  |  |  |  |  |
| Country | 5.6 | ID | 3 | RE |  | ALB | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient) and .03 (Insurance Policy), File: 2 (Patient) Subfile 2.312 (Insurance Type) File: 3.13 (Insured's Country) | Loop 2010C: N404 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Country Code, if |  |  |  |  |  |  |  |  |
| not USA, found in the N4 Subscriber City, State, Zip Code segment field 04 Country Code |  |  |  |  |  |  |  |  |
| Other Geographic Designation | 5.8 | ST | 3 | RE |  | 312 | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient) and .03 (Insurance Policy), File: 2 (Patient) Subfile 2.312 (Insurance Type) Field: 3.14 (Insured's Country Subdivision) | Loop 2010C: N407 Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Country Subdivision Code, if |  |  |  |  |  |  |  |  |
| the Subscriber's address is outside the US and there are Subdivision Codes, |  |  |  |  |  |  |  |  |
| found in the N4 Subscriber City, State, Zip Code segment field 07 Country Subdivision code |  |  |  |  |  |  |  |  |
| Guarantor Date/Time Of Birth | 8 | DTM | 8 | RE | FALSE | 19451212 | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient) and .03 (Insurance Policy), File: 2 (Patient) Subfile 2.312 (Insurance Type) Field: 3.01 (Insured's DOB) | Loop 2010C: DMG02 Subscriber Birth Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| YYYYMMDD - this field will be populated with the Subscriber's Birth Date from the |  |  |  |  |  |  |  |  |
| DMG Subscriber Demographic segment, field 02 Subscriber Birth Date |  |  |  |  |  |  |  |  |
| Guarantor Administrative Sex | 9 | IS | 1 | RE | FALSE |  | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient) and .03 (Insurance Policy), File: 2 (Patient) Subfile: 2.312 (Insurance Type) Field: 3.12 (Insured's Sex) | Loop 2010C: DMG03 Subscriber Gender Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Subscriber Gender Code |  |  |  |  |  |  |  |  |
| 'F' (Female) |  |  |  |  |  |  |  |  |
| 'M' (Male) |  |  |  |  |  |  |  |  |
| 'U' (Unknown) |  |  |  |  |  |  |  |  |
| from the DMG Subscriber Demographic segment field 03 Subscriber Gender |  |  |  |  |  |  |  |  |
| Guarantor Employment Status | 20 | IS | 2 | RE | FALSE | AU |  | Loop 2010C: INS08 Employment Status Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Subscriber's Employment |  |  |  |  |  |  |  |  |
| Status Code to indicate military status |  |  |  |  |  |  |  |  |
| 'AO' (Active Military Overseas) |  |  |  |  |  |  |  |  |
| 'AU' (Active Military USA) |  |  |  |  |  |  |  |  |
| 'DI' (Deceased) |  |  |  |  |  |  |  |  |
| 'PV' (Previous) |  |  |  |  |  |  |  |  |
| 'RU' (Retired Military USA) |  |  |  |  |  |  |  |  |
| Contact Relationship | 48 | IS | 2 | RE | FALSE | 1 | This field will reflect data found in File: 356.22 (278) Field: .02 (PATIENT), File: 2 (PATIENT) Subfile: 2.312 (INSURANCE TYPE) Field: 16 (PT.RELATIONSHIP TO INSURED) | Loop 2010D: INS02 Relationship to Insured Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Dependent Relationship to Insured Code |  |  |  |  |  |  |  |  |
| '01' (Spouse) |  |  |  |  |  |  |  |  |
| '19' (Child) |  |  |  |  |  |  |  |  |
| 'G8' (Other Relationship) |  |  |  |  |  |  |  |  |

### RPA_I08 Response MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Ex Val | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however it is an HL7 required field, so it will be populated with |  |  |  |  |  |  |  |  |
| 'AE' for any application errors reported by FSC |  |  |  |  |  |  |  |  |
| 'AR' for AAA reject reasons reported by HCCH |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 1231456 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field |  |  |  |  |  |  |  |  |
| for HL7, so it should be set to the Message Control ID (MSH.10) of the |  |  |  |  |  |  |  |  |
| original Request 278 message |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 10 | RE | FALSE | AAA 2010C | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'AAA 2010C', identifying this data is from segment AAA in the 2010C loop. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | C | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2010C: AAA04 Follow-up Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field value will be set to the value found in the AAA segment 04 Follow-up Action Code field. |  |  |  |  |  |  |  |  |
| Value should be: |  |  |  |  |  |  |  |  |
| 'C' - please correct and resubmit |  |  |  |  |  |  |  |  |
| 'N' - resubmission not allowed |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CWE | 483 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 | RE |  | 58 | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) | Loop 2010C: AAA03 Reject Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Reject Reason Code (if applicable) |  |  |  |  |  |  |  |  |
| '58' - invalid/missing date of birth |  |  |  |  |  |  |  |  |
| '64' - invalid/missing patient ID |  |  |  |  |  |  |  |  |
| '65' - invalid/missing patient name |  |  |  |  |  |  |  |  |
| '66' - invalid/missing patient gender code |  |  |  |  |  |  |  |  |
| etc... |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 80 | RE |  | Error text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be used for FSC to report text describing an error condition that was not reported by the HCCH via X12 |  |  |  |  |  |  |  |  |

### RPA_I08 Response PID segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Patient Identifier List | 3 | CX | 292 | R | TRUE |  |  |  |
| ID Number | 3.1 | ST | 50 | R |  | ACCT NUMBER | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Subfile 2..312 (Insurance Type) Field: 2 (Group Number) or File: 2 (Patient) Field: .09 (Social Security Number) | Loop 2010D: REF02 Dependent Supplemental Identifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC populate this field with the Dependent Supplemental Identifier, |  |  |  |  |  |  |  |  |
| either the Patient Account Number or the Patient SSN, found in the |  |  |  |  |  |  |  |  |
| REF Dependent Supplemental Identification segment, loop 2010D, field 02 |  |  |  |  |  |  |  |  |
| Assigning Authority | 3.4 | HD | 3 | RE |  |  |  |  |
| � Namespace ID | 3.4.1 | IS | 3 | RE |  | EJ | This field will reflect data found in FIle: 356.22 Field: .02 (Patient), File: 2 (Patient) Subfile: 2.312 (Insurance Type) Field: 2 (Group Number) or File: 2 (Patient) Field: .09 (Social Security Number) | Loop 2010D: REF01 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| FSC will populate this field with the Dependent Supplemental Reference Identification Qualifier, |  |  |  |  |  |  |  |  |
| which will identify the value in PID.3.1 |  |  |  |  |  |  |  |  |
| 'EJ' (Patient Account Number) |  |  |  |  |  |  |  |  |
| 'SY' (SSN) |  |  |  |  |  |  |  |  |
| from the REF Dependent Supplemental Identification segment, |  |  |  |  |  |  |  |  |
| field 01 Reference Identification Qualifier |  |  |  |  |  |  |  |  |
| Patient Name | 5 | XPN | 264 | R | FALSE |  |  |  |
| Family Name | 5.1 | FN | 194 | RE |  |  |  |  |
| � Surname | 5.1.1 | ST | 60 | R |  | SMITH | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .01 (Name) | Loop 2010D: NM103 Dependent Last Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is Not the Subscriber, the Patient's Last Name will be placed into |  |  |  |  |  |  |  |  |
| this field from the NM1 Dependent Name segment, field 03 Dependent Last Name |  |  |  |  |  |  |  |  |
| Given Name | 5.2 | ST | 35 | RE |  | JOHN | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .01 (Name) | Loop 2010D: NM104 Dependent First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, the Patient's First Name will be placed in this |  |  |  |  |  |  |  |  |
| field from the NM1 Dependent Name segment, field 04 Dependent First Name |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 5.3 | ST | 25 | RE |  | W | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .01 (Name) | Loop 2010D: NM105 Dependent Middle Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, the Patient's Middle Name or Initial will be |  |  |  |  |  |  |  |  |
| placed into this field from NM1 Dependent |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) | 5.4 | ST | 10 | RE |  | JR | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .01 (Name) | Loop 2010D: NM107 Dependent Name Suffix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, the Patient's suffix will be placed into this field from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| Date/Time of Birth | 7 | DTM | 8 | RE | FALSE | 20040508 | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .03 (Date of Birth) | Loop 2010D: DMG02 Dependent Birth Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| YYYYMMDD - If the Patient is not the Subscriber, the Patient's Birth Date will be placed in this field, from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| Administrative Sex | 8 | IS | 1 | RE | FALSE | F | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File 2 (Patient) Field: .02 (Sex) | Loop 2010D: DMG03 Dependent Gender Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populate this field with the Dependent Gender Code |  |  |  |  |  |  |  |  |
| 'F' (Female) |  |  |  |  |  |  |  |  |
| 'M' (Male) |  |  |  |  |  |  |  |  |
| 'U' (Unknown) |  |  |  |  |  |  |  |  |
| Patient Address | 11 | XAD | 292 | RE | FALSE |  |  |  |
| Street Address | 11.1 | SAD | 184 | RE |  |  |  |  |
| Street or Mailing Address | 11.1.1 | ST | 55 | RE |  | 123 MAIN STREET | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .111 (Street Address [Line 1] | Loop 2010D: N301 Dependent Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, this field will be populated with the |  |  |  |  |  |  |  |  |
| Dependent Address Line 1 from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| Other Designation | 11.2 | ST | 55 | RE |  | Apt 5 | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .112 (Street Address [Line 2] | Loop 2010D: N302 Dependent Address Line (2) |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, this field will be populated with the |  |  |  |  |  |  |  |  |
| Dependent Address Line 2 from the PATIENT (2) file, if one exists |  |  |  |  |  |  |  |  |
| City | 11.3 | ST | 30 | RE |  | RESTON | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .114 (City) | Loop 2010D: N401 Dependent City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, this field will be populated with the Patient's |  |  |  |  |  |  |  |  |
| City from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| State or Province | 11.4 | ST | 2 | RE |  | VA | This field will reflect data found in File: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .115 (State) | Loop 2010D: N402 Dependent State |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, this field will be populated with the |  |  |  |  |  |  |  |  |
| Dependent State from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 11.5 | ST | 15 | RE |  | 21121 | This field will reflect data found in File 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .116 (Zip Code) | Loop 2010D: N403 Dependent Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber, then this field will be populated with the |  |  |  |  |  |  |  |  |
| Dependent Zip code from the PATIENT (2) file |  |  |  |  |  |  |  |  |
| Country | 11.6 | ID | 3 | RE |  | ALB | This field will reflect data found in FIle: 356.22 (278) Field: .02 (Patient), File: 2 (Patient) Field: .1173 (Country) | Loop 2010D: N404 Dependent Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber and the Patient address is outside the US, |  |  |  |  |  |  |  |  |
| this field will contain the country code for the Dependent Country Code |  |  |  |  |  |  |  |  |
| Other Geographic Designation | 11.8 | ST | 3 | RE |  | IAN |  | Loop 2010D: N407 Dependent Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber and the Patient address is outside the US and |  |  |  |  |  |  |  |  |
| they have Subdivision Codes, this field will contain the Dependent Country Subdivision Code |  |  |  |  |  |  |  |  |
| Patient Account Number | 18 | CX | 250 | RE | FALSE |  |  |  |
| ID Number | 18.1 | ST | 80 | RE |  | 21312321 |  | Loop 2010D: NM109 Dependent Primary Identifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM109 Dependent Primary Identifier |  |  |  |  |  |  |  |  |
| Birth Order | 25 | NM | 2 | RE | FALSE | 1 |  | Loop 2010D: INS17 Birth Sequence Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If the Patient is not the Subscriber and there is a need to report Birth Order, this |  |  |  |  |  |  |  |  |
| field will contain the Birth Sequence Number of the Patient |  |  |  |  |  |  |  |  |

### RPA_I08 Response MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however it is an HL7 required |  |  |  |  |  |  |  |  |
| field, so it will be populated with |  |  |  |  |  |  |  |  |
| AE' for any application errors reported by FSC |  |  |  |  |  |  |  |  |
| AR' for AAA reject reasons reported by HCCH |  |  |  |  |  |  |  |  |
| Or this field will be set to 'AA' for TRN Patient Event Tracking Number segment |  |  |  |  |  |  |  |  |
| (loop 2000E), up to 2 of these 'AR' for AAA reject reasons reported by HCCH |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 1231456 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field for HL7, so it should be set to the |  |  |  |  |  |  |  |  |
| Message Control ID (MSH.10) of the original Request 278 message |  |  |  |  |  |  |  |  |
| Or, for TRN Patient Event Tracking Number segment, this field will be set to 'TRN 2000E' |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 50 | RE | FALSE | AAA 2010D | File: 356.22 (278) Subfile: 356.22105A (TRN SEGMENTS) Field: .04 (TRACE NUMBER) or Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) | Loop 2010D: or Loop 2000E: TRN02 Patient Event Trace Number or 'AAA 2000E' |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'AAA 2010D', identifying this data is from |  |  |  |  |  |  |  |  |
| segment AAA in the 2010D loop. |  |  |  |  |  |  |  |  |
| Or 'AAA 2000E' |  |  |  |  |  |  |  |  |
| Or, for TRN Patient Event Tracking X12 segment, this field will have the TRN02 Patient Event Trace Number in it |  |  |  |  |  |  |  |  |
| Field length has been expanded to 50 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | C | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2010D: AAA04 Follow-up Action Code or Loop 2000E: AAA04 Follow-up Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field value will be set to the value found in the AAA segment 04 Follow-up Action Code field. |  |  |  |  |  |  |  |  |
| Value should be: |  |  |  |  |  |  |  |  |
| 'C' - please correct and resubmit |  |  |  |  |  |  |  |  |
| 'N' - resubmission not allowed |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CWE | 483 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 10 | RE |  | 15 | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) or Subfile: 356.22105A (TRN SEGMENTS) Field: .05 (TRACE ASSIGNING ENTITY ID) | Loop 2010D: AAA03 Reject Reason Code or Loop 2000E: AAA03 Reject Reason Code or TRN03 Trace Assigning Entity Identifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Reject Reason Code (if applicable) |  |  |  |  |  |  |  |  |
| '15' - required application data missing |  |  |  |  |  |  |  |  |
| '33' - input errors |  |  |  |  |  |  |  |  |
| '52' - service dates not within provider |  |  |  |  |  |  |  |  |
| plan enrollment |  |  |  |  |  |  |  |  |
| '58' - invalid/missing date of birth |  |  |  |  |  |  |  |  |
| '64' - invalid/missing patient ID |  |  |  |  |  |  |  |  |
| etc... |  |  |  |  |  |  |  |  |
| Or for the TRN Patient Event Tracking Number segment, this field will have the |  |  |  |  |  |  |  |  |
| Trace Assigning Entity Identifier, which identifies organization that assigned the |  |  |  |  |  |  |  |  |
| trace number, 1st position must be either a 1 if EIN, 3 if a DUNS or 9 if user assigned |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 80 | RE |  | Error text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) or Subfile: 356.22105A Field: .06 (TRACE ASSIGNING ENTITY ID-2) | Loop 2010D: AAA segment Error text for FSC reported errors. Also, Loop 2000E: AAA segment Error text for FSC reported errors. Or Loop 2000E: TRN04 Trace Assigning Entity Additional ID |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be used for FSC to report text describing an error condition that was not reported by the HCCH via X12 |  |  |  |  |  |  |  |  |
| Or, for the TRN Patient Event Tracking Number segment, this field will have the |  |  |  |  |  |  |  |  |
| TRN04 Trace Assigning Entity Additional ID value. This is required when a |  |  |  |  |  |  |  |  |
| specific division or group of the company identified in TRN03 is needed |  |  |  |  |  |  |  |  |
| Message Waiting Number | 7 | NM | 2 | RE | FALSE | 1 | File: 356.22 (278) Subfile: 356.22105A (TRN SEGMENTS) Field: .03 (TRACE TYPE CODE) | Loop 2000E: TRN01 Trace Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For the TRN Patient Event Tracking X12 segment, this value will be set to one of |  |  |  |  |  |  |  |  |
| the following: |  |  |  |  |  |  |  |  |
| 1' (current transaction trace numbers, trace number assigned by the creator of the 278 request), |  |  |  |  |  |  |  |  |
| 2' (referenced transaction trace numbers) |  |  |  |  |  |  |  |  |

### RPA_I08 Response G2R Segment Group – PRB segment (Required)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action Code | 1 | ID | 2 | R | FALSE | CO |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application is not using this field for this X12 message, however, it is |  |  |  |  |  |  |  |  |
| required so it will be set equal to 'CO' |  |  |  |  |  |  |  |  |
| Action Date/Time | 2 | DTM | 8 | R | FALSE | 20040328 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| YYYYMMDD - The VistA application is not using this field for this X12 message, |  |  |  |  |  |  |  |  |
| however, this field is required, so it will be set equal to the date of the request |  |  |  |  |  |  |  |  |
| Problem ID | 3 | CWE | 250 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 1 | RE |  | 1 | File: 356.22 (278) Field: 2.02 (CERTIFICATION TYPE CODE) | Loop 2000E: UM02 Certification Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will set this value equal to the Certification Type Code value |  |  |  |  |  |  |  |  |
| '1' (appeal - immediate), |  |  |  |  |  |  |  |  |
| '2' (appeal - standard), |  |  |  |  |  |  |  |  |
| '3' (cancel), |  |  |  |  |  |  |  |  |
| '4' (extension), |  |  |  |  |  |  |  |  |
| 'I' (initial), |  |  |  |  |  |  |  |  |
| 'N' (reconsideration), |  |  |  |  |  |  |  |  |
| 'R' (renewal), |  |  |  |  |  |  |  |  |
| 'S' (revised) |  |  |  |  |  |  |  |  |
| Problem Instance ID | 4 | EI | 60 | R | FALSE |  |  |  |
| Entity Identifier | 4.1 | ST | 3 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application is not using this field for this X12 message, however, it is |  |  |  |  |  |  |  |  |
| required, so it will be set equal to 1 |  |  |  |  |  |  |  |  |
| Episode of Care ID | 5 | EI | 60 | R | FALSE |  |  |  |
| Entity Identifier | 5.1 | ST | 10 | R |  | UM 2000E |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will set this field value equal to 'UM 2000E' to indicate to |  |  |  |  |  |  |  |  |
| FSC that this data is for X12 segment UM, loop 2000E |  |  |  |  |  |  |  |  |
| Problem Management Discipline | 11 | CWE | 250 | RE | TRUE |  |  |  |
| Identifier | 11.1 | ST | 3 | RE |  | HS | File: 356.22 (278) Field: 2.01 (REQUEST CATEGORY) or 2.03 (SERVICE TYPE CODE) | Loop 2000E: UM01 Request Category Code or UM03 Service Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The 1st occurrence of this field will be populated with the Request Category Code |  |  |  |  |  |  |  |  |
| 'IN' (Individual when BHT06 = RU), |  |  |  |  |  |  |  |  |
| 'AR' (Admission review), |  |  |  |  |  |  |  |  |
| 'HS' (health services review), |  |  |  |  |  |  |  |  |
| 'SC' (specialty review) |  |  |  |  |  |  |  |  |
| The 2nd occurrence of this field will be used for the Service Type Code. |  |  |  |  |  |  |  |  |
| Examples: |  |  |  |  |  |  |  |  |
| '1' (Medical Care), |  |  |  |  |  |  |  |  |
| '2' (Surgical), |  |  |  |  |  |  |  |  |
| '3' (Consultation), |  |  |  |  |  |  |  |  |
| '4' (Diagnostic X-Ray), |  |  |  |  |  |  |  |  |
| etc_ |  |  |  |  |  |  |  |  |
| Problem Ranking | 18 | CWE | 250 | RE | FALSE |  |  |  |
| Identifier | 18.1 | ST | 3 | RE |  | U | File: 356.22 (278) Field: 2.13 (LEVEL OF SERVICE) | Loop 2000E: UM06 Level of Service Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Level of Service Code |  |  |  |  |  |  |  |  |
| '03' (emergency), |  |  |  |  |  |  |  |  |
| 'E' (elective), |  |  |  |  |  |  |  |  |
| 'U' (urgent) |  |  |  |  |  |  |  |  |

### RPA_I08 Response G2R Segment Group – PV1 segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Patient Class | 2 | IS | 1 | R | FALSE | U |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will not be using this field for this X12 message, however, |  |  |  |  |  |  |  |  |
| it is required by HL7, so it will be set equal to 'U' |  |  |  |  |  |  |  |  |
| Assigned Patient Location | 3 | PL | 2816 | RE | FALSE |  |  |  |
| Point of Care | 3.1 | IS | 2 | RE |  | 34 | File: 356.22 (278) Field: 2.05 (FACILITY TYPE) | Loop 2000E: UM04-1 Facility Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Facility Type Code. |  |  |  |  |  |  |  |  |
| Person Location Type | 3.6 | IS | 20 | RE |  | A | File: 356.22 (278) Field: 2.04 (FACILITY TYPE QUALIFIER) | Loop 2000E: UM04-2 Facility Code Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Facility Code Qualifier for field PV1.3.1 |  |  |  |  |  |  |  |  |
| 'A' (uniform billing claim form bill type), |  |  |  |  |  |  |  |  |
| 'B' (place of service codes for professional or dental services) |  |  |  |  |  |  |  |  |

### RPA_I08 Response AUT segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authorizing Payer, Company ID | 2 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 10 | R |  | REF 2000E |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| If this segment exists, this field will be populated with 'REF 2000E', which indicates to FSC that this data is for X12 |  |  |  |  |  |  |  |  |
| segment REF, loop 2000E |  |  |  |  |  |  |  |  |
| Or |  |  |  |  |  |  |  |  |
| 'HCR 2000E' |  |  |  |  |  |  |  |  |
| Text | 2.2 | ST | 50 | RE |  | REF number | FIle: 356.22 (278) Field: 17.01 (PREV. AUTHORIZATION #) or 17.02 (PREV. ADMIN. REFERENCE #) based in AUT.2.4 value, or for HCR segment, Subfile: 356.22103A (HCR SEGMENTS) Field: .05 (REVIEW DECISION REASON CODE) | Loop 2000E: HCR03 Review Decision Reason Code or REF02 Previous Administration Reference Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Previous Administration Reference Number that this request is providing |  |  |  |  |  |  |  |  |
| additional information for or follow-up on. |  |  |  |  |  |  |  |  |
| Or |  |  |  |  |  |  |  |  |
| HCR03 Review Decision Reason Code |  |  |  |  |  |  |  |  |
| Alternate Text | 2.5 | ST | 3 | RE |  | BB | For HCR segment, File: 356.22 (278) Subfile: 356.22103A (HCR SEGMENTS) Field: .06 (SECOND SURGICAL OPINION FLAG) | Loop 2000E: HCR04 Second Surgical Opinion Indicator or REF01 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
| BB' (authorization number for additional information request), |  |  |  |  |  |  |  |  |
| NT' (authorization number for follow-up request) |  |  |  |  |  |  |  |  |
| Or |  |  |  |  |  |  |  |  |
| HCR04 Second Surgical Opinion Indicator |  |  |  |  |  |  |  |  |
| Authorization Identifier | 6 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 6.1 | ST | 50 | RE |  | 3213 | File: 356.22 (278) Subfile: 356.22103A (HCR SEGMENTS) Field: .04 (REVIEW IDENTIFICATION NUMBER) | Loop 2000E: HCR02 Review Identification Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| HCR02 Review Identification Number |  |  |  |  |  |  |  |  |
| Universal ID | 6.3 | ST | 199 | RE |  | A1 | File:356.22 (278) Subfile: 356.22103A (HCR SEGMENTS) Field: .03 (CERTIFICATION ACTION CODE) | Loop 2000E: HCR01 Certification Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| HCR01 Certification Action Code |  |  |  |  |  |  |  |  |
| 'A1' (Certified in total), |  |  |  |  |  |  |  |  |
| 'A2' (Certified - partial), |  |  |  |  |  |  |  |  |
| 'A3' (not certified), |  |  |  |  |  |  |  |  |
| 'A4' (pended), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |

### RPA_I08 Response ZTP segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID | 1 | SI | 4 | R | FALSE | 1 | Fixed value of '1' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, but it is required, so a fixed value of ' 1 ' will be placed here |  |  |  |  |  |  |  |  |
| Date/Time Qualifier | 2 | CWE | 230 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 20 | R |  | 102 | File: 356.22 (278) Field: 2.18 (ACCIDENT DATE) or 2.19 (LAST MENSTRUAL PERIOD DATE) or 2.2 (ESTIMATED DOB) or 2.21 (ILLNESS DATE) or .07 (EVENT DATE) or 2.22 (DISCHARGE DATE) | Loop 2000E: DTP01 Date/Time Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| 439 - Accident Related |  |  |  |  |  |  |  |  |
| 484 - Last Menstrual Period |  |  |  |  |  |  |  |  |
| ABC - Estimated Date of Birth |  |  |  |  |  |  |  |  |
| 431 - Onset of Current Symptoms |  |  |  |  |  |  |  |  |
| AAH - when 278 is for Admission Review and Proposed or Actual Admission date is known |  |  |  |  |  |  |  |  |
| 435 - when 278 is for Admission Review and Proposed or Actual Admission date is unknown |  |  |  |  |  |  |  |  |
| 096 - Discharge Date when 278 is for Admission Review and Actual Discharge Date is known |  |  |  |  |  |  |  |  |
| 102 - Issue Date |  |  |  |  |  |  |  |  |
| 036 - Certification Expiration Date |  |  |  |  |  |  |  |  |
| 007 - Certification Effective Date |  |  |  |  |  |  |  |  |
| Date Time Period | 3 | DR | 51 | R | FALSE |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Date or range of dates being transmitted. |  |  |  |  |  |  |  |  |
| Range start date/time | 3.1 | DTM | 8 | R |  | 20100230 | File: 356.22 (278) Field: 2.18 (ACCIDENT DATE) or 2.19 (LAST MENSTRUAL PERIOD DATE) or 2.2 (ESTIMATED DOB) or 2.21 (ILLNESS DATE) or .07 (EVENT DATE) or 2.22 (DISCHARGE DATE) | DTP03 Accident Date, DTP03 Last Menstrual Date, DTP03 Estimated Date of Birth, DTP03 Onset of Current Symptoms or Illness Date, DTP03 Proposed or Actual Event Date, DTP03 Proposed or Actual Admission Date, DTP03 Proposed or Actual Discharge Date, DTP03 Certification Issue Date, DTP03 Certification Expiration Date, DTP03 Certification Effective Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Date will be in the following format: |  |  |  |  |  |  |  |  |
| CCYYMMDD |  |  |  |  |  |  |  |  |
| Loop ID | 4 | ST | 10 | R | FALSE | DTP 2000E |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The VistA application will populated this field with '2000E', indicating the data here is for X12 segment DTP, loop 2000E |  |  |  |  |  |  |  |  |

### RPA_I08 Response DG1 segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID - DG1 | 1 | SI | 4 | R | FALSE | 1 | Fixed value 1 through 12 | Loop 2000E: HI01 thru HI12 (up to 12 diagnosis codes) - integer value 1 through 12 |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to a sequential numerical value, from 1 through 12 |  |  |  |  |  |  |  |  |
| Diagnosis Code - DG1 | 3 | CWE | 250 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 30 | R |  | 473.1 | File: 356.22 (278) Subfile: 356.223 (PATIENT DIAGNOSIS) Field: .02 (DIAGNOSIS CODE) | Loop 2000E: HI01-2 Diagnosis Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Diagnosis Code related to this Patient Diagnosis |  |  |  |  |  |  |  |  |
| Diagnosis Date/Time | 5 | DTM | 8 | RE | FALSE | 20140410 | File: 356.22 (278) Subfile: 356.223 (PATIENT DIAGNOSIS) Field: .03 (DIAGNOSIS DATE) | Loop 2000E: HI01-4 Diagnosis Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| YYYYMMDD - This date is the Diagnosis Date |  |  |  |  |  |  |  |  |
| Diagnosis Type | 6 | IS | 2 | R | FALSE | W |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, so it will be set to 'W' |  |  |  |  |  |  |  |  |
| Diagnosis Classification | 17 | IS | 3 | RE | FALSE | ABF | File: 356.22 (278) Subfile: 356.223 (PATIENT DIAGNOSIS) Field: .01 (DIAGNOSIS TYPE) | Loop 2000E: HI01-1 Diagnosis Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Diagnosis Type Code, identifying the code list from which field DG1.3.1 is from. |  |  |  |  |  |  |  |  |
| example: |  |  |  |  |  |  |  |  |
| 'ABF' (International Classification of |  |  |  |  |  |  |  |  |
| Diseases Clinical Mod. Diagnosis (ICD-10-CM)), |  |  |  |  |  |  |  |  |
| ABJ' (International Classification of Diseases Clinical Mod. Admitting Diagnosis (ICD-10-CM)), |  |  |  |  |  |  |  |  |
| etc... |  |  |  |  |  |  |  |  |

### RPA_I08 Response ZHS segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID | 1 | SI | 10 | RE | FALSE | HSD 2000E | Fixed value 'HSD 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the value 'HSD 2000E', which indicates to FSC that this |  |  |  |  |  |  |  |  |
| data is for X12 segment HSD, loop 2000E |  |  |  |  |  |  |  |  |
| Quantity Qualifier | 2 | CWE | 99 | RE | FALSE |  |  |  |
| Identifier | 2.1 | ST | 2 | R |  | DY | File: 356.22 (278) Field: 4.01 (HCSD QUANTITY QUALIFIER) | Loop 2000E: HSD01 Quantity Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Health Care Services Delivery Quantity Qualifier |  |  |  |  |  |  |  |  |
| 'DY' (Days) |  |  |  |  |  |  |  |  |
| 'FL' (Units) |  |  |  |  |  |  |  |  |
| 'HS' (Hours) |  |  |  |  |  |  |  |  |
| 'MN' (Month) |  |  |  |  |  |  |  |  |
| 'VS' (Visits) |  |  |  |  |  |  |  |  |
| Benefit Quantity | 3 | NM | 15 | RE | FALSE | 123 | File: 356.22 (278) Field: 4.02 (HCSD SERVICE UNIT COUNT) | Loop 2000E: HSD02 Service Unit Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Health Care Services Delivery Service Unit Count |  |  |  |  |  |  |  |  |
| Unit or Basis for Measurement Code | 4 | CWE | 20 | RE | FALSE |  |  |  |
| Identifier | 4.1 | ST | 2 | R |  | DA | File: 356.22 (278) Field: 4.03 (HCSD UNITS OF MEASUREMENT) | Loop 2000E: HSD03 Unit or Basis for Measurement Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Health Care Services Delivery Unit or Basis for Measurement Code |  |  |  |  |  |  |  |  |
| 'DA' (Days) |  |  |  |  |  |  |  |  |
| 'MO' (Months) |  |  |  |  |  |  |  |  |
| 'VS' (Visit) |  |  |  |  |  |  |  |  |
| 'WK' (Week) |  |  |  |  |  |  |  |  |
| 'YR' (Years) |  |  |  |  |  |  |  |  |
| Sample Selection Modulus | 5 | ST | 6 | RE | FALSE | 1.5 | File 356.22 (278) Field: 4.04 (HCSD SAMPLE SELECTION MODULUS) | Loop 2000E: HSD04 Sample Selection Modulus |
| Implementation Note: |  |  |  |  |  |  |  |  |
| To specify the sampling frequency in terms of a modulus of the Unit of Measure, e.g., every fifth bag, every 1.5 minutes |  |  |  |  |  |  |  |  |
| Time Period Qualifier | 6 | CWE | 20 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 | R |  | 25 | File: 356.22 (278) Field: 4.05 (HCSD TIME PERIOD QUALIFIER) | Loop 2000E: HSD05 Time Period Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Health Care Services Delivery Time Period Qualifier. It is used if needed to provide |  |  |  |  |  |  |  |  |
| further information about the number and frequency of benefits. |  |  |  |  |  |  |  |  |
| '6' (Hour) |  |  |  |  |  |  |  |  |
| '7' (Day) |  |  |  |  |  |  |  |  |
| '21' (Years) |  |  |  |  |  |  |  |  |
| '22' (Service Year) |  |  |  |  |  |  |  |  |
| '23' (Calendar Year) |  |  |  |  |  |  |  |  |
| '24' (Year to Date) |  |  |  |  |  |  |  |  |
| '25' (Contract) |  |  |  |  |  |  |  |  |
| '26' (Episode) |  |  |  |  |  |  |  |  |
| '27' (Visit) |  |  |  |  |  |  |  |  |
| '28' (Outlier) |  |  |  |  |  |  |  |  |
| '29' (Remaining) |  |  |  |  |  |  |  |  |
| '30' (Exceeded) |  |  |  |  |  |  |  |  |
| '31' (Not Exceeded) |  |  |  |  |  |  |  |  |
| '32' (Lifetime) |  |  |  |  |  |  |  |  |
| '33' (Lifetime Remaining) |  |  |  |  |  |  |  |  |
| '34' (Month) |  |  |  |  |  |  |  |  |
| '35' (Week) |  |  |  |  |  |  |  |  |
| Number of Periods | 7 | NM | 20 | RE | FALSE | 5 | File: 356.22 (278) Field: 4.06 (HCSD PERIOD COUNT) | Loop 2000E: HSD06 Period Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Health Care Services Delivery Period Count. This is used if needed to |  |  |  |  |  |  |  |  |
| provide further information about the number and frequency of benefits. |  |  |  |  |  |  |  |  |
| Ship/Delivery or Calendar Pattern Code | 8 | CWE | 20 | RE | FALSE |  |  |  |
| Identifier | 8.1 | ST | 2 | R |  | A | File: 356.22 (278) Field: 4.07 (HCSD DELIVERY FREQUENCY) | Loop 2000E: HSD07 Delivery Frequency Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Heath Care Services Delivery Frequency Code. This is used if needed to provide further |  |  |  |  |  |  |  |  |
| information about the number and frequency of benefits. |  |  |  |  |  |  |  |  |
| '1' - 1st Week of the Month |  |  |  |  |  |  |  |  |
| '2' - 2nd Week of the Month |  |  |  |  |  |  |  |  |
| '3' - 3rd Week of the Month |  |  |  |  |  |  |  |  |
| '4' - 4th Week of the Month |  |  |  |  |  |  |  |  |
| '5' - 5th Week of the Month |  |  |  |  |  |  |  |  |
| '6' - 1st & 3rd Weeks of the Month |  |  |  |  |  |  |  |  |
| '7' - 2nd & 4th Weeks of the Month |  |  |  |  |  |  |  |  |
| '8' - 1st Working Day of Period |  |  |  |  |  |  |  |  |
| '9' - Last Working Day of Period |  |  |  |  |  |  |  |  |
| 'A' - Monday through Friday |  |  |  |  |  |  |  |  |
| 'B' - Monday through Saturday |  |  |  |  |  |  |  |  |
| 'C' - Monday through Sunday |  |  |  |  |  |  |  |  |
| 'D' – Monday |  |  |  |  |  |  |  |  |
| 'E' – Tuesday |  |  |  |  |  |  |  |  |
| 'F' – Wednesday |  |  |  |  |  |  |  |  |
| 'G' – Thursday |  |  |  |  |  |  |  |  |
| 'H' – Friday |  |  |  |  |  |  |  |  |
| 'J' – Saturday |  |  |  |  |  |  |  |  |
| 'K' – Sunday |  |  |  |  |  |  |  |  |
| 'L' - Monday through Thursday |  |  |  |  |  |  |  |  |
| 'M' – Immediately |  |  |  |  |  |  |  |  |
| 'N' - As Directed |  |  |  |  |  |  |  |  |
| 'O' - Daily Mon. through Fri. |  |  |  |  |  |  |  |  |
| 'P' - 1/2 Mon. & 1/2 Thurs. |  |  |  |  |  |  |  |  |
| 'Q' - 1/2 Tues. & 1/2 Thurs. |  |  |  |  |  |  |  |  |
| 'R' - 1/2 Wed. & 1/2 Fri. |  |  |  |  |  |  |  |  |
| 'S' - Once Anytime Mon. through Fri. |  |  |  |  |  |  |  |  |
| 'SG' - Tuesday through Friday |  |  |  |  |  |  |  |  |
| 'SL' - Monday, Tuesday and Thursday |  |  |  |  |  |  |  |  |
| 'SP' - Monday, Tuesday and Friday |  |  |  |  |  |  |  |  |
| 'SX' - Wednesday and Thursday |  |  |  |  |  |  |  |  |
| 'SY' - Monday, Wednesday and Thursday |  |  |  |  |  |  |  |  |
| 'SZ' - Tuesday, Thursday and Friday |  |  |  |  |  |  |  |  |
| 'T' - 1/2 Tue. & 1/2 Fri. |  |  |  |  |  |  |  |  |
| 'U' - 1/2 Mon. & 1/2 Wed. |  |  |  |  |  |  |  |  |
| 'V' - 1/3 Mon., 1/3 Wed., 1/3 Fri. |  |  |  |  |  |  |  |  |
| 'W' - Whenever Necessary |  |  |  |  |  |  |  |  |
| 'X' - 1/2 By Wed., Bal. By Fri. |  |  |  |  |  |  |  |  |
| 'Y' - None (Also Used to Cancel or |  |  |  |  |  |  |  |  |
| Override a Previous Pattern) |  |  |  |  |  |  |  |  |
| Ship/Delivery Pattern Time Code | 9 | CWE | 20 | RE | FALSE |  |  |  |
| Identifier | 9.1 | ST | 2 | R |  | A | File: 356.22 (278) Field: 4.08 (HCSD DELIVERY TIME PATTERN) | Loop 2000E: HSD08 Delivery Pattern Time Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Health Care Services Delivery Pattern Time Code. This is used if needed to |  |  |  |  |  |  |  |  |
| provide further information about the number and frequency of benefits. |  |  |  |  |  |  |  |  |
| 'A' - 1st Shift (Normal Working Hours) |  |  |  |  |  |  |  |  |
| 'B' - 2nd Shift |  |  |  |  |  |  |  |  |
| 'C' - 3rd Shift |  |  |  |  |  |  |  |  |
| 'D' - A.M. |  |  |  |  |  |  |  |  |
| 'E' - P.M. |  |  |  |  |  |  |  |  |
| 'F' - As Directed |  |  |  |  |  |  |  |  |
| 'G' - Any Shift |  |  |  |  |  |  |  |  |
| Y' - None (Also Used to Cancel or Override a Previous Pattern) |  |  |  |  |  |  |  |  |

### RPA_I08 Response PV1 segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Patient Class | 2 | IS | 1 | R | FALSE | U |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, it is required for HL7, so this field will be set to 'U' |  |  |  |  |  |  |  |  |
| Assigned Patient Location | 3 | PL | 2816 | RE | FALSE |  |  |  |
| Point of Care | 3.1 | IS | 10 | RE |  | CL1 2000E | Fixed value 'CL1 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to 'CL1 2000E'. This indicates to FSC that this data is for X12 segment CL1 in loop 2000E. |  |  |  |  |  |  |  |  |
| Admission Type | 4 | IS | 1 | RE | FALSE | 1 | File: 356.22 (278) Field: 7.01 (ADMISSION TYPE) | Loop 2000E: CL101 Admission Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Admission Type Code |  |  |  |  |  |  |  |  |
| '1' (EMERGENCY), |  |  |  |  |  |  |  |  |
| '2' (URGENT), |  |  |  |  |  |  |  |  |
| '3' (ELECTIVE), |  |  |  |  |  |  |  |  |
| '4' (NEWBORN), |  |  |  |  |  |  |  |  |
| '5' (TRAUMA), |  |  |  |  |  |  |  |  |
| '9' (INFORMATION NOT AVAILABLE) |  |  |  |  |  |  |  |  |
| Admit Source | 14 | IS | 1 | RE | FALSE | 1 | File: 356.22 (278) Field: 7.02 (ADMISSION SOURCE) | Loop 2000E: CL102 Admission Source Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Admission Source Code for X12 segment CL1 |  |  |  |  |  |  |  |  |
| '1' (physician referral), |  |  |  |  |  |  |  |  |
| '2' (clinic referral), |  |  |  |  |  |  |  |  |
| '3' (HMO referral), |  |  |  |  |  |  |  |  |
| '4' (transfer from hospital), |  |  |  |  |  |  |  |  |
| '5' (transfer from skilled nursing facility), |  |  |  |  |  |  |  |  |
| 6' (transfer from other health care facility), |  |  |  |  |  |  |  |  |
| '7' (emergency room), |  |  |  |  |  |  |  |  |
| '8' (court / law enforcement), |  |  |  |  |  |  |  |  |
| '9' (info not available) |  |  |  |  |  |  |  |  |
| Discharge Disposition | 36 | IS | 2 | RE | FALSE |  | File: 356.22 (278) Field: 7.03 (PATIENT STATUS) | Loop 2000E: CL103 Patient Status Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Patient Status Code |  |  |  |  |  |  |  |  |

### RPA_I08 Response OBR segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Universal Service Identifier | 4 | CWE | 705 | R | FALSE |  |  |  |
| Identifier | 4.1 | ST | 10 | R |  | CR1 2000E |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to 'CR1 2000E' to indicate to FSC that this data is for X12 segment CR1 in loop 2000E |  |  |  |  |  |  |  |  |
| Quantity/Timing | 27 | TQ | 2218 | RE | FALSE |  |  |  |
| Quantity | 27.1 | CQ | 722 | RE |  |  |  |  |
| � Quantity | 27.1.1 | NM | 16 | RE |  | 175 | File: 356.22 (278) Field: 18.06 (TRANSPORT DISTANCE) | Loop 2000E: CR106 Transport Distance |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be the Transport Distance |  |  |  |  |  |  |  |  |
| � Units | 27.1.2 | CWE | 2 | RE |  | DH | File: 356.22 (278) Field: 18.05 (DISTANCE UNITS) | Loop 2000E: CR105 Unit or Basis of Measurement |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Unit or Basis of Measurement qualifier for field OBR.27.1.1 |  |  |  |  |  |  |  |  |
| DH' (miles) |  |  |  |  |  |  |  |  |
| DK' (kilometers) |  |  |  |  |  |  |  |  |
| Placer Supplemental Service Information | 46 | CWE | 705 | RE | FALSE |  |  |  |
| Identifier | 46.1 | ST | 1 | RE |  | I | File: 356.22 (278) Field: 18.03 (AMBULANCE TRANSPORT CODE) | Loop 2000E: CR103 Ambulance Transport Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Ambulance Transport Code |  |  |  |  |  |  |  |  |
| 'I' (initial trip), |  |  |  |  |  |  |  |  |
| 'R' (return trip), |  |  |  |  |  |  |  |  |
| 'T' (transfer trip), |  |  |  |  |  |  |  |  |
| 'X' (round trip) |  |  |  |  |  |  |  |  |

### RPA_I08 Response RXA segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Give Sub-ID Counter | 1 | NM | 9 | R | FALSE | 5 | File: 356.22 (278) Field: 7.05 (TREATMENT SERIES NUMBER) | Loop 2000E: CR201 Treatment Series Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Treatment Series Number, which is the total number of treatments in the series of treatments. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 9 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Administration Sub-ID Counter | 2 | NM | 15 | R | FALSE | 2 | File: 356.22 (278) Field: 7.06 (TREATMENT COUNT) | Loop 2000E: CR202 Treatment Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Treatment Count |  |  |  |  |  |  |  |  |
| Field length has been expanded to 15 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Date/Time Start of Administration | 3 | DTM | 24 | R | FALSE | 20140428 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for X12 message, so will be set to date of transmission |  |  |  |  |  |  |  |  |
| Date/Time End of Administration | 4 | DTM | 24 | R | FALSE | 20140428 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| this field is not used for this X12 message, however, it is a required HL7 field, so it will be set to the date of message transmission |  |  |  |  |  |  |  |  |
| Administered Code | 5 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 5.1 | ST | 1 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, this field is required for HL7, so it will be set to 1 |  |  |  |  |  |  |  |  |
| Administered Amount | 6 | NM | 20 | R | FALSE | 0 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, it is a required field for HL7, so it will be set to 0 (zero) |  |  |  |  |  |  |  |  |
| Administration Notes | 9 | CWE | 697 | RE | TRUE |  |  |  |
| Identifier | 9.1 | ST | 3 | RE |  | C1 | File: 356.22 (278) Field: 7.07 (SUBLUXATION LEVEL CODE #1) or 7.08 (SUBLUXATION LEVEL CODE #2) | Loop 2000E: CR203 Subluxation Level Code and (2nd occurrence) CR204 Subluxation Level Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This multiple field will contain the Subluxation Level Code |  |  |  |  |  |  |  |  |
| 'C1' (cervical 1), |  |  |  |  |  |  |  |  |
| 'C2' (cervical 2), |  |  |  |  |  |  |  |  |
| 'IL' (ilium), |  |  |  |  |  |  |  |  |
| 'L1' (lumbar 1), |  |  |  |  |  |  |  |  |
| etc_ |  |  |  |  |  |  |  |  |

### RPA_I08 Response RXE segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Quantity/Timing | 1 | TQ | 2218 | RE | FALSE |  |  |  |
| Quantity | 1.1 | CQ | 15 | RE |  |  |  |  |
| � Quantity | 1.1.1 | NM | 15 | RE |  | 1 | File: 356.22 (278) Field: 8.07 (OXYGEN USE PERIOD HOUR COUNT) | Loop 2000E: CR508 Oxygen Use Period Hour Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Oxygen Use Period Hour Count. |  |  |  |  |  |  |  |  |
| This is the number of hours per period of oxygen use |  |  |  |  |  |  |  |  |
| Text | 1.8 | TX | 80 | RE |  | Up to 80 characters of free text | File: 356.22 (278) Field: 8.08 (RESPIRATORY THERAPIST ORDER) | Loop 2000E: CR509 Respiratory Therapist Order Text |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Respiratory Therapist Order Text. These are special orders for the respiratory therapist. |  |  |  |  |  |  |  |  |
| Give Code | 2 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 1 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, it is required for HL7. |  |  |  |  |  |  |  |  |
| Set to ' 1 ' |  |  |  |  |  |  |  |  |
| Give Amount - Minimum | 3 | NM | 15 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 response message, however, it is an HL7 required field, therefore it will be set to a fixed value if '1' |  |  |  |  |  |  |  |  |
| Give Units | 5 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 5.1 | ST | 1 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not required for this X12 message, but it is an HL7 required field, so it will be set to ' 1 ' |  |  |  |  |  |  |  |  |
| Pharmacist/Treatment Supplier's Verifier ID | 14 | XCN | 4751 | RE | TRUE |  |  |  |
| ID Number | 14.1 | ST | 1 | RE |  | A | File: 356.22 (278) Field: 8.01 (OXYGEN EQUIPMENT TYPE #1) and 8.02 (OXYGEN EQUIPMENT TYPE #2) | Loop 2000E: CR503 Oxygen Equipment Type Code and (2nd occurrence) CR504 Oxygen Equipment Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Equipment Type Code |  |  |  |  |  |  |  |  |
| 'A' (concentrator), |  |  |  |  |  |  |  |  |
| 'B' (liquid stationary), |  |  |  |  |  |  |  |  |
| 'C' (gaseous stationary), |  |  |  |  |  |  |  |  |
| 'D' (liquid portable), |  |  |  |  |  |  |  |  |
| 'E' (gaseous portable), |  |  |  |  |  |  |  |  |
| 'O' (other) |  |  |  |  |  |  |  |  |
| Number of Refills Remaining | 16 | NM | 15 | RE | FALSE | 1.5 | File: 356.22 (278) Field: 9.07 (PORTABLE OXYGEN SYSTEM FLOW) | Loop 2000E: CR516 Portable Oxygen System Flow Rate |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Portable Oxygen System Flow Rate. This the oxygen flow rate for a portable oxygen system in liters per minute |  |  |  |  |  |  |  |  |
| Total Daily Dose | 19 | CQ | 15 | RE | FALSE |  |  |  |
| Quantity | 19.1 | NM | 15 | RE |  | 3 | File: 356.22 (278) Field: 8.06 (DAILY OXYGEN USE COUNT) | Loop 2000E: CR507 Daily Oxygen Use Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Daily Oxygen Use Count, which is the number of times per day the patient must use oxygen |  |  |  |  |  |  |  |  |
| Give Rate Amount | 23 | ST | 15 | RE | FALSE | 0.75 | File: 356.22 (278) Field: 8.05 (OXYGEN FLOW RATE) | Loop 2000E: CR506 Oxygen Flow Rate |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Flow Rate in liters per minute. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 15 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Dispense Package Size Unit | 29 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 29.1 | ST | 1 | RE |  | A | File: 356.22 (278) Field: 9.08 (OXYGEN DELIVERY SYSTEM) | Loop 2000E: CR517 Oxygen Delivery System Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Delivery System Code |  |  |  |  |  |  |  |  |
| 'A' (nasal cannula), |  |  |  |  |  |  |  |  |
| 'B' (oxygen conserving device), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Supplementary Code | 31 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 31.1 | ST | 1 | RE |  | A | File: 356.22 (278) Field: 8.03 (OXYGEN EQUIPMENT TYPE #3) | Loop 2000E: CR518 Oxygen Equipment Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Oxygen Equipment Type Code |  |  |  |  |  |  |  |  |
| 'A' (concentrator), |  |  |  |  |  |  |  |  |
| 'B' (liquid stationary), |  |  |  |  |  |  |  |  |
| 'C' (gaseous stationary), |  |  |  |  |  |  |  |  |
| 'D' (liquid portable), |  |  |  |  |  |  |  |  |
| 'E' (gaseous portable), |  |  |  |  |  |  |  |  |
| 'O' (other) |  |  |  |  |  |  |  |  |

### RPA_I08 Response PRB segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action Code | 1 | ID | 2 | R | FALSE | UC |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 278 message, however, it is a required field for HL7, so a 'UC' will be placed into this field |  |  |  |  |  |  |  |  |
| Action Date/Time | 2 | DTM | 8 | R | FALSE | 20140422 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field for HL7, so this field will be populated |  |  |  |  |  |  |  |  |
| with the date of the message transmission, CCYYMMDD format |  |  |  |  |  |  |  |  |
| Problem ID | 3 | CWE | 705 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 1 | R |  | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .02 (CERTIFICATION TYPE CODE) | Loop 2000E: CR608 Certification Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Certification Type Code. |  |  |  |  |  |  |  |  |
| '1' (appeal - immediate), |  |  |  |  |  |  |  |  |
| '2' (appeal - standard), |  |  |  |  |  |  |  |  |
| '3' (cancel), |  |  |  |  |  |  |  |  |
| etc_ |  |  |  |  |  |  |  |  |
| Problem Instance ID | 4 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier | 4.1 | ST | 199 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 message, however, it is a required field for HL7, and so a '1' is placed in this field. |  |  |  |  |  |  |  |  |
| Episode of Care ID | 5 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 5.1 | ST | 10 | RE |  | CR6 2000E | Fixed value 'CR6 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the text 'CR6 2000E'. This identifies to FSC that this data is for X12 segment CR6 in loop 2000E |  |  |  |  |  |  |  |  |
| Problem Date of Onset | 16 | DTM | 8 | RE | FALSE | 20140412 | File: 356.22 (278) Field: 10.01 (HOME HEALTH START DATE) | Loop 2000E: CR602 Home Health Start Date |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the date covered home health services began. The date format will be CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem Onset Text | 17 | ST | 35 | RE | FALSE | 20140401-20140408 | File: 356.22 (278) Field: 10.02 (HOME HEALTH CERT. START) and 10.03 (HOME HEALTH CERT. END) | Loop 2000E: CR604 Home Health Certification Period |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the certification period covered by this plan of treatment. |  |  |  |  |  |  |  |  |
| Format of this date range will be CCYYMMDD-CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem Prognosis | 22 | CWE | 705 | RE | FALSE |  |  |  |
| Identifier | 22.1 | ST | 1 | RE |  | 6 | FIle: 356.22 (278) Field: 2.15 (PROGNOSIS CODE) | Loop 2000E: CR601 Prognosis Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Prognosis Code, indicating the physician's prognosis for the patient. |  |  |  |  |  |  |  |  |
| '1' (Poor) |  |  |  |  |  |  |  |  |
| '2' (Guarded) |  |  |  |  |  |  |  |  |
| '3' (Fair) |  |  |  |  |  |  |  |  |
| '4' (Good) |  |  |  |  |  |  |  |  |
| '5' (Very Good) |  |  |  |  |  |  |  |  |
| '6' (Excellent) |  |  |  |  |  |  |  |  |
| '7' (Less than 6 Months to Live) |  |  |  |  |  |  |  |  |
| '8' (Terminal) |  |  |  |  |  |  |  |  |

### RPA_I08 Response PSL segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Product/Service Line Item Number | 1 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier | 1.1 | ST | 9 | R |  | PWK 2000E | Fixed value 'PWK 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'PWK 2000E'. This allows FSC to identify this data to be for X12 segment PWK in loop 2000E |  |  |  |  |  |  |  |  |
| Product/Service Line Item Sequence Number | 3 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 message, however, it is required for this HL7 segment, so this value will be a |  |  |  |  |  |  |  |  |
| sequential integer, starting at 1, for each occurrence of this segment. |  |  |  |  |  |  |  |  |
| Product/Service Line Item Status | 6 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 6.1 | ST | 20 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for this X12 message, however, it is required for this HL7 segment, so this value will be set to '1' |  |  |  |  |  |  |  |  |
| Product/Service Code | 7 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 7.1 | ST | 20 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 message, however, it is required for this HL7 segment, so this field will be set to '1' |  |  |  |  |  |  |  |  |
| Health Document Reference Identifier | 19 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 19.1 | ST | 2 | RE |  | 3 | File: 356.22 (278) Field: 11.01 (REPORT TYPE CODE) | Loop 2000E: PWK01 Attachment Report Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Attachment Report Type Code, which indicates the title or contents of a document, report or supporting item. |  |  |  |  |  |  |  |  |
| 03' (report justifying treatment beyond utilization guidelines), |  |  |  |  |  |  |  |  |
| '04' (drugs administered), |  |  |  |  |  |  |  |  |
| '05' (treatment diagnosis), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Namespace ID | 19.2 | IS | 80 | RE |  | 4312 | File: 356.22 (278) Field: 11.03 (ATTACHMENT CONTROL NUMBER) | Loop 2000E: PWK06 Attachment Control Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Attachment Control Number |  |  |  |  |  |  |  |  |
| Universal ID | 19.3 | ST | 80 | RE |  | Up to 80 characters of free text | File: 356.22 (278) Field: 11.04 (ATTACHMENT DESCRIPTION) | Loop 2000E: PWK07 Attachment Description |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Attachment Description. This is a free-form description to clarify the related data |  |  |  |  |  |  |  |  |
| elements and their content |  |  |  |  |  |  |  |  |
| Processing Consideration Code | 20 | IS | 10 | RE | FALSE | AA | File: 356.22 (278) Field: 11.02 (REPORT TRANSMISSION CODE) | Loop 2000E: PWK02 Report Transmission Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Report Transmission Code. This code defines the timing, transmission method, or format |  |  |  |  |  |  |  |  |
| by which reports are to be sent. |  |  |  |  |  |  |  |  |
| AA' (available on request at provider site), |  |  |  |  |  |  |  |  |
| 'BM' (by mail), |  |  |  |  |  |  |  |  |
| 'EL' (electronically only), |  |  |  |  |  |  |  |  |
| 'EM' (E-mail), |  |  |  |  |  |  |  |  |
| 'FX' (by fax), |  |  |  |  |  |  |  |  |
| 'VO' (voice) |  |  |  |  |  |  |  |  |
| Restricted Disclosure Indicator | 21 | ID | 2 | R | FALSE | NA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message; however, it is required for this HL7 segment. Therefore, this field will be set to 'NA' |  |  |  |  |  |  |  |  |

### RPA_I08 Response NTE segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Comment | 3 | FT | 264 | RE | FALSE | Up to 264 characters of free form text | File: 356.22 (278) Subfile: 356.2212 (MESSAGE TEXT) Field: .01 (MESSAGE TEXT) | Loop 2000E: MSG01 Free form message text |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with Free Form Message Text. This is a free form format that allows the transmission of |  |  |  |  |  |  |  |  |
| text information to the UMO about the patient event. |  |  |  |  |  |  |  |  |
| Comment Type | 4 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier | 1 | ST | 10 | RE |  | MSG 2000E | Fixed value 'MSG 2000E' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'MSG 2000E'. This allows FSC to identify this data for the X12 segment MSG in loop 2000E |  |  |  |  |  |  |  |  |

### RPA_I08 Response G3O Segment Group (optional) (repeating) – PRD segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role | 1 | CWE | 705 | R | TRUE |  |  |  |
| Identifier | 1.1 | ST | 3 | RE |  | 71 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .01 (PROVIDER TYPE) | Loop 2010EA: NM101 Entity Identifier Code or PRV01 Provider Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated within this repeating segment group for 4 different X12 segments. |  |  |  |  |  |  |  |  |
| For the NM1 segment in loop 2010EA, this field will contain the Entity Identifier Code. |  |  |  |  |  |  |  |  |
| '71' (attending physician), |  |  |  |  |  |  |  |  |
| '72' (operating physician), |  |  |  |  |  |  |  |  |
| 'DD' (assistant surgeon), |  |  |  |  |  |  |  |  |
| 'FA' (facility), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| For the PRV segment in loop 2010EA, this field will contain the Provider Code. |  |  |  |  |  |  |  |  |
| 'AD' (admitting), |  |  |  |  |  |  |  |  |
| 'AS' (assistant surgeon), |  |  |  |  |  |  |  |  |
| 'AT' (attending), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Text | 1.2 | ST | 1 | RE |  | 1 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .02 (PERSON / NON-PERSON) | Loop 2010EA: NM102 Entity Type Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For the NM1 segments in loops 2010EA, this field will contain the Entity Type Qualifier. |  |  |  |  |  |  |  |  |
| '1' (person), |  |  |  |  |  |  |  |  |
| '2' (non-person entity) |  |  |  |  |  |  |  |  |
| Alternate Identifier | 1.4 | ST | 10 | R |  | NM1 2010EA | Fixed value 'NM1 2010EA' or 'PRV 2010EA' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain text which identifies the X12 segment and Loop the data is for. The possible values are: |  |  |  |  |  |  |  |  |
| 'NM1 2010EA' |  |  |  |  |  |  |  |  |
| 'PRV 2010EA' |  |  |  |  |  |  |  |  |
| Provider Name | 2 | XPN | 1262 | RE | TRUE |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Field length has been expanded to 1262 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Family Name | 2.1 | FN | 194 | RE |  |  |  |  |
| � Surname | 2.1.1 | ST | 60 | R |  | Organization Name | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) or File: 4 (INSTITUTION) Field: .01 (NAME) | Loop 2010EA: NM103 Patient Event Provider Last or Organization Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For the NM1 segments in loops 2010EA, 2010EB, and 2010EC, this field will contain the value for NM103, the Name field. |  |  |  |  |  |  |  |  |
| Given Name | 2.2 | ST | 35 | RE |  | JOHN | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010EA: NM104 Patient Event Provider First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider First Name |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 2.3 | ST | 25 | RE |  | Middle | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010EA: NM105 Patient Event Provider Middle Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider First Name |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) | 2.4 | ST | 10 | RE |  | JR | FIle: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010EA: NM107 Patient Event Provider Name Suffix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider Name Suffix |  |  |  |  |  |  |  |  |
| Provider Address | 3 | XAD | 513 | RE | FALSE |  |  |  |
| Street Address | 3.1 | SAD | 184 | RE |  |  |  |  |
| Street or Mailing Address | 3.1.1 | ST | 55 | RE |  | Address Line 1 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .111 (STREET ADDRESS 1) or File: 4 (INSTITUTION) Field: 4.01 (ST. ADDR.1(MAILING)) | Loop 2010EA: N301 Patient Event Provider Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA , this field will contain the Address Line 1 data |  |  |  |  |  |  |  |  |
| Other Designation | 3.2 | ST | 55 | RE |  | Suite 200 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .112 (STREET ADDRESS 2) or File: 4 (INSTITUTION) Field: 4.02 (ST. ADDR. 2 (MAILING)) | Loop 2010EA: N302 Patient Event Provider Address Line 2 |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA, this field will contain the Address Line 2 data |  |  |  |  |  |  |  |  |
| City | 3.3 | ST | 30 | RE |  | City | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .114 (CITY) or File: 4 (INSTITUTION) Field: 4.03 (CITY (MAILING)) | Loop 2010EA: N401 Patient Event Provider City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA , this field will contain the City name |  |  |  |  |  |  |  |  |
| State or Province | 3.4 | ST | 2 | RE |  | DC | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .115 (STATE) orFile: 4 (INSTITUTION)Field: 4.04 (STATE (MAILING)) | Loop 2010EA: N402 Patient Event Provider State or Province Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA, this field will contain the State or Province Code |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 3.5 | ST | 15 | RE |  | 21044 | File: 356.22 (278) Subfile: 256.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .116 (ZIP CODE) or File: 4 (INSTITUTION) Field: 4.05 (ZIP (MAILING)) | Loop 2010EA: N403 Patient Event Provider Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loops 2010EA, this field will contain the State or Province Code |  |  |  |  |  |  |  |  |
| Country | 3.6 | ID | 3 | RE |  | ALB | Not available in the Vista database for file 200 (NEW PERSON) or File 4 (INSTITUTION) | Loop 2010EA: N404 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment N4 in loop 2010EA, this field will contain the Patient Event Provider Country Code, if outside the USA |  |  |  |  |  |  |  |  |
| Other Geographic Designation | 3.8 | ST | 3 | RE |  | HID | Not available in the Vista database for file 200 (NEW PERSON) or File 4 (INSTITUTION) | Loop 2010EA: N407 Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment N4 in loop 2010EA, this field will contain the Country Subdivision Code if the Patent Event was outside the |  |  |  |  |  |  |  |  |
| USA and the country has subdivision codes |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 | PLN | 100 | RE | FALSE |  |  |  |
| ID Number | 7.1 | ST | 80 | R |  | ID12344 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: 41.99 (NPI) or File: 4 (INSTITUTION) Field: 41.99 (NPI) or Field: 4.043 (TAXONOMY CODE) | Loop 2010EA: NM109 Patient Event Provider ID or PRV03 Provider Taxonomy Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For X12 segment NM1 in loop 2010EA, this field will contain the Patient Event Provider ID. |  |  |  |  |  |  |  |  |
| For X12 segment PRV in loop 2010EA, this field will contain the Provider Taxonomy Code |  |  |  |  |  |  |  |  |

### RPA_I08 Response G3O Segment Group (optional) (repeating) – MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however it is an HL7 required field, so it will be populated with |  |  |  |  |  |  |  |  |
| AE' for any application errors reported by FSC |  |  |  |  |  |  |  |  |
| AR' for AAA reject reasons reported by HCCH |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 1231456 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field for HL7, so it should be set to the |  |  |  |  |  |  |  |  |
| Message Control ID (MSH.10) of the original Request 278 message |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 10 | RE | FALSE | AAA 2010EA | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'AAA 2010EA', identifying this data is from segment AAA in the 2010EA loop. |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | C | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2010EA: AAA04 Follow-up Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field value will be set to the value found in the AAA segment 04 Follow-up Action Code field. Value should be: |  |  |  |  |  |  |  |  |
| 'C' - please correct and resubmit |  |  |  |  |  |  |  |  |
| 'N' - resubmission not allowed |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CNE | 483 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 | RE |  | 15 | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) | Loop 2010EA: AAA03 Reject Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Reject Reason Code (if applicable) |  |  |  |  |  |  |  |  |
| '15' - required application data missing |  |  |  |  |  |  |  |  |
| '33' - input errors |  |  |  |  |  |  |  |  |
| 52' - service dates not within provider plan enrollment |  |  |  |  |  |  |  |  |
| '58' - invalid/missing date of birth |  |  |  |  |  |  |  |  |
| '64' - invalid/missing patient ID |  |  |  |  |  |  |  |  |
| etc... |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 80 | RE |  | Error text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be used for FSC to report text describing an error condition that was not reported by the HCCH via X12 |  |  |  |  |  |  |  |  |

### RPA_I08 Response G3O Segment Group (optional) (repeating) – NK1 segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID - NK1 | 1 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Field is not used for the X12 278 response message, however, it is required for HL7, so we will set this field to ' 1 ' |  |  |  |  |  |  |  |  |
| Name | 2 | XPN | 294 | RE | TRUE |  |  |  |
| Family Name | 2.1 | FN | 194 | RE |  |  |  |  |
| � Surname | 2.1.1 | ST | 60 | R |  | SMITH | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.05 (CONTACT LAST OR COMPANY NAME) | Loop 2010EB: NM103 Response Contact Last or Organization Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM103 Response Contact Last or Organization Name |  |  |  |  |  |  |  |  |
| Given Name | 2.2 | ST | 35 | RE |  | JOHN | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.05 (CONTACT FIRST NAME) | Loop 2010EB: NM104 Response Contact First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM104 Response Contact First Name |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 2.3 | ST | 25 | RE |  | MIDDLE | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.06 (CONTACT MIDDLE NAME) | Loop 2010EB: NM105 Response Contact Middle Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM105 Response Contact Middle Name |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) | 2.4 | ST | 10 | RE |  | JR | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.07 (CONTACT NAME SUFFIX) | Loop 2010EB: NM107 Response Contact Name Suffix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM107 Response Contact Name Suffix |  |  |  |  |  |  |  |  |
| Name Context | 2.9 | CWE | 1 | RE |  |  |  |  |
| � Identifier | 2.9.1 | ST | 1 | RE |  | 1 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.08 (IDENTIFICATION CODE QUALIFIER) | Loop 2010EB: NM102 Entity Type Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| '1' – Person |  |  |  |  |  |  |  |  |
| '2' - Non-person entity |  |  |  |  |  |  |  |  |
| Address | 4 | XAD | 513 | RE | FALSE |  |  |  |
| Street Address | 4.1 | SAD | 55 | RE |  |  |  |  |
| � Street or Mailing Address | 4.1.1 | ST | 55 | RE |  | 100 MAIN STREET | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5 (CONTACT ADDRESS LINE-1) | Loop 2010EB: N301 Response Contact Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N301 Response Contact Address Line |  |  |  |  |  |  |  |  |
| Other Designation | 4.2 | ST | 55 | RE |  | SUITE 200 | File: 356.22 (278) Subfile: 356.2213PA(PATIENT EVENT PROVIDER) Field: 5.01 (CONTACT ADDRESS LINE-2) | Loop 2010EB: N302 Response Contact Address Line 2 |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N302 Response Contact Address Line 2 |  |  |  |  |  |  |  |  |
| City | 4.3 | ST | 30 | RE |  | CITY | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.02 (CONTACT CITY NAME) | Loop 2010EB: N401 Additional Patient Information Contact City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N401 Additional Patient Information Contact City Name |  |  |  |  |  |  |  |  |
| State or Province | 4.4 | ST | 2 | RE |  | VA | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.03 (CONTACT STATE OR PROVINCE) | Loop 2010EB: N402 Additional Patient Information Contact State Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N402 Additional Patient Information Contact State Code |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 4.5 | ST | 15 | RE |  | 33311 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.04 (CONTACT ZIP CODE) | Loop 2010EB: N403 Additional Patient Information Contact Postal Code or Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N403 Additional Patient Information Contact Postal Code or Zip Code |  |  |  |  |  |  |  |  |
| Country | 4.6 | ID | 3 | RE |  | ALB | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.05 (CONTACT COUNTRY CODE) | Loop 2010EB: N404 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N404 Country Code |  |  |  |  |  |  |  |  |
| Other Geographic Designation | 4.8 | ST | 3 | RE |  | 321 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.06 (CONTACT COUNTRY SUBDIVISION) | Loop 2010EB: N407 Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N407 Country Subdivision Code |  |  |  |  |  |  |  |  |
| Contact Role | 7 | CWE | 705 | RE | FALSE |  |  |  |
| Identifier | 7.1 | ST | 2 | RE |  | 24 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.08 (IDENTIFICATION CODE QUALIFIER) | Loop 2010EB: NM108 Identification Code Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM108 Identification Code Qualifier |  |  |  |  |  |  |  |  |
| '24' (Employer's Identification Number), |  |  |  |  |  |  |  |  |
| '34' (SSN), |  |  |  |  |  |  |  |  |
| 46' (electronic transmitter identification number), |  |  |  |  |  |  |  |  |
| 'PI' (payer identification), |  |  |  |  |  |  |  |  |
| 'XV' (CMS plan ID), |  |  |  |  |  |  |  |  |
| 'XX' (CMS national provider identifier) |  |  |  |  |  |  |  |  |
| Text | 7.2 | ST | 80 | RE |  | 2ND FLOOR | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.09 (IDENTIFICATION CODE) | Loop 2010EB: NM109 Response Contact Identifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM109 Response Contact Identifier |  |  |  |  |  |  |  |  |
| Next of Kin / Associated Parties Employee Number | 12 | CX | 3503 | RE | FALSE |  |  |  |
| ID Number | 12.1 | ST | 15 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used as part of this X12 message, however, it is a required field, so we will set this field equal to 1 |  |  |  |  |  |  |  |  |
| Mother's Maiden Name | 26 | XPN | 294 | RE | TRUE |  |  |  |
| Family Name | 26.1 | FN | 194 | RE |  |  |  |  |
| Surname | 26.1.1 | ST | 50 | R |  | SURNAME |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for this X12 message, so it will be populated with 'SURNAME' |  |  |  |  |  |  |  |  |
| Contact Person's Name | 30 | XPN | 294 | RE | TRUE |  |  |  |
| Family Name | 30.1 | FN | 194 | RE |  |  |  |  |
| Surname | 30.1.1 | ST | 50 | R |  | SURNAME |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for this X12 message, so this field will be populated with 'SURNAME' |  |  |  |  |  |  |  |  |
| Next of Kin/Associated Party's Identifiers | 33 | CX | 3503 | RE | TRUE |  |  |  |
| ID Number | 33.1 | ST | 15 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not being used for the X12 message, so this field will be populated with a ' 1 ' |  |  |  |  |  |  |  |  |

### RPA_I08 Response G3O Segment Group (optional) (repeating) – CTD segment (Optional)

| Name | Seq |  | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Role | 1 |  | CWE | 705 | R | TRUE |  |  |  |
| Identifier | 1.1 |  | ST | 20 | R |  | PER 2010EB | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .04 (PER LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be set to 'PER 2010EB'. This identifies this data is from X12 segment PER in loop 2010EB |  |  |  |  |  |  |  |  |  |
| Contact Name | 2 |  | XPN | 294 | RE | TRUE |  |  |  |
| Family Name | 2.1 |  | FN | 194 | RE |  |  |  |  |
| � Surname | 2.1.1 |  | ST | 60 | R |  | SURNAME | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .06 (CONTACT NAME) | Loop 2010EB: PER02 Response Contact Name |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| PER02 Response Contact Name |  |  |  |  |  |  |  |  |  |
| Contact Communication Information | 5 |  | XTN | 2527 | RE | TRUE |  |  |  |
| Telecommunication Equipment Type | 5.2 |  | ID | 8 | RE |  | X.400 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: .07 (COMMUNICATION NO. QUALIFIER-1) Or Field: .08 (COMMUNICATION NO. QUAIFIER-2) Or Field: .09 (COMMUNICATION NO. QUALIFIER-3) | Loop 2010EB: (1st occurrence) PER03 Communication Number Qualifier or (2nd occurrence) PER05 Communication Number Qualifier or (3rd occurrence) PER07 Communication Number Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| PER03 Communication Number Qualifier |  |  |  |  |  |  |  |  |  |
| 'EM' (Electronic Mail), |  |  |  |  |  |  |  |  |  |
| 'FX' (facsimile), |  |  |  |  |  |  |  |  |  |
| 'TE' (telephone), |  |  |  |  |  |  |  |  |  |
| 'UR' (uniform resource locator (URL)) |  |  |  |  |  |  |  |  |  |
| This field is associated with an HL7 table, so a cross mapping is required |  |  |  |  |  |  |  |  |  |
| 'EM' ==> 'X.400' |  |  |  |  |  |  |  |  |  |
| 'FX' ==> 'FX' |  |  |  |  |  |  |  |  |  |
| 'TE' ==> 'PH' |  |  |  |  |  |  |  |  |  |
| 'UR' ==> 'Internet' |  |  |  |  |  |  |  |  |  |
| Any Text | 5.8 |  | ST | 256 | RE |  | (814) 555-1212 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 1 (CONTACT COMMUNICATION NUMBER-1) Or Field: 2 (CONTACT COMMUNICATION NUMBER-2) Or Field: 3 (CONTACT COMMUNICATION NUMBER-3) | Loop 2010EB: (1st occurrence) PER04 Response Contact Communication Number or (2nd occurrence) PER06 Response Contact Communication Number or (3rd occurrence) PER08 Response Contact Communication Number |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| PER04 Response Contact Communication Number |  |  |  |  |  |  |  |  |  |
| Contact Identifiers |  | 7 | PLN | 100 | RE | TRUE |  |  |  |
| ID Number |  | 7.1 | ST | 20 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, so this field will be populated with ' 1 ' |  |  |  |  |  |  |  |  |  |
| Type of ID Number |  | 7.2 | IS | 8 | R |  | VA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, so this field will be populated with ' VA' |  |  |  |  |  |  |  |  |  |

### RPA_I08 Response G3O Segment Group (optional) (repeating) – PRD segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role | 1 | CWE | 705 | R | TRUE |  |  |  |
| Identifier | 1.1 | ST | 3 | R |  | 45 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.02 (ENTITY IDENTIFIER CODE) | Loop 2010EC: NM101 Entity Identifier Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM101 Entity Identifier Code |  |  |  |  |  |  |  |  |
| '45' (Drop-off location), |  |  |  |  |  |  |  |  |
| 'FS' (Final Scheduled Destination), |  |  |  |  |  |  |  |  |
| 'ND' (Next Destination), |  |  |  |  |  |  |  |  |
| 'PW' (pickup address), |  |  |  |  |  |  |  |  |
| 'R3' (Next scheduled Destination) |  |  |  |  |  |  |  |  |
| Alternate Identifier | 1.4 | ST | 10 | RE |  | NM1 2010EC | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER Field: 4.01 (NM1 LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| 'NM1 2010EC' which indicates this data is |  |  |  |  |  |  |  |  |
| from X12 segment NM1 loop 2010EC |  |  |  |  |  |  |  |  |
| Provider Name | 2 | XPN | 294 | RE | TRUE |  |  |  |
| Family Name | 2.1 | FN | 194 | RE |  |  |  |  |
| Surname | 2.1.1 | ST | 60 | R |  | Location Name | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 4.04 (CONTACT LAST OR COMPANY NAME) | Loop 2010EC: NM103 Patient Event Transport Location Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM103 Patient Event Transport Location Name |  |  |  |  |  |  |  |  |
| Provider Address | 3 | XAD | 513 | RE | FALSE |  |  |  |
| Street Address | 3.1 | SAD | 55 | RE |  |  |  |  |
| Street or Mailing Address | 3.1.1 | ST | 55 | RE |  | Transport Location Address line 1 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5 (CONTACT ADDRESS LINE-1) | Loop 2010EC: N301 Patient Event Transport Location Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N301 Patient Event Transport Location Address Line |  |  |  |  |  |  |  |  |
| Other Designation | 3.2 | ST | 55 | RE |  | Suite 200 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.01 (CONTACT ADDRESS LINE-2) | Loop 2010EC: N302 Patient Event Transport Location Address Line 2 |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N302 Patient Event Transport Location Address Line 2 |  |  |  |  |  |  |  |  |
| City | 3.3 | ST | 30 | RE |  | Location City | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.02 (CONTACT CITY NAME) | Loop 2010EC: N401 Patient Event Transport Location City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N401 Patient Event Transport Location City Name |  |  |  |  |  |  |  |  |
| State or Province | 3.4 | ST | 2 | RE |  | VA | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.03 (CONTACT STATE OR PROVINCE) | Loop 2010EC: N402 Patient Event Transport Location State Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N402 Patient Event Transport Location State Code |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 3.5 | ST | 15 | RE |  | 21211 | File: 356.22 (278) Subfile: 356.2213PA (PATIENT EVENT PROVIDER) Field: 5.04 (CONTACT ZIP CODE) | Loop 2010EC: N403 Patient Event Transport Postal Code or Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N403 Patient Event Transport Postal Code or Zip Code |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 | PLN | 100 | RE | TRUE |  |  |  |
| ID Number | 7.1 | ST | 20 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message; however, it is required for HL7. |  |  |  |  |  |  |  |  |
| Fixed value of '1' used. |  |  |  |  |  |  |  |  |
| Type of ID Number | 7.2 | IS | 8 | R |  | QA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message; however it is an HL7 required field. |  |  |  |  |  |  |  |  |
| Fixed value of 'QA' |  |  |  |  |  |  |  |  |

### RPA_I08 Response G3O Segment Group (optional) (repeating) – MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |  |  |  |  |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AE' (MSA standard value for application error) or 'AR" (MSA standard value for application reject) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 312313 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |  |  |  |  |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Message Control ID (MSH.10) or original request 278 message |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 10 | RE | FALSE | AAA 2010EC | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) |  |  |  |  |  |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This field will be populated with 'AAA 2010EC', identifying this data is from segment AAA in the 2010EC loop. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | N | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2010EC: AAA04 Follow-up Action Code |  |  |  |  |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AAA04 Follow-up Action Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 'C' (please correct and resubmit), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 'N' (resubmission not allowed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CWE | 483 | RE | FALSE |  |  |  |  |  |  |  |  |  |  |
| Identifier | 6.1 | ST | 10 | RE |  | 15 | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) | Loop 2010EC: AAA03 Reject Reason Code |  |  |  |  |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AAA03 Reject Reason Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '15' (required application data missing), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '33' (input errors), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52' (service dates not within provider plan enrollment), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '57' (invalid/missing date(s) of service), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 80 | RE |  | Error text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) |  |  |  |  |  |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Implementation Note: |
| For FSC: This is text that can be used to identify an error that was not reported by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| the HCCH as a standard X12 AAA error condition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (optional) (repeating) – MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| AE' (MSA standard value for application error) or 'AR" (MSA standard value for application reject) |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  |
| For TRN Service Trace Number segment, this field will be set to 'AA' |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 1232132 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, it is a required field for HL7, so it should be set to the |  |  |  |  |  |  |  |  |
| Message Control ID (MSH.10) of the original Request 278 message |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  |
| For TRN Patient Event Tracking Number segment, this field will be set to 'TRN 2000F' |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 50 | RE | FALSE | AAA 2000F | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) | Loop 2000F: TRN02 Service Trace Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| AAA 2000F', which identifies this data is from X12 segment AAA loop 2000F |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  |
| For TRN Service Trace Number X12 segment, this field will have the TRN02 Service Trace Number in it |  |  |  |  |  |  |  |  |
| Field length has been expanded to 50 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | C | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2000F: AAA04 Follow-up Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| AAA04 Follow-up Action Code |  |  |  |  |  |  |  |  |
| 'C' (please correct and resubmit), |  |  |  |  |  |  |  |  |
| 'N' (resubmission not allowed) |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CWE | 483 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 | RE |  | 33 | File: 356.22 (278) Subfile: 356.221105A (TRN SEGMENTS) Field: .05 (TRACE ASSIGNING ENTITY ID) Or Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) | Loop 2000F: TRN03 Trace Assigning Entity ID or AAA03 Reject Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| AAA03 Reject Reason Code |  |  |  |  |  |  |  |  |
| '15' (required application data missing), |  |  |  |  |  |  |  |  |
| '33' (input errors), |  |  |  |  |  |  |  |  |
| 52' (service dates not within provider plan enrollment), |  |  |  |  |  |  |  |  |
| '57' (invalid/missing date(s) of service), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Or |  |  |  |  |  |  |  |  |
| Or for the TRN Service Tracking Number segment, this field will have the Trace Assigning Entity Identifier, which |  |  |  |  |  |  |  |  |
| identifies organization that assigned the trace number |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 199 | RE |  | Error text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) Or Subfile: 356.22105A (TRN SEGMENTS) Field: .06 (TRACE ASSIGNING ENTITY ID-2) | Loop 2000F: TRN04 Trace Assigning Entity Additional ID |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For FSC: This is text that can be used to identify an error that was not reported by the HCCH as a standard X12 AAA error condition. |  |  |  |  |  |  |  |  |
| Or |  |  |  |  |  |  |  |  |
| For the TRN Service Tracking Number segment, this field will have the TRN04 Trace Assigning Entity Additional ID value. |  |  |  |  |  |  |  |  |
| Message Waiting Number | 7 | NM | 2 | RE | FALSE | 1 | File: 356.22 (278) Subfile: 356.22105A (TRN SEGMENTS) Field: .03 (TRACE TYPE CODE) | Loop 2000F: TRN01 Trace Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For the TRN Service Tracking X12 segment, this value will be set to one of the following: |  |  |  |  |  |  |  |  |
| 1' (current transaction trace numbers, trace number assigned by the creator of the 278 request), |  |  |  |  |  |  |  |  |
| 2' (referenced transaction trace numbers) |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (optional) (repeating) – PRB segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action Code | 1 | ID | 2 | R | FALSE | CO | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .01 (REQUEST CATEGORY) | Loop 2000F: UM01 Request Category Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Request Category Code. |  |  |  |  |  |  |  |  |
| 'HS' (Health Services Review), |  |  |  |  |  |  |  |  |
| 'SC' (Specialty Care Review) |  |  |  |  |  |  |  |  |
| This field is associated with an HL7 table, so a mapping is required. |  |  |  |  |  |  |  |  |
| 'HS' ==> 'CO' |  |  |  |  |  |  |  |  |
| 'SC' ==> 'AD' |  |  |  |  |  |  |  |  |
| Action Date/Time | 2 | DTM | 8 | R | FALSE | 20140422 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This X12 message is not using this field; however, it is required for this HL7 segment. Therefore, this field will be set |  |  |  |  |  |  |  |  |
| with the message transmission date, in format CCYYMMDD |  |  |  |  |  |  |  |  |
| Problem ID | 3 | CWE | 705 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 1 | R |  | I | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .02 (CERTIFICATION TYPE CODE) | Loop 2000F: UM02 Certification Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The field will contain the Certification Type Code |  |  |  |  |  |  |  |  |
| '1' (appeal - immediate), |  |  |  |  |  |  |  |  |
| '2' (appear - standard), |  |  |  |  |  |  |  |  |
| '3' (cancel), |  |  |  |  |  |  |  |  |
| '4' (extension), |  |  |  |  |  |  |  |  |
| 'I' (initial), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Text | 3.2 | ST | 2 | RE |  | 1 | Field: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .03 (SERVICE TYPE CODE) | Loop 2000F: UM03 Service Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Service Type Code |  |  |  |  |  |  |  |  |
| '1' (medical care), |  |  |  |  |  |  |  |  |
| '2' (surgical), |  |  |  |  |  |  |  |  |
| '3' (consultation), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Problem Instance ID | 4 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier | 4.1 | ST | 199 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This X12 message will not be using this field, however, this field is required for this HL7 segment, therefore, we will set this field equal to '1' |  |  |  |  |  |  |  |  |
| Episode of Care ID | 5 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 5.1 | ST | 10 | RE |  | UM 2000F | Fixed value 'UM 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to 'UM 2000F' This identifies the data from this segment is from X12 segment XM loop 2000F |  |  |  |  |  |  |  |  |
| Problem Classification | 10 | CWE | 705 | RE | FALSE |  |  |  |
| Identifier | 10.1 | ST | 2 | RE |  | 34 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .05 (FACILITY TYPE) | Loop 2000F: UM04-1 Facility Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Facility Type Code. |  |  |  |  |  |  |  |  |
| This code identifies where services were or may be performed; the 1st and 2nd positions of the Uniform Bill Type |  |  |  |  |  |  |  |  |
| Code for Institutional Services, or the Place of Service Codes for Professional and Dental services |  |  |  |  |  |  |  |  |
| Text | 10.2 | ST | 2 | RE |  | A | Field 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .04 (FACILITY TYPE QUALIFIER) | Loop 2000F: UM04-2 Facility Code Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with a Facility Code Qualifier |  |  |  |  |  |  |  |  |
| 'A' (uniform billing claim form bill type), |  |  |  |  |  |  |  |  |
| B' (place of service codes for professional or dental services) |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – AUT segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authorizing Payer, Company ID | 2 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 2.1 | ST | 10 | R |  | REF 2000F |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'HCR 2000F' or 'REF 2000F' This is for FSC to be able to determine that this data is for |  |  |  |  |  |  |  |  |
| X12 segment REF in loop 2000F |  |  |  |  |  |  |  |  |
| Text | 2.2 | ST | 50 | RE |  | REV123 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 9.01 (PREV. AUTHORIZATION) or 9.02 (PREV. ADMIN, REFERENCE) or Subfile: 356.22103A (HCR SEGMENTS) Field: .05 (REVIEW DECISION REASON CODE) | Loop 2000F: HCR03 Review Decision Reason Code or REF02 Previous Review Authorization Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Previous Review Authorization Number if there is one |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  |
| HCR03 Review Decision Reason Code |  |  |  |  |  |  |  |  |
| Alternate Text | 2.5 | ST | 3 | RE |  | BB | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 9.01 (PREV. AUTHORIZATION) or 9.02 (PREV. ADMIN, REFERENCE) or Subfile: 356.22103A (HCR SEGMENTS) Field: .06 (SECOND SURGICAL OPINION FLAG | Loop 2000F: HCR04 Second Surgical Opinion Indicator or REF01 Reference Identification Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with a Reference Identification Qualifier |  |  |  |  |  |  |  |  |
| 'BB' (authorization number), |  |  |  |  |  |  |  |  |
| 'NT' (Administrator's reference number) |  |  |  |  |  |  |  |  |
| Or |  |  |  |  |  |  |  |  |
| HCR04 Second Surgical Opinion Indicator |  |  |  |  |  |  |  |  |
| Authorization Identifier | 6 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 6.1 | ST | 50 | RE |  | 32132 | File: 356.22 (278) Subfile: 356.22103A (HCR SEGMENTS) Field: .04 (REVIEW IDENTIFICATION NUMBER) | Loop 2000F: HCR02 Review Identification Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| HCR02 Review Identification Number |  |  |  |  |  |  |  |  |
| Universal ID | 6.3 | ST | 2 | RE |  | A1 | File: 356.22 (278) Subfile: 356.22103A (HCR SEGMENTS) Field: .03 (CERTIFICATION ACTION CODE) | Loop 2000F: HCR01 Certification Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| HCR01 Certification Action Code |  |  |  |  |  |  |  |  |
| 'A1' (Certified in total), |  |  |  |  |  |  |  |  |
| 'A2' (Certified - partial), |  |  |  |  |  |  |  |  |
| 'A3' (not certified), |  |  |  |  |  |  |  |  |
| 'A4' (pended), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – ZTP segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date/Time Qualifier | 2 | CWE | 230 | R | FALSE |  |  |  |
| identifier | 2.1 | ST | 3 | R |  | 472 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to the Date / Time Qualifier |  |  |  |  |  |  |  |  |
| '472' (Service) |  |  |  |  |  |  |  |  |
| '102' (Issue) |  |  |  |  |  |  |  |  |
| '036' (Expiration) |  |  |  |  |  |  |  |  |
| '007' (Effective) |  |  |  |  |  |  |  |  |
| Date Time Period | 3 | DR | 51 | R | FALSE |  |  |  |
| range start date/time | 3.1 | DTM | 8 | R |  | 20100230 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: .11 (SERVICE DATE) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will contain the Proposed or Actual Service Date in the format CCYYMMDD |  |  |  |  |  |  |  |  |
| Loop ID | 4 | ST | 10 | R | FALSE | DTP 2000F |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to 'DTP 2000F'. This allows FSC to determine that this data is for X12 segment DTP in loop 2000F |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 |  |  |  |  |  |  |  |  |
| characters to allow for X12 value |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – DG1 segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID - DG1 | 1 | SI | 4 | R | FALSE | 1 |  | Loop 2000F: HI01 thru HI12 (up to 12 Health Care Code information) |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to a sequential numerical value, from 1 through 12 |  |  |  |  |  |  |  |  |
| Diagnosis Code - DG1 | 3 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 3.1 | ST | 30 | R |  | 463.1 | File: 356.22 (278) Subfile: 356.22107A (HI SEGMENTS) Field: .03 (INDUSTRY CODE) | Loop 2000F: HI01-2 Industry Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| HI01-2 Industry Code |  |  |  |  |  |  |  |  |
| Diagnosis Classification | 17 | IS | 3 | RE | FALSE | LOI | File: 356.22 (278) Subfile: 356.22107A (HI SEGMENTS) Field: .02 (CODE LIST QUALIFIER CODE) | Loop 2000F: HI01-1 Code List Qualifier Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| HI01-1 Code List Qualifier Code |  |  |  |  |  |  |  |  |
| LOI' (Logical Observation Identifier Names and Codes (LOINC) Codes) |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – PSL segment (Optional) (Repeating)

| Name |  | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Product/Service Line Item Number |  | 1 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier |  | 1.1 | ST | 10 | R |  | SV1 2000F |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated with one of the following values. These values provide FSC the information to determine |  |  |  |  |  |  |  |  |  |
| what X12 segment and loop this data applies to. |  |  |  |  |  |  |  |  |  |
| SV1 2000F' |  |  |  |  |  |  |  |  |  |
| 'SV2 2000F' |  |  |  |  |  |  |  |  |  |
| 'SV3 2000F' |  |  |  |  |  |  |  |  |  |
| 'TOO 2000F' |  |  |  |  |  |  |  |  |  |
| Product/Service Line Item Sequence Number |  | 3 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not required for this X12 message, however, it is a required field for the HL7 segment, so this field will be |  |  |  |  |  |  |  |  |  |
| populated with a sequential number, starting at 1 |  |  |  |  |  |  |  |  |  |
| Product/Service Line Item Status |  | 6 | CWE | 697 | R | FALSE |  |  |  |
| Identifier |  | 6.1 | ST | 20 | R |  | P |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message; however, this field is required for this HL7 segment. So this field will be set to 'P' for processed |  |  |  |  |  |  |  |  |  |
| Product/Service Code |  | 7 | CWE | 697 | R | FALSE |  |  |  |
| Identifier |  | 7.1 | ST | 3 | R |  | 542 | File 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.01 (PROCEDURE CODE TYPE) | Loop 2000F: SV101-1 Product or Service ID Qualifier or SV202-1 Product or Service ID Qualifier or SV301-1 Product or Service ID Qualifier or TOO01 Code List Qualifier Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field will be populated as follows: |  |  |  |  |  |  |  |  |  |
| SV101-1 Product or Service ID Qualifier |  |  |  |  |  |  |  |  |  |
| HC' (Health Care Financing Administration Common Procedural Coding System (HCPCS) codes), |  |  |  |  |  |  |  |  |  |
| 'N4' (national drug code in 5-4-2 format |  |  |  |  |  |  |  |  |  |
| SV202-1 Product or Service ID Qualifier |  |  |  |  |  |  |  |  |  |
| HC' (health care financing administration common procedural coding system (HCPCS) codes), |  |  |  |  |  |  |  |  |  |
| ID' (international classification of diseases), |  |  |  |  |  |  |  |  |  |
| 'N4' (national drug code in 5-4-2 format), |  |  |  |  |  |  |  |  |  |
| 'ZZ' (mutually defined) |  |  |  |  |  |  |  |  |  |
| SV301-1 Product or Service ID Qualifier |  |  |  |  |  |  |  |  |  |
| AD' (American Dental Association Codes) |  |  |  |  |  |  |  |  |  |
| TOO01 Code List Qualifier Code |  |  |  |  |  |  |  |  |  |
| JP' (universal national tooth designation system) |  |  |  |  |  |  |  |  |  |
| Text |  | 7.2 | ST | 48 | RE |  | D9221 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) File: 1.02 (PROCEDURE #1) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .01 (TOOTH CODE) | Loop 2000F: SV101-2 Procedure Code or SV202-2 Procedure Code or SV301-2 Procedure Code or TOO02 Tooth Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV101-2 Procedure Code |  |  |  |  |  |  |  |  |  |
| SV202-2 Procedure Code |  |  |  |  |  |  |  |  |  |
| SV301-2 Procedure Code |  |  |  |  |  |  |  |  |  |
| TOO02 Tooth Code |  |  |  |  |  |  |  |  |  |
| Alternate Text |  | 7.5 | ST | 48 | RE |  | D9221 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.03 (PROCEDURE #2) | Loop 2000F: SV101-8 Procedure Code or SV202-8 Procedure Code or SV301-8 Procedure Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV101-8 Procedure Code |  |  |  |  |  |  |  |  |  |
| SV202-8 Procedure Code |  |  |  |  |  |  |  |  |  |
| SV301-8 Procedure Code |  |  |  |  |  |  |  |  |  |
| Product/Service Code Modifier |  | 8 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier |  | 8.1 | ST | 2 | RE |  | 21 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.04 (PROCEDURE MODIFIER #1) | Loop 2000F: SV101-3 Procedure Modifier or SV202-3 Procedure Modifier or SV301-3 Procedure Modifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV101-3 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| SV202-3 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| SV301-3 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| Text |  | 8.2 | ST | 2 | RE |  | 24 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.05 (PROCEDURE MODIFIER #2) | Loop 2000F: SV101-4 Procedure Modifier or SV202-4 Procedure Modifier or SV301-4 Procedure Modifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV101-4 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| SV202-4 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| SV301-4 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| Alternate Identifier |  | 8.4 | ST | 2 | RE |  | 25 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.06 (PROCEDURE MODIFIER #3) | Loop 2000F: SV101-5 Procedure Modifier or SV202-5 Procedure Modifier or SV301-5 Procedure Modifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV101-5 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| SV202-5 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| SV301-5 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| Alternate Text |  | 8.5 | ST | 2 | RE |  | 27 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.07 (PROCEDURE MODIFIER #4) | Loop 2000F: SV101-6 Procedure Modifier or SV202-6 Procedure Modifier or SV301-6 Procedure Modifier |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV101-6 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| SV202-6 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| SV301-6 Procedure Modifier |  |  |  |  |  |  |  |  |  |
| Product/Service Code Description |  | 9 | ST | 80 | RE | FALSE | ORAL EVALUATION | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.08 (PROCEDURE DESCRIPTION) | Loop 2000F: SV101-7 Procedure Code Description or SV202-7 Procedure Code Description or SV301-7 Procedure Code Description |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV101-7 Procedure Code Description |  |  |  |  |  |  |  |  |  |
| SV202-7 Procedure Code Description |  |  |  |  |  |  |  |  |  |
| SV301-7 Procedure Code Description |  |  |  |  |  |  |  |  |  |
| Product/Service Quantity |  | 12 | CQ | 721 | RE | FALSE |  |  |  |
| Quantity |  | 12.1 | NM | 15 | RE |  | 2 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.11 (UNIT COUNT) | Loop 2000F: SV104 Service Unit Count or SV205 Service Unit Count or SV306 Service Unit Count |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV104 Service Unit Count |  |  |  |  |  |  |  |  |  |
| SV205 Service Unit Count |  |  |  |  |  |  |  |  |  |
| SV306 Service Unit Count |  |  |  |  |  |  |  |  |  |
| Units | 12.2 |  | CWE | 2 | RE |  |  |  |  |
| � Identifier | 12.2.1 |  | ST | 2 | RE |  | UN | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.1 (UNITS CODE) | Loop 2000F: SV103 Unit or Basis for Measurement Code or SV204 Unit or Basis for Measurement Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV103 Unit or Basis for Measurement Code |  |  |  |  |  |  |  |  |  |
| 'F2' (International Unit), |  |  |  |  |  |  |  |  |  |
| 'MJ' (minutes), |  |  |  |  |  |  |  |  |  |
| 'UN' (unit) |  |  |  |  |  |  |  |  |  |
| SV204 Unit or Basis for Measurement Code |  |  |  |  |  |  |  |  |  |
| 'DA' (days), |  |  |  |  |  |  |  |  |  |
| 'F2' (international unit), |  |  |  |  |  |  |  |  |  |
| 'UN' (unit) |  |  |  |  |  |  |  |  |  |
| Product/Service Unit Cost |  | 13 | CP | 20 | RE | FALSE |  |  |  |
| Price |  | 13.1 | MO | 20 | R |  |  |  |  |
| � Quantity |  | 13.1.1 | NM | 10 | R |  | 2 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.07 (RATE) | Loop 2000F: SV206 Service Line Rate |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV206 Service Line Rate |  |  |  |  |  |  |  |  |  |
| The rate per unit of associate revenue for hospital accommodation |  |  |  |  |  |  |  |  |  |
| Product/Service Gross Amount |  | 15 | CP | 765 | RE | FALSE |  |  |  |
| Price |  | 15.1 | MO | 20 | R |  |  |  |  |
| � Quantity |  | 15.1.1 | NM | 18 | R |  | 100 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 1.09 (SERVICE LINE AMOUNT) | Loop 2000F: SV102 Service Line Amount or SV203 Service Line Amount or SV302 Service Line Amount |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV102 Service Line Amount |  |  |  |  |  |  |  |  |  |
| SV203 Service Line Amount |  |  |  |  |  |  |  |  |  |
| SV302 Service Line Amount |  |  |  |  |  |  |  |  |  |
| Product/Service Clarification Code Type |  | 17 | IS | 48 | RE | FALSE | 313 | File 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.06 (REVENUE CODE) or Field: 3.06 (PLACEMENT STATUS) | Loop 2000F: SV201 Service Line Revenue Code or SV305 Prosthesis, Crown, or Inlay Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV201 Service Line Revenue Code |  |  |  |  |  |  |  |  |  |
| SV305 Prosthesis, Crown, or Inlay Code |  |  |  |  |  |  |  |  |  |
| Field length has been expanded to 48 |  |  |  |  |  |  |  |  |  |
| characters to allow for X12 value |  |  |  |  |  |  |  |  |  |
| Restricted Disclosure Indicator |  | 21 | ID | 2 | R | FALSE | NA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| This field is not used for the X12 message, however, this field is required in the HL7 segment, so this field is set to 'NA' |  |  |  |  |  |  |  |  |  |
| Side of body |  | 33 | CWE | 697 | RE | FALSE |  |  |  |
| Identifier |  | 33.1 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.01 (ORAL CAVITY DESIGNATION #1) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .02 (TOOTH SURFACE #1) | Loop 2000F: SV304-1 Oral Cavity Designation Code or TOO03-1 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV304-1 Oral Cavity Designation Code |  |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |  |
| TOO03-1 Tooth Surface Code |  |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |
| Text |  | 33.2 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.02 (ORAL CAVITY DESIGNATION #2) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .03 (TOOTH SURFACE #2) | Loop 2000F: SV304-2 Oral Cavity Designation Code or TOO03-2 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV304-2 Oral Cavity Designation |  |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |  |
| TOO03-2 Tooth Surface Code |  |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |
| Alternate Identifier |  | 33.4 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.03 (ORAL CAVITY DESIGNATION #3) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .04 (TOOTH SURFACE #3) | Loop 2000F: SV304-3 Oral Cavity Designation Code or TOO03-3 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV304-3 Oral Cavity Designation |  |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |  |
| TOO03-4 Tooth Surface Code |  |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |
| Alternate Text |  | 33.5 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.04 (ORAL CAVITY DESIGNATION #4) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .05 (TOOTH SURFACE #4) | Loop 2000F: SV304-4 Oral Cavity Designation Code or TOO03-4 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV304-5 Oral Cavity Designation Code |  |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |  |
| TOO03-4 Tooth Surface Code |  |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |
| Original Text |  | 33.9 | ST | 3 | RE |  | B | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 3.05 (ORAL CAVITY DESIGNATION #5) or Sub-Subfile: 356.22164PA (TOOTH INFORMATION) Field: .06 (TOOTH SURFACE #5) | Loop 2000F: SV304-5 Oral Cavity Designation Code or TOO03-5 Tooth Surface Code |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV304-5 Oral Cavity Designation |  |  |  |  |  |  |  |  |  |
| (Code source 135: American Dental Association Codes) |  |  |  |  |  |  |  |  |  |
| TOO03-5 Tooth Surface Code |  |  |  |  |  |  |  |  |  |
| 'B' (Buccal), |  |  |  |  |  |  |  |  |  |
| 'D' (Distal), |  |  |  |  |  |  |  |  |  |
| 'F' (Facial), |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV209 Nursing Home Residential Status Code |  |  |  |  |  |  |  |  |  |
| 1' (transferred to intermediate care facility - mentally retarded), |  |  |  |  |  |  |  |  |  |
| '2' (newly admitted), |  |  |  |  |  |  |  |  |  |
| '3' (newly eligible), |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |
| Validation |  | 47 | ID | 1 | RE | FALSE | N | File: 356.22 (278) Subfile: 256.2216PA (SERVICE LINE) Field: 2.05 (EPSDT?) | Loop 2000F: SV111 EPSDT Indicator |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV111 EPSDT Indicator |  |  |  |  |  |  |  |  |  |
| 'N' (No), |  |  |  |  |  |  |  |  |  |
| 'Y' (Yes) |  |  |  |  |  |  |  |  |  |
| Comment |  | 48 | ST | 1 | RE | FALSE | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 2.09 (NURSING HOME LEVEL OF CARE) | Loop 2000F: SV120 Nursing Home Level of Care or SV210 Nursing Home Level of Care |
| Implementation Note: |  |  |  |  |  |  |  |  |  |
| SV120 Nursing Home Level of Care |  |  |  |  |  |  |  |  |  |
| '1' (skilled nursing facility), |  |  |  |  |  |  |  |  |  |
| '2' (intermediate care facility), |  |  |  |  |  |  |  |  |  |
| '8' (hospice), |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |
| SV210 Nursing Home Level of Care |  |  |  |  |  |  |  |  |  |
| '1' (skilled nursing facility), |  |  |  |  |  |  |  |  |  |
| '2' (intermediate care facility), |  |  |  |  |  |  |  |  |  |
| 3' (intermediate care facility - mentally retarded), |  |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – ZHS segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Set ID | 1 | SI | 9 | RE | FALSE | HSD 2000F |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with 'HSD 2000F'. This allows FSC to identify this data is for X12 segment HSD in loop 2000F |  |  |  |  |  |  |  |  |
| Field length has been expanded to 9 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Quantity Qualifier | 2 | CWE | 99 | RE | FALSE |  |  |  |
| identifier | 2.1 | ST | 2 | R |  | DY | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.01 (HCSD QUANTITY QUALIFIER) | Loop 2000F: HSD01 Quantity Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Quantity Qualifier |  |  |  |  |  |  |  |  |
| 'DY' (days), |  |  |  |  |  |  |  |  |
| 'FL' (units), |  |  |  |  |  |  |  |  |
| 'HS' (hours), |  |  |  |  |  |  |  |  |
| 'MN' (month), |  |  |  |  |  |  |  |  |
| 'VS' (visits) |  |  |  |  |  |  |  |  |
| Benefit Quantity | 3 | NM | 15 | RE | FALSE | 123 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.02 (HCSD SERVICE UNIT COUNT) | Loop 2000F: HSD02 Service Unit Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Unit Count |  |  |  |  |  |  |  |  |
| Unit or Basis for Measurement Code | 4 | CWE | 20 | RE | FALSE |  |  |  |
| Identifier | 4.1 | ST | 2 | R |  | DA | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: HCSD UNITS OF MEANSUREMENT) | Loop 2000F: HSD03 Unit or Basis for Measurement Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Unit or Basis for Measurement Code |  |  |  |  |  |  |  |  |
| 'DA' (days), |  |  |  |  |  |  |  |  |
| 'MO' (months), |  |  |  |  |  |  |  |  |
| 'WK' (week) |  |  |  |  |  |  |  |  |
| Sample Selection Modulus | 5 | ST | 6 | RE | FALSE | 1.5 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.04 (HCSD SAMPLE SELECTION MODULUS) | Loop 2000F: HSD04 Sample Selection Modulus |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Sample Selection Modulus. |  |  |  |  |  |  |  |  |
| To specify the sampling frequency in terms of a modulus of the Unit of Measure, e.g., every fifth bag, every 1.5 minutes |  |  |  |  |  |  |  |  |
| Time Period Qualifier | 6 | CWE | 20 | RE | FALSE |  |  |  |
| Identifier | 6.1 | ST | 2 | R |  | 21 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.05 (HCSD TIME PERIOD QUALIFIER) | Loop 2000F: HSD05 Time Period Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Time Period Qualifier |  |  |  |  |  |  |  |  |
| '6' (hour), |  |  |  |  |  |  |  |  |
| '7' (day), |  |  |  |  |  |  |  |  |
| '21' (years), |  |  |  |  |  |  |  |  |
| '26' (episode), |  |  |  |  |  |  |  |  |
| '27' (visit), |  |  |  |  |  |  |  |  |
| '34' (month), |  |  |  |  |  |  |  |  |
| '35' (week) |  |  |  |  |  |  |  |  |
| Number of Periods | 7 | NM | 3 | RE | FALSE | 5 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.06 (HCSD PERIOD COUNT) | Loop 2000F: HSD06 Period Count |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Period Count |  |  |  |  |  |  |  |  |
| Used if needed to provide further information about the number and frequency of benefits. |  |  |  |  |  |  |  |  |
| Ship/Delivery or Calendar Pattern Code | 8 | CWE | 20 | RE | FALSE |  |  |  |
| Identifier | 8.1 | ST | 2 | R |  | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.07 (HCSD DELIVERY FREQUENCY) | Loop 2000F: HSD07 Delivery Frequency Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Delivery Frequency Code |  |  |  |  |  |  |  |  |
| '1' (1st week of the month), |  |  |  |  |  |  |  |  |
| '2' (2nd week of the month), |  |  |  |  |  |  |  |  |
| 'A' (Monday through Friday), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Ship/Delivery Pattern Time Code | 9 | CWE | 20 | RE | FALSE |  |  |  |
| Identifier | 9.1 | ST | 1 | R |  | A | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 5.08 (HCSD DELIVERY TIME PATTERN) | Loop 2000F: HSD08 Delivery Pattern Time Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Delivery Pattern Time Code |  |  |  |  |  |  |  |  |
| '1' (1st week of the month), |  |  |  |  |  |  |  |  |
| '2' (2nd week of the month), |  |  |  |  |  |  |  |  |
| 'A' (Monday through Friday), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – PSL segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Product/Service Line Item Number | 1 | EI | 424 | R | FALSE |  |  |  |
| Entity Identifier | 1.1 | ST | 10 | R |  | PWK 2000F | Fixed value 'PWK 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is set to 'PWK 2000F'. This provides FSC a way to determine that this data is for X12 segment PWK in loop 2000F |  |  |  |  |  |  |  |  |
| Product/Service Line Item Sequence Number | 3 | SI | 4 | R | FALSE | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This X12 message is not using this field, however it is a required field for this HL7 segment, so this field will be populated with a sequential numeric value, starting at 1 |  |  |  |  |  |  |  |  |
| Product/Service Line Item Status | 6 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 6.1 | ST | 20 | R |  | P |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however this field is required for this HL7 segment, so this field is populated with 'P' |  |  |  |  |  |  |  |  |
| Product/Service Code | 7 | CWE | 697 | R | FALSE |  |  |  |
| Identifier | 7.1 | ST | 1 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, however, it is an HL7 required field, so it will be set to ' 1 ' |  |  |  |  |  |  |  |  |
| Health Document Reference Identifier | 19 | EI | 424 | RE | FALSE |  |  |  |
| Entity Identifier | 19.1 | ST | 2 | RE |  | 3 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 6.01 (REPORT TYPE CODE) | Loop 2000F: PWK01 Attachment Report Type Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Attachment Report Type Code |  |  |  |  |  |  |  |  |
| 03' (report justifying treatment beyond utilization guidelines), |  |  |  |  |  |  |  |  |
| '04' (drugs administered), |  |  |  |  |  |  |  |  |
| '05' (treatment diagnosis), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Namespace ID | 19.2 | IS | 80 | RE |  | Att123 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 6.03 (ATTACHMENT CONTROL NUMBER) | Loop 2000F: PWK06 Attachment Control Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Attachment Control Number |  |  |  |  |  |  |  |  |
| Universal ID | 19.3 | ST | 80 | RE |  | Description of Attachment | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 6.04 (ATTACHMENT DESCRIPTION) | Loop 2000E: PWK07 Attachment Description |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Attachment Description |  |  |  |  |  |  |  |  |
| Processing Consideration Code | 20 | IS | 10 | RE | FALSE | 5 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Field: 6.02 (REPORT TRANSMISSION CODE) | Loop 2000F: PWK02 Report Transmission Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Report Transmission Code |  |  |  |  |  |  |  |  |
| 03' (report justifying treatment beyond utilization guidelines), |  |  |  |  |  |  |  |  |
| '04' (drugs administered), |  |  |  |  |  |  |  |  |
| '05' (treatment diagnosis), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Restricted Disclosure Indicator | 21 | ID | 2 | R | FALSE | NA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This X12 message is not using this field, however it is required for this HL7 segment, so this field will be populated with 'NA' |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – NTE segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Comment | 3 | FT | 264 | RE | FALSE | Up to 264 characters of free form text | File: 356.22 (278) Subfile 356.2216PA (SERVICE LINE) Field:356.22167 (MESSAGE TEXT) | Loop 2000F: MSG01 Free Form Message Text |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with Free Form Message Text |  |  |  |  |  |  |  |  |
| Comment Type | 4 | CWE | 250 | RE | FALSE |  |  |  |
| Identifier | 4.1 | ST | 10 | RE |  | MSG 2000F | Fixed value 'MSG 2000F' |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is set to 'MSG 2000F'. FSC can use this information to know that this data is for X12 segment MSG in loop 2000F |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – PRD segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role | 1 | CWE | 250 | R | TRUE |  |  |  |
| Identifier | 1.1 | ST | 3 | RE |  | 1T | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .01 (PROVIDER TYPE) | Loop 2010FA: NM101 Entity Identifier Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM101 Entity Identifier Code |  |  |  |  |  |  |  |  |
| '1T' (physician, clinic or group practice), |  |  |  |  |  |  |  |  |
| '72' (operating physician), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Text | 1.2 | ST | 1 | RE |  | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .02 (PERSON / NON-PERSON) | Loop 2010FA: NM102 Entity Type Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Entity Type Qualifier |  |  |  |  |  |  |  |  |
| '1' (person), |  |  |  |  |  |  |  |  |
| '2' (non-person entity) |  |  |  |  |  |  |  |  |
| Alternate Identifier | 1.4 | ST | 10 | R |  | NM1 2010FA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with one of the following, which allows FSC to determine which X12 segment and loop the data belongs to. 'NM1 2010FA' |  |  |  |  |  |  |  |  |
| Provider Name | 2 | XPN | 264 | RE | TRUE |  |  |  |
| Family Name | 2.1 | FN | 194 | RE |  |  |  |  |
| � Surname | 2.1.1 | ST | 60 | R |  | Organization Name | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) or File: 4 (INSTITUTION) Field: .01 (NAME) | Loop 2010FA: NM103 Service Provider Last Name or Organization Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with the Service Provider Last Name or Organization Name |  |  |  |  |  |  |  |  |
| Given Name | 2.2 | ST | 35 | RE |  | John | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010FA: NM104 Service Provider First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider First Name |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 2.3 | ST | 25 | RE |  | Middle | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field:: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010FA: NM105 Service Provider Middle Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider Middle Name |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) | 2.4 | ST | 10 | RE |  | JR | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) | Loop 2010FA: NM107 Service Provider Name Suffix |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider Name Suffix |  |  |  |  |  |  |  |  |
| Provider Address | 3 | XAD | 292 | RE | TRUE |  |  |  |
| Street Address | 3.1 | SAD | 184 | RE |  |  |  |  |
| � Street or Mailing Address | 3.1.1 | ST | 55 | RE |  | Address Line 1 | FIle: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .111 (STREET ADDRESS 1) or File: 4 (INSTITUTION) Field: 4.01 (ST. ADDR 1 (MAILING)) | Loop 2010FA: N301 Service Provider Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider Address Line 1 |  |  |  |  |  |  |  |  |
| Other Designation | 3.2 | ST | 55 | RE |  | Suite 2 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .112 (STREET ADDRESS 2) or File: 4 (INSTITUTION) Field: 4.02 (ST. ADDR. 2 (MAILING)) | Loop 2010FA: N302 Service Provider Address Line 2 |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Providers Address Line 2, if available |  |  |  |  |  |  |  |  |
| City | 3.3 | ST | 30 | RE |  | City | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .114 (CITY) or File: 4 (INSTITUTION) Field: 4.03 (CITY (MAILING)) | Loop 2010FA: N401 Service Provider City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider City Name |  |  |  |  |  |  |  |  |
| State or Province | 3.4 | ST | 2 | RE |  | VA | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .115 (STATE), File: 4 (INSTITUTION) Field: 4.04 (STATE (MAILING)) | Loop 2010FA: N402 Service Provider State or Province Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider State or Province Code |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 3.5 | ST | 15 | RE |  | 20133 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .116 (ZIP CODE) or File: 4 (INSTITUTION) Field: 4.05 (ZIP (MAILING)) | Loop 2010FA: N403 Service Provider Postal Zone or Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider Postal Zone or Zip Code |  |  |  |  |  |  |  |  |
| Country | 3.6 | ID | 3 | RE |  | ALB | Not available in the Vista database for file 200 (NEW PERSON) or File 4 (INSTITUTION) | Loop 2010FA: N404 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider's Country Code of location, if outside of the USA |  |  |  |  |  |  |  |  |
| Other Geographic Designation | 3.8 | ST | 3 | RE |  | JAL | Not available in the Vista database for file 200 (NEW PERSON) or File 4 (INSTITUTION) | Loop 2010FA: N407 Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is populated with the Service Provider's Country Subdivision Code, if outside the USA and Subdivision Codes exist |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 | PLN | 100 | RE | FALSE |  |  |  |
| ID Number | 7.1 | ST | 80 | R |  | 123456789 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field .03 (PROVIDER), File: 200 (NEW PERSON) Field 41.99 (NPI) or File: 4 (INSTITUTION) Field: 41.99 (NPI) or Field: 4.043 (TAXONOMY CODE) | Loop 2010EA: NM109 Service Provider ID |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For NM109, this field is the Service Provider ID. This will be the NPI number for the service provider |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – MSA segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgment Code | 1 | ID | 2 | R | FALSE | AR |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| AE' (MSA standard value for application error) or 'AR" (MSA standard value for application reject) |  |  |  |  |  |  |  |  |
| Message Control ID | 2 | ST | 199 | R | FALSE | 231233 | File: 356.22 (278) Field: .12 (MESSAGE CONTROL ID) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Message Control ID (MSH.10) or original request 278 message |  |  |  |  |  |  |  |  |
| Text Message | 3 | ST | 10 | RE | FALSE | AAA 2010FA | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .02 (AAA LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Set to 'AAA 2010FA'. Indicates this data is from X12 AAA segment loop 2010FA |  |  |  |  |  |  |  |  |
| Field length has been expanded to 10 characters to allow for X12 value |  |  |  |  |  |  |  |  |
| Delayed Acknowledgment Type | 5 | ST | 1 | RE | FALSE | C | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .05 (FOLLOW-UP ACTION CODE) | Loop 2010FA: AAA04 Follow-up Action Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| AAA04 Follow-up Action Code |  |  |  |  |  |  |  |  |
| 'C' (please correct and resubmit), |  |  |  |  |  |  |  |  |
| 'N' (resubmission not allowed) |  |  |  |  |  |  |  |  |
| Error Condition | 6 | CWE | 483 | RE | FALSE |  |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| The CE data type for this field is an oversight in the publication of v2.6. The CE data type is no longer supported as of v2.6. |  |  |  |  |  |  |  |  |
| Identifier | 6.1 | ST | 2 | RE |  | 15 | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: .04 (REJECT REASON CODE) | Loop 2010FA: AAA03 Reject Reason Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| AAA03 Reject Reason Code |  |  |  |  |  |  |  |  |
| '15' (required application data missing), |  |  |  |  |  |  |  |  |
| '33' (input errors), |  |  |  |  |  |  |  |  |
| 52' (service dates not within provider plan enrollment), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| Text | 6.2 | ST | 80 | RE |  | Error text | File: 356.22 (278) Subfile: 356.22101A (AAA SEGMENTS) Field: 1 (FSC ERROR MESSAGE) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| For FSC: This is text that can be used to identify an error that was not reported by the HCCH as a standard X12 AAA error condition. |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – PRD segment (Optional) (Repeating)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Provider Role | 1 | CWE | 705 | R | TRUE |  |  |  |
| Identifier | 1 | ST | 3 | R |  | AS | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .01 (PROVIDER TYPE) | Loop 2010FA: PRV01 Provider Code or Loop 2010FB: NM101 Entity Identifier Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| PRV01 Provider Code |  |  |  |  |  |  |  |  |
| (see list provided in requirements document) |  |  |  |  |  |  |  |  |
| examples: |  |  |  |  |  |  |  |  |
| 'AS' (assistant surgeon), |  |  |  |  |  |  |  |  |
| 'OP' (operating), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| or 'L5' (Contact) |  |  |  |  |  |  |  |  |
| Text | 2 | ST | 1 | RE |  | 1 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .02 (PERSON / NON-PERSON) | Loop 2010FB: NM102 Entity Type Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM102 Entity Type Qualifier |  |  |  |  |  |  |  |  |
| '1' (person), |  |  |  |  |  |  |  |  |
| '2' (non-person entity) |  |  |  |  |  |  |  |  |
| Alternate Identifier | 4 | ST | 10 | RE |  | PRV 2010FA | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 4.01 (NM1 LOOP IDENTIFIER) |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be set to 'PRV 2010FA' This identifies this data to be from X12 segment PRV loop 2010F |  |  |  |  |  |  |  |  |
| Or |  |  |  |  |  |  |  |  |
| This field will be set to 'NM1 2010FB' |  |  |  |  |  |  |  |  |
| Provider Name | 2 | XPN | 264 | RE | TRUE |  |  |  |
| Family Name | 1 | FN | 194 | RE |  |  |  |  |
| Surname | 1 | ST | 50 | R |  | SMITH | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .03 (PROVIDER), File: 200 (NEW PERSON) Field: .01 (NAME) or File: 4 (INSTITUTION) Field: .01 (NAME) | Loop 2010FB: NM103 Response Contact Last or Organization Name |
| Given Name | 2 | ST | 35 | RE |  | JOHN | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 4.05 (CONTACT FIRST NAME) | Loop 2010FB: NM104 Response Contact First Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM104 Response Contact First Name |  |  |  |  |  |  |  |  |
| Second and Further Given Names or Initials Thereof | 3 | ST | 25 | RE |  | MIDDLE | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 4.06 (CONTACT MIDDLE NAME) | Loop 2010FB: NM105 Response Contact Middle Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM105 Response Contact Middle Name |  |  |  |  |  |  |  |  |
| Suffix (e.g., JR or III) | 4 | ST | 10 | RE |  | JR | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 4.07 (CONTACT NAME SUFFIX) | Loop 2010FB: NM107 Response Contact Name Suffix |
| Provider Address | 3 | XAD | 292 | RE | FALSE |  |  |  |
| Street Address | 1 | SAD | 184 | RE |  |  |  |  |
| � Street or Mailing Address | 1 | ST | 55 | RE |  | 123 Street Address | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 5 (CONTACT ADDRESS LINE-1) | Loop 2010FB: N301 Response Contact Address Line |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N301 Response Contact Address Line |  |  |  |  |  |  |  |  |
| Other Designation | 2 | ST | 55 | RE |  | Suite 200 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 5.01 (CONTACT ADDRESS LINE-2) | Loop 2010FB: N302 Response Contact Address Line 2 |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N302 Response Contact Address Line 2 |  |  |  |  |  |  |  |  |
| City | 3 | ST | 30 | RE |  | City | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 5.02 (CONTACT CITY NAME) | Loop 2010FB: N401 Additional Service City Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N401 Additional Service City Name |  |  |  |  |  |  |  |  |
| State or Province | 4 | ST | 2 | RE |  | VA | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 5.03 (CONTACT STATE OR PROVINCE) | Loop 2010FB: N402 Additional Service Information State or Province Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N402 Additional Service Information State or Province Code |  |  |  |  |  |  |  |  |
| Zip or Postal Code | 5 | ST | 15 | RE |  | 21011 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 5.04 (CONTACT ZIP CODE) | Loop 2010FB: N403 Additional Service Information Postal Zone or Zip Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N403 Additional Service Information Postal Zone or Zip Code |  |  |  |  |  |  |  |  |
| Country | 6 | ID | 3 | RE |  | ALB | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 5.05 (CONTACT COUNTRY CODE) | Loop 2010FB: N404 Country Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N404 Country Code |  |  |  |  |  |  |  |  |
| Other Geographic Designation | 8 | ST | 3 | RE |  | 233 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 5.06 (CONTACT COUNTRY SUBDIVISION) | Loop 2010FB: N407 Country Subdivision Code |
| Implementation Note: |  |  |  |  |  |  |  |  |
| N407 Country Subdivision Code |  |  |  |  |  |  |  |  |
| Provider Identifiers | 7 | PLN | 100 | RE | TRUE |  |  |  |
| ID Number | 1 | ST | 80 | R |  | 123456789 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 4.02 (ENTITY IDENTIFIER CODE) | Loop 2010FA: PRV03 Provider Taxonomy Code or NM109 Response Contact Identifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM109 Response Contact Identifier |  |  |  |  |  |  |  |  |
| or |  |  |  |  |  |  |  |  |
| PRV03 Provider Taxonomy Code |  |  |  |  |  |  |  |  |
| Type of ID Number | 2 | IS | 3 | R |  | 24 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 4.03 (ENTITY TYPE QUALIFIER) | Loop 2010FA: PRV02 Reference Identification Qualifier or NM108 Identification Code Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| NM108 Identification Code Qualifier |  |  |  |  |  |  |  |  |
| '24' (employer's identification number), |  |  |  |  |  |  |  |  |
| '34' (ssn), |  |  |  |  |  |  |  |  |
| etc._ |  |  |  |  |  |  |  |  |
| or Reference Identification Qualifier of 'PXC' (Health Care Provider Taxonomy Code) |  |  |  |  |  |  |  |  |

### RPA_I08 Response G5O Segment Group (Optional) (Repeating) – CTD segment (Optional)

| Name | Seq | DT | Len | Opt | Rep | Example Value | VistA Input/Storage | X12 Segment / Field Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Role | 1 | CWE | 705 | R | TRUE |  |  |  |
| Identifier | 1.1 | ST | 10 | R |  | PER 2010FB |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field will be populated with 'PER 2010FB' |  |  |  |  |  |  |  |  |
| This identifies this data is from X12 segment PER loop 2010FB |  |  |  |  |  |  |  |  |
| Contact Name | 2 | XPN | 294 | RE | TRUE |  |  |  |
| Family Name | 2.1 | FN | 194 | RE |  |  |  |  |
| � Surname | 2.1.1 | ST | 60 | R |  | Joe Contact | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .06 (CONTACT NAME) | Loop 2010FB: PER02 Response Contact Name |
| Implementation Note: |  |  |  |  |  |  |  |  |
| PER02 Response Contact Name |  |  |  |  |  |  |  |  |
| Contact Communication Information | 5 | XTN | 2527 | RE | TRUE |  |  |  |
| Telecommunication Equipment Type | 5.2 | ID | 8 | RE |  | EM | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: .07 (COMMUNICATION NO. QUALIFIER-1) or Field: .08 (COMMUNICATION NO. QUALIFIER-2) or Field: .09 (COMMUNICATION NO. QUALIFIER-3) | Loop 2010FB: PER03 Communication Number Qualifier or PER05 Communication Number Qualifier or PER07 Communication Number Qualifier |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Communication Number Qualifier |  |  |  |  |  |  |  |  |
| 'EM' (Electronic Mail), |  |  |  |  |  |  |  |  |
| 'EX' (telephone extension), |  |  |  |  |  |  |  |  |
| 'FX' (facsimile), |  |  |  |  |  |  |  |  |
| 'TE' (telephone), |  |  |  |  |  |  |  |  |
| 'UR' (uniform resource locator (URL) |  |  |  |  |  |  |  |  |
| Any Text | 5.8 | ST | 256 | RE |  | (888) 555-1212 | File: 356.22 (278) Subfile: 356.2216PA (SERVICE LINE) Sub-Subfile: 356.22168PA (SERVICE PROVIDER) Field: 1 (CONTACT COMMUNICATION NUMBER-1) or Field: 2 (CONTACT COMMUNICATION NUMBER-2) or Field: 3 (CONTACT COMMUNICATION NUMBER-3) | Loop 2010FB: PER04 Response Contact Communication Number or PER06 Response Contact Communication Number or PER08 Response Contact Communication Number |
| Implementation Note: |  |  |  |  |  |  |  |  |
| Response Contact Communication Number |  |  |  |  |  |  |  |  |
| Contact Identifiers | 7 | PLN | 100 | RE | TRUE |  |  |  |
| ID Number | 7.1 | ST | 20 | R |  | 1 |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, but it is an HL7 required field, so it will be populated with a '1' |  |  |  |  |  |  |  |  |
| Type of ID Number | 7.2 | IS | 8 | R |  | VA |  |  |
| Implementation Note: |  |  |  |  |  |  |  |  |
| This field is not used for this X12 message, so it will be populated with a fixed 'VA' |  |  |  |  |  |  |  |  |