# Technology
Resource Guide

# Provided by:

# Patient Safety Center of Inquiry
8900 Grand Oak Circle (118M)
Tampa, FL 33637
813-558-3900
http://www.visn8.va.gov/patientsafetycenter/

# Last Revised: 7/29/2026

# Technology Resource Guide

Note: Inclusion in this guide does not constitute endorsement by the Tampa Patient Safety Center of Inquiry in Tampa or of the Veterans Health Administration. The products in this guide are merely examples of various types of equipment available in the marketplace at a particular period in time. We have no more information about particular equipment, purchasing or contracting than is presented here. Please do not request purchasing information from this office. The listings show the URL for a manufacturer’s web site. We do not provide links to those sites in this document.

TOP

## Choose Category:

## Safe Patient Handling Equipment

## Beds by Care Setting

## Equipment by Task

## Safe Patient Handling Equipment (click on link to go to listing):

| STANDARD EQUIPMENT |  |
| --- | --- |
| Bathing/Showering/Toileting | Bed Systems (Frames/Surfaces/Assist Devices/Supine Transport—Manual and Powered) |
| Car Lifts/Vehicle Extraction | Ceiling Lifts |
| Chairs: Dependency/Geri/Specialty/Wheelchairs (Seated Transport) | Floor-Based Lifts |
| Lateral Transfer Aids and Repositioning Devices (Air Assisted, Friction Reducing, Manual and Powered, Transfer Boards) | Lifts (other) |
| Other Mobility Aids | Sit-to-Stand Lifts |
| Sling Guide | Standing Assist Aids |
| BARIATRIC EQUIPMENT |  |
| Bariatric Ambulatory/Mobility Aids | Bariatric Bathing/Showering/Toileting |
| Bariatric Beds/Support Surfaces/Transportation | Bariatric Ceiling/Gantry Lifts |
| Bariatric Chairs (Transfer/Dependency/Wheeled and Cushions) | Bariatric Lateral Transfer Systems |
| Bariatric Powered Lifts | Bariatric Repositioning Systems |

##  TOP

## Bathing/Showering/Toileting	         					         TOP

| Bathing/Showering/Toileting PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| Andermac/Anderlift Lift and Transport System anderlift.com | When lifted, transported, weighed or bathed, the patient is resting on a "hammock" created by a custom sheet and nestled in the frame. | Four pieces of equipment in one - a bather, bed scale, transportation system and power patient lift. Up to 800 lb. capacity |
| Arjo/Huntleigh / Alenti arjohuntleigh.com | Bathing transport and transfer for semi-dependent residents. | Part of an integrated bathing system. |
| Arjo/Huntleigh / Bolero arjohuntleigh.com | Battery-powered lift bath trolley. Handset controlled height-adjustment. | Straight steering device for corridors. |
| Arjo/Huntleigh / Calypso arjohuntleigh.com | Bathing and showering lift for semi-dependent residents. | Can also be used for toileting. |
| Arjo/Huntleigh / Calypso PUR arjohuntleigh.com | Bathing and showering lift for semi-dependent residents. | Seat is easy to clean and disinfect. |
| Arjo/Huntleigh / Century Bath arjohuntleigh.com | Key-hole shaped sit-bath with therapy options including Hydromassage and Hydrosound. | Auto-fill option. |
| Arjo/Huntleigh / Freedom Bath arjohuntleigh.com | Side-entry roll door for access. Appropriate in both independent and assisted environments. | Integrated contoured seat. |
| Arjo/Huntleigh / Miranti arjohuntleigh.com | Bath trolley lift with power drive, powered height adjustment and powered backrest. | Handset operation of all functions. |
| Arjo/Huntleigh / Parker Bath arjohuntleigh.com | Assisted bathing solution with side entry, variable speed air-spa, & soap dispenser. | Wall-mounted cleaner/disinfection system. |
| Arjo/Huntleigh / Primo arjohuntleigh.com | Straight-edged bath solution available in shorter version for more mobilized residents and longer for dependent residents. | Height-adjustable. |
| Arjo/Huntleigh / Rhapsody arjohuntleigh.com | Keyhole-shaped bath solution available in shorter version for more mobilized residents and longer for dependent residents. | Height-adjustable. |
| Horcher Lifting Systems/Forelli Shower Trolley horcher.com | Shower trolley with side panels that raise and lower electronically. | Foot controls on both sides of the chassis. |
| Horcher Lifting Systems/Lena Bathtub horcher.com | Patient-care lifting bathtub | Accepts lifts along the entire tub length; temperature presets. |
| Horcher Lifting Systems/Mona Bath Stretcher Lift horcher.com | Stretcher allows single caregiver to move patient from bed to bath. | 350 lb. capacity; Digital scale optional. |
| Horcher Lifting Systems/Nina Bath Seat Lift horcher.com | Bath lift with swivel seat for transfers between bed, bath, shower and toilet or as mode of transport. | Height adjusted via hand controls. |
| Horcher Lifting Systems/Swing horcher.com | Sitting/lying bathtub | Side entry; electric lowering and tilting. |
| Mangar USA / Archimedes mangarusa.com | Battery operated bath lift. Lifts a person weighing up to 330 lbs. | Lightweight, portable, comes apart for travel. |
| Mangar USA / Bathing Cushion mangarusa.com | Inflatable bath cushion for people with reasonable upper body stability but who need assistance getting in and out of a bathtub. | Very light, cleans easily with commercial bathroom cleaner. |
| Polaris Medical / Novum 1000 Patient lift patientbathing.com | Bathing transport and transfer for semi-dependent residents. 440 lbs | Integrates with existing bathing systems. High weight capacity. |
| Polaris Medical / Novum 2000 Lift bath trolley patientbathing.com | Battery-powered lift bath trolley. Handset controlled height-adjustment. 440lbs | Integrates with existing bathing systems. High weight capacity 440 lbs. |
| Polaris Medical / Hektor Patient lift patientbathing.com | Bathing transport and transfer for semi-dependent residents. 330 lbs | Can integrate into most bath systems. Provides comfort with modern looks. |
| Polaris Medical / Ikarus Lift bath trolley patientbathing.com | Battery-powered lift bath trolley. Handset controlled height-adjustment. | Comfortable PU foam top comfortable for residents |
| Polaris Medical / GK Magic Recumbent bathing system patientbathing.com | Height adjustable bathing system with varying options. Key hole shaped. | Specialized pipeless jets, ultrasonic cleaning and air spa. Mandatory disinfection systems |
| Polaris Medical / GK Basic+ Recumbent bathing system patientbathing.com | Height adjustable bathing system with varying options. Simplified easy to use layout. | Specialized pipeless jets, cleaning and air spa. |
| Polaris Medical / GK Magic Recumbent bathing system patientbathing.com | Specialized stainless steel full bath designed for burn units. | Specialized for burn units |
| Polaris Medical / GK Relax Raise and Tilt bathing system patientbathing.com | Tilt and Recline bathing system, height adjustable. | Integrates with existing floor lifts and ceiling lifts |
| Polaris Medical / GK Hera Sit in bathing system patientbathing.com | Side door bathing solution which is lift accessible. | Integrated liquids dispensing, mandatory disinfection. Pipeless jets, ultrasonic cleaning |
| Polaris Medical / GK Dayton Sit in bathing system patientbathing.com | Key whole shape is lift accessible with deep immersion. | Specialized pipeless jets, cleaning and air spa.. |
| Polaris Medical / GK Primus Showeer Trolley patientbathing.com | Shower trolley used for shallow baths and also for debridement in ER’s. | High weight capacity. Hydraulic or battery operated. Adjustable back rests. Straight steering and central locking brakes |
| T.H.E. Medical/Aquarius Recumbent Tub themedical.com | Extra wide tub with whirlpool and air spa options. 100% effective infection control system. | Touchpad control panel. |
| T.H.E. Medical/Atlantic Pre-Filling Side Entry Reclining Bathing System themedical.com | Low water volume free-standing tub. 12-jet air spa system. | One touch recline feature. |
| T.H.E. Medical/TR 900 Bathing System themedical.com | Height-adjustable bathtub with keyhole shaped tub for more width at the head end. | Can be equipped with a built-in cleaning and a whirlpool system. |
| T.H.E. Medical/TR 1700 Bathing System themedical.com | Height-adjustable bathtub with thermostatic mixer for bathtub filling and shower. | For bathrooms with limited space. |
| T.H.E. Medical/TR 2000 Hydraulic Shower Trolley themedical.com | Side-mounted column to allow stretcher to overlap bed surface further. | Side rails can be folded inward when passing through narrow doors. 330 lb. capacity. |
| T.H.E. Medical/TR 3000 Electric Shower Trolley themedical.com | Battery operated shower trolley with side-mounted column to allow stretcher to overlap bed surface further. | Side rails can be folded inward when passing through narrow doors. 330 lb. capacity. |
| T.H.E. Medical/Winnipeg Side Entry Tub themedical.com | Free-standing wall tub with low threshold door for entry with or without a transfer device. | Contoured interior for comfort. |

## Bed Systems
(Frames/Surfaces/Assist Devices/Supine Transport—Manual and Powered)

| Bed Systems PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| BEDS |  |  |
| ArjoHuntleigh/Enterprise Range arjohuntleigh.com | Hospital bed range. Profiling system for pressure and shear reduction. | Low height of 12”. 550 lb. capacity. Many options available. |
| ArjoHuntleigh/Contoura® Range arjohuntleigh.com | Four-section electric profiling hospital bed. | Electronically operated backrest, knee-break and height adjustment. Many models available. |
| ArjoHuntleigh/Minuet® 2 Range arjohuntleigh.com | Long term care bed with fully electric operation of the bed. | Aids individual independence and reduces moving and handling risks to the caregiver. 450 lb. capacity. |
| Carroll Hospital Group/Spirit Select™ carrollhospitalgroup.com | Acute care low bed. Includes bed exit alarm and weight monitoring. | 10” low height, 35 ¼” raised height. 500 lb. capacity. |
| Carroll Hospital Group/Spirit Select™ carrollhospitalgroup.com | Acute care low bed. Includes bed exit alarm and weight monitoring. | 10” low height, 35 ¼” raised height. 500 lb. capacity. |
| Guldmann/BedLift guldmann.net | Can be used with a regular bed when there is need for electric height adjustment. | Can use in non-hospital bed. |
| Guldmann/Insertion Bed guldmann.net | Can be used in an ordinary bed with motors for operation of back, thigh, and leg sections as well as for height adjustment. | Can use in non-hospital bed. |
| Guldmann/Care Bed guldmann.net | Care bed with 2-4 motors with detachable wooden head/foot boards. | Bed is prepared for steel bed guards. |
| Guldmann/Care Bed guldmann.net | Care bed with 2-4 motors with fixed wooden head/food boards. | Wooden bed guards. |
| Hill-Rom/Advantra™ 2 hill-rom.com | Med/surg bed with alerts to help prevent patient falls. | Bedside controls, less patient repositioning. |
| Hill-Rom/CareAssist® ES hill-rom.com | Med/surg bed with Intellidrive® power transport feature. | One button Dining Chair® feature. |
| Hill-Rom/TotalCare® Bariatric Plus hill-rom.com | Converts from bed to full chair position to avoid numerous patient transfer and repositioning tasks. | Optional Low Air Loss Pressure Redistribution Surface. |
| Hill-Rom/TotalCare® Connect hill-rom.com | ICU bed with continuous rotation therapy and head of bed alarm. | Microclimate surface for wound healing and to promote healthy skin. |
| Hill-Rom/VersaCare® hill-rom.com | Low profile med/surg bed. | Boost feature to assist repositioning patient in bed. Many mattress options available. |
| Hill-Rom/VersaCare® TC hill-rom.com | Bed system for step down and transitional care. | Intellidrive® power transport. |
| Stryker/Electric Bed stryker.com | Med/surg bed with low bed height and full coverage side rails. | Retractable fifth-wheel mobility. Manual model also available. |
| Stryker/GoBed II stryker.com | Med/surg bed with low bed height and full coverage side rails. | Retractable fifth-wheel mobility. |
| Stryker/S3 stryker.com | Med/surg bed that monitors the patient’s position in three zones and has 30 degree head of bed angle. | One-hand operation of side rails, center-mounted brake, one-touch in-bed scale. |
| Transfer Master/Hi-Lo Bed transfermaster.com | Bed with transfer height as low as 14”. Available in twin, full and queen size. | Has the capacity to sit someone to almost 90 degrees. |
| MATTRESS REPLACEMENT/OVERLAYS |  |  |
| ArjoHuntleigh/Acer arjohuntleigh.com | Portable mattress replacement system. | For patients at risk from complications of immobility associated with acute respiratory failure. |
| ArjoHuntleigh/Alpha Response arjohuntleigh.com | Semi-automatic pressure redistribution system. | Can be used from general ward and acute care to private or nursing home care. |
| ArjoHuntleigh/Alpha Trancell Deluxe arjohuntleigh.com | Alternating pressure redistribution. | Prevention of pressure ulcers and management of superficial tissue damage. |
| ArjoHuntleigh/AlphaX cell® arjohuntleigh.com | Active pressure redistributing mattress overlay. | For patients at medium risk for pressure sore development, or existing pressure sores up to and including stage 2. |
| ArjoHuntleigh/AlphaX cell® Double Bed System arjohuntleigh.com | Active pressure redistributing mattress overlay. | For couples who want to share a bed when one partner has a requirement for a pressure redistributing support surface. |
| ArjoHuntleigh/Auto Logic 110 arjohuntleigh.com | Fully automatic pressure redistributing mattress. | Alternating and constant low pressure modes. |
| ArjoHuntleigh/Auto Logic 200 arjohuntleigh.com | Automatic self setting mattress/cushion combination. Alternating pressure redistribution. | Constant low pressure mode for patients unable to tolerate a moving surface. |
| ArjoHuntleigh/Bari Breeze arjohuntleigh.com | Low air loss, turn assist, mattress replacement system. | 1,000 lb. capacity. |
| ArjoHuntleigh/Breeze arjohuntleigh.com | Low air loss mattress replacement system. | Portable. For hospital or community use. |
| ArjoHuntleigh/ConformX® arjohuntleigh.com | Mattress replacement system with low interface pressures. | Suitable for radiographic use. |
| ArjoHuntleigh/DFS3 arjohuntleigh.com | Dynamic flotation system. | Automatically adjusts to the weight, size and position of the patient. |
| ArjoHuntleigh/Pentaflex® arjohuntleigh.com | Foam-based support system with internal air channels. | Support zones varied to anatomical distribution |
| ArjoHuntleigh/Trinova arjohuntleigh.com | Three cell cycle for optimum low pressure. Simulates normal body movement. | Integrated seating system and an assessment mode for step down. |
| Hill-Rom/Acucair® hill-rom.com | Continuous air flow therapy surface. | The surface provides continuous low pressure regardless of head elevation, position changes or punctures. |
| Hill-Rom/Clinitron® II Air Fluidized Therapy hill-rom.com | Support surface to promote wound/skin disorder healing. | Can be defluidized to facilitate patient positioning during pulmonary care and Trendelenburg and Reverse Trendelenburg. CPR procedures can be performed immediately and without a cardiac board. |
| Hill-Rom/Clinitron® Rite Hite® Air Fluidized Therapy hill-rom.com | Support surface combining air fluidized therapy and low air loss therapy. | Can be used with patients whose regimen requires head elevation. |
| Hill-Rom/Envision® E700 hill-rom.com | Rental therapy mattress replacement for VersaCare® and TotalCare® Connect Beds | Addresses pressure ulcer healing issues: pressure, shear, friction, patient immobility and microclimate management. |
| Hill-Rom/Flexi Care® Eclipse hill-rom.com | 5-zoned low air loss mattress. | 300 lb. capacity. Moisture vapor permeable sleep surface. |
| Hill-Rom/Synergy® Air Elite hill-rom.com | Low air loss therapy surface. | Individual air cushions connected to side panel with CPC connector for easy detachment and cleaning. |
| Hill-Rom/TotalCare Spo2rt® Surface hill-rom.com | Critical care support surface with Intelidrive® for transport. | Full chair and stand assist with chair egress as well as one button boost function. |
| Hill-Rom/VersaCare A.I.R.™ hill-rom.com | 3 independent zones of continuous low pressure. | Turn assist allows single caregiver to perform patient examinations, linen changes, and other routine tasks. |
| Hill-Rom/V-Cue® Dynamic Air Therapy® hill-rom.com | Low air loss surface and pressure relief mattress. | Adjustable pressure relief mattress allows the unit to be transferred from ICU to Med-Surg frame without removing the patient from the surface. |
| Span America/PressureGuard® APM2 Safety Supreme spanamerica.com | Multi-mode air therapy with raised safety perimeter. | Switch from alternating pressure to lateral rotation or powered flotation. |
| Stryker/Epic II stryker.com | Critical care bed that includes integrated scale and bed exit monitoring system. | Accepts independent support surfaces. |
| Stryker/InTouch stryker.com | Critical care bed with many customizable features including touch screen interface to patient metrics, protocol reminders, turn schedules, etc. | Accepts independent support surfaces. |
| STRETCHERS |  |  |
| Andermac/Anderlift Lift and Transport System anderlift.com | When lifted, transported, weighed or bathed, the patient is resting on a "hammock" created by a custom sheet and nestled in the frame. | Four pieces of equipment in one - a bather, bed scale, transportation system and power patient lift. Up to 800 lb. capacity. |
| Hill-Rom/Electric Stretcher hill-rom.com | Powered stretcher with optional active hand brake and IV pole transport device. | Side-rail controls allow patients to electrically adjust the stretcher for their own comfort. |
| Hill-Rom/Procedural Stretcher hill-rom.com | For use in ER, OR, PACU or outpatient settings. | Optional active hand brake and IV pole transport device. |
| Hill-Rom/Surgical Stretcher hill-rom.com | For use in hospital or ambulatory surgery settings with integrated IV pole transport device. | Articulating head section and wrist rests. |
| Hill-Rom/Transport Stretcher hill-rom.com | Stretcher with a variety of mattress options depending upon the setting. | 700 lb. capacity. |
| Hill-Rom/Trauma Stretcher hill-rom.com | Stretcher appropriate for use in trauma and emergency. | Optional active hand brake. Integrated IV pole transport device option. |
| Stryker/Prime Stretcher stryker.com | Powered transport stretcher. Choice of available surfaces depending upon area of use. | Patients can adjust their own positions without calling a caregiver for assistance. 700 lb. capacity. |
| Wy’East Medical/TotaLift-II wyeastmed.com | Chair that converts to stretcher/procedure/transport table. | 400 lb. capacity. |
| FALL MONITORS |  |  |
| Carroll Hospital Group/Spirit Select™ carrollhospitalgroup.com | Acute care low bed. Includes bed exit alarm and weight monitoring. | 10” low height, 35 ¼” raised height. 500 lb. capacity. |
| Stanley Healthcare Solutions/BedCheck stanleyhealthcare.com | Fall prevention/alarm system with control unit and mat. | Selectable alarm sounds can be used for high risk patients. |
| Stryker/Epic II stryker.com | Critical care bed that includes integrated scale and bed exit monitoring system. | Accepts independent support surfaces. |

## Car Lifts/Vehicle Extraction						 TOP

| Car Lifts/Vehicle Extraction PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| Access Unlimited / Multi-Lift accessunlimited.com | Lift works by a 12 volt motor with the power from the electrical system of your car to perform transfers (car, bath, home). Available in a wide variety of slings. | Fits every care on the market including minivan, trust, SUV or RV. |
| ArjoHuntleigh/Maxi Lite arjohuntleigh.com | Battery-powered floor-based lift for general lifts and transfers including chair to car. | 350 lb. capacity. |
| EZ Way/EZ Out Vehicle Extraction Lift ezlifts.com | Accommodates various wheel well configurations; movable mast for variety of vehicle door styles; single hand control; 500 lb. capacity. | Telescoping mast for tall trucks or short cars. |
| Braun Corporation / Van Entry Systems braunability.com | Designed for full-size and minivans for level transfers from wheelchair or scooter to seat. | Rotation of the seat up to 140 degrees. |
| Liko North America/Golvo liko.com/na | Flexible lifting strap is extended into a vehicle to facilitate transfer. | Permits transfers such as supine from rear seat; accommodates SUVs and higher-than-normal vehicles. |
| Medcare Products/Car Extractor medcarelifts.com | Floor-based lift with boom for car transfers. 30” reach and 40” lifting range. | 475 lb. capacity. |
| Technimotion / Proxi-Motion technimotion.com | Floor-based lift. No log-rolling required with Smart Insertion slings. Performs car transfers. Battery operated. Tabletop attachments allows lift to function as overbed table when not in use. | 400 lb. weight capacity. Strap adjustment system. |
| T.H.E. Medical/Vehicle Extractor themedical.com | For patient transfers to and from vehicles. | 500 lb. capacity. |

## Ceiling Lifts									 TOP

| Ceiling Lifts PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| ArjoHuntleigh/Maxi Sky 440 arjohuntleigh.com | Lifts up to 440 lbs. Allows “swing” transfer of life cassette and patient from one track to another. | Portable. |
| ArjoHuntleigh/Maxi Sky 6000 arjohuntleigh.com | Ceiling lift, lifts up to 600 lbs. Soft start and stop motor control. | Emergency stop and lower devices. |
| ArjoHuntleigh/Maxi Sky 1000 arjohuntleigh.com | Allows a single caregiver to perform transfers of bariatric residents under handset control without stress or strain and with no manual lifting. | Can be mounted on semi-permanent gantry or permanently mounted in the ceiling. 1,000 lb. capacity. |
| BHM Medical/V3 bhm-medical.com | Portable ceiling lift motor. | 440 lb. capacity |
| BHM Medical/V4 bhm-medical.com | Fixed ceiling lift. Has return to charge feature. | 440 or 600 lb. capacities available. |
| BHM Medical/V10 bhm-medical.com | Fixed ceiling lift. Has return to charge feature. | Lifting capacity of 1,000 lbs. |
| BHM Medical/Voyager Duo bhm-medical.com | Fixed ceiling lift. | Lifting capacity of 220/440 lbs. |
| BHM Medical/Voyager Portable bhm-medical.com | Portable ceiling lift motor. Reacher allows disengagement of the strap for easy suspension and removal. | 440 lb. capacity |
| EZ Way, Inc./EZ Ceiling Lift ezlifts.com | Fixed motor ceiling lift. Weight capacities ranging from 650 to 1,000 lbs. | Curtain track pass-through feature; many track configurations available. |
| EZ Way, Inc./EZ Portable Ceiling Lift ezlifts.com | Portable motor ceiling lift. | 450 lb. capacity. |
| Guldmann/GH2 Ceiling Hoist System guldmann.net | System is available as small, free-standing, self-sustaining and removable unit or as wall- or ceiling-mounted system. | Available with many motor options and customizable ceiling track configurations. Capacities from 200 to 1,000 lbs. |
| Guldmann/GH3 Ceiling Hoist System guldmann.net | Modular system that can be mounted on ceiling, wall or support pillars. System is available as small, free-standing, self-sustaining and removable unit or as wall- or ceiling-mounted system. | Available with many motor options and customizable ceiling track configurations. Capacities from 200 to 1,100 lbs. |
| Horcher Lifting Systems/UniLift horcher.com | Available in 350 and 500 lb. capacities. Patients with sufficient upper body strength may operate lift independently. | Can use one lift in a variety of locations and an unlimited number of track configurations |
| Liko North America/Free Span liko.com/na | Gantry system that can be moved from room to room. | 880 lb. capacity. |
| Liko North America/Likorall liko.com/na | Single motor lifts up to 507 lbs., dual motor version lifts 1,014 lbs. | Self -braking motor. Lift stops automatically if power is interrupted. Available in single- or multi-track and room-to-room. |
| Liko North America/Multirall liko.com/na | Portable/reversible with quick-release hook system. | Self -braking motor. Lift stops automatically if power is interrupted. Available in single- or multi-track and room-to-room. |
| Medcare Products/Medcare Lift medcarelifts.com | Single motor/single track ceiling lift system. | 450 or 625 lb. capacity. |
| Medcare Products/Portable Lift medcarelifts.com | Ceiling lift motor that can be moved from room to room. | 425 lb. capacity. |
| Medcare Products/Bariatric Lift medcarelifts.com | Ceiling lift with heavy duty motor to accommodate bariatric patients. | 1,000 lb. capacity. |
| Molift/Nomad moliftinc.com | Portable ceiling lift. | 4-point suspension integrated into lifting motor; integrated handles; climbs from room-to-room. Emergency stop and lowering. |
| Polaris Medical / Wispa Lite polarismedicalinc.com | Compact and light portable ceiling lift . Room to room option (under door header) available | 440 lb. capacity. |
| Polaris Medical / Wispa Lift polarismedicalinc.com | Fixed ceiling lift can be equipped with tracking and gates. | 440 or 600 lb. capacity. |
| Polaris Medical / Wispa Bariatric polarismedicalinc.com | Fixed or temporary set up. | 1000 lb capacity |
| RoMedic/Reda romedic.com | Ceiling list design that can be used with permanent or portable motor applications. | Wide lifting range, 505 lb. capacity. |
| RoMedic/Rise 450M romedic.com | Ceiling track hoist. Ca be used on a single track with or without curves and switches or a traverse/gantry system. | 450 lb. capacity. Battery charge life display. Emergency stop function and emergency lowering. Automatic switch off after 2 min. when not in use. |
| RoMedic/Rise625T romedic.com | Built in traverse motor for powered travel along the track. Can be a single track, with or without curves and switches, or a traverse/gantry system. | 625 lb. capacity. Battery charge life display. Emergency stop function and emergency lowering. Automatic switch off after 2 min. when not in use. |
| RoMedic/RisePorto romedic.com | Ceiling lift motor that can be moved from room to room. | Portable. 440 lb. capacity. |
| Surehands/Freedom Bridge handimove.com | Free-standing rail system on which you can mount a permanent or portable motor. | When rail attachment to ceiling or wall is not possible. |
| Surehands/2000 Series handimove.com | Portable ceiling lift system that can be configured for various applications. | Available in single room or room-to-room models. |
| Surehands/HM2500 handimove.com | Permanent ceiling lift system that can be configured for various applications. | Manual, electric or infra red wireless controls available. 440 lb. capacity. |
| Surehands/Wall-to-Wall™ handimove.com | Patient lift that can be mounted on the wall in situations where a ceiling installation is not possible. | 320 lb. capacity |
| T.H.E. Medical/Cirrus themedical.com | Manual or power traverse options. Curtain track solution available. 6-pt spreader bar. | 600 lb. and 750 lb. capacities available. Available with infection control strap. |
| T.H.E. Medical/Stratus Portable themedical.com | 6-pt sling attachment system with auto-leveling side bars. | Capacities to 750 lbs. Infection control strap. Portable. |
| T.H.E. Medical/Pinnacle themedical.com | High-output lifting motor with oversize spreader bars. | Extra long, extra wide lifting strap. Capacities to 1,000 lbs. |
| Waverley Glen/Griffin Lift waverleyglen.com | Lightweight and portable with pneumatic hand control. | 425 lb. capacity. Infra red control optional |
| Waverley Glen/P-440 Portable Lift waverleyglen.com | Lightweight and portable. | 440 lb. capacity. |
| Waverley Glen/Transportable Lift waverleyglen.com | Charger included in base price. | 440 lb. capacity. |

## Chairs: Dependency/Geri/Specialty/Wheelchairs (Seated Transport)

| Chairs/Seated Transport PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| 21st Centrun Scientific/H-Frame Bounder Wheelchair 21stcenturyscientific.com | Power chair provides transport for the user in rain or snow without slipping. | H-frame uses 2.5" by 1.5" by .125" wall steel-TIG welded for superior strength. |
| ArjoHuntleigh/Aura Seating System arjohuntleigh.com | Seating cushions with pressure redistributing alternating cell systems for standard hospital and home chairs. | Standard support surface or battery-powered options available. |
| ArjoHuntleigh/TotaLift II Transfer Chair arjohuntleigh.com | Can be used to transfer patient laterally from bed to stretcher, then transport them in sitting or reclining position. | 400 lb. capacity. Avoids caregiver having to lift the patient at all. |
| ArjoHuntleigh/TC-300 Treatment Chair arjohuntleigh.com | 23-37” high treatment/transport chair. Foot pedal operation, one-step Trendelenburg. | 400 lb. capacity. Accessories for various procedures available. |
| Hill-Rom/Barton® I-440 Transfer Chair hill-rom.com | Converts from a flat transfer chair to an upright mobile chair. Adjustable head, foot and arm rest. | 400 lb. capacity. Positioning and transfer system option. |
| Hill-Rom Art-of-Care® Recliners hill-rom.com | A variety of models available. | Positioning handles, locking casters, accessory hooks, optional removable arm, tray table and accessory brackets are available. |
| Horcher Lifting Systems/CosyChair horcher.com | Chair that can be used in many settings with gas strut-operated back and footrest. | Lambs wool cover on seat and footrest. |
| Cambridge Technologies/ Care Chair camtecproducts.com | Converts from chair to stretcher laterally and eliminates patient transfers. | Sitting on the head end will not tip. Available in regular and bariatric models. |
| Medcor/Care Cliner medcor.net | Used in dialysis, blood collection, geriatric care, oncology, respiratory, critical care and cardiac rehab. | Pressurized gas cylinder helps elevate back from deep recline position. Arms adjust for lateral transfers. |
| Invacare/Clinical Recliner invacare.com | Provides patient seating. Used in patient procedures such as dialysis, blood collection, day surgery, EEK/EKG care, oncology, outpatient clinic, E.R., room and maternity. | Varied density foam cradles the patient to sustain proper positioning and posture. |
| Invacare/HTR Tilt and Recline Chair invacare.com | Provides transport and a range of angles in tilt adjustment, which allows the user to redistribute body weight as needed. | Fold down back provides easy access to personal grooming and transfers. |
| PHS West / Ergo-Express PTC1 phswest.com |  |  |
| SMT Health Systems/Rock-King smthealthsystems.com | A wheelchair that allows the user to rock back while seated with dual positions. | Dual position rocker lock and rocking mechanism. |
| Stryker/Stretcher Chair stryker.com | Used to move patients from a prone position to a seated one. | Ergonomic seat tilt allows a recliner position, 24" wide and 74" length patient surface. |
| Wy’East Medical/TotaLift-II wyeastmed.com | Chair that converts to stretcher/procedure/transport table. | 400 lb. capacity. |

## Floor-Based Lifts								 TOP

| Floor-Based Lifts PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| Andermac/Anderlift Lift and Transport System anderlift.com | When lifted, transported, weighed or bathed, the patient is resting on a "hammock" created by a custom sheet and nestled in the frame. | Four pieces of equipment in one - a bather, bed scale, transportation system and power patient lift. Up to 800 lb. capacity |
| ArjoHuntleigh/Maxi 500 arjohuntleigh.com | Battery-powered floor-based lift. | 500 lb. capacity. Can lift patient from the floor. |
| ArjoHuntleigh/Maxi Lite arjohuntleigh.com | Battery-powered floor-based lift for general lifts and transfers including chair to car.. | 350 lb. capacity. |
| ArjoHuntleigh/Maxi Move arjohuntleigh.com | Battery-powered floor-based lift. | Built-in scale. 500 lb. capacity. |
| ArjoHuntleigh/Maxi Twin arjohuntleigh.com | Dynamic Positioning System (DPS) which can lift the patient from the floor without having to lift the patients head or back to attach the sling and a soft stretcher which is able to lift patients from the floor in the supine position. | 375 lb. capacity. Allows rotation of patient 360 degrees with dual masts. |
| BHM Medical/Ergolift II bhm-medical.com | Floor-based lift. | 400 lb. capacity |
| BHM Medical/Ergolift 600 bhm-medical.com | Floor-based lift for bariatric patients. | 600 lb. capacity. Base expands for use with wider bariatric chairs. |
| EZ Way, Inc./EZ Light Lift ezlifts.com | Portable floor based lift that can be folded for easy storage or transport. | 300 lb. capacity. |
| EZ Way, Inc./EZ Way Bariatric Patient Lift ezlifts.com | Floor-based lift for bariatric patients. | 1,000 lb. capacity |
| EZ Way, Inc./Smart Lift ezlifts.com | Floor lift with many features for improved safety. | 500 and 600 lb. capacities. |
| Guldmann/GL2 guldmann.net | Mobile lifter (electric). with manual width adjustment. | Maximum load 330 lbs. |
| Guldmann/GL3 guldmann.net | Mobile lifter with both electric width adjustment and lifting function. | Maximum load 330 lbs. |
| Guldmann/GL4 guldmann.net | Mobile lifter with electric parallel width adjustment and electric lifting function. | Maximum load 330 lbs. |
| Guldmann/GLHD guldmann.net | Mobile lifter available with electric width adjustment. | Maximum load 440 lbs. |
| Horcher Lifting Systems/Diana horcher.com | Electric floor based lift for patients with limited mobility. | 440 lb. capacity. |
| Horcher Lifting Systems/Lexa horcher.com | Floor based lift that can take up patients sitting, lying, or from the floor and move them to a sitting or lying position for transport to toilet, wheelchair or bed. | Sway-free transport. |
| Invacare/Reliant Lifts invacare.com | Floor-based lift for patient/resident transfers. Many models available. | 300 to 600 lb. capacity. |
| Liko North America/Golvo liko.com/na | Floor-based lift that can be used to lift a patient from the floor, do a lateral transfer, or outfitted for ambulation training. | 440 lb. capacity. |
| Liko North America/Uno 100/102 liko.com/na | Floor-based lift residential and home-care settings. | 385/440 lb. capacity. |
| Liko North America/Uno 200 liko.com/na | With the different settings on the lift mast and the actuator, Uno 200 can be used for lifting from the floor as well as to high beds or gurneys. | 440 lb. capacity. |
| Liko North America/Viking L liko.com/na | Floor-based lift for common lifting needs. | 550 lb. capacity. Can be outfitted with arm rests for gait training. |
| Liko North America/Viking M liko.com/na | Floor-based lift for common lifting needs. Weighs only 77 lbs. | 450 lb. capacity. Can be outfitted with arm rests for gait training. |
| Liko North America/Viking 2 liko.com/na | Floor-based lift for transfers between bed and wheelchair, to and from the toilet and bathtub or for lifting someone who has fallen to the floor. Weighs only 55 lbs. | 350 lb. capacity. |
| Liko North America/Viking X2 liko.com/na | Floor-based lift. Narrow frame for use where doorways are narrow or space is limited. | 350 lb. capacity. |
| Mangar USA/Camel mangarusa.com | A battery operated pneumatic lifting cushion. Lifts a patient weighing up to 700 lbs. from the floor. With back rest. | Compact, lightweight, can be used in tight spaces, such as closet or bathroom; may be used outdoors. |
| Mangar USA/Elk mangarusa.com | A battery operated pneumatic lifting cushion. Lifts a patient weighing up to 1,000 lbs. from the floor. Without back rest. | Compact, lightweight, can be used in tight spaces such as closet or bathroom; may be used outdoors. |
| McAuley Medical/SuperLIFT mcauleymedical.com | Device for transferring or transporting a patient in the supine position. Powered lift and base expansion/contraction feature. | Lifts up to 500 lbs. |
| Medcare Products/Medcare Lift medcarelifts.com | Transfers patients of up to 475 lbs. | Expandable base design |
| Medcare Products/Care Bâr Lift medcarelifts.com | Floor based lift for bariatric patients. | Lifts up to 600 lbs. |
| Medline/Electric Elevating Lift medline.com | Battery-operated lifts used to transfer patients up to 600 lbs. 6-point cradles, broad lifting ranges. Sling made of soft, easy-to-clean, long-lasting polyester material. | Low-profile base legs; 6-point sling cradles. |
| Molift/Partner 205 moliftinc.com | Battery operated, lifting capacity of 450 lbs., 4-point suspension, 70” lifting capability from the bottom of the suspension that lifts from floor, bed, or sitting. Stretcher, Scale & Supporting Arms available as accessories. | Automatic service warning (counts lifts and alerts user when it is due for service), emergency stop, electronic & manual lowering standard on all lifts. |
| Molift/Partner 255 moliftinc.com | Battery operated, lifting capacity up for 505 lbs. (230) and 560 lbs. (255) with 4-point suspension, vertical lifting column, lifts from floor, bed, or sitting. Works with Stretcher, Scale & Supporting Arms available as accessories. | Automatic service warning (counts lifts and alerts user when it is due for service), emergency stop, electronic & manual lowering standard on all lifts. |
| Polaris Medical / Fionia home lift polarismedicalinc.com | Battery-operated lifts used to transfer patients in home setting Only 21” wide to fit through most doorways | Battery pack is interchangeable. Weight capacity 209 lbs |
| Polaris Medical / Mobile Floor Lifts polarismedicalinc.com | Range of lifts from 365lbs, 440lbs and 550lbs. Power opening base very wide stance with long arm length options for extended reach. | Varying weight capacities for specific needs. Can be supplied with wall or on board charging. |
| Polaris Medical / Selandia Bariatric polarismedicalinc.com | Battery operated, lifting capacity of 660 lbs., 3-point suspension, Lifting capability from floor, bed, or sitting. Stretcher, Scale available as accessory | Heavy duty lift but quite easy to move around. |
| RoMedic/Carina 350 romedic.com | Collapsible for transport or storage, portable floor-based lift with either electric or manual base-widening. | 350 lb. capacity. |
| RoMedic/Eva400 romedic.com | Mobile patient hoist to lift from a seated or prone position. Can be used for gait training. | Lightweight aluminum. Adjustable mast and base. 400 lb. capacity. |
| RoMedic/Eva450 romedic.com | Mobile patient hoist to lift from a seated or prone position. Can be used for gait training. | Lightweight aluminum. Adjustable mast and base. 450 lb. capacity. |
| RoMedic/Eva600 romedic.com | Mobile patient hoist to lift from a seated or prone position. Can be used for gait training. | Lightweight aluminum. Adjustable mast and base. 600 lb. capacity. |
| RoMedic/Rino450EE romedic.com | Floor-based lift that can lift a patient from the bed or floor in seated or supine position. | 450 lb. capacity. |
| RoMedic/Rino450EM romedic.com | Floor-based lift that can lift a patient from the bed or floor in seated or supine position. | 450 lb. capacity. Can also be equipped with a digital scale. |
| SMT Health System/Volaro Series 4 smthealthsysems.com | Lifts patient from floor, commode, wheelchair, bed and recliner. | 450 lb. capacity; many options and slings available. |
| SMT Health System/Volaro Series 5 smthealthsysems.com | Battery powered lift to lift patient from floor, commode, wheelchair, bed and recliner. | 400 lb. capacity; many options and slings available. |
| Technimotion Medical/Proxi-Motion technimotion.com | Floor-based lift for lifting, repositioning, transfers and sit-to-stand assistance. | 450 lb. weight capacity. Strap adjustment system. |
| T.H.E. Medical/Ultralift 300XH themedical.com | Floor-based lift with 6-pt spreader bar; extra-long boom. | 600 lb. capacity; low base allows access beneath low beds. |
| Vancare/Vanderlift vancare.com | Floor-based lifts with many options. | 400-1,000 lb. capacity models available. |
| Waverley Glen/F550 waverleyglen.com | Electronic lift and transfer device. | Dual wheeled castors, legs spread to allow more stability. 550 lb. capacity. |

## Lateral Transfer Aids and Repositioning Devices 
(Air Assisted, Friction Reducing, Manual and Powered, Transfer Boards)

Many devices are available for accomplishing lateral transfers as well as repositioning a patient in a bed or a chair. Friction reducing devices are often constructed of smooth synthetic fabrics. These facilitate the lateral transfer or repositioning of patients that can offer limited or no assistance. The reduction of forces required to execute the transfer minimize biomechanical loading of the caregiver’s arms and back. Properly designed handles and pull straps can improve the caregivers’ grasp and reduce forward reach during transfers.  Air-assisted devices are specialty transfer mattresses installed beneath dependent patients onto which a portable air supply is attached that inflates the mattress. Air flows through perforations in the underside of the mattress, reducing the forces required to execute a lateral transfer. Other devices include transfer boards and powered transfer devices. There are also devices for dependency and wheelchairs that will keep that patient from sliding.

| Lateral Transfer and Repositioning Aids MFG./PRODUCT/URL | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| AIR-ASSISTED LATERAL TRANSFER AIDS |  |  |
| Airpal/Patient Air Lift airpal.com | A flexible nylon mattress that allows lateral transfers of patients from bed to stretcher by the release of low air pressure through a patented chamber. | Patients remain on transfer pad for all ancillary products, including radiology, CT scan, radiation therapy, heart catheterization, physical therapy, labor and delivery. |
| Airpal/RAMP™ airpal.com | Rapid airway management positioning system. | Achieves proper ear to sterna notch position, alignment of the airway axes and improves glottic exposure. |
| EZ Way/EZ Matt ezlifts.com | Air-assisted mattress for lateral transfers. | No weight limit. |
| Hovertech/HoverJack hovermatt.com | Lifts patient from floor to bed or stretcher height. | No weight limit. |
| Hovertech/Hovermatt hovermatt.com | A waterproof, anti-bacterial anti-stain, nylon mattress that allows for lateral transfers of patients. | No weight limit. Available in single-patient use as well as split-leg for use in the lithotomy position or half-matt for use during cystology and ob/gyn procedures. |
| Hovertech/HoverTrans Safety Slide hovermatt.com | For evacuation of non-ambulatory patients during emergency. | Uses Teflon-infused polyester for a smooth sliding surface and a bed mattress for patient support and protection. |
| McAuley Medical/AirSlide mcauleymedical.com | Dual-technology lateral transfer system. Air mattress and slide sheet all in one. | Absence of air holes, stitching and Velcro decreases infection control issues. |
| Smart Medical/Liftaem smartmedtechnology.com | Air-assisted device to assist with lateral patient transfers and patient repositioning. | Can remain on mattress during procedures such as x-rays, CT scans, etc. 1,000 lb. capacity. Single patient use (disposable). Fully-contained cart with blower and storage compartment. |
| FRICTION REDUCING DEVICES |  |  |
| ArjoHuntleigh/Maxislide arjohuntleigh.com | An ergonomically-designed device having ultra-low friction properties, that will aid caregivers in moving or positioning patients while at the same time encouraging good posture without causing undue stress to either the patient or the caregivers. | Once the patient has been moved up in bed, there is minimal risk of them sliding back down again after being released. |
| ArjoHuntleigh/Maxi Slide arjohuntleigh.com | Sliding system that offers a wide range of patient handling maneuvers. | Easy to place and remove from under the patient. |
| Ergosafe/ErgoSlide LT4400 ergosafeproducts.com | One piece non-mechanical device to laterally transfer from one surface to another without involving multiple caregivers, lifting the patient, or using mechanical equipment. | One piece heavy-duty nylon tube with four sets of handles that folds for easy storage. |
| EZ Way/EZ Slide Sheets ezlifts.com | Friction reducing device to facilitate patient repositioning, lateral transfers and sling insertion. | Available in various sizes and flat or roller/tube configurations. |
| Liko/Handy Sheet liko.com/na | Friction-reducing device that can be used for the insertion and removal of slings from patients in the supine or seated position. Can also be used to assist in patient repositioning and lateral transfers. | Available in 2 sizes. |
| Liko/Handy Tube liko.com/na | Friction-reducing material in a tube configuration to ease patient repositioning in bed. Also comes with straps for lateral transfers of supine patients. | Available in 3 sizes. |
| McAuley Medical/Slide Sheets mcauleymedical.com | Friction reducing devices in various sizes to ease patient repositioning. Washable. | Disposable versions available. |
| McAuley Medical/Rotating Seat Cushion mcauleymedical.com | Friction reducing devices that assists in positioning/rotating a patient while in a seated position for transfers. | Can be used on a chair or in a car. |
| McAuley Medical/One Way Slide Cushion mcauleymedical.com | Can be used in a chair, bed or car to help the patient avoid sliding out of position. | Material slide one way and locks to maintain posterior position. |
| Phil-E-Slide/Lateral Transfer Aid phil-e-slide.us.com | Used to move a supine patient from one level flat surface to another. | Has six durable handles where pull straps are attached. |
| Polaris Medical / Polaris Slides polarismedicalinc.com | Used to assist persons movement in bed or chair. With handles or without. Wide selection available. | Available as patient specific disposable slings in many different fabrics and types. |
| Polaris Medical / One Way Slide cushion polarismedicalinc.com | Used to position and prevent sliding of patient in bed or chair. | Material slide one way and locks to maintain position. Wide selection available with bariatric sizes |
| Polaris Medical / Twist Assist Swivel cushion polarismedicalinc.com | Cushioned high quality swivel cushion used for chair transfer and standing pivots | Used in hospitals, home care and other areas where seating solutions are required |
| Polaris Medical / Positioning cushion polarismedicalinc.com | Allows for proper positioning of wheelchair user in chair with easy removal afterwards. | Easy to use and remove. |
| RoMedic/AutoSlide romedic.com | Used in car to allow users to turn and move sideways easily. | Cushion attaches to the back of the car seat so that the user cannot slide forward. |
| RoMedic/Easy Slide romedic.com | Used for transfers between beds and x-ray or operating tables, shower trolleys, stretchers, etc. | Can be used for turning users on x-ray or operating tables. |
| RoMedic/MoveMaster romedic.com | Sliding mat with handles at one end to assist in various transfers and repositioning the patient in bed. | Can be used when transferring to shower chairs or to the toilet. |
| RoMedic/OneWaySlide romedic.com | Used to improve sitting positions in chairs or wheelchairs. | Allow patient to maintain position without sliding forward. |
| RoMedic/OneWaySlide Tubular romedic.com | Same as OneWaySlide but tubular material for more active users. | Allow patient to maintain position without sliding forward. |
| RoMedic/TurnSheet romedic.com | Placed on a mattress cover which is smooth in the middle and has an anti-slip surface at the edges, this minimizes the risk of the user sliding out of the bed. | Does not interfere with properties of pressure-reducing mattresses. |
| RoMedic/TurnTableSoft romedic.com | Facilitates turning by the use of two round discs with friction-reducing material on one side. | Material resists soiling. |
| Scan Medical/Safety Sure Transfer Slide scanmedical.com | A padded tube of fabric used to transfer a patient from one surface to another without lifting. | Various sizes allow use in different situations and with varying dependency levels of patients. |
| Slide-Mat/Slide Mat slidemat.com | Eliminates the need for lifting or log-rolling to effectively transfer or reposition. | Comfortable, radiotranslucent, compact and lightweight. |
| Sundance Tortoise P.A.D. www.sundancesolutions.com | Turning, repositioning and offloading system designed to reposition patients while offloading the sacrum and greater trochanter using a combination of fluidized technology and positive air displacement. | Aids in keeping the patient in a desired position and helps the caregiver reposition (boost) the patient up in bed. |
| SureHands/One-Way Sheet handimove.com | Glides in one direction only to maintain seated posture. | Comes in sizes to use on a chair or in a bed. |
| SureHands/Rotating Cushion handimove.com | Used for turning in seated position on chair or bed. Padded. | Allows independent use. |
| SureHands/Rotation Disk handimove.com | Assists in transfers from bed to chair or toilet. | Patient stands on disk, caregiver can easily pivot them to transfer. |
| SureHands/Sliding Mat handimove.com | Used for lateral transfers. | Padded version of the slide sheet. |
| SureHands/Sliding Sheet handimove.com | Friction reducing device to assist in patient repositioning. | Lessens caregiver strain in performing moves and transfers. |
| Waverley Glen/Four Way Glide waverleyglen.com | Can be used to reposition the patient and then maintain that position. | Bottom is low friction nylon, top is quilted to lock system in place. |
| Waverley Glen/MultiGlide waverleyglen.com | Can be used to reposition the patient in multiple directions. | Various sizes available. |
| Waverley Glen/One Way Glide waverleyglen.com | Material slides in one direction only. | Allows patient to maintain position in bed or chair. |
| Waverley Glen/PediTurn waverleyglen.com | Assists in pivoting patient for transfer. | Low-friction disk with non-slip material on the outer surface. |
| Waverley Glen/Swivel Cushion waverleyglen.com | Assists patients to rotate while in a seated position. | Low-friction circular cushion with quilted cotton cover. |
| Wright Products/Slipp Patient Mover wrightproductsinc.com | Used for patient repositioning and transfers. | Easy portability (4 lbs.) and storage. |
| Wy’East Medical/Pink Slipp wyeastmed.com | Friction reducing device with HD Poly Warm-Touch Technology.™ It glides against itself, reducing skin shear and making lateral transfer and repositioning easier to accomplish. | Individually packaged in a ready-to-use Stat Pak, fits in a pocket or dispenser for immediate use. |
| MECHANICAL LATERAL TRANSFER DEVICES |  |  |
| ArjoHuntleigh/TotaLift II Transfer Chair arjohuntleigh.com | Can be used to transfer patient laterally from bed to stretcher, then transport them in sitting or reclining position. | 400 lb. capacity. Avoids caregiver having to lift the patient at all. |
| Astir Technologies/ PowerNurse astirtechnologies.com | Conveyor belt device that allows caregiver to transfer patient without lifting. Remote control device allows you to select three modes of belt control. | Faster transfers, decreases nursing MSD injuries. Hand-held remote control. |
| BAMS Manufacturing/PRS prsbacksaver.com | Device that can be attached to a bed or procedure table to do lateral transfer or reposition the patient up in bed. | Requires only one caregiver. Attachment available to rotate the patient or can use a sling to support arm or leg. |
| Barton Medical Corp./Safe T Trans bartonmedical.com | Device enables a non-ambulatory person to be transferred from surface to surface. | Can be use in various positions e.g. as a bed, gurney bed, lounge chair, or wheelchair. |
| ConMediSys /MediGLIDER™ conmedisys.com | Patient transfer device with dual-end operational controls, single-use sanitary disposable sheets. Transfers patient in 1-3 minutes. | 500 lb. capacity; eliminates lateral transfers. |
| EZ Way/Repositioner ezlifts.com | Attached to the bed, a sheet is fed from a roller at the foot of the bed to a roller at the head of the bed. As the handle turns the roller the sheet moves the patient toward the head of the bed. | Can be rewound for additional use. |
| Technimotion/Shuttle Motion technimotion,com | Transfer and bedside chair that reclines and extends horizontally over the center of the patient bed. | 350 lb. capacity. |
| T.H.E. Medical/Power Glide Lateral Transfer System themedical.com | Provides all-electric lateral transfers to/from bed/stretcher, OR table, medical imaging table, or other supine surface. | Once the transfer is complete can be used as a transport stretcher. |
| Wy’East Medical/Totalift II wyeastmed.com | Transfer of your patient laterally from bed to stretcher without lifting, then adjusts to a sitting or reclining position. | Converts from a chair to a horizontal stretcher position. 400 lb. capacity |
| Wy’East Medical/Totalift PTS wyeastmed.com | Uses the Wy'East Medical patented system to carry the patient from bed to stretcher with no lifting. | Integrated bumper system cushions jolts and reduces damage to hospital furniture. |
| TRANSFER BOARDS |  |  |
| Composite Medical Distributors/Durable Wood Transfer Board ultraslick.com | Lightweight transfer board; 250 lb. capacity; 8” wide; lacquer finish. | Available in 13”, 18”, 24”, 27” and 30” lengths, with or without hand holes. |
| Composite Medical Distributors/UltraSlick® Transfer Board ultraslick.com | Lightweight transfer board; Composite outer layer; 250 lb. capacity; lacquered finish on back to allow for easy placement. | Outer layer never requires waxing or refinishing. Available in 18”, 24”, 27” and 30” lengths—with and without holes. |
| McAuley Medical/Rollbord mcauleymedical.com. | Utilizes a conveyor belt action to move a patient from surface to surface easily. | Anti-microbial, anti-static material. Disposable cover sheet. |
| McAuley Medical/Glide Board mcauleymedical.com. | Low friction, medical trade plastic device to assist in transfers from wheelchair to chair, car. | Easy to clean, available in many configurations and sizes. |
| McAuley Medical/Turn Plate mcauleymedical.com. | Assist the caregiver in pivoting the patient for seated transfers. Rubber grip on bottom. | Use on any type of flooring. |
| Polaris Medical / Q2 board polarismedicalinc.com | Low friction, medical trade plastic device to assist in transfers from wheelchair to chair, car. | Antimicrobial. 627lbs weight capacity. Optionally available with an integrated patient specific slide sheet. |
| Polaris Medical / Lateral Assist Transfer board polarismedicalinc.com | Transfer board with removable cover to assist in transfers from one level surface to another. | Comes in 2 fold or 4 fold for EMS and storage purposes. Comes complete with storage bag. |
| RoMedic/Easy Glide romedic.com | Used for transfers between wheelchairs and beds, toilets, and car seats. | Made of strong, flexible polyethylene. Anti-slip tape on the underside prevents the board from sliding during transfer. Available in four sizes. |
| RoMedic/Easy Glide Oval romedic.com | Used for lying transfers, higher up in bed, turning in bed and transfers from one bed to another. | Note: Should have firm underlying surface on both sides when doing transfers. |
| RoMedic/Easy Glide Extra Long romedic.com | Used for lying transfers, higher up in bed, turning in bed and transfers from one bed to another. | Note: Should have firm underlying surface on both sides when doing transfers. |
| RoMedic/RollerSlide romedic.com | Used for lying transfers, bridges gaps and moves with the users during the transfer. | Ultra low friction cover can be laundered. |
| SureHands/Rolbord handimove.com | Used for bed to bed or bed to shower chair transfers. | Makes transfers easier for the patient. |
| SureHands/Transfer Board handimove.com | Used for transfers to/from objects of the same height. | Promotes independent use. |
| Waverley Glen/Onward Turntable waverleyglen.com | Assists in pivoting patient for transfer. | Injection molded plastic for institutional use. |
| Waverley Glen/Two-Move Lateral Transfer Boards waverleyglen.com | Transfer board with removable cover to assist in transfers from one level surface to another. | Available in two widths as well as pediatric. |

## Lifts (Other)									 TOP

| Lifts (Other) PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| Andermac/Anderlift anderlift.com | Complete bedside treatment system that lifts, transports, bathes weighs and turns. | Multi functional- a bather, bed scale, transportation system, power patient lift and turning system. |
| Aquatic Access/Pool Lifts aquaticaccess.com | Lifts for above-ground, below-ground and indoor pools as well as spas. | Many configurations and installation options available. |
| Diaco/Dental Chair diaco.co.uk | Mobile dental platform designed for wheelchair patients. Patient remains in their own wheelchair at all times while being positioning for dental care treatment. | No need for manually moving the patient, suitable for manual and electric wheelchairs and all patients up to 1,100 lbs. |
| Horcher Lifting Systems/Caesar Pool Lift horcher.com | Floor- or ceiling-mounted pool lift to assist in pool entry and exit without the aid of a caregiver. | Manual keypad at entry and water level; emergency off switch; 440 lb. capacity. |
| LiftSeat/LS400 liftseat.com | Powered toilet lift that can be installed over the toilet in the bathroom or used as a bedside commode. | Supports patient sit-to-stand motion path and reduces weight-bearing stress on staff. 400 lb. capacity. |
| LiftSeat/LS600 liftseat.com | Powered toilet lift that can be installed over the toilet in the bathroom or used as a bedside commode. | Supports patient sit-to-stand motion path and reduces weight-bearing stress on staff. 650 lb. capacity. |
| Liko North America/FreeSpan Pool Lift liko.com/na | For use in a swimming pool are, one leg is installed in the pool and the other on the edge of the pool. | Available in different lengths and used with the Likorall or Multirall systems. |
| Martin / Martin Chair-A-Table martininnovations.com | Wheelchair integrates with patient exam table without having to lift the patient onto the table. | No lift, 1,000 lb. capacity. |
| Midmark/625 Barrier-Free Examination Table midmark.com | Exam table with 18” to 37” adjustable seat height. Lithotomy, supine, elevated, low and barrier-free positions. | 650 lb. capacity. Many options and accessories available. |
| REDpoint International/Versatilt versatilt.com | Wheelchair tilting device that allows patients to be examined by dentists, doctors and other healthcare providers while the occupants remain in their wheelchair. | Eliminates the need for difficult, dangerous transfers. Portable, easy to transport from place to place. Accommodates manual and electric wheelchairs. |
| RMT-Aquatics/PAL Portable Aquatic Lift grouprmt.com | Assortment of completely portable and deck mounted semi-portable, battery operated pool lifts | Make any pool or spa accessible, and can be conveniently stored away when not in use. |
| SureHands/HM2500 handimove.com | Permanent ceiling lift system that can be configured for various applications. | 440 lb. capacity. |
| SureHands/Motorized Wheelchair to Water™ Pool Lift handimove.com | Portable lift device on casters that can be rolled out to a pool and used for wheelchair to water transfers. | 297.6 lb. capacity. |
| T.H.E. Medical/TR 9650 Mobile Patient Combilift themedical.com | Bath lift that converts from a patient lift to a stretcher lift to a chair lift. | Can be used to transfer a supine or seated dependent patient into a bathtub. |

## Other Mobility Aids								 TOP

| Other Mobility Aids PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| Anderlift / Lift and Transport System anderlift.com | When lifted, transported, weighed or bathed, the patient is resting on a "hammock" created by a custom sheet and nestled in the frame. | Four pieces of equipment in one - a bather, bed scale, transportation system and power patient lift. 500 lb. capacity. |
| Brewer Company / Access High-Low Exam Table brewercompany.com | Powered exam table converts from chair to table and back again. Has grab bars, storage and foot control. | 450 lb. weight capacity; 18” low height, 28” when flat |
| Bruno Independent Living Aids / Electra-Ride II bruno.com | Battery powered stair lift for patient transport. | Safety sensor foot plate and a 45 degree rotation seat at both top and bottom of stairs. Accommodates up to 350 lbs. |
| Dane Technologies / PowerPal 1000 danetechnologies.com | A workplace safety solution that eliminates the strain and pain of pushing/pulling heavy wheeled loads, battery powered; custom hitches; handles up to 2,000 lbs. | Compact & low profile, clean look, easy maneuverability and quick operation. With ergonomic design and quick to learn features. |
| Dane Technologies / Wheelchair Mover danetechnologies.com | Power-assisted wheelchair mover transports patients up to 750 lbs. | 510k cleared, Class II Medical Powered Device assists in making wheelchair transport safer for the operator to reduce potential injury and physical strain while ensuring a safer mover for patients including ramps. Connects and disconnects to standard wheelchairs. |
| Diaco / Dental Chair diaco.co.uk | Mobile dental platform designed for wheelchair patients. Patient remains in their own wheelchair at all times while being positioning for dental care treatment. | No need for manually moving the patient, suitable for manual and electric wheelchairs and all patients up to 1,100 lbs. |
| DJ Products / Cart Caddy Lite djproducts.com | A battery powered, electric tug designed with enough power to push or pull carts that can typically be pushed or pulled by one person. | Designed to prevent the strains and pains often associated with manually pushing and pulling heavy carts and equipment. 1,500 lb. capacity. |
| DJ Products / Cart Caddy Shorty djproducts.com | A battery powered, electric tug designed with enough power to push or pull carts that can typically be pushed or pulled by one person. | Designed to prevent the strains and pains often associated with manually pushing and pulling heavy carts and equipment. 3,000 lb. capacity. |

## Sit-to-Stand Lifts

Also called Standing and Raising Aids, these lifts are used for patients with partial weight-bearing capability that need assistance getting up from a seated position for ambulation, or for transfers from a seated to seated position (chair to wheelchair, bed to wheelchair).

| Sit-to-Stand Lifts PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| ArjoHuntleigh/Sara 3000 arjohuntleigh.com | Powered features enable a single caregiver to provide routine handling activities such as transfers and toileting. | 400 lb. capacity. |
| ArjoHuntleigh/Sara Lite arjohuntleigh.com | Raises and lowers semi-dependent patients for transfers and toileting. | 385 lb. capacity. |
| ArjoHuntleigh/Sara Plus arjohuntleigh.com | Standing aid that can provide ambulation and balance training. | 420 lb. capacity. |
| ArjoHuntleigh/Stedy arjohuntleigh.com | Allows patients to stand themselves up before and after toilet transfers. | Encourages independence through participation in the transfer. 264 lb. capacity. |
| ArjoHuntleigh/Walker arjohuntleigh.com | Walking frame to allow patient to begin ambulation safely. | 300 lb. capacity. |
| BHM Medical/Ergostand bhm-medical.com | Sit-to-stand lift for patients with some physical strength. Can be used for dressing, transfers and hygiene care. | 400 lb. capacity. |
| EZ Way, Inc./EZ Bariatric Sit-to-Stand ezlifts.com | Sit-to-stand lift that can assist in ambulation. | 800 lb. capacity. |
| EZ Way, Inc./EZ Light Stand ezlifts.com | Portable sit-to-stand lift that folds for storage or transport. | Performs transfers or ambulation. |
| EZ Way, Inc./EZ Smart Stand ezlifts.com | Sit-to-stand lift with many electronic safety features. | 400 and 500 lb. capacity. |
| EZ Way, Inc./EZ Way Stand ezlifts.com | Sit-to-stand lift that can assist with ambulation. | 400 and 500 lb. capacity. |
| Horcher Lifting Systems/Raisa horcher.com | Facilitates standing and light transport. Available in motorized or manual versions. | Has a 350 lb. lifting capacity. |
| Invacare/Reliant invacare.com | Serves a range of dependency levels in transfers. | Multi-functional slings allow it to be a standing lift or seated transfer lift. |
| Liko North America/Sabina Lift liko.com/na | Sit-Stand lift for users who can actively participate in transfer process. | For load training, promotion of circulation and stimulation of natural body functions. Various models and options. |
| Medline/Stand Assist Lift medline.com | Battery operated lifts used to assist patients up to 600 lbs. | Low-profile base legs; 6-point sling cradles. |
| Polaris Medical / Sit-Stand Lifts polarismedicalinc.com | Range of lifts from 365lbs, 440lbs and 550lbs. Power opening base very wide stance with long arm length options for extended reach. | Varying weight capacities for specific needs. Can be supplied with wall or on board charging. |
| RoMedic/MiniLift125 romedic.com | Mobile sit-to-stand lift. | 275 lb. capacity |
| RoMedic/MiniLift200 romedic.com | Mobile sit-to-stand lift. | 440 lb. capacity |
| RoMedic/Rena 440DS romedic.com | Used for sit-to-stand and pivot transfers. | 440 lb. capacity |
| RoMedic/Rena 440EE romedic.com | Used in combination with the appropriate sling to lift and transfer patients. | 440 lb. capacity |
| RoMedic/Rena 440EM romedic.com | Used in combination with the appropriate sling to lift and transfer patients. | 440 lb. capacity |
| RoMedic/RiseUp600 romedic.com | Used in combination with the appropriate sling or vest to lift and transfer patients. | 595 lb. capacity. |
| Stand Aid of Iowa/Stand Aid stand-aid.com | Device for physically challenged individuals. Allows users to be vertically stabilized with 100% weight bearing. Available in a manual model with hydraulic or powered lift and drive. | Accommodates individuals 4'2" to 7' tall up, from 400 to 550 lbs. |
| T.H.E. Medical/Steady Aid 3500X themedical.com | Sit-to-stand lift with all electric powered base widening and removable foot plate for ambulation. | Optional digital weigh scale; 350, 450 and 750 lb. capacities. |
| Vancare/Vera vancare.com | Helps patients into a standing position. | 350 lb. capacity and adjustable foot and knee stand. Also available in 600 lb. capacity. |
| Waverley Glen/S440 waverleyglen.com | Sit-to-stand lift. | Patient must be able to bear weight and have some degree of self-support. |

## Standing Assist Aids								 TOP

| Standing Assist Aids PRODUCT/MANUFACTURER | DESCRIPTION | KEY ADVANTAGE |
| --- | --- | --- |
| Brown Engineering Corp / Bed-Bar bedbar.com | Support rail people use for assistance in and out of bed. | Applicable for people of any age or physical condition, short-term or long-term use. |
| BHM Medical/Ministand bhm-medical.com | Provides standing, sitting and toileting assistance and an efficient way to dress, clean and transfer individuals who requires some additional support. | 350 lb. capacity. |
| Care Catalog Services / Safe-T-Pole caremedical.com | Floor-to-ceiling pole assists with standing, sitting, transferring, or climbing stairs. | No tools required for installation. |
| Clarke Healthcare Products/ArcoRail clarkehealthcareproducts.com | Used for repositioning patients and providing assistance getting in and out of bed. | Handrail that rotates 360 degrees and locks in four positions. |
| Easy Stand / Evolv™ easystand.com | Stand aid with options including hip supports, lateral supports and head support. | Contoured seat provides a maximum over-center stretch for various body types. |
| Easy Stand / Glider™ easystand.com | Stand aid that allows users to move the handles with their arms (or a caregiver can assist) which creates a reciprocal movement in the legs. | The full range seat on the Glider stander has hinged break-away sections for each leg that allow full leg extension while standing. |
| EZ Way/Stand Aid ezlifts.com | Manual standing assist aid with a padded seat that swings into place for transport. | 400 lb. capacity. |
| Healthcraft Products, Inc. / SuperPole healthcraftproducts.com | The pole is held solid by a jackscrew expansion between floor and ceiling. The pole provides assistance in standing or moving. | No need for wall mounts, versatile- can be used beside a bed, bath, toilet, or chair |
| Hovertech International / HoverLift Sit-2-Stand Lift hovermatt.com | Sit-to-stand lift for patient transfers and transport. | 440 lb. capacity |
| Hovertech International / HoverLift 3-in-1 Lift hovermatt.com | Converts from total patient lift to sit-to-stand lift to ambulating lift. | 550 lb. capacity for sit-to-stand and lifting functions. |
| Life Stand USA / Life Stand Wheelchair lifestandusa.com | Fully motorized chair allows transfers from sitting to standing with power assistance and allows mobility in either position. | Remote swing away joystick allows for easier access to tables and desks. |
| Rand-Scot, Inc. / Easy Pivot Lifts randscot.com/lifts | Device used for transfer of patients from a wheelchair to a chair or bed. | Wide wheel base reduces chance of tipping. |
| RoMedic/QuickMove romedic.com | Supports patient when rising to standing position and during transfers. Must have some arm strength and be weight bearing. | 374 lb. capacity. |
| RoMedic/Return 7500 romedic.com | Used to lift or transfer care recipients that still have some strength left to or from beds, wheelchairs or toilets. | Can be used in a rehab unit for activating care recipients. |
| Stand Aid of Iowa, Inc. / Econo Stand stand-aid.com | Hand operated hydraulic lift used to aid users from a wheelchair to standing position. | Allows users to be vertically stabilized with 100% weight bearing. |
| T.H.E. Medical / Steady Aid 3500x themedical.com | Sit-to-stand mobile lift with dual controls, removable foot plate & knee pad, calf strap, power base widening. | 450 lb. capacity with leg support harness. |
| T.H.E. Medical/UltraTurn Sit-To-Stand Lift themedical.com | Manual sit-to-stand lift. | 300 lb. capacity |
| T.H.E. Medical/Ultramove Standing Frame themedical.com | Manual standing frame to assist patients that require minimal assistance. | 400 lb. capacity, adjustable base. |
| Uplift Technologies, Inc. / Uplift Seat Assist up-lift.com | Assist user to get up easier from a surface. | Portable and weighs only 9 pounds. |
| Technimotion Medical / Promi-Motion II technimotion.com | Floor-based lift. No log-rolling required with Smart Insertion slings. Performs car transfers. Battery operated. Tabletop attachments allows lift to function as overbed table when not in use. | 400 lb. weight capacity. The arms, which fold out of the way when not in use, enable seated frontal lifts for transfers or toileting and sit-to-stand applications. |
| Waverley Glen/SA400 waverleyglen.com | Stand aid with padded split seat. | 400 lb. capacity. |

## Bariatric Ambulatory/Mobility Aids

| Bariatric Ambulatory/Mobility Aids Mfg./Product | Description | Key Advantage |
| --- | --- | --- |
| Hill-Rom/Walker hill-rom.com | Height-adjustable walker | Capacity up to 800 lbs. |
| Medline/Bariatric Heavy Duty Walker medline.com | Walker | Over the toilet design, folds to 4.25” for storage or transporting |
| Polaris Medical / walkers and rollators polarismedicalinc.com | Walkers 415lbs and rollators 650lbs | Varying weight capacities for specific needs. |
| Sizewise/Bari-Walker sizewise.net | Wheeled walker | Weight capacity 750 lbs. |
| Sten-Barr/Bariatric Walker stenbarr.com | Walker | Specifically designed for obese patients. Weight capacity up to 850 lbs. |
| TFI Healthcare/Walkers tfihealthcare.com | Walkers available in various widths, heights, with and without wheels. | Capacities up to 700 lbs. |
| TriLine/Bari Walker trilinemedical.com | Adjustable walker from 31-39” in 1” increments. | 750 lb. capacity, 27” wide. |

Bariatric Bathing/Showering/Toileting

| Bariatric Bathing/Showering/Toileting Mfg./Product | Description | Key Advantage |
| --- | --- | --- |
| ActiveAid/Shower/Commode Chair Series 720 activeaid.com | Chair used to assist patients in the bathroom. | Available in four seat widths and accommodates up to 600 lbs. |
| Broda Seating/Bari385 Transport Commode Shower Chair seatingisbelieving.com | Suspension system and individually-conforming straps. 65 back recline position. | Reduces heat and moisture build up that lead to skin breakdown. 800 lb. capacity. |
| ConvaQuip/Bariatric Bath Seats convaquip.com | Assists residents getting into tub. | Certified for 850 lbs. |
| ConvaQuip/Bedside Commode convaquip.com | For residents getting into bathroom. | Certified for 850-1,500 lbs. |
| ConvaQuip/Obese Bath Seats convaquip.com | Patient support while bathing. | Accommodates 850 lbs. |
| Hill-Rom/Shower/Commode Chair hill-rom.com | Allows patient toileting and showering with easier patient transfer. | Capacity up to 800 lbs. |
| Invacare/Bariatric Drop Arm Commode invacare.com | Steel construction. Drop arms facilitate access. | 1,000 lb. capacity. |
| Medline/Bariatric Extra Wide Commode medline.com | Snap-on-toilet seat with 500 lb. capacity. | 24 inches wide. |
| Medline/Bariatric Reclining Shower Chair medline.com | Shower chair. | Reclines. |
| Medline/Bariatric Shower Chair medline.com | Shower chair. | 900 lb. capacity. |
| Medline/Bariatric Shower Chair/Commode medline.com | Shower/commode chair. | 700 lb. capacity. |
| Polaris Medical / commodes / shower commodes polarismedicalinc.com | Shower/commode chair. Back recline shower commode also height adjustable models | Full line of shower and commodes self propelled versions up tto 715lbs |
| Sizewise/Bari-Shower Commode sizewise.net | Stainless steel construction. Can be used at bedside, in shower, over toilet, or as transport. | Weight capacity is 750 lbs.; removable arm. |
| Sizewise/Drop Arm Commode sizewise.net | 24” model can be used at bedside or over toilet with a weight capacity of 600 lbs. 36” wide model can be used at the bedside with a weight capacity of 1,000 lbs. | Removable back and arms, padded seat. |
| Sten-Barr/Bariatric Commode stenbarr.com | Reinforced steel construction commode. | 700 lb. capacity. |
| Sten-Barr/Bariatric Shower Chair stenbarr.com | Shower Chair. | 750 lb. capacity. |
| TFI Healthcare/Commode/Shower Chair tfihealthcare.com | Various commodes and shower chairs available for bedside or over toilet. | Capacities to 850 lbs. |

## Bariatric Beds/Support Surfaces/Transport

| Bariatric Beds/Mattresses/Transport Mfg./Product | Description | Key Advantage |
| --- | --- | --- |
| Andermac/Anderlift Lift and Transport System anderlift.com | When lifted, transported, weighed or bathed, the patient is resting on a "hammock" created by a custom sheet and nestled in the frame. | Four pieces of equipment in one - a bather, bed scale, transportation system and power patient lift. Up to 800 lb. capacity. |
| Burke/Tri-Flex II™ Bed burkebariatric.com | Bed that accommodates patients up to 1,000 lbs. | Accommodates extra tall patients, folds and rolls, easy storage. |
| Burke/Tri-Flex II Air™ Transport Air Mattress System burkebariatric.com | Support surface with alternating pressure, low air loss and push button width adjustment from 37” to 48”/54”. | Maintains air support during transport. 1,000 lb. capacity |
| Burke/Tri-Lift™ Bed burkebariatric.com | Bed that convert to chair position and allows the patient to walk out of the end of the bed. | Adjustable widths. Available with optional air mattress and impulse drive system. |
| Burke/Barri Float™ burkebariatric.com | Therapeutic pressure reducing mattress system specially designed for the morbid obese patient to lower interface pressure & temperature. | Enclosed side bolsters for support. |
| Camtec/ExpandaCare Bariatric Bed 3954 camtecproducts.com | Bed/Transport | Adjusts from its full width of 54” to 39-1/2” in less than one minute. This allows it to pass through doorways and elevator entries as narrow as 40 inches, resulting in fewer transfers. |
| Camtec/Bariatric Bed 4400 camtecproducts.com | 5-function bed. Raises from 19” to 30.” Controlled by 10-button pendant. | Accessories available including scale. 1,000 lb. capacity. |
| Camtec/Bariatric Bed 4700 camtecproducts.com | 2-function bed. Electrically-driven head and foot elevations. | General-purpose unit is used primarily in nursing and private homes. 600 lb. capacity. |
| Carroll Hospital Group/Spirit Plus carrollhospitalgroup.com | Low bed with many options and features. | 8 ¾” low bed position, high position of 34”. 500 lb. capacity. |
| Carroll Hospital Group/Spirit Select™ carrollhospitalgroup.com | Acute care low bed. Includes bed exit alarm and weight monitoring. | 10” low height, 35 ¼” raised height. 500 lb. capacity. |
| ConvaQuip/5500B Bariatric Bed convaquip.com | Patient bed with four actuators and battery back-up system. | 750 or 1,000 lb. capacity. Sit-to-stand and cardio chair positioning. Foot-end egress. Scale included. |
| Dane Technologies/Wheelchair Mover danetechnologies.com | Power-assisted wheelchair mover transports patients up to 750 lbs. | 510k cleared, Class II Medical Powered Device assists in making wheelchair transport safer for the operator to reduce potential injury and physical strain while ensuring a safer mover for patients including ramps. Connects and disconnects to standard wheelchairs. |
| Hill-Rom/Excel Care® ES Bed hill-rom.com | Bed with adjustable, frame, width and length; patient-assist siderail; scale. | Seat deflates to help patient get in and out of bed more easily. 1,000 lb. capacity. |
| Hill-Rom/Magnum II Patient Care System hill-rom.com | Multi functional bed, chair & transport vehicle. | Capacity up to 800 lbs. |
| Hill-Rom/TotalCare Bariatric Plus System hill-rom.com | Bed with power assisted transport system and FullChair position. | Pulmonary therapies including continuous lateral rotation therapy and percussion/vibration therapy. 500 lb. capacity. |
| Hill-Rom/Triflex™ II Bariatric Bed hill-rom.com | Multiple positions. High-low from 14” to 28.” Independent head and knee positioning. | 1,000 lb. capacity. |
| Invacare/BARPKG750 Invacare.com | Split spring design; battery back-up. Package includes bed and mattress. | Bed ends, half rails standard. 650 lb. capacity |
| KCI/BariAir Therapy System kci1.com | Risk management bed for patients up to 850 lbs. | Continuous lateral turn up to 20 degrees to aid in patient positioning, promote comfort and shift pressure points. |
| KCI/BariKare Patient Care System kci1.com | Pressure reduction bed for acute and extended care settings. | Side rails extend straight out 5.75" on each side, for a total added width of 11.5." Allows patient to exit from front of bed. 850 lb. capacity. |
| KCI/BariMaxx II Therapy System kci1.com | Pressure reduction bed system. | Accommodates up to 1,000 lbs. |
| PHS West/Ergo-Express PTC1 phswest.com | Attendant-driven motorized transport chair. Seat swivels 180 degrees. Flip up, adjustable, removable arms.20 miles per charge (onboard charger). | Power-on-demand for inclines. 750 lb. capacity. |
| Roho Group Dry Flotation Kit therohogroup.com | Mattress conforms so that patients' soft tissue does not deform. | No weight limit, lightweight and portable. |
| SenTech/Stage IV 3000 Plus sentechmedical.com | Multi-zoned, multi-therapy system. | Therapy for patients up to 990 lbs. Air retained in case of power loss. |
| SenTech/Stage IV Millennium 3 Plus sentechmedical.com | Multi-zoned, multi-therapy system. Interactive set up screen and internal instructions. | Therapy for patients up to 995 lbs. Air retained in case of power loss. |
| SenTech/Thera Turn Millennium Plus sentechmedical.com | Multi-zoned, multi-therapy system. Four pre-programmed turning angles, choice of angle of hold. | Therapy for patients up to 750 lbs. Air retained in case of power loss. |
| Sizewise /Bari-Rehab Platform™ sizewise.net | Bed with extra width of 39" and 48". | The weight capacity is 1,000 lbs. Retractable deck available. |
| Sizewise /Bari-Rehab Platform 2™ sizewise.net | Bed with extra width of 39" and 48". | The weight capacity is 1,000 lbs. Retractable deck available. |
| Sizewise/SW Evolution™ sizewise.net | Bariatric low bed with low resting height of 9”. | 850 lb. capacity. |
| Sizewise/SW Low Boy® sizewise.net | Bariatric low bed with low resting height of 7”. | 850 lb. capacity. |
| Sizewise/Mighty Air sizewise.net | Low air loss therapy mattress. | Static and alternating modes; side wall bolster support. 1,000 lb. capacity. |
| Sizewise/SW Pulsate sizewise.net | Low air loss therapy mattress. | Static and pulsating modes; available as mattress or overlay. |
| Sizewise/Shuttle A Series sizewise.net | Transport chair that can be used in chair position or converts to exam/rehab table. | Fully electric. 650 lb. capacity. |
| Sizewise/Shuttle B Series sizewise.net | Transport chair that can be used in chair position or converts to exam/rehab table. | Fully electric. 1,000 lb. capacity. |
| Sizewise/Shuttle B Series LTC sizewise.net | Transport chair that can be used in chair position or converts to exam/rehab table. | Fully electric. 1,000 lb. capacity. |
| Sizewise/Big Turn sizewise.net | Full body lateral rotation mattress with low air loss and pulsation therapies. | 1,000 lb. capacity |
| Span America/PressureGuard CFT spanamerica.com | Constant force therapy, self-adjusting air therapy mattress. | Proven option for prevention in the high risk ICU/CCU environment. 750 lb. capacity. |
| Span America/GeoMattress Atlas spanamerica.com | Mattress designed to meet the unique skin integrity challenges of the bariatric client. | 600 and 750 lb. capacities. |
| Span America/PressureGuard Easy Air® XL Mattress spanamerica.com | Low air loss/alternating pressure mattress system. | Optional pendant controller. 1,000 lb. capacity. |
| Span America/PressureGuard® APM Bariatric Mattress spanamerica.com | Alternating pressure air mattress system. | Digital control unit, 8 comfort settings. 750 lb. capacity. |
| Sten-Barr/SB Bari 600 Bed Frame stenbarr.com | Bed specifically designed to obese patients. | Accommodates up to 600 lbs. |
| Sten-Barr/SB Bari 1000 Bed Frame stenbarr.com | Bed specifically designed to obese patients. | Accommodates up to 1,000 lbs. |
| Stryker/M-1 Roll-In System stryker.com | Ambulance cot with a portable litter. | 500 lb. capacity. |
| Stryker/MX-PRO Bariatric Transport stryker.com | Ambulance cot specifically designed for the bariatric patient. | 1600 or 850 lb. capacity units available. |
| Stryker/MX-PRO Incubator Transport stryker.com | Ambulance cot specifically designed for incubators. | Positive action features for adjusting height help prevent inadvertent drops. |
| Stryker/MX-PRO R3 stryker.com | Ambulance cot. | 650 lb. capacity. |
| TriLine/Bari 10A Bariatric Bed Frame trilinemedical.com | Patient support with pendant control; electronic bed scales. | Battery backup. 1,050 lb. capacity. Width expansion for comfort and transport. |
| TriLine/Barton Chair 330/660 trilinemedical.com | Transportation chair. Tilt in space position converts from chair to stretcher/rehab/care surface. | Adjusts to fit most hospital beds and stretchers. Capacities up to 660 lbs. |
| TriLine/Turn 1000™ Lateral Turning and Low Air Loss Surface trilinemedical.com | Lateral rotation mattress for treatment and prevention of pressure ulcers & respiratory conditions. | “Autofirm” mode for maximum air inflation; 4 turn angles and 4 turn set times. 1,000 lb. capacity. |
| Wy'East Medical/TotalLift PTS Transfer Stretcher wyeastmed.com | Transfer patient from stretcher to bed with no lifting. | 400 lb. capacity. |

## Bariatric Ceiling/Gantry Lifts

| Bariatric Ceiling/Gantry Lifts Product | Description | Key Advantage |
| --- | --- | --- |
| ArjoHuntleigh/Maxi Sky 1000 arjohuntleigh.com | Designed to lift and transfer patients from bed to chair or commode. | 1,000 lb. capacity. |
| BHM Medical/V10 Ceiling Track Lift bhm-medical.com | Ceiling track lift. 1,000 lb. capacity. | Can be installed on a permanent ceiling mounted track or the flexibility of a free standing semi-permanent rack is available. |
| Chiltern Invadex/Patient Handling System chilterninvadex.com.uk | Free-standing gantry system for patient handling. | Multi-functional as bed, chair, or transport vehicle. 800 lb. capacity. |
| Guldmann/G2 Ceiling Hoist System guldmann.net | System is available as small, free-standing, self-sustaining and removable unit or as wall- or ceiling-mounted system. | Available with many motor options and customizable ceiling track configurations. Capacities 1,000 lbs. |
| Guldmann/G3 Ceiling Hoist System guldmann.net | Modular system that can be mounted on ceiling, wall or support pillars. System is available as small, free-standing, self-sustaining and removable unit or as wall- or ceiling-mounted system. | Available with many motor options and customizable ceiling track configurations. Capacities from 200 to 1,100 lbs. |
| Horcher/UniLift horcher.com | Ceiling lift for patient transfers with 500 lb. capacity. Also available is a stretcher system with 440 lb. capacity. | Tracks can be mounted to ceiling or wall. |
| Liko/Ceiling-Mounted Ultra Twin liko.com/na | Bariatric ceiling lift. | Dual motor design facilitates positioning of patient for comfort and security. 1,100 lb. capacity. |
| Liko/Liko Twin Free Span liko.com/na | Bariatric ceiling lift. | Free-standing system that is not affixed to the walls or the ceiling. Dual motor design facilitates positioning of patient for comfort and security. 1,100 lb. capacity. Available with Ultra Twist accessory for 360 degree rotation. |
| Medcare/Bariatric Ceiling Lift medcarelifts.com | Single motor/single track ceiling lift system. | 1,000 lb. capacity |
| Polaris Medical / Wispa Bariatric polarismedicalinc.com | Bariatric ceiling lift | 1,000 lb. capacity |
| T.H.E Medical/Pinnacle High Capacity themedical.com | Heavy duty fixed lift w/an extra wide, extra long lifting strap for safety. Includes dual controls, emergency down, curtain pass-through & standardized face plate. | 800 or 1,000 lb. capacity. Infection control strap. |
| Waverley Glen/Transactive Plus waverleyglen.com | Heavy-duty ceiling track system. | Lifts up to 800 lbs. |
| Sizewise/Bari-Lift and Transfer: Overhead sizewise.net | Gantry system that is not mounted in the ceiling or wall. Designed to help patients out of bed and into a wheelchair or shower/commode chair. | Available in 750 or 1,000 lb. versions. |
| Sten-Barr/Assist Lift 1000 stenbarr.com | Gantry system that is not mounted in the ceiling or wall. Designed to help patients out of bed and into a wheelchair or shower/commode chair. | 1,000 lb. capacity. |

## Bariatric Lateral Transfer Systems					 TOP

| Bariatric Lateral Transfer Systems Mfg./Product | Description | Key Advantage |
| --- | --- | --- |
| Hill-Rom/Airpal Patient Air Lift hill-rom.com | Cushioned film of air transfers patient from bed to stretcher in a variety of care settings. | 1,200 lb. capacity. |
| Hovermatt/Hovertech International hovermatt.com | A waterproof, anti-bacterial anti-stain, nylon mattress that allows for lateral transfers of patients. | No weight limit. Available in single-patient use as well as split-leg for use in the lithotomy position or half-matt for use during cystology and ob/gyn procedures. |
| HoverJack /Hovertech International hovermatt.com | Lifts patient from floor to bed or stretcher height. | No weight limit. |
| Polaris Medical / Lateral Assist polarismedicalinc.com | Strong padded transfer board with nylon cover the transfers from one surface to another | 625 weight capacity. Two models available. EMS/EMT version available |
| Smart Medical Technology/Liftaem smartmedtechnology.com | Air-assisted device to assist with lateral patient transfers and patient repositioning. | Can remain on mattress during procedures such as x-rays, CT scans, etc. 1,000 lb. capacity. Single patient use (disposable). Fully-contained cart with blower and storage compartment. |
| Sizewise/SW Air Transfer sizewise.net | Air-assisted device to assist with lateral patient transfers and patient repositioning. | Capacity exceeding 1,000 lbs. |
| Wy'East Medical/TotalLift PTS Transfer Stretcher wyeastmed.com | Transfer patient from stretcher to bed with no lifting. | 400 lb. capacity. |

## Bariatric Repositioning Systems						 TOP

| Bariatric Repositioning Systems Mfg./Product | Description | Key Advantage |
| --- | --- | --- |
| ArjoHuntleigh/Maxi Slide arjohuntleigh.com | Sliding system that offers a wide range of patient handling maneuvers. | Easy to place and remove from under the patient. |
| ErgoSafe/ErgoSlide® 2101 ergosafe-products.com | Assists caregivers in repositioning a patient in bed while reducing the risk of injury to the caregiver by not lifting, but sliding the patient. | Using ergonomically-inclined handles for gripping, caregivers can maneuver patients to the desired position without having to lift. |
| EZ Way/Repositioner ezlifts.com | This repositioner is a crank-powered device, made of vinyl, which allows caregivers to move patients up and down in bed. | Allows one person to facilitate repositioning. |
| Liko/Liko Sheet & Multi Strap liko.com | Reposition using ceiling lifts. | Can be left under patient. |
| McAuley Medical/AirSlide mcauleymedical.com | Allows staff to laterally transfer patients on a comfortable cushion of air without lifting. | No weight limit; small, lightweight and portable; Ergonomic Extension Handles included. |
| Polaris Medical / Polaris Slides polarismedicalinc.com | Sliding systems that offer a wide range of patient handling maneuvers. | Bariatric sheets and non-slide sheets. All ways slide combines all. |
| SenTech/Thera Turn Millennium Plus sentechmedical.com | Multi-zoned, multi-therapy system. Four pre-programmed turning angles, choice of angle of hold. | Therapy for patients up to 750 lbs. Air retained in case of power loss. |
| Sizewise/SW Air Transfer sizewise.net | Air-assisted device to assist with lateral patient transfers and patient repositioning. | Capacity exceeding 1,000 lbs. |
| Waverley Glen/Repositioning Systems waverleyglen.com | Range of sliding sheets, transfer boards, and repositioning aids. | Allows for safe transfer and repositioning for both bariatric patient and caregiver. |

## Bariatric Powered Lifts							 TOP

| Bariatric Powered Lifts Mfg./Product | Description | Key Advantage |
| --- | --- | --- |
| Andermac/Anderlift Lift and Transport System anderlift.com | When lifted, transported, weighed or bathed, the patient is resting on a "hammock" created by a custom sheet and nestled in the frame. | Four pieces of equipment in one - a bather, bed scale, transportation system and power patient lift. Up to 800 lb. capacity. |
| AryCare/C1000 Lift arycare.com | Unit lifts and transfers; remains over bed. | Lifts, repositions or transfers patients to wheelchair. 1,000 lb. capacity |
| ArjoHuntleigh/Tenor Lift arjohuntleigh.com | Patient lift. | Weight capacity of 704 lbs. (320kg) giving increased flexibility to handle heavier patients with safety and security. |
| BHM/ErgoLift 600 bhm-medical.com | Floor-based lift. 600 lb. capacity. | Motorized, secure base. Quick access control box. |
| ConvaQuip/2000x Lift convaquip.com | Transfers those patients needing help from chair to bed or toilet. | Electronic anti-entrapment system and 600 lb lifting capacity. |
| ConvaQuip/9750 Lift convaquip.com | Transfers those patients needing help from chair to bed or toilet. | 1,000 lb lifting capacity. |
| Hill-Rom/Liftem Mark IV Portable Lift hill-rom.com | Patient transferlift with remote control operation for all functions. | No physical movement of the lift required to facilitate the transfer. 700 lb. capacity. |
| Invacare/Reliant 600 invacare.com | High lift range allows transfer to high surfaces. Base legs adjust and lock. | 600 lb. capacity. Manual lowering if power lost. Available with standard or powered base. |
| Liko/Viking XL Mobile Lift liko.com/na | Mobile floor lift. | General purpose bariatric lift with numerous options and accessories for bariatric applications. 550 and 660 lb. capacities available. |
| Polaris Medical / King 550/660 polarismedicalinc.com | Mobile floor lift. | Weight capacities up to 660lbs |
| RoMedic/Eva 600EE romedic.com | Mobile lift with electrical base width adjustment. | 600 lb. capacity. |
| Sizewise/Bari Lift & Transfer sizewise.net | Floor-based lift to help patients out of bed into a wheelchair or shower/commode chair. | Weight capacity is 750 lbs. and 1,000 lbs. Height adjustable from 78” to 94.” Scale and variety of slings available. |
| Sten-Barr/SB Mobile Lift 600 stenbarr.com | Easy to use mobile lift helps transfer patient from bed to wheel chair or commode. | Weight capacity of 600 lbs. |
| THE Medical/Titan-X themedical.com | Electric/hydraulic power system for patient transfers. | 1000 lb. lifting capacity. |
| TriLine/Bariatric Lift trilinemedical.com | Floor-based lift. | 600 lb. capacity. |
| Vancare/Vanderlift II B450 vancare.com | Patient lift. 4-pt. sling connection. | Emergency lowering. 450 lb. capacity |
| Vancare/Vanderlift II B600 vancare.com | Patient lifting system. 4-pt. sling connection. | Emergency lowering. 600 lb. capacity. |

## Bariatric Chairs (Transfer/Dependency/Wheeled and Cushions)

| Bariatric Transfer/Dependency Chairs Mfg./Product | Description | Key Advantage |
| --- | --- | --- |
| 21st Century/Big Bounder H-Frame Powered Wheelchair 21stcenturyscientific.com | Provides transportation. | Power seating systems: recline, tilt, seat elevate, and/or power headrest. Available in up to 1,000 lb. capacity. |
| 21st Century/Big Bounder Power Wheelchair 700 21stcenturyscientific.com | Provides transportation. | Accommodates 21” or wider seats. |
| 21st Century/Bounder Plus Conventional Wheelchair 21stcenturyscientific.com | Provides transportation. | Accommodates users up to 500 lbs. |
| Blue Chip Medical/Gel Pro® Elite Wheelchair Cushion bluechipmedical.com | A wheelchair cushion used for pressure relief. | Stabilizes and reduces rocking motion. |
| Broda/Chairs and Recliners seatingisbelieving.com | Custom seating; chairs, recliner chairs. | Accommodates up to 900 lbs. |
| Burke/Scout BOSS 6 Series burkebariatric.com | Powered wheelchairs with various options. | Up to 675 lb. capacity. |
| Camtec/CareChair camtecproducts.com | Infinite positioning adjustments. Foot and leg rests independently adjustable. | Converts from chair to rehab/care transport surface. Various capacities available |
| ConvaQuip/Wheelchairs convaquip.com | Powered transport/wheelchairs. | Electronic braking prevents the chair from moving when the power is off or the joystick is in the neutral position. 600 to 660 lb. capacity. |
| Hill-Rom/Art of Care Large Capacity Chair hill-rom.com | Chair with 30” or 40” seat widths available. | 750 lb. capacity |
| Hill-Rom/Barton I-660 Transfer Chair hill-rom.com | Converts from a flat transfer chair to an upright mobile chair. Adjustable head, foot and arm rest. | Allows a single caregiver to laterally transfer patients. 660 lb. capacity. |
| Hill-Rom/Wheelchair hill-rom.com | Patient transport. | Anti-tip mechanism, accommodates up to 800 lbs. |
| Invacare/Electric Wheelchairs/Transport Chairs invacare.com | Variety of chairs available | Up to 700 lb. capacity. |
| Invacare/MVP Wheelchair invacare.com | Custom wheelchair. | Adjustable angle back option on the square frame offers four discrete positions. |
| Invacare/Solara Wheelchair invacare.com | Tilt-in-space manual transport for bariatric residents. | A built-in 2" drop base facilitates remarkably low seat-to-floor heights. |
| Martin Innovations/Chair-A-Table®/Wheelchair martininnovations.com | An examination table with a proprietary coupling wheelchair | Allows a single caregiver to transfer 1000+ lbs. to an examination table with no lifting |
| MedLine/Shuttle Heavy Duty Wheelchair medline.com | Transport with wide seats to accommodate 600 lbs. | Back and seat have polypropylene mesh liner to support additional weight. |
| Polaris Medical / Heavy Duty Wheelchair polarismedicalinc.com | Transport with wide seats to accommodate 1000 lbs. | Many varieties available. |
| Pride Mobility/Quantum 600XL pridemobility.com | Power wheelchair for patient transport. Enhanced climbing and transitioning capability for rough terrain. | 400 lb. capacity. |
| Pride Mobility/Quantum 1420 pridemobility.com | Power wheelchair for patient transport. Midwheel drive design | 500 lb. capacity. |
| Pride Mobility/Quantum 1424 pridemobility.com | Power wheelchair for patient transport. Midwheel drive design | 450 lb. capacity. |
| Pride Mobility/Quantum 1650 pridemobility.com | Power wheelchair for patient transport. Midwheel drive design | 650 lb. capacity. |
| Pride Mobility/Quantum 6000Z HD pridemobility.com | Power wheelchair for patient transport. Enhanced climbing and transitioning capability for rough terrain. | 450 lb. capacity. |
| Rehab Seating/Ascender Chair rehabseating.com | Chair with foot rests and arm rests. | Accommodate 300, 500, and 1000 lb. capacity. |
| Sentech/Chair sentechmedical.com | Converts any chair to an alternating pressure relief system. | Therapy and support for patients weighing up to 550 lbs. Models for bedside or mobile applications. |
| Sentech/Recliner sentechmedical.com | Six multiple positions including deep recline; feet can be elevated higher than head. | Therapy and support for patients weighing up to 550 lbs. |
| Sizewise/Bari-Chair sizewise.net | Wheelchair with removable arm rests and independently adjustable leg rests. | 750 lb. capacity. Available in 24” and 36” wide models. |
| Sizewise/Bari-Recliner sizewise.net | Wheelchair with power recline feature, removable arm rests and independently adjustable leg rests. | 750 lb. capacity. Available in 26” and 30” wide models. |
| Sizewise/SW Bedside Recliner sizewise.net | Patient chair with footrest and multiple reclining positions. | 1,000 lb. capacity. Moisture barrier feature on the seat and back. |
| Sizewise/Shuttle A Series sizewise.net | Transport chair that can be used in chair position or converts to exam/rehab table. | Fully electric. 650 lb. capacity. |
| Sizewise/Shuttle B Series sizewise.net | Transport chair that can be used in chair position or converts to exam/rehab table. | Fully electric. 1,000 lb. capacity. |
| Sizewise/Shuttle B Series LTC sizewise.net | Transport chair that can be used in chair position or converts to exam/rehab table. | Fully electric. 1,000 lb. capacity. |
| Sten-Barr/Bariatric Wheel Chair stenbarr.com | Specifically designed to obese patients. | Accommodates up to 750 lbs. |
| Sunrise Medical/Jay Plus Back sunrisemedical.com | Specifically designed to address the pelvic and trunk stability of bariatric patients. | Weight capacity of 650 lbs. |
| Sunrise Medical/Jay Plus Cushion sunrisemedical.com | A wheelchair cushion that can be customized to address the posture and skin protection needs of the bariatric client. | Weight capacity of 650 lbs. |
| TriLine/Barton Chair 330/660 trilinemedical.com | Transportation chair. Tilt in space position converts from chair to stretcher/rehab/care surface. | Adjusts to fit most hospital beds and stretchers. Capacities up to 660 lbs. |
| TriLine/Bari Recliner trilinemedical.com | Wheeled chair with power recline feature. | 600 lb. capacity, 26” wide. |
| TriLine/Bari Recliner Wheelchair trilinemedical.com | Manual base with lower recline. Removable reclining full length arms and headrest extension. Elevating leg rests; up to 45 recline. | 600 lb. capacity. |
| TriLine/Bari Wheelchair trilinemedical.com | Manual bariatric wheelchair. | 600 lb. capacity. |

## Beds by Care Setting							 TOP

### Homecare/Long Term Care Beds

### Med/Surg Beds

This guide is a resource to assist you in choosing the type of bed you need for your particular area of care. It is by no means a complete list. You should do your own research and contact manufacturers for current models and specifications as they may have changed since our last update.

Beds can be purchased in many different configurations. Frames, various siderail configurations, headboards, footboards, mattresses and many accessories can be purchased separately or together depending upon your needs. Specifications included are for the widest and longest sizes available, for other height and width options see the manufacturers’ web sites.

We have not addressed the various types of mattresses available in the marketplace, but depending upon your patient’s needs, you can get standard, foam, and specialty surfaces (such as air-assisted pressure relief or treatment surfaces). Some of these mattress options also feature anti-microbial covers. It is important to remember that all beds have different deck dimensions and that not all mattresses fit all beds. Please consult the manufacturer to assure that you are using the correct size mattress for your bed.

Available options can include:

- Exit alarm

- Foot board at end of frame to keep patient from sliding down

- Ability to adjust the height of the bed to a comfortable and safe working position for the caregiver as well as to lower the bed to the floor for patients at risk for falls.

- Power assist to use as transport device

- Scale

- Turn assist

- Chair position

## Homecare/Long Term Care Beds					 TOP

Homecare beds are generally manufactured for ease of use by home caregivers or for long-term care settings. The head and footboards are usually available in wood-grain laminates to be more aesthetically-pleasing.

| Manufacturer | Product | Features | Weight Capacity | Specifications |
| --- | --- | --- | --- | --- |
| Adden Furniture, Inc. | Electric Health Bed | 3 motors; 2-position vascular footrest; 6-IV locations; Rails available in Full, ¾ and ½ lengths; Grid deck. | 500 lbs. | - Sleep surface: 35” x 80” |
| Adden Furniture, Inc. | Manual Health Bed | 3-crank; Rails available in Full, ¾ and ½ lengths; Grid deck. | 500 lbs. | - Sleep surface: 35” x 80” |
| Basic American Metal Products | WhisperLite™ II Semi-Electric Homecare Bed (WL0208) | Quiet, smooth operation; Lightweight | 450 lbs. | - Bed Height: Min. 15", Max. 24" (at deck level). - Overall Bed Size: 87" L x 36" W. |
| Basic American Metal Products | Bariatric Bed (ABL-B600) | Full Electric 3-function operation; Simple assembly, no special tools required | 600 lbs. | - Bed height: low 17" to high 27". - Overall bed dimensions: 46.5" width (with rails) and 87 length. - Sleeping surface dimensions: 42" width and 80" length. |
| Basic American Metal Products | Universal Homecare Bed (SD0204A) | Slip clutches to eliminate unsynchronized components | 450 lbs. | - Sleep surface 36”x80” |
| Carroll Hospital Group | Spirit | Low resting height of 8.75;” Fiber composite deck detaches into four sections for thorough sanitization; Trendelenburg, reverse Trendelenburg, chair positioning, CPR, and auto contour functions. | 500 lbs. | - Sleep surface: 36” x 80” - Overall frame: 35.5” x 91.5” - High position: 34” - Low position: 8.75” |
| Graham-Field Health Products | Eze-lok Bed | Single pedal to secure bed to the floor at any height; 12” low deck height; Manual, Semi-Electric or Full-Electric | 450 lbs. | - 76”-84” lengths - 12” low deck height |
| Graham-Field Health Products | Zenith Bed | Steel-wire grid or pan sleep deck; 7.9” to 30” deck heights | 500 lbs. | - 76” or 80” length - 7.9” to 30” deck height |
| Hill-Rom USA, Inc. | Resident LTC Bed | Footend control lockout panel and floor brake; separate articulation of head and knee areas. | 300 lbs. | - Size: 42”x91.5” - Deck height range: 13.875-30” - Sleep deck: 36” x 80” |
| Hill-Rom USA, Inc. | Low Bed | Restraint-Free Assist Rail; Height adjustable; Attendant and patient-accessible controls with lock-out. | 500 lbs. | - Sleep surface: 35” x 80” - Overall surface: 41” x 89.5” - Low position: 6.75” - High position: 33” |
| Invacare Continuing Care Group | ARRO Bed | Detachable mattress deck; 14-function attendant control panel; Restraint-free assist rail; Immobilization system | 500 lbs. | - Sleep surface: 35” x 80” - Overall surface: 41” x 89.5” - High position: 33” - Low position: 6.75” |
| Invacare Continuing Care Group | Echo | Detachable mattress deck; Weightless tuck-away siderails; Restraint-free Carroll Assist Rail; Hand controls | 450 lbs. | - Sleep surface: 76” or 80” length - Overall: 41” x 84” or 88” |
| Invacare Continuing Care Group | SC900DLX | Mattress deck available in perforated pan or grid styles; Built-in wall protection; Restraint-free assist rails; Hand controls | 450 lbs. | - Sleep surface: 38” x 76”/80” - Overall: 36” x 80”/84” |
| Invacare Continuing Care Group | Solo | Detachable mattress deck, 9-function attendant control panel; Restraint-free Carroll Assist Rail; 6-function hand control | 400 lbs. | - Sleep surface: 35” x 76”/80” - Overall: 41” x 80”/84” |
| Invacare Continuing Care Group | 820DLX Series | Built-in wall protection; 6-function hand control; Electrical or manual operations; Restraint-free Carroll Assist Rail; Mattress deck available in perforated pan or grid style. | 450 lbs. | - Overall: 36” x 82” - Sleep surface width: 76” or 80” - High position: 26” - Low position: 13.5” |
| Invacare Continuing Care Group | SoloPro | Mobility at any bed height; Restraint-free Carroll Assist Rail; 6-function hand control; Detachable mattress deck. | 400 lbs. | - Overall: 41” x 84” or 88” - Sleep surface width: 76” or 80” - High position: 25” - Low position: 9” |
| Joerns Healthcare Inc. | Ultra Care Bed 770 | EasyLift leg lift; Four-section sleep surface; staff control; two-position assist handle. | 500 lbs. | - Overall: 35” x 80” - Travel range: 7-30” |
| Joerns Healthcare Inc. | Ultra Care Bed 790 | Four motors; 4-section sleep surface; staff control; two position assist handle; rolls at any height. | 500 lbs. | - Overall: 35” x 80” - Travel range: 9-30” |
| Joerns Healthcare Inc. | Ultra Care Bed 795 | Four motors; 4-section sleep section; staff control; 2-position assist handle; rolls at any height. | 500 lbs. | - Overall: 35” x 80” - Travel range: 9-30” |
| Joerns Healthcare Inc. | Ultra Care Bed 530 | 4-section sleep surface; staff control. | 500 lbs. | - Overall: 35” x 80” - Travel range: 8 7/8” x 32” |
| Joerns Healthcare Inc. | Ultra Care Bed 330 | 4-section sleep surface; staff control. | 450 lbs. | - Overall: 35” x 80” |
| NOA Medical Industries, Inc. | NOA Vertical Riser | Raise & lower with the wheels locked; Braking swivel casters. | 500 lbs. | - Overall: 40’ x 89” - Deck height range: 7”-26” - Sleep deck: 84” |
| NOA Medical Industries, Inc. | NOA AP Riser | Descends to just 7” from the floor, rises to convenient working height; Multiple positions for therapeutic & emergency use. | 500 lbs. | - Overall: 40” x 89” - Deck height range: 7”-26” - Sleep deck” 84” |
| NOA Medical Industries, Inc. | NOA Manual Resident | Manual; Three cranks stow away for safety | 500 lbs. | - Overall length: 40” x 89” - Deck height range: 12”-23” |
| NOA Medical Industries, Inc. | NOA Electric Resident | Electric; Rib deck sleeping surface; Central locking system; Head deck, foot deck & height controlled by 3 full-function hospital-grade motors. | 500 lbs. | - Overall length: 40” x 89” - Deck height range: 12”-23” |
| NOA Medical Industries, Inc. | NOA Onyx Bed | Dementia, Alzheimer’s, Restraint-Free and Psychiatric Settings; No edges for safety. | 450 lbs. | - Width (without rails): 34.5” - Length: 80” - 3” min. height w/o casters. - 7” min height w/casters |
| Stryker Medical | LTC Bed | Interchangeable caster & stationary bed leg system; zero-transfer gap with siderails are down and tucked under bed; Chaperone® bed exit system optional. | 350 lbs. | - Overall: 42” x 87” - Low position: 12” - High position: 28” - Patient surface: 35” x 80” |
| Sunrise Medical | Guardian® Full-Electric Bed | 7 models available; Split-spring design; Hand pendant control | 450 lbs. | - Overall: 36” x 88” - Surface height range: 16-24.5” |
| Sunrise Medical | SunTec Quick-Change Full Electric Bed | Patient/caregiver hand control; Split-spring design; Changes from manual to semi-electric or full electric without the use of tools. | 500 lbs. | - Overall: 35” x 85.75” - Mattress deck: 35” x 80” |
| Sunrise Medical | SunTec Grid-Style Full Electric Shaft Bed | Grid sleeping surface; emergency crank clips to bed for ready access. | 500 lbs. | - Overall: 36” x 87.75” - Mattress deck: 36” x 80” |
| Sunrise Medical | SunTec Spring-Style Full Electric Shaft Bed | Split spring; Hand control; Self-leveling system avoids out-of-sync operation. | 500 lbs. | - Overall: 36” x 88.5” - Mattress deck: 36” x 80” |
| Sunrise Medical | SunTec Vantage Full Electric Bed | Split spring design; Longer head section for patient comfort; Knee operation only in the safe, low-bed height position. | 500 lbs. | - Overall: 35” x 85.75” - Mattress deck: 35” x 80” |

## Med/Surg Beds									 TOP

| Manufacturer | Product | Features | Weight Capacity | Specifications |
| --- | --- | --- | --- | --- |
| Carroll Hospital Group | Sprit Plus | Low resting height of 8.75;” Fiber composite deck completely detaches for thorough sanitization; Trendelenburg, reverse Trendelenburg, chair positioning, CPR, and auto contour functions. | 500 lbs. | - Sleep surface: 36” x 80” - Overall frame: 35.5” x 93.5” - High position: 34” - Low position: 8.75” |
| Hill-Rom USA, Inc. | Century + Bed | Optional battery back-up; Addresses entrapment area issues; Siderails with embedded nurse and patient controls on both sides; Patient lockout controls. | 500 lbs. | - Sleep surface: 36” x 80” - Overall frame: 36” x 88.5” - High position: 29.875” - Low position: 18.125” |
| Hill-Rom USA, Inc. | VersaCare® Bed System | Low height, minimized siderail gaps; LowChair® position; Firm surface for patient transfers, optional IntelliDrive® power-assisted transport. | 500 lbs. | - Sleep surface: 35.5” x 86” - Overall frame: 40” x 94.5” - High position: 37” - Low position: 18” |
| Hill-Rom USA, Inc. | CareAssist® ES Bed | Angle indicators for degree of head section and Trendelenburg positions; Instant CPR release; One-button Dining Chair® position; Built in scale; Nightlight. | 500 lbs. | - Sleep deck: 36” x 84” - Frame: 39.74” x 94.04” - High position: 32.5” - Low position: 15.75” |
| Stryker Medical | GoBed II® | Chaperone® bed exit system; Zone Control® technology; Integrated pump holder on footboard; Low height; Centrally-located 5th wheel to assist in transport. | 500 lbs. | - Overall: 40” x 94.25” - High position: 29” - Low position 14.5” - Patient surface: 36” x 84” |
| Stryker Medical | Electric & Manual Beds | Centrally-located steer system; Optional removable crank handles for manual bed positioning. | 500 lbs. | - Overall: 41.5” x 90.5” - High position: 30” - Low position: 13” - Patient surface: 35” x 80” |
| Stryker Medical | Secure II | Chaperone® bed exit system; Zone Control® technology; Single-button in-bed scale; Retractable bed frame; iBed Awareness option to monitor brake, siderail position, bed exit and low bed height. | 500 lbs. | - Overall: 42.5” x 93” - High position: 30” - Low position: 16” - Patient surface: 35” x 84” |

## Equipment by Task								 TOP

## Algorithms 1: Transfer to and From: Bed to Chair, Chair to Toilet, Chair to Chair, or Car to Chair

### Beds

### Car Lifts/Vehicle Extraction

### Ceiling Lifts

### Dependency/Geri/Specialty Chairs/Wheelchairs

### Floor-Based Lifts

### Friction Reducing Lateral Aids

### Repositioning Devices

### Sliding Boards

### Slings

### Standing Assist Aids

## Algorithm 2: Lateral Transfer to and from: Bed to Stretcher, Trolley

### Air Assisted Lateral Transfer Aids

### Beds/Transport

### Ceiling Lifts

### Dependency/Geri/Specialty Chairs/Wheelchairs/Seated Transport

### Friction Reducing Lateral Aids

### Mechanical Lateral Aids

### Slings

## Algorithm 3: Transfer to and from: Chair to Stretcher or Chair to Exam Table

### Ceiling Lifts

### Dependency/Geri/Specialty Chairs/Wheelchairs/Seated Transport

### Floor-Based Lifts

### Slings

### Standing Assist Aids

## Algorithm 4: Reposition in Bed: Side-to-Side, Up in Bed

### Beds

### Ceiling Lifts

### Floor-Based Lifts

### Friction Reducing Lateral Aids

### Repositioning Devices

### Slings

## Algorithm 5: Reposition in Chair: Wheelchair and Dependency Chair

### Ceiling Lifts

### Dependency/Geri/Specialty Chairs/Wheelchairs/Seated Transport

### Floor-Based Lifts

### Repositioning Devices

### Slings

### Standing Assist Aids

## Algorithm 6: Transfer Patient Up from Floor

### Floor-Based Lifts

### Slings

## Slings Guide									 TOP

These tables are guides for assisting you in seeing the variety of sling types available from various manufacturers. Please go to the manufacturer's web site for the latest information and specifications (see list at end). You can view sling types by vendor, or sling types by function/task.

## Sling Types by Vendor

| Vendor | Sling Name | Details | Material Used | Sizes Available |
| --- | --- | --- | --- | --- |
| Alpha Modalities, LLC | Am-Seat Sling | Reusable, fullback seated transfer sling; center and side handles; peri-area open; dignity strap; can be used with bilateral amputee. | Reusable: SWL 500 kg. | S, M, L, X, XX Color coded |
|  | AM-TSL | Reusable, full length log roll/repositioning sling |  | 660 lb. (300 kg.) or 1000 lb. (454 kg.) |
|  | AM-BariVest | Bariatric reusable walking sling; patient must bear weight; designed for fall prevention. |  | 400-600 lb. (181-272 kg.) or 600-800 lb. (272-362 kg.) |
|  | AM-LingSling | Holds limbs while doing dressing change or peri-care; closed cell foam padding; can also be used to hold patient on side for back care. |  | 60” x 8” long |
| ARJO | General purpose unpadded sling | Has head support, used with Maxilift, Dextra, Bianca, Trixie, Maxi Move, Maxi Twin, Maxi 500, Maxi Sky, Maxi Lite, Opera, Tempo and Bravo. | 100% Standard polyester, latex free; clips & loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | General purpose padded sling | Has reinforced, padded leg pieces for comfort, used with the above lifts. | 100% Standard polyester, spacer fabric, latex free; clips & loops. | XS, S, M, L, XL Color coded |
|  | Mesh unpadded sling | Bathing/showering sling for use with the above lifts. | 100% Standard polyester mesh, latex free; clips & loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | Mesh padded sling | Bathing/showing sling with padded leg pieces for comfort, used with the above lifts. | 100% Standard polyester mesh, spacer fabric, latex free; clips & loops | XS, S, M, L, LL, XL Color coded |
|  | Mesh unpadded sling with extended leg (+100 mm) | Bathing/showing sling with additional fabric on thigh sections for use with the above lifts. | 100% Standard polyester, latex free; clips & loops | M, L, LL, XL Color coded |
|  | Extended leg padded sling (+100mm) | For users who require additional fabric and padded support around the thigh area. For use with the above lifts. | 100% Standard polyester, latex free, clips & loops | S, M, L, LL, XL, XXL Color coded |
|  | Comfort deluxe sling | Provides additional comfort in pressure areas such as the legs and shoulders. For use with the above lifts. | 100% Polyester, spacer fabric, latex free; clips & loops | XS, S, M, L., LL, XL, XXL Color coded |
|  | Toilet sling with head support | Toileting, built-in head support with extra padding under arms. | 100% Polyester, spacer fabric, latex free; clips & loops | XS, S, M, L, LL, XL Color coded |
|  | Pediatric | Special elephant pattern fabric and adjustable head support. | 100% Standard polyester, spacer fabric, latex free, light blue with printed elephant images; clips & loops | XXS, XS, S Color coded |
|  | Right leg single amputee sling | Suitable for above- or below-knee or total hip amputee patients | 100% Standard polyester, latex free; clips & loops | M, L, LL, XL Color coded |
|  | Left leg single amputee sling | Suitable for above- or below-knee or total hip amputee patients. | 100% Standard polyester, latex free; clips & loops | M, L, LL, XL Color coded |
|  | Double amputee sling | Suitable for above-knee or total hip double amputee patients. | 100% Standard polyester, latex free; clips & loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | Double amputee sling with commode aperture | Commode sling suitable for above-knee or total hip double amputee patients. | 100% Standard polyester, latex free; clips & loops | S, M, L, LL, XL, XXL Color coded |
|  | Double amputee mesh sling | Suitable for bathing/showing patients who are above-knee or total hip double amputees. | 100% Standard polyester mesh, latex free; clips & loops | XS, S, M, L, LL, XL Color coded |
|  | Double amputee mesh sling with commode aperture | Commode sling suitable for bathing/showering patients who are above-knee or total hip double amputees. | 100% Standard polyester mesh, latex free; clips & loops | S, M, L, LL, XL Color coded |
|  | Amputee sling with Velcro flap | Suitable for above- or below-knee double amputee users and is applied like a normal sling (without need for log rolling) | 100% Standard polyester, latex free; clips & loops | XS, S, M, L, LL, XL Color coded |
|  | General purpose all day sling | Extra thin fabric with wicking action to prevent moisture build-up. | 100% Polyester all-day fabric, latex free; clips & loops | XS, S, M, L, LL Color coded |
|  | Double amputee all day sling | Extra thin fabric with wicking action to prevent moisture build-up. | 100% Polyester all-day fabric, latex free; clips & loops | XS, S, M, L, LL Color coded |
|  | General purpose all day mesh unpadded sling | Flat mesh can be left under the user or used for bathing/showering. | 100% Polyester all day mesh fabric, latex free; clips & loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | Double amputee all day mesh sling | Flat mesh can be left under the user or used for bathing/showering. Suitable for patients who are above-knee or total hip double amputees | 100% Polyester all day mesh fabric, latex free; clips & loops. | XS, S, M, L, LL Color coded |
|  | Bariatric loop deluxe padded sling | Reinforced material and stitching in the most stressed areas with extra padding. For use with Tenor and Maxi Sky1000 lifts. | 100% Standard polyester, spacer fabric, latex free; loops only. | M, L, XL, XXL Color coded |
|  | Bariatric loop deluxe divided leg hammock sling | Reinforced material and stitching in the most stressed areas with extra padding and divided leg often preferred with Tenor and Maxi Sky1000 lifts. | 100% Standard polyester, spacer fabric, latex free; loops only | M, L, XL, XXL Color coded |
|  | Bariatric loop basic padded sling | Standard sling with padding in the legs. For use with the Tenor and Maxi Sky1000 lifts. | 100% Standard polyester, latex free; loops only | M, L, XL, XXL Color coded |
|  | Bariatric loop basic hammock sling | Reinforced material and stitching in the most stressed areas with extra padding. For use with Tenor and Maxi Sky1000 lifts. | 100% Standard polyester, latex free; loops only | M, L, XL, XXL Color coded |
| Guldmann Basic Slings | Basic | Supports whole body excluding head (goes up to neck). For bed to wheelchair transfers. |  |  |
|  | Low | Supports body up to shoulder height (goes under armpits level). For chair-bed transfers. |  |  |
|  | High | Supports whole body including head |  |  |
| Guldmann Active Slings | Micro | Supports upper body & upper thighs | Polyester | Kids, XS, S, M, L, XL, XXL |
|  | Micro Plus | Supports upper body, side & chest, upper thigh | Polyester | Kids, XS, S, M, L, XL, XXL |
|  | Trainer | Supports chest & shoulder areas, detachable leg straps | Polyester | 10-14, XS, S, M, L, XL, XXL |
| Guldmann Custom Slings | Sit-on | Supports thigh bone & higher on back for persons sitting in sling for a long time. | Net |  |
|  | Sit-on high (not suitable for patients with pressure sores) | Supports thigh bone & higher on back for persons sitting in sling for a long time. | Net |  |
|  | Amputee | Supports whole body up to shoulder height & pelvis region | Polyester | S, M, L |
| Horcher | UPG-1. 2 piece quick sling (1 for thoracic, other for both legs) | Wheelchair users | Slip resistant aerotex, mesh or fleece cover | M |
| Horcher Standard Slings | 3 piece quick sling (1 for thoracic, 2 pieces for each leg) | Good for toileting | Mesh/aerotex, fleece cover | M |
|  | Universal Sling | Transfers, bathing, toileting. No head support. | Mesh, nylon, parasilk, padded fabric | XS, S, M, L, XL |
|  | Universal with head support | Same as above but with head support. | Mesh, nylon, parasilk, padded fabric, head support. | XS, S, M, L, XL |
|  | Double amputee sling | Full bottomed sling | Mesh, nylon, parasilk | S, M, L |
| Horcher Comfort Slings | Double amputee sling with head support | Full bottomed sling with head support | Nylon, parasilk, padded fabric, head support, clips | M, L |
|  | Lexa Sling | Head support that cradles body | Mash, nylon, parasilk, padded fabric, head support, clips | XS, S, M, L, XL |
|  | Toileting sling with head support | Open bottomed sling | Nylon, padded fabric, head support, clips | XS, S, M, L, XL |
|  | Toileting sling without head support | Open bottomed sling | Mesh, padded fabric, loops, clips | M, L, XL |
| Horcher Standing Aid Sling | Raisa corset sling | Goes around thoracic | Nylon, Aerotex, clips | M, L, XL |
| Molift Inc. | Molift Easy Sling | Padded body and legs. Can be ordered with or without head support. | Polymide material (Cordua), with polyester straps, padded with polyester foam. | XXS, XS, S, M, L, M, XL, XXL Color coded |
|  | Molift Easy Toilet Sling | Padded body and legs; two buckles on waist band; anti-slip surface. Can be ordered with or without head support. | Polymide material (Cordua), with polyester straps, padded with polyester foam. | XXS, XS, S, M, L, M, XL, XXL Color coded |
|  | Molift Easy Amputee Sling | Firm padding; two buckles on waist; use for single or bilateral amputation. Can be ordered with or without head support. | Polymide material (Cordua), with polyester straps, padded with polyester foam. | XXS, XS, S, M, L, M, XL, XXL Color coded |
|  | Molift Basic Bathing Sling | Mesh polyester; same basic layout as Molift Easy sling; multiple positioning loops for adjustment. Can be ordered with or without head support. | Mesh Polymide material (Cordua) with polyester straps. | XXS, XS, S, M, L, M, XL, XXL Color coded |
|  | Molift Ambulating Vest | Firm padding on body and legs; twin-belt system; professional mountain climbing straps with a high waist combination with padded groin straps (also suitable for toilet situations). | Polymide material (Cordua), with polyester straps, padded with polyester foam. | S, M, L Color coded |
|  | Molift Easy Raiser Sling | Firmly padded body for active lifts; anti-slip surface. | Polymide material (Cordua), with polyester straps, padded with polyester foam. |  |
| Polaris Medical | Disposable Sling | Has head support, used with a variety of patient lifts | Non-woven blend | XS, S, M, L, LL, XL, XXL Color coded |
| Polaris Medical | Non Slip Standing Aid Sling | Non slip back buckle fastening | Nylon and Polyester. Versions with different size chest belts available | XS, S, M, L, LL, XL, XXL Color coded |
| Polaris Medical | General purpose unpadded or padded sling | Has head support, used with multiple lifts | 100% Standard polyester, latex free; loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | Universal Sling | With or without head support, used with multiple lifts including ceiling lifts | 100% Standard polyester, latex free; loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | Hygiene Sling | Toilet transfers, padded with or without head support | 100% Standard polyester, latex free; loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | Hammock Sling | Transfers, non padded with head and lateral support | 100% Standard polyester, latex free; loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | Pediatric Sling | With head support for children and smaller persons | 100% Standard polyester, latex free; loops | XS, S, M, L, LL, XL, XXL Color coded |
|  | Positioning Sling | All ways slides total bed positioning system | Integrated evacuation offered optionally | Single and Double sizes |
|  | Bathing Sling | Mesh sling for bathing | 100% Standard polyester, latex free; loops | XS, S, M, L, LL, XL, XXL Color coded |
| Surehands | Universal/standard | Has a cut-out for toileting, supports whole body, with or without head support | Solid fabric or mesh |  |
|  | Hygiene Sling | Toilet transfers |  |  |
| Technimotion | Smart Insertion Slings | No log-rolling required. Segmented plastic inserts provide rigidity. Thigh sling can be used for seated transfers. 400 lb. cap. | Washable polyester fabric. Also available in non-woven material as single patient use (disposable). | M, L |
|  | One-piece slings | Same components of Smart Insertion Slings connected with a supporting piece of fabric. Requires log-rolling for insertion. 400 lb. cap. | Washable polyester fabric. Also available in non-woven material as single patient use (disposable). | M, L |
|  | The Palaytte | Designed for seated transfers and patients with compromised upper body strength. Cushioned interface can be left under patient between transfers. 400 lb. cap. | Washable polyester fabric. | S, M, L |
|  | Standing Aid Sling | Used with Proxi-Motion lifts. Padded support keeps sling in a safe position during ambulation. Cap. 300 lbs. | Washable polyester fabric. Also available in non-woven material as single patient use (disposable). | M, L. |

## Sling Types by Function/Task

| Function/Task | Sling Type/ Category | Criteria | Special Considerations | Manufacturer |
| --- | --- | --- | --- | --- |
| Vertical transfers (to and from bed, wheelchair, commode, dependency chair, etc.); toileting; bathing; repositioning in chair; fall rescue | Seated | Patient can tolerate sitting position and has adequate hip & knee flexion. | Consider presence of wounds for sling application and patient positioning. Consider precautions of total hip replacement patients. | ARJO BHM Medical Guldmann Horcher Invacare Liko Polaris Medical Surehands Vancare Waverley Glen Wy’East Medical |
| Toileting, vertical transfers (to and from bed, wheelchair, commode, dependency chair, etc.); functional sit-stand training/support; dressing; pericare | Standing | Patient can grasp and hold handle with at least one hand, has at least partial weight bearing capability, has upper body strength, and is cooperative and can follow simple commands. | Consider presence of wounds for sling application and patient positioning. | ARJO BHM Medical Guldmann Horcher Invacare Liko Polaris Medical Surehands Vancare Waverley Glen Wy’East Medical |
| Lateral transfers (to and from bed, stretcher, shower trolley or gurney); making an occupied bed; toileting; bathing; repositioning in bed; fall rescue | Supine | Patient cannot tolerate sitting position and has restricted hip & knee flexion. Patient can tolerate supine position. Need more than one caregiver. | Do not use if patient has respiratory compromise or if wounds present may affect transfers or positioning. | ARJO BHM Medical Horcher Invacare Polaris Medical Vancare Waverley Glen Wy’East Medical |
| Ambulation training and support | Walking | Partial weight bearing, level of cooperation, consult Dr. & therapist for readiness | Do not use if patient has hypotension or wounds are present that affect transfers and positioning. | ARJO BHM Medical Horcher Liko Polaris Medical Waverley Glen Wy’East Medical |

| Ambulation training and support | Standing | Patient can grasp and hold handle with at least one hand, has at least partial weight bearing capability, has upper body strength, and is cooperative and can follow simple commands. | Consider presence of wounds for sling application and patient positioning. | ARJO BHM Medical Horcher Liko Polaris Medical Waverley Glen Wy’East Medical |
| --- | --- | --- | --- | --- |
| Limb support for wound debridement, surgical procedures, bathing | Limb support-lift straps | Sustained holding of any extremity to perform pt. care | Wounds, comfort, circulation, neurovascular and joint considerations, if task is of long duration | Liko Polaris Medical Note: lift straps not on web site, please contact manufacturer directly |

Features extracted from manufacturer’s specification sheets.

Weight capacities include patient, mattress, bedding, and all accessories.

Features extracted from manufacturer’s specification sheets.

Weight capacities include patient, mattress, bedding, and all accessories.